412_13_SD04

Environment Agency

NEC3 professional services contract (PSC)

Scope

Project / contract information

| Project name | RIVER OUSE TRIBS MODELLING (YORK) |
| --- | --- |
| Project 1B1S reference |  |
| Contract reference | J01042 |
| Date | 21/11/2017 |
| Version number | 0 |
| Author | B Cackett |

Revision history

| Revision date | Summary of changes | Version number |
| --- | --- | --- |

This Scope should be read in conjunction with the latest version of the current National Survey Specification at the Contract Date. In the event of conflict, this Scope shall prevail. The services are to be compliant with the National Survey Specification.

1	Introduction	3

1.1	Background	3

1.2	Objectives	4

2	Specified Requirements	5

2.1	Project Management	5

2.2	Product Delivery	5

2.3	Previous Studies and Data Sources	5

2.4	Survey	6

General Requirements applicable to all surveys,	6

2.4.1	Survey Control	6

2.4.2	Channel Surveys	7

3	Health and Safety	8

4	Risk Management	9

5	Specifications and Guidance	10

5.1	Guidance documents	10

6	Programme	11

6.1	Requirements of the Programme	11

7	Services and other things provided by the Employer	12

7.1	Data and information management and intellectual property rights	12

7.2	Licensing information	12

7.3	Data management and metadata	12

7.4	Open Data	12

7.5	Data security	12

# Introduction

## Background

Purpose of Survey:

This survey is being commissioned to produce channel cross sections, as well as details of other relevant features as required to build hydraulic models of the catchments. These models will inform the Environment Agency’s flood risk mapping and ongoing management of flood risk with partners such as York City Council.

----------------------------------------------------------------------------------------------------------------

Location and Extent of Survey:

The survey will be carried out within York. The watercourses are;

- Holgate Beck

- Germany Beck

- Bur Dike/Burdyke

- Blue Beck

The map below shows the general extents and location.

## Objectives

This contract is for the provision of a variety of survey outputs as described in Section 2.

The Consultant shall undertake survey as required and in all cases in accordance with the National Technical Specification for Surveying Services v4.0.  The Consultant shall deliver the outputs as described in Section 2 of this Scope.

Install suitable survey control

Carry out channel cross section survey

Produce the following deliverables (in brief);

  - Model input data

  - Drawings

  - Report

# Specified Requirements

The services include, but are not limited to, the provision of the items specified in Section 2.

## Project Management

Day to day management of project delivery shall be the responsibility of the Consultant.

In managing the services the Consultant shall undertake the following:

Maintain weekly contact with the Employer’s Project Manager such that the Employer is fully informed of progress and issues.

The Consultant shall raise an early warning where changes to the Scope or the accepted Programme occur or where significant issues arise on a project.  Reports are to be submitted to the Employer as soon as they are realised by the Consultant to allow informed decisions to be made by the project team. The early warning shall provide the Consultant’s best indication of the effect on the forecast and the Programme at the time of it being raised.

The Consultant shall capture lessons learnt relevant to the delivery of this project and share them with the Employer.

## Product Delivery

In delivering the services the Consultant shall:

- Ensure all deliverables and products show evidence of a quality control system. General quality tolerances should consist of:

- a)	All first draft deliverables shall be free from significant error and be consistent with other documents, avoiding repetition.

- b)	All first draft products shall tell a logical story with a clear audit trail of consultation and decision making processes, supported by robust evidence, which can be easily transferred and interpreted by future users.

- c)	All first draft deliverables shall be grammatically correct and clearly presented, with consistent formatting.

- d)	All first draft deliverables shall satisfy the relevant latest guidance and legislative requirements.

- e)	All first draft deliverables shall conform to the latest and necessary guidance and meet the template requirements, unless otherwise agreed with the Employer in advance of submission.

- f)	Products shall be suitable for the audience and written using ‘plain English’.

- Ensure all drawings are produced at the scale described below in Section 2.4.

## Previous Studies and Data Sources

The Consultant shall ensure that appropriate use is made of existing data, to avoid duplicating work already undertaken.

Existing channel survey data is available for:

Holgate Beck

Germany Beck

This data should be used by the Consultant to locate newly observed cross sections in identical locations (as far as practicable).

The Consultant is responsible for any new data requirements to meet the Scope outcomes.

The Consultant shall obtain Ordnance Survey mapping data and other relevant data products from the Partner Data Portal website and comply with the licensing of that site.

## Survey

The Consultant shall:

- Obtain all consents and approvals required for the survey to be undertaken.

- When on site, carry with them at all times the letter of introduction, or similar, supplied by the Employer.

- Deliver the survey as described in this Scope.

- Provide final outputs of the survey to the Employer in a digital format.

- Ensure that all survey results including the survey report are to be delivered as a zipped file email attachment if the zipped file is smaller than 10Mb or via DEFRA Sharefile (link supplied by Employer upon request).

### General Requirements applicable to all surveys,

The following is to be copied and pasted into the survey report as a record of the survey requirement.

The Consultant shall:

- Submit all documents with the EA job number in the file name.

- Name drawing files A-J01042-BB.dwg where A may be:

| Letter | Meaning |
| --- | --- |
| - L | - Long section |
| - X | - Cross sections |
| - T | - Topographic plan |

BB is a sequential sheet number – 01, 02 etc.

- Submit weekly progress reports to EA Project Manager by email / telephone.

### 2.4.1	Survey Control

The Consultant shall:

- Install sufficient survey control such that no surveyed data is further than 3km from an E2 grade survey station.

- Wherever possible existing monuments should be re-observed (existing survey stations have been supplied in J01042_EACS.shp).

- As per V4.0 of the EA Technical Specification all GNSS survey observations should be converted to OSGB and ODN via OSGB/TN15.

- Submit RINEX data for all reference and rover observations with the survey report.

- Number EACS using numbers provided by the Employer. The Consultant shall contact the Employer when the number of each grade station is known.

- Provide EACS description cards as .doc and .pdf format files with the EACS reference as the filename.

### 2.4.2	Channel Surveys

The Consultant shall:

- Supply channel survey data in ISIS .DAT format.

- Supply data in EACSD V3.2 format and include CES1 and CES2 data. EACSD shall be passed through the Environment Agency’s on-line validator program and the validation certificate presented in the survey report.

- Survey channel cross sections at locations indicated on the contract mapping.

- Survey channel hard and soft bed levels. Separate digital data files are to be provided for hard and for soft bed data.

- Extend cross sections 10m beyond the river bank into the flood plain (where possible).

- Number cross-sections in the format XXXX_YYYY where XXXX is the channel name:

BLUE – Blue Beck

GERM – Germany Beck

BURD – Burdyke

HOLG – Holgate Beck

And YYYY is the section chainage taken from the supplied OuseTribs_CentreLine.shp, starting at 0 at the outfall/downstream end. Where a tributary meets the main channel listed above, substitute the last letter of the channel name listed above with the number of the tributary, increasing upstream e.g. GER1_0008, GER2_0015.

- Present cross-sections at 1:100 natural scale, longitudinal sections at 1:2500 horizontal and 1:100 vertical scale or as agreed with the EA project manager.

- Survey any weir or drop structure if their head of water is 0.1m or greater found on site.

- Survey a structure section for all pipes crossing the water course found on site which have a diameter greater than 0.15m.

- Show ghosted on the section drawing for upstream elevations of bridges and culverts, the downstream soffit, top of parapet, invert, bed level and bank crests.

- Survey and show on the long profile outfalls entering the watercourse of 250mm diameter or greater (re: para 9.3.5).

- Produce gaugeboard description cards for all gaugeboards on the surveyed watercourses using template gaugeboard_blank.doc.

# Health and Safety

- Health and safety is of the highest priority for the Employer. The Consultant shall promote and adopt safe working practices for their own activities and Subconsultants working for them and shall, in consultation with the Employer, deliver surveys that take due regard for health and safety in their performance with regard to the public and anyone else concerned.

- The Consultant shall demonstrate that they have followed the principles of hazard identification, elimination and management in their survey work.

- The Consultant shall report to Employer any near miss, hazard or accident that happens during the delivery of the services, in addition to any statutory body such as HSE.

- The Consultant shall report to the Employer any brilliant safety behaviours highlighted during the delivery of the services.

# Risk Management

- The Consultant shall take the lead in project risk management throughout the survey.

# Specifications and Guidance

## Guidance documents

The Consultant shall undertake the appraisals in accordance with the specifications and guidance listed below.

- National Technical Specification for Surveying Services v4.0

# Programme

## Requirements of the Programme

- In delivering the services the Consultant shall submit all products to the Employer initially as ‘draft for review’ followed by ‘final for acceptance’. Formal advance notice of submission shall be given to the Employer and all product submissions will be subject to a two week review period by the Employer. The submission dates and review periods shall be included on the Programme for each project.

- All key activities required to deliver the services shall be identified on the Programme.

- The tasks within the project are to be divided as follows;

| 1 | Survey of Holgate Beck and supply draft deliverables |
| --- | --- |
| ##### 2 | ##### Survey of Burdyke and supply draft deliverables |
| 3 | Survey of Blue Beck and supply draft deliverables |
| 4 | Survey of Germany Beck and supply draft deliverables |
| 5 | Final report and delivery of full survey dataset |

# Services and other things provided by the Employer

## Data and information management and intellectual property rights

All of the data listed as being supplied to the Consultant as part of this study remains the Intellectual Property (IP) of the Employer.

## Licensing information

Licences for LIDAR Data, Ordnance Survey mapping, model, survey, hydrometric and historical data will be provided to the Consultant upon award of this commission.

## Data management and metadata

The survey deliverables are to adhere to the file structure stated in National Technical Survey Specification v4.0.

## Open Data

The consultant must make due consideration of the Open Data and make steps to ensure no personal data is provided.

## Data security

- All model and survey information will be provided to the Consultant in an encrypted format (using WinZip 128 bit encryption) according to Employer data security policy.  Once the commission is completed, all the original data sent to the Consultant, which is classed as commercially sensitive, shall be returned to the Employer in an encrypted format using WinZip 128 bit encryption.

- Project deliverables such as model files, survey data or anything of a personal nature such as questionnaires or address data must also be returned in an encrypted format using WinZip 128 bit encryption.