St Richards  Hospital

Spitalfield Lane

Chichester

West Sussex

PO19 6SE

Tel: 01243 788122

www.westernsussexhospitals.nhs.uk

Company

06/07/2017

Dear Sir

Re: Invitation to offer for the supply of DRAFT SPECIFICATION FOR Western Sussex Hospitals Foundation Trust CCTV Tender

On site Contact for Queries on the specification

Please contact Nick Rossiter on Worthing Hospital:  Direct Tel 07557 545364 or 01903 285 222 ext 84379

Email: nick.rossiter@wsht.nhs.uk

10.	Timeframe for tender processing (target dates)

| Activity | Target Date |
| --- | --- |
| Advert Placed in Contracts Finder | 06/07/2017 |
| Tenders issued by the Trust | 24/07/2017 |
| Contractors site visit | By Arrangement |
| Tender return date | 04/08/2017 |
| Tender review | 08/08/2017 |
| Contractor appointment by Trust | Soon After Review TBA |

11.       Evaluation of Tenders

| Criteria | Percentage Weighting |
| --- | --- |
| Price | 60% |
| Operational/Support | 20% |
| Technical | 15% |
| Training | 5% |

Commercial Evaluation

The price will be compared on the basis of equipment and service provision.

Tender prices will be scored on a comparative basis, with the cheapest compliant Tender   (excluding any Tenders that the Authority rejects as being abnormally low or non-compliant) receiving 100% of the available marks (60% following weighting).  All other Tenders will be compared against that lowest Tender using the formula:

(A / B) x100

A = price of lowest compliant Tender

B = price of the Tender being scored

12.        Note to Tenderers

Notes for Tenderers

The Tenderer will note that:

- in order for a tender return to be deemed compliant a response must be submitted in respect of the following Information Request.  Unless indicated otherwise, all these elements will attract a score during evaluation. The appropriate weighting and maximum available score have been included for information.   Any elements omitted will be scored as zero which will result in a lower overall score being achieved.

- The response must be compiled in the order of the questions asked.  Responses that require the Evaluation Panel to refer to other documents to identify your answer are not acceptable and may result in your submission being rejected.

- The following scoring mechanism will apply to each question:

| Score | Criteria |
| --- | --- |
| 0 | No response to question or response which is completely inadequate. |
| 1 | Very limited response to the question. Requirements not fulfilled. |
| 2 | Fair response, but lacking sufficient detail to make a clear judgement on acceptability. |
| 3 | An acceptable answer, complete in detail and relevance and (if appropriate) demonstrating current competency in the required areas. |
| 4 | An acceptable answer that fulfils the criteria of Score 3 and includes methodology / ideas which may result in increased quality of service. |
| 5 | An outstanding answer which demonstrates total comprehension, current delivery of the requirement and includes methodology / ideas which will provide significant additional quality of service. |

- Any changes to the information provided must be immediately notified to the Trust.

- In forming its Offer, the Tenderer will bear in mind the Key Objectives of the Trust as set out in this tender document.

Document No. 4

NHS CONTRACT FOR WESTERN SUSSEX HOSPITALS NHS FOUNDATION TRUST CCTV TENDER

Ref:

Draft Specification

Please provide a CCTV Provision

CCTV Cameras and Associated equipment to include Supply, Install and Commission

Listed below are the number of identified units on each site

Southlands Hospital

Worthing Hospital

St Richards Hospital

Number of Units identified on each site

| Site Name | St Richards | Worthing | Southlands |
| --- | --- | --- | --- |
| CCTV Cameras | 26 Internal 4 External | 38 Internal 7 External | 1 Internal 5 External |
| Digital Video Recorders (DVRS) | 2 | 3 | See Note Below |
| Video Monitors | 2 | 3 | See Note Below |
| UPS Power Units | 1 | 1 | See Note Below |
| Video Recording Switch Unit | 1 | 0 | See Note Below |

Southlands Note – The existing DVR is to be removed from Opthalmology Managers office and relocated in existing CCTV cabinet in IT Server room.

New CCTV Camera Locations

St Richards Hospital – PDF Plans will be provided to contractors upon site visit

Worthing Hospital – PDF Plans will be provided to contractors upon site visit

Southlands Hospital – PDF Plans will be provided to contractors upon site visit

Below is an example of the type of Cameras and Video Recording equipment we require.

Cameras

Internal Camera Specification

Samsung SCD-5083R 1/3" vandal resistant Super HAD colour CCD cameras, 1000 tvl, 3 - 10mm vari-focal auto-iris lens, Samsung super dynamic range (or equivalent), 30m IR illumination mounted in the twenty-six positions identified on the embedded drawings pages 3-6 below.

External Camera Specification

Samsung SCV-5083RP 1/3" vandal resistant Super HAD colour CCD cameras, 1000 tvl, 3 - 10mm vari-focal auto-iris lens, Samsung super dynamic range (or equivalent), 30m IR illumination mounted in the four positions identified on the embedded drawings pages 3-6 below

Recording Equipment

Digital Video Recorders (DVRS)

Bosch 5000 series 16 channel 2tb digital video recorders are to be supplied, installed and commissioned in the existing CCTV cabinet in Stillman House.

This will allow continuous or motion detection controlled recording for up to 32 cameras.

Certain cameras may be accommodated on existing recorders. (Please assess this on site visits)

Video Monitors

Samsung SMT 1934 19” video monitors (or equivalent) will be supplied and fitted (wall mounted) to display images from the DVRs. Display can be a single image from any camera or a variety of multi-screen displays.

UPS Power Unit

The DVRs will be connected via a UPS unit to protect against power surges.

Video Recorder Switching Unit

A Raritan EZ-116-UK KVM switcher will allow a single desktop monitor, keyboard and mouse to control up to 16 video recorders. Selection will be made by means of a push-button for each video recorder.

Below is a table of requirements. Please provide all details on a separate sheet

| Service | Action to take | Frequency | Comments / Response Provided | Weighting | Score 0 - 5 |
| --- | --- | --- | --- | --- | --- |
| Operational / Support | Are you able to provide all the equipment listed in the specification. If No and you would provide alternative equipment please provide full details. |  | YES/NO | 20 |  |
| Technical | - How do you ensure the Trust complies with Information Commissioners Office regulations for CCTV. |  | YES/NO | 5 |  |
| Technical | - Please provide full details of the number of cameras your recording equipment will accommodate. |  | YES/NO | 5 |  |
| Technical | - Are your offered cameras suitable for the location, bearing in mind the light levels and the size of the area to be viewed by each camera? |  | YES/NO If No please give reasons and any solutions |  |  |
| Technical | - On Assessment do you feel the cameras sited so that they are secure and protected from vandalism? |  | YES/NO If No please give reasons and any solutions (different locations) | 5 |  |
| Training | - What training do you offer on operating your equipment. |  | YES/NO | 3 |  |
| Training | - What charges are applicable for training- please provide full details and any costs. |  | YES/NO | 2 |  |

If any part is NO please provide details.

Please use separate sheet for answers if required.