Specification of Required Works - Festival Gardens

- Beginning April – end of October cut the grass at least eighteen times to a length of 5cm, carry out weeding, tidy grass edges, dead head roses and remove suckers and saplings and carry out watering as necessary.

- Prune the shrubs within the garden twice per year.

- Annual treatment of the grass - aeration, scarify and one feed.

- Multi-purpose feed  to be dug into the flowerbeds during Spring and organic material to be dug in during October/November.

- During the autumn cut back the perimeter hedge to reveal railings and any periphery hedging/trees overhanging neighbouring gardens.

- Sweep the internal pathways, three visits during the Autumn/Winter months and remove leaves from grass.

- Apply an algae remover to the steps in the Spring.

- All waste to be removed from site each visit.

- Spring bulbs, 100 daffodils, to be planted in Autumn

- To clean and maintain the wooden sign (installed 2014).

11. To attend review meetings with members of the Town Council/Town Clerk as 	  and 	 when requested.

Specification of Required Works - Jim Shields Garden

- Beginning April – end of October cut the grass at least eighteen times to a length of 5cm, remove weeds, tidy grass edges and carry out watering as

necessary.

Any additional works identified by the Town Council as being required for improvement and enhancement in either garden will be quoted for and invoiced separately.