Hailsham Memorial Institute Club

Western Road

Hailsham

BN27 3DN

HTC005/2024

Specification

- Supply and erect scaffolding access tower and double handrail approx. 60 lineal metres.

- Strip existing felt roof covering inclusive of insulation and remove from site as rubbish.

- Raise existing perimeter and all upstands with 100mm x 50mm as edge protector for insulation.

- Supply and fix 100mm white UPVC fascia to perimeter- approx. 60 lineal metres.

- Supply and fix 75mm Aluminium perimeter trim to perimeter – approx. 60 lineal meters.

- Supply and fix 18mm OSB smartply to 5 rooflights and 6 redundant air fans, (removed by others).

- Supply and fix code 4 lead outlets.

- Supply and lay Icopal vapour barrier fully torched.

- Supply and mechanically fix 100mm ply backed PUR insulation board.

- Supply and lay Icopal Thermovent base layer laid loose.

- Supply and lay Icopal Premium underlay fully torched.

- Supply and lay Icopal Protorch charcoal mineral as cap sheet to field area and all edging details.

- Any defective joists to be separately quoted and agreed via contract variations.

- Any defective Decking must be noted, photographed and quoted separately via contract variations.

- A minimum guarantee of 20 years.

Specification compiled by Hailsham Town Council on behalf of The Hailsham Memorial Institute.

HTC take no responsibility for the accuracy of the specification and request that all contractors conduct a full site visit as part of their due diligence.