Contractor questions – 17 November 2020

1.	Where are the Wind posts to be positioned, they are detailed but not shown on the layout 	plan ?

2.	Fixing detail/spec for Blockwork back to Columns, and centres ?

3.	Is Screed required to roof Slab, We have answered no just the 110mm RC Slab

4.	C2C Consulting Engineers general note 8 describes 50mm concrete blinding but on their 	drawings a depth of 150mm MOT is shown ?

5.	Is the Engineer providing a Civils drawing ?

Responses

- Apologies, the wind posts appear to have been on a non printing layer - 2No locations for wind posts (doors each side of steel framed area);

- We will update drawing with frame tie details for concrete columns. Ancon PPS ties at 450mm vert ctrs with debonding sleeves.

- No screed

- We will update the note to state that 50mm blinding is required below he ground beams only.

- We did not price for Civils design. Your details are sufficient.