SYSTEM REQUIREMENT DOCUMENT THE BATUK DIGITAL RADIO SERVICE

PART 1 GENERAL DESCRIPTION

1. Introduction

1.1.   This is a System Requirement Document (SRD) for the British Army Training Support Unit Kenya (BATUK) digital communications system. It defines the functionality and performance requirements of the overall system that will satisfy the user’s requirements. It does not predicate any particular solution or equipment so that it can be used as a basis for measuring and assessing potential system proposals at the Invitation To Tender (ITT) stage, procurement of a system and to verify the functionality and performance of the final delivered system for acceptance into service.

2. General Description

2.1.    The primary role of the BATUK training area communication system is to enable Command, Control and Communication for multiple users on a number of nets within the geographical coverage of the BATUK operation.

2.2.    BATUK consists of two administrative nodes located on the outskirts of Nairobi and Nanyuki. Training takes place throughout the year and encompasses different types of individual and collective training.  This is conducted over varied terrain, some leased by the UK and some belonging to the Kenyan Department of Defence (KDoD).

2.3.    There has been a significant increase in the level and intensity of training requirements and the current system is not able to meet these evolving requirements. A key component of the sustainability matrix is the provision of a robust, reliable and supportable training areas communications system that permits BATUK and all its enablers to construct and deliver training.

2.4.    The existing communication system is inadequate in coverage and capacity. A replacement system must be compatible with the increased volume of users, tempo of operations and geographical expanse of the training areas. The preferred solution shall be digital which will allow flexibility (multi users and groups), control, intelligent audio routing, with a limited data capability, robust and interference free communications.

PART 2 SYSTEM REQUIREMENTS TABLE

3.  Structure of SRD

3.1	The SRD defines the functionality and performance requirements of the overall system that will satisfy the user’s requirements. It does not predicate any particular solution or equipment so that it can be used as a basis for measuring and assessing potential system proposals at the Invitation To Tender (ITT) stage, procurement of a system and to verify the functionality and performance of the final delivered system for acceptance into service.

3.2	The SRD defines the functionality and performance requirements of the overall support system and is structured in accordance with the SRD guidelines contained in the Acquisition System Guidance (ASG).

3.3	The Requirements Management process provides a platform for the inevitable trade-offs between capability, cost and time. These decisions and those related to technology readiness may necessitate modifications to, or trading of, the System Requirements. All requirement changes and the associated justification arguments will be captured in future versions of the SRD.

3.4	The SRD contains the following two parts:

  - System Requirements – General.  The SRD contains information that relates to the overall capability need and records how the need arose.

  - System Requirements – Table.  The Table contains the complete, structured, set of individual requirements for the support contract for a radio communication system.

- Attributes

  - The following attributes have been recorded into the SRD:

- ID. Uniquely identifies the record. Keeps the identity of the record should the Hierarchical SR Number or other data change.

- System Requirement. Defines the required service, outcome, effect or constraint.

- Status. When approved, each requirement is declared as a ‘candidate’. They may later be ‘traded’ or ‘cancelled’ but should never be deleted.

- Priority. The System Requirements (SRs) are prioritised as, Mandatory, Key, 1, 2 or 3. All SRs are deemed essential and the allocated priority is only a mechanism for determining who and how requirements may be traded. The Authority reserves the right to modify the priorities set out below as risks to the programme are identified. The requirements in this SRD are defined as:

- Mandatory. Requirements that must be met and represent legal obligations.

- Key System Requirement  Assumed to be untradeable without formal agreement of the Networks Team. Trading or amendment after an agreed business case will require re-submission for approvals.

- Priority 1 – A high priority requirement. Trading will require reference back to HQ Trg Branch via the Networks Team.

- Priority 2 – A medium priority requirement. Trading will require reference back to HQ Trg Branch via the Networks Team.

- Priority 3 – A low priority requirement. Trading can be decided by the Networks Team.

- Justification. The justification for each statement outlines the reason why the requirement is a system need. This draws on the higher-level concepts/doctrine for justification.

- Threshold Measure of Effectiveness. The Threshold Measure of Effectiveness quantifies the requirement or constraint. It also defines Service Acceptance expectations and the solution trade-space where appropriate. The Threshold MOE is the worst-case value beyond which there will be insufficient operational benefit to justify the requirement.

- Objective Measure of Effectiveness. The Objective Measure of Effectiveness is the best-case value beyond which there will be insufficient additional operational benefit achieved.

- Validation Category. The Validation Category is the criteria through which it can be demonstrated that the requirement is met.

- Remarks. The remarks paragraph contains amplifying statements of justification (where necessary) or important points of note, or may be used to highlight concerns.

- SR Owner.  Owner of the ID.

5	Assumptions

5.1	For the purpose of this SRD it is assumed that:

- The Contractor will have access to MoD sites.

b.       The contractor will have to arrange their own transport to and from Kenya including the transport of equipment.

c. 	The contractor will be responsible for purchasing any land, masts to provide the coverage as defined in the SRD.

d.       The contractor will be responsible for obtaining any necessary licences to operate in Kenya.

e.       The contractor must abide to any local laws whilst in the Country.

f.        The contractor must ensure all staff access to medical facilities.

g.     Handheld Radio safe working distances.  The following assumptions can be made:

- Handheld User may be carrying live ammunition and/or an armed gun, on his person.

- Handheld User will leave radio within vehicle when physically refuelling vehicle from a jerry can. (Specified in user’s Standard Operating Procedures (SOP)).

- Handheld User will be >50 metres from a fuel bowser when vehicle is being refuelled. (Specified in User’s SOP).

- Handheld User will be >50 metres from a helicopter whilst it is being refuelled. (Specified in user’s SOP).

- Handheld User will be >50 metres from a rocket launcher (example representing any kind of armament which is electronically armed) when using radio. (Specified in User’s SOP).

h.	All equipment must conform to Health and Safety Regulations and a Safety Case and a Safety and Environmental Case Report will be produced by the Contractor.

- Constraints

6.1	The Contractor will be responsible for Safety Management as suppliers.

6.2	The MoD (end users) will be responsible for Safety Management as users.

6.3	It is the responsibility of the Contractor to follow all safety procedures or protocols that apply to the sites being visited. Local site instructions and safe working practices must to be adhered to at all times.

Appendices:

A   	Contractor’s Help Desk

B   	Support Meetings

C   	Quality

D   	Contractor Equipment Safety and Environmental Management Systems Requirement.

E   	Contractor Equipment Maintenance Requirements

F   	Technical Support Levels

G   	Quality Plan Template

H   	SECR Template

I   	SEMP Template

J   	Glossary

Appendix A

Contractor’s Help desk

Request for Support	The Help-Desk service shall be run in both English and local Kenyan languages and be in operation and staffed 24 hours per day, every day. It will manage the resolution of all calls made by the User Authorised Support Representative(s). The Help-Desk should answer all telephoned requests for support within 30 seconds. In addition all faxed or email requests for support should be acknowledged within one hour of receipt.

When requesting Support, the User’s Authorised Support Representative(s) shall provide the following information:

- User name;

- Site details the location or unit requiring support;

- Name of person requesting support;

- Contact details of the person to whom the Contractor should direct any response or request for additional information;

- Any other information necessary to enable the Contractor to identify the component affected;

- A full description of the problem, including any error messages and the circumstances leading to the call.

A unique call reference number shall be provided and maintained for each call for support made by the User. All information relating to the call and all actions taken will be retrievable utilising the unique call reference number.

The Contractor shall measure and report quarterly on the Service Levels centrally to the Authority. The Contractor may use any reasonable and practical method to determine the Availability of the Contractor Help Desk on a routine basis; such method shall be defined in the Contractors response to the ITT.

  - Initial Diagnosis and Call-Back	The Help-Desk will call back the originator (or nominated alternative contact), confirming that the call is being addressed, obtain any other information that may be required and agree the subsequent course of action. If the call can be resolved at this stage, (or the call does not require support), the call shall be closed with the agreement of the User

  - Call progress Monitoring	The Contractor will monitor and proactively progress calls against the service restored targets detailed in the SRD and will notify the User contact whenever any target response time is not met.

  - Problem / Fault Resolution   A call can only be closed with the agreement of the Authorised Support Representative. The Help-Desk will log and monitor the action plans and any failure to achieve the plan will be escalated by the Contractor and/or user.

  - Emergency Software Change	Emergency software changes may, with the prior agreement of the User, be undertaken in order to restore service. Emergency software changes will be fully documented, but of a temporary nature and will remain in effect until a formal scheduled software release is available and implemented. Permanent resolution of software faults will be addressed by periodic software releases.

  - Help Desk Service Levels

    - The Contractor Help Desk shall achieve the following Service Levels:

    - 99% Availability measured monthly. The Contractor Help Desk shall be considered Available to Users if enquiries and requests can be submitted through voice, fax and email channels listed above;

    - 95% of all Help Desk enquiries measured monthly shall be actioned or escalated to User’s satisfaction within 4 Normal Working Hours.

    - The User shall also be provided with alternative telephone and pager contact numbers in order to implement an escalation procedure, when it is not possible to obtain support or escalation via the Help Desk. These alternative telephone and pager contact numbers shall be for management representatives and not service engineers.

Appendix B

Support Meetings

1.   Support meetings between the Authority and the Contractor shall be held on a Quarterly basis.

2.   The meetings shall be by mutual consent take the form of an audio teleconference, video teleconference or face to face at a location decided by the Authority.

3.   The meeting organisation and record keeping shall be the responsibility of the Contractor.

4.   The Terms of Reference for the review meetings shall be agreed by the Authority and the Contractor but as a minimum the following tasks shall be covered:

a.   Highlight any new user requirements for future equipment.

b.   Monitor the performance of the system being provided under the MOD contract,
 e.g. conformance to SLA’s, quality, ordering, billing and system availability, items in the repair loop.

c.   Act as a focal point for user complaints and letters of plaudit.

d.   Address any safety issues that arise.

f.    Address any CP&F issues that arise.

g.   Address issues regarding training volume planning.

5.   In advance of the review meeting the Contractor shall produce a periodic report detailing:

a.   Progress since last period;

b.   Planned actions for next period;

c.   Performance against service level requirements detailed in the SRD.

6.    In addition each quarter the Contractor shall provide the User with up to date management information, including but not limited to a list of:

a.   All faults logged in the preceding quarter, indicating down time to date for each, their current status and progress;

b.   All calls closed in the preceding quarter, indicating the closure details and the down time for each call, as well as a summary sheet detailing the overall clearance rates for the current quarter;

c.   All currently open calls, indicating their status and progress including down time to date for each;

d.   The distribution of ‘time to answer’ of all Help-Desk telephone calls.

e.   Report any safety related issue(s) along with corrective action being taken.

7.   The quarterly reports and quarterly management information shall be supplied in electronic form and in MS Word, MS Excel, PDF or other mutually agreed format.

Appendix C

Quality

Contractual Requirements

Quality Assurance shall be carried out to the satisfaction of the Authority. The Contractual Quality requirements are:-

| AQAP 2105, Edition 1 | NATO Requirements for Deliverable Quality Plans http://www.nato.int/docu/stanag/aqap2105/aqap-2105-e.pdf |
| --- | --- |
| AQAP 2110, Edition 1 | NATO Quality Assurance Requirements for Design, Development and Production. http://www.nato.int/docu/stanag/aqap2110/aqap2110e.pdf |
| AQAP 2110 | Design Development & Production |
| DEF STAN 05-57, Issue 5 | Configuration Management of Defence Material UK Defence Standardization |
| DEF STAN 05-61, Part 1, Issue 4 | Quality Assurance Procedural Requirements – Concessions UK Defence Standardization |
| DEF STAN 05-61 Part 4 | Contractor Working Party |
| DEF STAN 05-99, Issue 2 | Managing Government Furnished Assets in Industry UK Defence Standardization |

1.2 Quality Plan

The Contractor shall provide a Quality Plan written in accordance with AQAP 2105 and the requirements of this section of this document. This should include, but not exclusively, the following: -

- Project description

- Organisation and responsibilities

- Resource Management

- Processes (General requirements)

- Documentation requirements

1.3 Certificate of Conformity

The Contractor shall provide a certificate of conformity for each service item supplied.

Certificate of Conformity is a document, signed by the Supplier, which states that the product conforms with contractual requirements.

Appendix D

Contractor Equipment Safety and Environmental Management Systems Requirement.

Mandated Processes for Safety and Environmental Protection (S&EP).  In July 2004 the use of a single Acquisition Safety and Environmental Management System (ASEMS) was mandated (DEF STAN 00-56 Safety Management Requirements for Defence Systems).

At the core of the ASEMS there are two Systems Manuals:

- Project Oriented Safety Management System (POSMS)

- Project Oriented Environmental Management System (POEMS)

Compliance with POSMS and POEMS will ensure that a Safety and Environmental Management System is robust, proportionate to a project’s levels of risk and is compatible with the DE&S corporate reporting requirements.

More information can be found at:

Mandated Processes for Safety and Environmental Protection » Safety and Environmental Protection (S&EP) » Acquisition Operating Framework (AOF)

The Contractor shall submit a safety and environmental systems management manual within 4 weeks of the contract award.

The Networks Team will provide ASEM Guidance Notes to aid the Contractor to meet their obligation at this section on contract award.

Acquisition Safety & Environmental Management (ASEM).

The contractor shall provide a Safety Policy statement and Safety Manual meeting the requirements of BS/OHSAS 18001 Occupational Health and Safety.

The contractor shall provide an Environmental Policy and Procedures statement in accordance with ISO 14001.

Appendix E

Contractor Equipment Maintenance Requirements

The Contractor shall be responsible for full maintenance on all items such as the Network ie repeaters,  PMR Handsets, and associated ancillaries in accordance with the equipment manufacturer’s instruction, specification and warranties, unless specifically advised by the User.

The repair by the Contractor shall extend to all faults and damage and not only that identified originally by the User.

Where damage is caused as a result of negligence on the part of the Contractor, then the Contractor shall be totally responsible for the repair or replacement of any item.

Where the Contractor considers it necessary to modify any manufacturer’s maintenance specification, instruction or recommendation, such a modification shall only be undertaken with the prior permission of the User and upon detailed written submission of the proposal

The Contractor shall be responsible for the repair of all faults and damage exhibited or suffered by items of PMR Handsets and associated ancillaries where such faults or damage are consistent with Fair Wear and Tear.

Fair Wear and Tear shall be considered as damage and / or defects PMR Handsets or part thereof, which occurs during normal and proper use of the equipment in connection with the User’s function.

The Contractor shall be responsible for the administration, management and repair of all faults associated with individual items of supplied PMR equipment and their associated ancillaries, either hand portable or vehicle mounted.

Authority damage will be considered on a case by case bases taking into consideration time leased and whether item is repairable or beyond economic repair.

Appendix F

Technical Support Levels

CIS / EW Technical Support (Delivery & Levels).  Battlefield Equipment Support Doctrine (BESD) defines levels of ES; however, these definitions do not fully define the activities associated with the maintenance of CIS / EW capability, such as software installation, information management, system administration etc.  The definitions below set out the levels and delivery methods of CIS / EW technical support, which includes maintenance.

Levels of Support

Levels.  In line with BESD there are four clearly defined Levels of technical support:

- Level 1.  Servicing and day-to-day activity generally conducted locally by the qualified, operator or engineer responsible for the delivery of the service or manning of an installation, this requires little or no additional test tools, equipment, applications or remote system access.  This may include basic operations such as:

    - Compliance with Installation Maintenance Instruction (IMI).

    - Consumable replenishment.

    - Functional testing (use of Built In Test Equipment (BITE)).

    - Fault diagnosis and corrective maintenance by replacement, adjustment or minor repair.

    - Local support to information managers, network monitoring and software / hardware release management.

    - Basic Local Area Network (LAN) administration and local application configuration.

    - Minor modification.

- Level 2.  Conducted by engineering staff with the appropriate specialist training and experience.  This requires some additional test tools, equipment or applications.  This may be conducted locally or remotely.  This may include functions such as:

    - All Level 1 tasks.

    - Greater level of fault diagnosis and corrective maintenance by replacement, adjustment and repair.  Forward based limited Level 2 support may include the greater level of fault diagnosis, but limited corrective maintenance ability (cable repair, minor software workarounds).

    - Advanced, system administration and configuration, application configuration, support to information and LAN managers, across a Metropolitan Area Network (MAN), formation, or multiple sites.

    - Basic interfacing and system interoperability tasks, including:

        - Inter-boundary International Standard Organisation (ISO) model layer 1 connection management.

        - Inter-boundary ISO model layer 2 and 3 gateway configuration.

        - Formation wide information flow management.

        - Formation wide bandwidth prioritisation and management.

- Level 3.  Conducted by specialist engineering staff with appropriate additional training and experience.  This activity will usually be conducted remotely.  This requires some additional test tools, equipment and applications, which may or may not be IT based.  Inter and intra system fault analysis and diagnosis will routinely be addressed through remote access.  Functions carried out may include:

    - All Level 1 & 2 tasks.

    - Advanced, multi-system administration and configuration, cross system application configuration, support to Force information managers and MAN managers, across multiple systems (Formation and Force wide).

    - Advanced interfacing and system interoperability tasks including:

        - Inter-system end to end service delivery, ISO model layers 4 – 7 monitoring and management.

        - Force wide information flow management.

        - Force and reach back wide bandwidth prioritisation and management.

    - Technical Base lining of an Installation or System.

- Level 4.  Level 4 is typically conducted out of theatre by defence contractors or specialist organisations, in support of the system design and operating authorities.  These organisations will typically have the ability to remote access, monitor and configure deployed systems as well as model deployed systems within specialist facilities.  This requires routine use of highly specialist, bespoke, test tool, equipment and applications.  Functions carried out include:

    - All Level 1, 2 and 3 tasks.

    - Major or highly technical repairs, upgrades and reconditioning.

    - Major or highly technical repairs, upgrades and reconditioning.

    - Advanced, global WAN, system of system administration and configuration tasks, assistance to WAN managers, system of system application configuration.

    - Advanced interfacing and system of system interoperability tasks including:

        - Multiple, joint and combined gateway and system end to end service delivery, across the ISO model.

        - Joint and combined system delivery.

        - Multiple, joint and combined information flow management.

        - Multiple, joint and combined bandwidth prioritisation and management.

Delivery of Support

  - Local.  This is the physical replacement, reloading and initialisation of equipment, sub-assemblies, Line Replaceable Units (LRU) or the reloading of software, firmware or soft protocols by direct interfacing with the system or equipment being supported.  This may be conducted by:

    - The individual responsible for Level 1 support.

    - Forward based engineering staff (intimate support, previously referred to as 1st Line).  Potentially only up to limited Level 2 support.

    - Rear or centralised engineering staff deploying forward to provide Level 2/3 support to resolve a specific issue.

  - Remote.  The reloading, initialising, restarting of software, firmware, protocol stacks, or other software or electronic, data, objects through remote access via the appropriate CIS.  This may be conducted by rear or centralised engineering staff offering Level 2/3/4 support, either via remote access or via faulty elements being shipped rearwards for attention

Appendix G

Quality Plan Template

Appendix H

SECR Template

Appendix I

SEMP Template

Appendix J

| GLOSSARY |  |
| --- | --- |
| AOF | Acquisition Operating Framework |
| BATUK | British Army Training Unit Kenya |
| BS | British Standard |
| CAT | Category |
| CCK | Communications Commission of Kenya (CCK) |
| CIDA | Coordinating Installation Design Authority |
| CIS | Communication Information Systems |
| CoC | Chain of Command |
| CS95 | Combat Soldier 95 uniform (CS95) |
| E2E | End-to-End |
| EMC | Electromagnetic Compatibility Services |
| FOC | Full Operating Capability |
| GFA | Government Furnished Assets |
| HFT | Hybrid Foundation Training (HFT) |
| H&S | Health and Safety |
| HQLF | Headquarters Land Forces |
| ICS | Information Communication Systems |
| ICT | Information and Communications Technology |
| IOC | Initial Operating Capability |
| ISD | In Service Date |
| ISO | International Standard Organisation |
| ISS | Information Systems & Services (Organisation) |
| JSP | Joint Service Publication |
| KDoD | Kenyan Department of Defence |
| KSR | Key Service Requirement |
| LRU | Line Replaceable Unit |
| LWC | Land Warfare Centre |
| MOU | Memorandum of Understanding |
| MoD | Ministry of Defence |
| MSR | Main Supply Route |
| OSD | Out of Service Date |
| PTT | Press to Talk |
| POEMS | Project Oriented Environmental Management System |
| SCIDA | Site Coordinating Installation Design Authority |
| SLA | Service Level Agreement |
| SPOC | Single Point of Contact |
| SSUN | Single Statement of User Need |
| TLB | Top Level Budget |
| TLC | Through Life Capability |
| SEMP | Systems Engineering Management Plan |
| SR | Service Requirement |
| SRD | Service Requirements Document |
| VfM | Value for Money |

BATUK MAIN

Base Station x1

MSR linking Nairobi and Nanyuki (200 km approx)

BATUK Fwd HQ Nanyuki

Base Station x1

Archers Post Training Area (KDoD)

Range Control

Base Station x1

Training Estate Leased from Private Land Owners.

KDoD Training Area

MSR linking Nanyuki and Archers Post (180 km approx)

Installation Maintenance Instruction (IMI).  The IMI details the maintenance tasks required on a particular installation type, in an easy to follow logical sequence.  It is drawn from the maintenance tables contained in the 601 section of the individual equipment AESPs that make up the installation or as detailed in the Interactive Electronic Technical Publication (IETP).  The IMI is produced by the RSIT with the extant version available via the Army Electronic Battle Box (Land Component Handbook (ICS) Section 3.1.4.3.1 Installation Maintenance Instructions).