Schedule 1 Specification

Replacement of Passenger Lift Westport House Specification

General Description: (Existing)

We are Purbeck District Council West Port House Worgret Road Wareham Dorset BH20 4PP within this building we have the following installation -:

Two Storey Lift which currently has installed a hydraulic passenger lift – 2 floors, manufactured in 1995.

| Location | West port House Worgret Road Wareham Dorset BH20 4PP |
| --- | --- |
| Number of Lifts | One |
| Type of Lift | Passenger |
| Manufacturer/Installer | Coastal Passenger Lifts |
| Date of Installation | 1995 |
| Date of Modernisation (if any) | N/A |
| Load Capacity | 8 Person 630Kg |
| Operating Speed | 0.6m/s |
| Levels Served | Two |
| Car Entrances | 1 x Two Panel Side Opening |
| Landing Entrances | 2 x Two Panel Side Opening |
| Control System | Automatic Push Button |
| Drive System | Hydraulic Indirect Acting – Two Speed |
| Machine Room Location | Second Floor Right Hand of Shaft |
| Lift Maintenance Contractor | Lift Services Stannah Ltd |

The location is to the right of the main stairway and the lift size is a maximum of 630kg or 8 persons.

1.1 Scope of Works

Preparation

1.1.1	To provide a fixed price to fully remove the existing lift and all of its components, tracks, fixings, redundant electrical equipment, cables and trunking etc. and for fully making good the fully stripped out lift shaft and existing lift room.

1.1.2	The works must include for removing any support walls at the bottom of the lift shaft and making good to the shaft walls and floor, including any required tanking to prevent water ingress into the lift shaft.

1.1.3	Any structural or remedial works required to the lift shaft must also be included for within the tender sum.

1.2 Technical

1.2.1	A new fully functional lift to fit into the existing lift shaft must be designed and installed to meet the requirements of this document.  All works included within this documentation must be included for within the tender sum, any omissions, deviations or clarifications must be requested in writing at tender stage otherwise deemed to be included.

1.2.2 Current Installation Characteristics

1.3 Information

1.3.1	The current lift is a Coastal Passenger 8 person 630kg electric hydraulic passenger goods lift servicing 2 floors in a fully occupied Office which will remain occupied for the duration of the works.

1.3.2      It is highly recommended that a site visit is carried out to assist with your Tender process.

end