SYSTEM REQUIREMENT DOCUMENT THE BATSUB DIGITAL RADIO SERVICE

PART 1 – SYSTEM REQUIREMENTS - GENERAL

Structure of SRD

      - This is a System Requirement Document (SRD) for a leased, dedicated, civilian radio system to enable the British Army Training Support Unit Belize (BATSUB) staff, exercising troops and other users, to facilitate, construct and deliver Hybrid Foundation Training (HFT) for British Army units and international agencies conducting training in Belize.

      - The SRD defines the functionality and performance requirements of the overall system that will satisfy the user’s requirements. It does not predicate any particular solution or equipment so that it can be used as a basis for measuring and assessing potential system proposals at the Invitation To Tender (ITT) stage, procurement of a system and to verify the functionality and performance of the final delivered system for acceptance into service.

3.	The SRD defines the functionality and performance requirements of the overall support system and is structured in accordance with the SRD guidelines contained in the Acquisition System Guidance (ASG).

4.	The Requirements Management process provides a platform for the inevitable trade-offs between capability, cost and time. These decisions and those related to technology readiness may necessitate modifications to, or trading of, the System Requirements. All requirement changes and the associated justification arguments will be captured in future versions of the SRD.

5.	The SRD contains the following two parts:

  - System Requirements – General.  The SRD contains information that relates to the overall capability need and records how the need arose.

  - System Requirements – Table.  The Table contains the complete, structured, set of individual requirements for the support contract for a radio communciation system.

System Description

- A fully managed end to end digitised radio communication system for BATSUB, to include ongoing training of personnel; replacement, repair and technical management of the infrastructure equipment, software and hardware.  The system will allow flexibility (multi users and groups), control, intelligent audio routing, robust, and interference free communications.

- In order to do this, the successful bidder will need to provide the following:

a.	Network.  This must be a digitised system with all associated software and hardware.  It is to provide sufficient talk groups and channels per talk group; each talk group not exceeding 200 subscribers, which would incorporate multi-group and multi-user communities.

b.	Frequencies.  Allocation, management and control of frequencies is managed by Public Utilities Commission (PUC) and BATSUB is to have dedicated and sole use of any frequencies used by the radio system.  This will require policing and strict control by the successful bidder to prevent other agencies from using assigned frequencies, or the possibility of frequency cross contamination.

c.	Radios.  These will need to be a varied mix of handheld, vehicle mounted and permanent base stations.  Vehicle and handheld units should be programmable and capable of integrating GPS services.

d.	Coverage.  Coverage of 95% or greater is required between training areas and along designated Main Supply Routes (MSRs).

(1)  The jungle canopy can cause temporary communication blackouts.  Any solution should cater for this eventuality and may require the use of mobile repeaters.

(2)	The service will cover the following areas:

- The Main Supply Route (MSR) between Belize City and the Guatemalan border to the West.

- Manatee North

- Manatee River

- Baldy Beacon

- Mountain Pine Ridge

- 1963 Line

- Guacamalo Bridge

- Sibun Gorge

- Sittee Range

(3)	Expansion to other training areas is anticipated to cover the following locations:

- Gallon Jug

- Yalbac

- New River Lagoon

- Seven Hills Ranges

- Hill Bank

- Spanish Creek

- Hattyville Range

- Deep River

It should be noted that should the training move beyond designated areas, the radio system would only be required on an “on demand” basis, rather than as a permanent fixture.  Coverage for these locations should be a minimum of 75% and 12 months’ notice would be given prior to training commencing in those areas.

e.	Management.  A fully managed end to end digitised radio communication system for BATSUB to include ongoing training of personnel; replacement, repair and technical management of the (infrastructure) equipment software and hardware.  It is expected that the successful bidder will have resources in country to provide 24/7/365 cover to ensure no loss of communications either through equipment failure or network faults.  The training areas cover a mixed expanse of land ranging from rolling hills to dense jungle.  This can lead to contact with predators, poachers and other animals or creatures which endanger the life of individuals.

- Mast Locations.  It is imperative that any potential bidder has written confirmation from the respective land owner, Belizean organisation, or agencies authorising them to either install masts or co-locate equipment on existing masts prior to award of any contract.

g.	Timings.  The system must be fully operational by 31 Jul 17.

History

- BATSUB is a permanent UK Collective Training Establishment (CTE) with its main administrative node located at Price Barracks, on the outskirts of Belize City.  The conduct of British Army training in Belize is codified within a Memorandum of Understanding between the two respective governments. Training will take place throughout the year and encompasses collective and special to arm training conducted over varied terrain ranging from rolling hills to dense jungle.

- BATSUB previously had a bespoke communications system to meet a very different training requirement based on analogue technology and this was mothballed as BATSUB drew down in 2011.  A new radio system was installed in 2014 when BATSUB was directed to facilitate minor exercises requiring communications over certain training areas.  With the intent to reinstate BATSUB and increase training, the communication system needs uplifting to cope with the increased volume of users and the geographical expanse of the training areas.

- The increase in scope and intensity of training has been coupled with a requirement to ensure training is sustainable in the short and longer term. A key component of the sustainability matrix is the provision of a robust, reliable and supportable communications system that permits BATSUB and all its enablers to facilitate training.

Acquisition Strategy

- The Belize Radio System contract will be a five year leased solution.

Priorities

- See the System Requirements table at Part 2.

Assumptions

- For the purpose of this SRD it is assumed that:

- The Contractor will have access to MoD sites.

b.       The contractor will have to arrange their own transport to and from Belize including the transport of equipment.

c. 	The contractor will be responsible for purchasing any land, masts to provide the coverage as defined in the SRD.

d.       The contractor will be responsible for obtaining any necessary licences to operate in Belize.

e.       The contractor must abide to any local laws whilst in the Country.

f.        The contractor must ensure all staff access to medical facilities.

- Handheld Radio safe working distances.  The following assumptions can be made:

- Handheld User may be carrying live ammunition and/or an armed gun, on his person.

- Handheld User will leave radio within vehicle when physically refuelling vehicle from a jerry can. (Specified in user’s Standard Operating Procedures (SOP)).

- Radios are not to be used within 50 metres of any electronically initiated weapons device ie. Javelin.

- Handheld User will be >50 metres from a fuel bowser when vehicle is being refuelled. (Specified in User’s SOP).

- Handheld User will be >50 metres from a helicopter whilst it is being refuelled. (Specified in user’s SOP).

- Radios are not to be used within 50 metres of aircraft armed with any electronically initiated weapons devices such as infra-red countermeasures (flares, chaff or munitions).

- All equipment must conform to Health and Safety Regulations and a Safety Case and a Safety and Environmental Case Report will be produced by the Contractor.

Constraints

- For the purpose of this SRD, assume that:

- All MOD installations are to comply with;

- MOD Installation Standards Policy and Governance of the physical and environmental aspects of MOD Information Communications Technology (ICT), in coordination with the Defence Co-ordinating Installation Design Authority (CIDA) procedures.

- UK regulations.

- other regulatory standards.

- The Contractor will be responsible for Safety Management as suppliers and maintainers.

- The MoD (end users) will be responsible for Safety Management as users.

Glossary

- A Glossary of abbreviations is at Annex D.

Part 2 – SYSTEM REQUIREMENTS TABLE

Attributes

- The following attributes have been recorded into the SRD:

- ID. Uniquely identifies the record. Keeps the identity of the record should the Hierarchical SR Number or other data change.

- System Requirement. Defines the required service, outcome, effect or constraint.

- Priority. The System Requirements (SRs) are prioritised as, Mandatory, Key, 1, 2 or 3. All SRs are deemed essential and the allocated priority is only a mechanism for determining who and how requirements may be traded. The Authority reserves the right to modify the priorities set out below as risks to the programme are identified. The requirements in this SRD are defined as:

- Mandatory. Requirements that must be met and represent legal obligations.

- Key System Requirement (Summarised in Part 2). Assumed to be untradeable without formal agreement of the Networks Team. Trading or amendment after an agreed business case will require re-submission for approvals.

- Priority 1 – A high priority requirement. Trading will require reference back to HQ Training Branch Field Army via the Networks Team.

- Priority 2 – A medium priority requirement. Trading will require reference back to HQ Training Branch, Field Army via the Networks Team.

- Priority 3 – A low priority requirement. Trading can be decided by the Networks Requirements Team.

- Justification. The justification for each statement outlines the reason why the requirement is a system need. This draws on the higher-level concepts/doctrine for justification.

- Threshold (Measure of Effectiveness). The Threshold Measure of Effectiveness quantifies the requirement or constraint. It also defines Service Acceptance expectations and the solution trade-space where appropriate. The Threshold MOE is the worst-case value beyond which there will be insufficient operational benefit to justify the requirement.

- Object (Measure of Effectiveness). The Objective measure of Effectiveness is the best-case value beyond which there will be insufficient additional operational benefit achieved.

- Validation Category. The Validation Category is the criteria through which it can be demonstrated that the requirement is met.

    - Technical Assessment.  Verification achieved by assessing technical documentation and specifications or by inspection of a proposed solution.

    - Modelling and Simulation.  Verification achieved by predicting the behaviour of a system based on limited known information.

    - System Test.  Verification achieved through a defined activity or experiment in controlled conditions.

    - Certification.  Verification achieved by issue of a certificate of an appropriate authority that the solutions meet a defined standard.

    - User Trial.  Verification achieved by users conducting a series of tests to replicate operating conditions.

    - ITE&A.  Integrated Test, Evaluation and Acceptance to confirm the solution meets the endorsed user requirements.  Plan provided by the contractor and accepted by the users.

    - In-Service Special Evaluation (In-Service Sp. Eval).  Verification achieved by assessment of the system when in service.

- Status. When approved, each requirement is declared as a ‘candidate’. They may later be ‘traded’ or ‘cancelled’ but should never be deleted.

- Remarks. The remarks paragraph contains amplifying statements of justification (where necessary) or important points of note, or may be used to highlight concerns.

Annexes:

Annex A	Contractor’s Helpdesk

Annex B	Support Meetings

Annex C       Quality

- Annex D	Contractor Equipment Safety and Environmental Management Systems Requirement.

- Annex E	Contractor Equipment Maintenance Requirements

Annex F       Leaflet 1 Technical Support Levels

Annex G	Glossary

Part 2 – SYSTEM REQUIREMENTS TABLE

| ID | System Requirement | Pty | Justification | Threshold | Objective | Validation Category | Status | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Equipment |  |  |  |  |  |  |  |
| 1 | Infrastructure |  |  |  |  |  |  |  |
| 1.0 | The Authority requires a service fully managed by the Supplier. | KSR | The user does not want to own the equipment. | The solution is provided as a leased and managed service, including all mobile assets. | Same as Threshold. | Technical Assessment. | Candidate | In this context a totally managed service means that all equipment is leased and that maintenance (including second line battery maintenance), configuration data to enable routine programming and reprogramming of handsets and any software upgrades are to be carried out by the Contractor. |
| 1.1 | The solution shall interconnect all sites | KSR | For safety reasons It is imperative that all sites can communicate with each other (all-informed network). | The main sites and routes must be interconnected: - Main Supply Route (MSR). - Belize City - Price Barracks. - Belmopan. - Specified Training Areas as outlined in Belize Training Areas Map GSGS 5935 Edition 1. | All sites and routes must be interconnected. | Technical Assessment (including Coverage Predictions, Project Plan, Risk Management and Controls etc) Tested through User Field Trials/ verification, validation & Test Matrix Site Acceptance Test (SAT) | Candidate | The system should act as a communications cloud. Providing a user has coverage they should be able to contact any other user within coverage. See Limited Distribution Maps - Training Areas as outlined in Belize Training Areas Map GSGS 5935 Edition 1. The training areas outlined on the map are for guidance purposes only. 3 sites to operate concurrently, with a surge capability for a further 2 sites. |
| 1.2 | The solution will provide 95% radio coverage over the following specified BATSUB Training areas: Requirement 1 - Manatee North - Manatee River - Baldy Beacon - Mountain Pine Ridge - 1963 Line - Guacamalo Bridge - Sibun Gorge - Sittee Range Areas 6-13 as per Belize Trg areas map | M | Level of coverage is required to ensure safe training of all personnel within the training areas. | The solution will provide 95% coverage over the specified BATSUB Training areas. | The solution shall provide 100% coverage over specified BATSUB Training areas. | Technical Assessment (including Coverage Predictions, Project Plan, Risk Management and Controls etc) Tech Field Trial. Ops field trial. In-service Sp evaluation SAT | Candidate | This coverage has been broken down into two groups of training areas Areas where coverage is suspect or non-existent should be clearly documented. Requirement 1 is to reach FOC by 31 July 2017. Tenderers are to submit a detailed project plan in MS Project detailing how this will be achieved. All repeater stations are to have a 8 hour UPS capability |
| 1.3 | Requirement 2 The solution will provide 75% radio coverage over the BATSUB Training Areas as defined under Options at serial 18. 1-5, 14&15 as per Belize Trg areas map. | M | # Level of coverage is essential when training is taking place within this training area to ensure safe training of all personnel within the training area. | The solution will provide 75% radio coverage by land mass of the training areas shown on Training Areas Map GSGS 5935 Edition 1. | The solution shall provide 100% coverage. | Technical Assessment (including Coverage Predictions, Blind Spots identified, Project Plan, Risk Management and Controls etc) Tech Field Trial. Ops field trial. In-service Sp evaluation. SAT | Candidate | Areas where coverage is suspect or non-existent should be clearly documented. Requirement 2 is to be 12 months from Tasking Notice. All repeater stations are to have a 8 hour UPS capability |
| 1.4 | # The solution will provide coverage over the Main Supply Route (MSR) | KSR | There is a risk of accident or breakdown within this developing and potentially dangerous country. It is essential that personnel can call for support or inform the Chain of Command of any incident in a timely manner. The level of coverage is required to ensure that all personnel (exercising and staff) can communicate with BATSUB HQ (Price Barracks) during all road moves. | The solution will provide 90% coverage over the Main Supply Route (MSR) | The solution will provide 100% coverage over the Main Supply Route (MSR) | Technical Assessment (including Coverage Predictions, Project Plan, Risk Management and Controls etc) Tech Field Trial. Ops field trial. In-service Sp evaluation SAT | Candidate | This is to form part of Requirement 1. |
| 1.5 | # The solution shall support multiple concurrent group calls. | KSR | A number of users may be training in the same training area/vicinity; all must be able to communicate. | Support a minimum of 4 concurrent group calls | Support 8 concurrent group calls. | Technical Assessment Tech Field Trial. Ops field trial. In-service Sp evaluation. SAT | Candidate | Coverage required in specified training areas and MSR. |
| 1.6 | # Talk Groups | KSR | Contractual | The solution should provide a minimum of 8 talk groups | The solution should provide up to 12 talk groups | Technical Assessment Tech Field Trial. Ops field trial. In-service Sp evaluation. SAT | Candidate |  |
| 1.7 | # Voice recording | M | Contractual | 100% | 100% | Technical Assessment SAT | Candidate | All conversations to be recorded at end of exercises and passed to the Authority on suitable media able to play on Windows machine (disc) and deleted on PMR system. |
| 2 | Terminal Equipment |  |  |  |  |  |  |  |
| 2.1 | The solution shall provide a system that delivers handheld, vehicle and desktop radio capability. Handheld radios will be provided with suitable carry case and the ability to be attached to military webbing. Vehicle and desktop units will be supplied with suitable fixings to ensure stable use. The solution shall support multiple call types | KSR | There is a requirement for personnel on foot, in vehicles and in command posts (field or office environments) to communicate. To allow user choice, ease of use and increase usage efficiency | The solution shall provide at 95% the delivery of handheld equipment and delivery plus installation of vehicle and base-station equipment. The solution shall support group calls, individual calls and broadcast calls The following quantities are required: 80 handheld radios (including 2 batteries per radio), 22 vehicle radios, 5 base stations and mobile repeaters as required. | 100% | Technical Assessment SAT Manufacturers’ documentation and industry standards | Candidate | All equipment supplied for the purpose of fitting into a vehicle shall be EEC type-approval marked (‘e’ marked) to indicate compliance with the Commission Directive 2004/104/EC as last amended. All vehicle fits are to conform to the current version of FCS 1362:2010 UK CODE OF PRACTICE for the installation of mobile radio and related ancillary equipment in land based vehicles. Vehicle fits are to use the same radios as desk tops to reduce training requirements. All equipment is to be CE compliant. The service shall provide a range of harness options that will enable effective and secure carrying of equipment included as part of the leased service. The Contractor shall ensure that the installation, commissioning & testing of mobile equipment in vehicles is in accordance with the manufacturer’s requirements. |
| 2.2 | The solution must provide the capability to make direct 1-to-1 calls between radios. | 1 | There is a requirement to communicate outside the network, e.g. if outside coverage. | The systems must permit the use of 2 simultaneous direct 1-to-1 calls. | Same as Threshold. | Technical Assessment SAT | Candidate | These calls are simplex calls that do not use any infrastructure, aka back to back radio calls. |
| 2.3 | The solution shall have a facility to store and recall frequently used channels/talk groups. | 1 | To ease user operation and ensure usage efficiency. | Store and recall 10 separate channels (Talkgroups) | Store and recall 15 separate channels (Talkgroups) | Technical Assessment SAT | Candidate | Normally selected by rotary control on radio terminal. |
| 2.4 | The handheld solution shall operate for prolonged periods without requiring recharging. | 1 | Users on foot will not have regular access to a main supply where batteries can be charged. | Shall operate for 8 hrs on a 5/5/90 duty cycle without the need to change battery. | Shall operate for 10 hrs on a 5/5/90 duty cycle without the need to change battery. | Technical Assessment from manufacturers publications and user trials | Candidate | Handheld to have a visual battery level indicator |
| 2.5 | A catalogue of replacement items shall be provided. | 2 | To replace lost, stolen, maltreated or additional items. | Main mobile items to be priced separately. | Main mobile items and all ancillaries to be priced separately. | Technical Assessment | Candidate | Asset Data should include at a minimum the following parameters: Handset identity serial number. Despatched location. Software Configuration. Vehicle fit details i.e. Make, Model, registration number. Building & room location of desktop fits. Ear microphones are classed as consumable due to hygiene regulations |
| 2.6 | Handheld elements of the system shall be as compact as possible. | 2 | Space and weight is at a premium and therefore system should be as small and light as possible. | Handheld radios must weigh less than 800g, | Same as Threshold. | Manufacturers publication Product Samples | Candidate |  |
| 2.7 | The solution shall include GPS. | KSR | There is a requirement for accurate positioning of personnel. In the event of an incident it would aid in the locating the exact place where the incident took place and aid in the recovery of personnel and equipment. | GPS coverage in all areas specified in para 1.2 & 1.3. | Same as threshold | SAT Technical trials. | Candidate |  |
| 3 | Ancillaries |  |  |  |  |  |  |  |
| 3.1 | Battery maintenance shall be easy & minimal | 2 | There is a requirement that any battery maintenance is very simple as this will cut down on resource costs. | Fully charged within 6 hours | Fully charged within 2 hours | Manufacturers publication Technical assessment and user trials | Candidate | Definition of second line maintenance is any operation other than charging the batteries e.g. charge failure, battery leak or physical damage. 6 x 10 Multiple battery chargers and 20 single chargers are required |
| 3.2 | Batteries should be replaced on failure. | 2 | To maintain the efficiency and integrity of the system. | Batteries must be replaced after 500 cycles. | Same as threshold | Technical Assessment | Candidate | All other batteries to be replaced in line with manufacturer’s instructions as part of the managed service. |
| 4 | Equipment - General |  |  |  |  |  |  |  |
| 4.1 | The solution shall provide an end-to-end managed system. | 1 | BATSUB is not resourced to manage the network; as a consequence it is imperative that a comprehensive management system is provided. | The configuration and management of the bearer system and all supplied mobile assets shall be the sole responsibility of the supplier. | Same as Threshold. | Technical Assessment, including supportability plan/methodology. | Candidate | BATSUB do not have the ability to manage User Groups, i.e. they cannot add a radio to a user group. Support levels to be as outlined in Leaflet 1 Annex F. |
| 4.2 | The solution shall operate on licensed frequencies and bandwidths. | M | Legislative. It is a legal requirement to operate on licensed frequencies and bandwidths in Belize. | Must comply with all Belizean communications regulations. | Same as Threshold. | Technical Assessment Certification. | Candidate | The Public Utilities Commission (PUC) is the sole authority for regulations and standards within Belize even if the item already meets EU or other external regulations. |
| 4.3 | The solution shall meet legal regulations for equipment certification within Belize and the UK. | M | It is a legal requirement to ensure that all communications equipment is certified for use in Belize by the PUC. | 100% compliant. | Same as Threshold. | Technical Assessment Certification. | Candidate | The PUC is the sole authority for certifying communications equipment within Belize, even if the item already meets EU or other external regulations. All equipment must be CE certified. |
| 5 | Ergonomics |  |  |  |  |  |  |  |
| 5.1 | Users shall be able to remove/replace power source without tools. | 1 | The solution must be well designed and not require additional tools to remove or replace the power source. | Wearing CS95 gloves. | Same as Threshold. | Technical Assessment User Trials | Candidate | Wearing military gloves. Relates in the main to Hand portable battery replacement in the field. |
| 5.2 | Users shall be able to connect ancillary equipment to the solution with ease. | 2 | Users must be able to connect ancillary equipment to the solution wearing standard military uniform, including issued gloves. | External power, External microphone, remote Press To Talk (PTT), Headphones. | All ancillary equipment wearing CS95 gloves. | Technical Assessment User Trials | Candidate | This relates to connecting remote earpiece and separate speaker. Generally desktop radios have external microphone and speaker arrangement and vehicle sets have external fist mic and speaker arrangement; this is acceptable. |
| 6 | Interoperability |  |  |  |  |  |  |  |
| 6.1 | The system shall not interfere or affect in any way other systems. | M | There is a requirement for this solution to operate near other communications and weapons systems and it is imperative that they do not cause any interference. | No adverse effects are caused | Same as Threshold. | Technical Assessment SAT | Candidate | All electronic equipment to be CE-marked to indicate compliance and with the Automotive EMC Directive 95/54/EC for vehicle fits . |
| 6.2 | The system should operate free of radio interference from any other systems | 1 | System is a critical safety net and requires unrestricted access to its allocated frequencies. | No adverse interference received | Same as threshold | Technical trials | Candidate | System needs to be interference free from other radio spectrum users e.g. local emergency services. |
| 6.3 | System availability | M | Contractual | 99.5% | 99.7% | Technical assessment | Candidate | System is defined as repeater and interlinking capability. |
| 6.4 | The system shall have a high availability figure | M | Critical Health, Safety and Administrative system. | A call should be have a 90% probability of success. | A call should have a 95% probability of success. | Technical Assessment User Trials SAT | Candidate | Contractor to propose how this will be monitored. |
| 6.5 | The system shall have satisfactory Speech Quality | 1 | Contractual | PESQ862 3.5 | PESQ862 5.0 | Technical Assessment User Trials SAT | Candidate | Perceptual Evaluation of Speech Quality Measurement (PESQ) – ITU-P862 PESQ provides a score in the range of 1 to 5 where 1 is unacceptable & 5 is excellent. |
| 7 | Availability |  |  |  |  |  |  |  |
| 7.1 | Sufficient quantities of mobile equipment shall be provided | 1 | There must be sufficient equipment to support all types of users. | 80 handheld radios (including 2 batteries and sufficient chargers), 22 vehicles and 5+ base station installations. | As Threshold. | Technical Assessment SAT | Candidate | For clarity, this means three batteries in total per handset. All desktops are to have a 4hour UPS capability and are to be installed iaw SCIDA regulations contained within JSP604. Vehicle fist are to be IAW ID 2.1 |
| 7.2 | The solution should be able to meet surge demands | 1 | There must be sufficient equipment to support a surge requirement of 20%. | Additional 16 handheld radios, 6 vehicle borne radios and 1 base station to be available within 21 calendar days. | As Threshold. | Technical Assessment | Candidate | 20% surge capacity required within 21 calendar days. The Service shall provide the option to call off additional PMR handsets and associated equipment terminals for planned and unplanned events. The price of calling off additional equipment should be linked to the amount of notice given and term required i.e. week, month or annually. |
| 7.3 | Supplied radio equipment shall be reliable | 1 | Radio equipment must be reliable throughout the length of the contract. Defective radio equipment should be replaced. | Defective equipment replaced within 24 hrs | Same as threshold | Technical Assessment | Candidate | This is a critical system for the Health and Safety of users. |
| 7.4 | The radio equipment shall be robust | KSR | The nature of use, and user throughput, will mean that the solution needs to be robust and able to withstand a degree of forceful handling. | The solution must be able to withstand high humidity, rough handling and low level impacts due to being dropped. | The solution must be able to withstand high humidity, rough handling and low level impacts due to being dropped. | Technical Assessment User trials | Candidate | Mil-Std-810 - Environmental Engineering Considerations and Laboratory Tests. Shall meet IP54 or better and Mil Std 810-D+E Shall meet IP54 or better and mil stds 810-C/D/E IP64 |
| 8 | Logistics/Support Arrangements |  |  |  |  |  |  |  |
| 8.1 | The user shall have a single point of contact (SPOC), i.e. a service or help desk for queries and support on a 24 hour, 7 days a week and 365 days a year basis. | 1 | A single point of contact is required to enable fault and query resolution. | SPOC must be able to escalate and request Level 1 to 4 support (See Leaflet 1 Annex E for details). | Same as Threshold | Technical Assessment Past Experience. | Candidate | See Annex A |
| 8.2 | The solution shall be fully supported from Level 1 to Level 4 (See Leaflet 1 for details) | 1 | BATSUB have no technical support staff. | BATSUB have no technical support staff. | Same as Threshold. | Technical Assessment. | Candidate | No BATSUB maintenance or maintenance activities, i.e. fuse & battery replacement. |
| 8.3 | The solution shall be sustainable for the length and duration of the contract. | M | The equipment must be sustainable to meet the Military Training Plan. | The leased solution must be sustainable for 5 years from Full Operational Capability (FOC). | The leased solution must be sustainable for 5 years from Full Operational Capability (FOC). | Technical Assessment. | Candidate | Any obsolescence issues arising are to be managed out by the contractor during the contract term. |
| 9 | Documentation |  |  |  |  |  |  |  |
| 9.1 | The user shall be provided with relevant training documentation | M | Training documentation is required for user. | Quick start guide to be issued with every radio. | Same as Threshold. | Technical Assessment. Examples from previous contracts. | Candidate |  |
| 9.2 | The solution shall provide a stores inventory which identifies the main equipment, sub-assemblies and ancillaries. | 1 | This is required to ensure that all mobile items are correctly accounted for by users and management. | Every item should have a part no, main items should be serialised. | Same as Threshold. | Technical Assessment. | Candidate | Relates mainly to user mobile equipment. Accountability by military however will still be necessary due to numbers of users involved and their distribution. |
| 10 | Training |  |  |  |  |  |  |  |
| 10.1 | The delivery of the solution shall be accompanied by the delivery of appropriate equipment specific training courses. | KSR | Comprehensive training shall be provided to all personnel required to use the equipment. | Course must enable the student to; Be competent in use of equipment | Same as Threshold | Tech field trial. Ops/User field trial. In-service Sp evaluation | Candidate |  |
| 10.2 | The PMR solution should require very little training for all handheld, vehicle fitted radios and base station users | 1 | There will be a large turnover of users, consequently the radios should be simple to use and require very little training. | The solution should require no more than 2 hours training for handheld and 4 hours training for base station and vehicle fitted. | The solution should require no more than 1 hours training for handheld and 2 hours training for base station and vehicle fitted. | Technical Assessment User Trial | Candidate |  |
| 11 | Personnel |  |  |  |  |  |  |  |
| 11.1 | On-Site Engineering support will be required for all field training exercises. | 1 | During exercises on the training areas it is imperative that this training opportunity is fully utilised and not stopped or delayed due to issues with the communications system. | An on-site engineer is required at the exercise control (EXCON) during live firing exercises. This will be up to 200 days per exercise over a twelve month period. | Same as Threshold | Technical Assessment | Candidate | Specific dates to be set by Trg Br/BATSUB 200 days of support shall be provided as part of managed service. |
| 11.2 | Additional On Site support shall be provided on a call off basis. | 1 | Further equipment training may be required | A daily rate for training support to be provided. | Same as Threshold | Technical Assessment | Candidate |  |
| 11.3 | Additional Management and ‘Train the Trainer’ Courses shall be available on a call off basis. | 1 | May be required due to staff turnover and/ or absences. | Price per course. | Same as Threshold. | Technical Assessment User Trial/Documentation/Course Syllabus. | Candidate |  |
| 12 | Health and Safety |  |  |  |  |  |  |  |
| 12.1 | The solution shall be safe to use in all scenarios including near ammunition, dust and fuel. | M | The solution must comply with UK Health and Safety Regulations. | Approved safety case. Provide a Safety Management System. | Same as Threshold | Documentation, including Safety Case/Plan. | Candidate | Initial first issue of Safety & Environmental Case Report to be made available to the Authority within 30 working days of contract award. Handheld radio’s and batteries shall be suitable for military training near ammunition, dust and hazardous refuelling environments (bulk fuel and/or jerry cans), temperatures in the environments, and the operating range of the radio’s. For ammunition, MOD will be required to carry out a HERO assessment for safe working distance to munitions, including countermeasures, once makes and models have been provided by supplier. |
| 12.2 | The Contractor shall follow all safety procedures or protocols that apply to the sites being visited. Local site instructions and safe working practices must be adhered to at all times. | M | Health, Safety, Environmental, Fire | 100% Compliance | Same as Threshold | Technical Assessment | Candidate |  |
| 12.3 | The Contractor will be required to produce and maintain Safety Cases for any equipment supplied. Including Safety Environmental Management Plans | M | Health, Safety, Environmental, Fire | 100% Compliance | Same as Threshold | Technical Assessment | Candidate | The Contractor is responsible for the provision of safety equipment including personnel protection equipment and clothing. The Contractor must adhere to the Provision and Use of Work Equipment Regulations 1998 (PUWER). The Contractor shall show compliance with the requirements of the MoD Project Orientated Safety Management System (POSMS) and related Joint Services Publications for Land Systems Equipment Safety Assurance under the current MoD policy for managing equipment safety. The Contractor shall provide a Safety Policy statement and Safety Manual, showing they are meeting the requirements of BS/OHSAS 18001 Occupational Health and Safety. Tenderers are to articulate their experience in this area. Separate costs should be provided with any quotation provided. Safety Cases are to submitted in accordance with DEFSTAN 00-56 and or DEFSTAN 00-972 Safety Case to follow the following format if no airfield involved: SEMP to follow the following format: |
| 12.4 | The Contractor shall produce and maintain a Safety and Environmental Case and Hazard Log, which shall be reviewed on a yearly basis or sooner if the circumstances dictate e.g. after major accidents/incidents or major equipment changes. The Contractor shall have the Safety Case independently assessed by a suitably qualified Independent Safety Assessor of their choice. | M | Health, Safety, Environmental, Fire | 100% Compliance | Same as Threshold | Technical Assessment Documentation, including Safety Case | Candidate |  |
| 12.5 | The Contractor shall produce (within 21 days of contract let) and maintain, a Project Hazard Log. | M | Health, Safety, Environmental, Fire | Project Hazard Log produced within 21 days | Project Hazard Log produced within 14 days | Technical Assessment | Candidate |  |
| 12.6 | The Contractor shall produce (within 21 days of contract let) and maintain a Project Method Statement on how activities are to be conducted safely. The Method Statement document shall define the procedures to be employed whilst carrying out the routine maintenance and call out support of the Belize Training Areas Digital Radio system. The document must be made available to the Authority within 14 days of commencement of the Contract. | M | Health, Safety, Environmental, Fire | Project Method Statement produced within 21 days | Project Method Statement produced within 14 days | Technical Assessment | Candidate |  |
| 12.7 | - The Contractor must show compliance with the requirements of the MoD Project Orientated Environmental Management System (POEMS), related Joint Services Publication 418 MOD Corporate Environmental Protection Manual, and should demonstrate ISO 14001. | M | Health, Safety, Environmental, Fire | 100% Compliant | 100% Compliant | Technical Assessment | Candidate |  |
| 12.8 | The Contractor shall provide and Environmental Policy and Procedures statement in accordance with ISO 14001. | M | Health, Safety, Environmental, Fire | 100% Compliant | 100% Compliant | Technical Assessment | Candidate |  |
| 12.9 | The Contractor shall be responsible for carrying out a Hazard Analysis and Risk Assessment for each site to be visited. Risk Assessment forms are to be compiled and maintained. New hazards or changes to existing hazards shall be recorded and reported to the Authority. | M | Health, Safety, Environmental, Fire | 100% Compliance | 100% Compliant | Technical Assessment | Candidate |  |
| 12.10 | Assessments in accordance with the Control of Substances Hazardous to Health regulations (CoSHH) are to be carried out for each site to be visited. The Contractor shall compile and maintain a file containing the relevant Safety Data Sheets for all chemical substances etc. that are to be used to carry out any maintenance activity. Particular attention shall be paid to working with electricity, radiation and working at height as appropriate to the activities being undertaken. | 2 | Health, Safety, Environmental, Fire | 100% Compliance | 100% Compliant | Technical Assessment | Candidate |  |
| 12.11 | The Contractor shall provide a minimum of two personnel to be present at all times for undertaking work at remote unmanned sites. No ‘lone-worker’ processes or procedures will be allowed. The Contractor shall be responsible for their own First Aid provision for both equipment and trained First Aiders, particularly at remote unmanned sites. | M | Health, Safety, Environmental, Fire | 100% Compliance | 100% Compliant | Technical Assessment | Candidate | No “Lone-worker” processes will be accepted. |
| 13 | Organisation |  |  |  |  |  |  |  |
| 13.1 | The solution should not require any changes to BATSUB organisation | 1 | BATSUB personnel numbers will not be increased as a result of any new communications system. | No change to BATSUB organisation and staffing levels. | Same as Threshold | Technical Assessment | Candidate |  |
| 14 | Quality Management |  |  |  |  |  |  |  |
| 14.1 | The solution shall provide an assured quality of service | M | A quality solution is required given its importance to personnel safety. | Provide a quality ‘End to End’ (E2E) Management system. | Same as Threshold | Technical Assessment Certification to recognised Quality Standard | Candidate | See Annex C for Quality Targets. |
| 14.2 | A Quality Plan will be required IAW AQAP2105 The Contractor shall provide a quality plan in accordance with AQAP 2105 & DEFCON 602A and the requirement of this section of the document. This should include, but not exclusively, the following; - Project description - Organisation and responsibilities - Resource Management - Processes (General Requirements) - Documentation - Certificate of Conformity The contractor shall provide a Certificate of conformity for each service item supplied. | M | Quality Assurance | 30 days | 60 days | Technical Assessment | Candidate | A draft plan should be supplied with the tender, with a full plan within 60 days for the chosen supplier. Note days refer to calendar days Template to follow: |
| 14.3 | Quality Assurance shall be carried out to the satisfaction of the Authority. The Contractual Quality requirements are: BS EN ISO 9001:2008 Quality Management Systems. AQAP 2105, Edition 1 NATO Requirements for Deliverable Quality Plans AQAP 2110, Edition 1 NATO Quality Assurance Requirements for Design, Development and Production. DEF STAN 05-57, Issue 5 Configuration Management of Defence Material DEF STAN 05-61, Part 1, Issue 4 Quality Assurance Procedural Requirements – Concessions DEF STAN 05-95, Issue 3 Quality System for the Design development, Supply and Maintenance of Software DEF STAN 05-99, Issue 2 Managing Government Furnished Assets in Industry | M | Policy/Standardization | 100% Compliance | Same as Threshold | Technical Assessment | Candidate | The service will conform to Industry and Defence standards for equipment and how it is managed and delivered. If not issued by a UKAS recognised 3rd party certification body, then relevant local standard. |
| 14.4 | All equipment must be NATO codified by the contractor. | M | MOD policy Standardisation | 100% complaint | 100% compliant | Technical Assessment | Candidate | NATO codification must be undertaken in accordance with the Defence Policy and Procedures for NATO codification. JSP 886 |
| 15 | Doctrine & Concepts |  |  |  |  |  |  |  |
| 15.1 | The solution shall support BATSUB field training and routine business | 2 | Concept of Use | The solution should be capable of sustaining 24/7 operation | Same as Threshold | Technical Assessment | Candidate |  |
| 16 | Delivery |  |  |  |  |  |  |  |
| 16.1 | Full Operating Capability (FOC). | 1 | The user requires the full system to be in place before training can commence in the specified training areas | Defined as all deliverables received and accepted by the user. | Same as Threshold. | Technical Assessment (including Coverage Predictions, Project Plan, Risk Management and Controls etc) System Acceptance Test | Candidate | FOC is defined as a fully working system with all infrastructure, network and testing complete. |
| 17 | Maintenance |  |  |  |  |  |  |  |
| 17.1 | The Service Provider will fully maintain the system | 1 | To ensure the ongoing reliability of the system | Maintenance should ensure ongoing functionality of the system | Same as Threshold | Technical Assessment | Candidate | See Annex E |
| 18 | Options |  |  |  |  |  |  |  |
| 18.1 | Requirement 2 Additional training areas may need to be used in the future and they are: Gallon Jug Yalbac New River Lagoon Seven Hills Ranges Hill Bank Spanish Creek Hattyville Range Deep River | KSR | Level of coverage is essential when training is taking place within this training area to ensure safe training of all personnel within the training area. | The solution will provide 75% radio coverage for troops conducting training on areas shown on Training Areas Map GSGS 5935 Edition 1. | The solution shall provide 100% coverage | Technical Assessment (including Coverage Predictions, Blind Spots identified, Project Plan, Risk Management and Controls etc) Tech Field Trial. Ops field trial. In-service Sp evaluation. | Candidate | Areas where coverage is suspect or non-existent should be clearly documented. The contractor will be given a minimum of twelve months’ notice to provide coverage for these additional training areas. |
| 19 | Performance Metrics |  |  |  |  |  |  |  |
| 19.1 | Failure to send out an engineer within agreed timescales to rectify a fault. | M | Contractual | Per repair | Per repair | Technical Assessment | Candidate | 5% reduction in one quarterly payment |
| 19.2 | Failure to meet help desk targets. | M | Contractual | Less than 30 seconds | As Threshold | Technical Assessment | Candidate | 5% reduction in one quarterly payment if 10% of the call are not answered iaw the Annex A during one quarter |
| 19.3 | System (Repeater availability). | M | Contractual | 99.5% | 99.5% | Technical Assessment |  | Contractors are to propose how they would monitor availability over a 7 day period (Monday to Sunday) for repeater availability at all sites. Failure of two 7 days periods within a three month period will incur a 5% reduction in one quarterly payment |
| 19.4 | Failure to meet FOC date for Requirement 1 | M | Contractual | 31 July 2017 | 30 June 2017 | Technical Assessment | Candidate | For each month FOC is delayed there will be 10% reduction in the monthly payment until FOC is declared. Additionally failure to meet contractual FOC date will result in the contractor funding all charges incurred by the Authority in extending the existing support arrangements. |
| 19.5 | Failure to meet FOC date for Requirement 2 | M | Contractual | TBA | TBA | Technical Assessment | Candidate | For each month FOC is delayed there will be a 10% reduction in the monthly payment until FOC is declared. |
| 19.6 | Failure to meet a surge requirement | M | Contractual | Per instance | Per instance | Technical Assessment | Candidate | 5% reduction in one monthly payment. |
| 19.7 | Failure to replace defective equipment within agreed timescales | M | Contractual | Per instance | Per instance | Technical Assessment | Candidate | 5% reduction in one monthly payment. |

Annex A

- Contractor’s Help desk

- Request for Support

- The Help-Desk service shall be run in both English and local Belizean languages and be in operation and staffed 24 hours per day, every day. It will manage the resolution of all calls made by the User Authorised Support Representative(s). The Help-Desk should answer all telephoned requests for support within 30 seconds. In addition all faxed or email requests for support should be acknowledged within one hour of receipt.

- When requesting Support, the User’s Authorised Support Representative(s) shall provide the following information:

- User name;

- Site details the location or unit requiring support;

- Name of person requesting support;

- Contact details of the person to whom the Contractor should direct any response or request for additional information;

- Any other information necessary to enable the Contractor to identify the component affected;

- A full description of the problem, including any error messages and the circumstances leading to the call.

- A unique call reference number shall be provided and maintained for each call for support made by the User. All information relating to the call and all actions taken will be retrievable utilising the unique call reference number.

- The Contractor shall measure and report quarterly on the Service Levels centrally to the Authority.

- The Contractor may use any reasonable and practical method to determine the Availability of the Contractor Help Desk on a routine basis; such method shall be defined in the Contractors response to the ITT.

  - Initial Diagnosis and Call-Back	The Help-Desk will call back the originator (or nominated alternative contact), confirming that the call is being addressed, obtain any other information that may be required and agree the subsequent course of action. If the call can be resolved at this stage, (or the call does not require support), the call shall be closed with the agreement of the User

  - Call progress Monitoring	The Contractor will monitor and proactively progress calls against the service restored targets detailed in the SRD and will notify the User contact whenever any target response time is not met.

  - Problem / Fault Resolution   A call can only be closed with the agreement of the Authorised Support Representative. The Help-Desk will log and monitor the action plans and any failure to achieve the plan will be escalated by the Contractor and/or user.

  - Emergency Software Change	Emergency software changes may, with the prior agreement of the User, be undertaken in order to restore service. Emergency software changes will be fully documented, but of a temporary nature and will remain in effect until a formal scheduled software release is available and implemented. Permanent resolution of software faults will be addressed by periodic software releases.

  - Help Desk Service Levels.  The Contractor Help Desk shall achieve the following Service Levels:

- 99% Availability measured monthly. The Contractor Help Desk shall be considered Available to Users if enquiries and requests can be submitted through voice, fax and email channels listed above;

- 95% of all Help Desk enquiries measured monthly shall be actioned or escalated to User’s satisfaction within 4 Normal Working Hours.

- The User shall also be provided with alternative telephone and pager contact numbers in order to implement an escalation procedure, when it is not possible to obtain support or escalation via the Help Desk. These alternative telephone and pager contact numbers shall be for management representatives and not service engineers.

Annex B

- Support Meetings

- Support meetings between the Authority and the Contractor shall be held on a Quarterly basis.

- The meetings shall be by mutual consent take the form of an audio teleconference, video teleconference or face to face at a location decided by the Authority.

- The meeting organisation and record keeping shall be the responsibility of the Contractor.

- The Terms of Reference for the review meetings shall be agreed by the Authority and the Contractor but as a minimum the following tasks shall be covered:

      - Highlight any new user requirements for future equipment.

- b.	Monitor the performance of the system being provided under the MOD contract, e.g. conformance to SLA’s, quality, ordering, billing and system availability, items in the repair loop.

- c.	Act as a focal point for user complaints and letters of plaudit.

- d.	Address any safety issues that arise.

- f. 	Address any P2P Catalogue issues that arise.

- g.	Address issues regarding training volume planning.

- In advance of the review meeting the Contractor shall produce a periodic report detailing:

- a.	Progress since last period;

- b.	Planned actions for next period;

- c.	Performance against service level requirements detailed in the SRD.

- In addition each quarter the Contractor shall provide the User with up to date management information, including but not limited to a list of:

  - All faults logged in the preceding quarter, indicating down time to date for each, their current status and progress;

  - All calls closed in the preceding quarter, indicating the closure details and the down time for each call, as well as a summary sheet detailing the overall clearance rates for the current quarter;

  - All currently open calls, indicating their status and progress including down time to date for each;

  - The distribution of ‘time to answer’ of all Help-Desk telephone calls.

  - Report any safety related issue(s) along with corrective action being taken.

- The quarterly reports and quarterly management information shall be supplied in electronic form and in MS Word, MS Excel, PDF or other mutually agreed format.

Annex C

- Quality

- Contractual Requirements

- 1.	Quality Assurance shall be carried out to the satisfaction of the Authority. The Contractual Quality requirements are: -

| AQAP 2105, Edition 1 | NATO Requirements for Deliverable Quality Plans http://www.nato.int/docu/stanag/aqap2105/aqap-2105-e.pdf |
| --- | --- |
| AQAP 2110, Edition 1 | NATO Quality Assurance Requirements for Design, Development and Production. http://www.nato.int/docu/stanag/aqap2110/aqap2110e.pdf |
| AQAP 2110 | Design Development & Production |
| DEF STAN 05-57, Issue 5 | Configuration Management of Defence Material UK Defence Standardization |
| DEF STAN 05-61, Part 1, Issue 4 | Quality Assurance Procedural Requirements – Concessions UK Defence Standardization |
| DEF STAN 05-61 Part 4 | Contractor Working Party |
| DEF STAN 05-99, Issue 2 | Managing Government Furnished Assets in Industry UK Defence Standardization |

- Quality Plan

- 2.	The Contractor shall provide a Quality Plan written in accordance with AQAP 2105 and the requirements of this section of this document. This should include, but not exclusively, the following: -

- Project description

- Organisation and responsibilities

- Resource Management

- Processes (General requirements)

- Documentation requirements

- Certificate of Conformity

- 3.	The Contractor shall provide a certificate of conformity for each service item supplied.

- 4.	Certificate of Conformity is a document, signed by the Supplier, which states that the product conforms to contractual requirements.

Annex D

- Contractor Equipment Safety and Environmental Management Systems Requirement

1.	Mandated Processes for Safety and Environmental Protection (S&EP)

In July 2004 the use of a single Acquisition Safety and Environmental Management System (ASEMS) was mandated (DEF STAN 00-56 Safety Management Requirements for Defence Systems).

2.	At the core of the ASEMS there are two Systems Manuals:

- Project Oriented Safety Management System (POSMS)

- Project Oriented Environmental Management System (POEMS)

3.	Compliance with POSMS and POEMS will ensure that a Safety and Environmental Management System is robust, proportionate to a project’s levels of risk and is compatible with the DE&S corporate reporting requirements.

4.	More information can be found at:

- Mandated Processes for Safety and Environmental Protection » Safety and Environmental Protection (S&EP) » Acquisition Operating Framework (AOF)

5.	The Contractor shall submit a safety and environmental systems management manual within 4 weeks of the contract award.

6.	The Networks Team will provide ASEM Guidance Notes to aid the Contractor to meet their obligation at this section on contract award.

- Acquisition Safety & Environmental Management (ASEM).

7.	The contractor shall provide a Safety Policy statement and Safety Manual meeting the requirements of BS/OHSAS 18001 Occupational Health and Safety.

8.	The contractor shall provide an Environmental Policy and Procedures statement in accordance with ISO 14001.

Annex E

Contractor Equipment Maintenance Requirements

- The Contractor shall be responsible for full maintenance on all items such as the Network i.e. repeaters,  PMR Handsets, and associated ancillaries in accordance with the equipment manufacturer’s instruction, specification and warranties, unless specifically advised by the User.

- The repair by the Contractor shall extend to all faults and damage and not only that identified originally by the User.

- Where damage is caused as a result of negligence on the part of the Contractor, then the Contractor shall be totally responsible for the repair or replacement of any item.

- Where the Contractor considers it necessary to modify any manufacturer’s maintenance specification, instruction or recommendation, such a modification shall only be undertaken with the prior permission of the User and upon detailed written submission of the proposal

- The Contractor shall be responsible for the repair of all faults and damage exhibited or suffered by items of PMR Handsets and associated ancillaries where such faults or damage are consistent with Fair Wear and Tear.

- Fair Wear and Tear shall be considered as damage and / or defects PMR Handsets or part thereof, which occurs during normal and proper use of the equipment in connection with the User’s function.

- The Contractor shall be responsible for the administration, management and repair of all faults associated with individual items of supplied PMR equipment and their associated ancillaries, either hand portable or vehicle mounted.

- Authority damage will be considered on a case by case bases taking into consideration time leased and whether item is repairable or beyond economic repair.

Annex F

Leaflet 1 Technical Support Levels

    - CIS / EW Technical Support (Delivery & Levels).  Battlefield Equipment Support Doctrine (BESD) defines levels of ES; however, these definitions do not fully define the activities associated with the maintenance of CIS / EW capability, such as software installation, information management, system administration etc.  The definitions below set out the levels and delivery methods of CIS / EW technical support, which includes maintenance.

Levels of Support

    - Levels.  In line with BESD there are four clearly defined Levels of technical support:

- Level 1.  Servicing and day-to-day activity generally conducted locally by the qualified, operator or engineer responsible for the delivery of the service or manning of an installation, this requires little or no additional test tools, equipment, applications or remote system access.  This may include basic operations such as:

    - Compliance with Installation Maintenance Instruction (IMI).

    - Consumable replenishment.

    - Functional testing (use of Built In Test Equipment (BITE)).

    - Fault diagnosis and corrective maintenance by replacement, adjustment or minor repair.

    - Local support to information managers, network monitoring and software / hardware release management.

    - Basic Local Area Network (LAN) administration and local application configuration.

    - Minor modification.

- Level 2.  Conducted by engineering staff with the appropriate specialist training and experience.  This requires some additional test tools, equipment or applications.  This may be conducted locally or remotely.  This may include functions such as:

    - All Level 1 tasks.

    - Greater level of fault diagnosis and corrective maintenance by replacement, adjustment and repair.  Forward based limited Level 2 support may include the greater level of fault diagnosis, but limited corrective maintenance ability (cable repair, minor software workarounds).

    - Advanced, system administration and configuration, application configuration, support to information and LAN managers, across a Metropolitan Area Network (MAN), formation, or multiple sites.

    - Basic interfacing and system interoperability tasks, including:

        - Inter-boundary International Standard Organisation (ISO) model layer 1 connection management.

        - Inter-boundary ISO model layer 2 and 3 gateway configuration.

        - Formation wide information flow management.

        - Formation wide bandwidth prioritisation and management.

- Level 3.  Conducted by specialist engineering staff with appropriate additional training and experience.  This activity will usually be conducted remotely.  This requires some additional test tools, equipment and applications, which may or may not be IT based.  Inter and intra system fault analysis and diagnosis will routinely be addressed through remote access.  Functions carried out may include:

    - All Level 1 & 2 tasks.

    - Advanced, multi-system administration and configuration, cross system application configuration, support to Force information managers and MAN managers, across multiple systems (Formation and Force wide).

    - Advanced interfacing and system interoperability tasks including:

        - Inter-system end to end service delivery, ISO model layers 4 – 7 monitoring and management.

        - Force wide information flow management.

        - Force and reach back wide bandwidth prioritisation and management.

    - Technical Base lining of an Installation or System.

- Level 4.  Level 4 is typically conducted out of theatre by defence contractors or specialist organisations, in support of the system design and operating authorities.  These organisations will typically have the ability to remote access, monitor and configure deployed systems as well as model deployed systems within specialist facilities.  This requires routine use of highly specialist, bespoke, test tool, equipment and applications.  Functions carried out include:

    - All Level 1, 2 and 3 tasks.

    - Major or highly technical repairs, upgrades and reconditioning.

    - Major or highly technical repairs, upgrades and reconditioning.

    - Advanced, global WAN, system of system administration and configuration tasks, assistance to WAN managers, system of system application configuration.

    - Advanced interfacing and system of system interoperability tasks including:

        - Multiple, joint and combined gateway and system end to end service delivery, across the ISO model.

        - Joint and combined system delivery.

        - Multiple, joint and combined information flow management.

        - Multiple, joint and combined bandwidth prioritisation and management.

Delivery of Support

    - Delivery.  CIS / EW technical support can be delivered either locally or remotely:

  - Local.  This is the physical replacement, reloading and initialisation of equipment, sub-assemblies, Line Replaceable Units (LRU) or the reloading of software, firmware or soft protocols by direct interfacing with the system or equipment being supported.  This may be conducted by:

    - The individual responsible for Level 1 support.

    - Forward based engineering staff (intimate support, previously referred to as 1st Line).  Potentially only up to limited Level 2 support.

    - Rear or centralised engineering staff deploying forward to provide Level 2/3 support to resolve a specific issue.

  - Remote.  The reloading, initialising, restarting of software, firmware, protocol stacks, or other software or electronic, data, objects through remote access via the appropriate CIS.  This may be conducted by rear or centralised engineering staff offering Level 2/3/4 support, either via remote access or via faulty elements being shipped rearwards for attention

Annex G

Glossary

| Acronym | Definition |
| --- | --- |
| AC/DC | Alternating/Direct current |
| AES | Advanced Encryption Standard |
| AQAP | Allied Quality Assurance Publications |
| ASG | Acquisition System Guidance |
| AST | Agreed Service Time |
| BATSUB | British Army Training Support Unit Belize |
| BS | British Standard |
| CE | Conformité Européene |
| CIS | Communications Information Systems |
| CM | Configuration Management |
| CoSHH | Control of Substances Hazardous to Health |
| COTS | Commercial Off the Shelf |
| DEFCON | Defence Conditions |
| DefStan | Defence Standard |
| DFTS | Defence Fixed Telephone Services |
| DT | Down Time |
| DTL | Delivery Team Leader |
| EMC | Electromagnetic Compatability |
| EU | European Union |
| FOC | Full Operating Capability |
| FRB | Failure Review Board |
| GPS | Global Positioning System |
| GPT | Government Provided Telephone |
| GQA | Government Quality Assurance |
| GQAR | Government Quality Assurance Representative |
| GSGS | Georgraphical Section of the General Staff |
| HERO | Hazards of Electromagnetic Radiation to Ordnance |
| HFT | Hybrid Foundation Training |
| ID | Identifier |
| IEEE | Institute of Electrical and Electronic Engineers |
| IET | Institute of Engineering and Technology |
| ISA | Independent Safety Auditor |
| ISD | In Service Date |
| ISO | International Organisation for Standardisation |
| ISS | Information Systems and Services |
| ITE&A | Integrated Test, Evaluation and Acceptance |
| ITEAP | Integrated Test Evaluation and Acceptance Plan |
| ITT | Invitation to Tender |
| ITU | International Telecommunication Union |
| JSP | Joint Service Publication |
| KPI | Key Performance Indicator |
| KSR | Key System Requirement |
| LFE | Learning from Experience |
| MOD | Ministry of Defence |
| MOE | Measure of Effectiveness |
| MSR | Main Supply Route |
| NARO | Naval Aircraft Repair Organisation |
| NATO | North Atlantic Treaty Organisation |
| OEM | Original Equipment Manufacturers |
| OHSAS | Occupational Health and Safety Advisory Service |
| P2P | Purchase to Payment |
| PMO | Programme Management Office |
| POEMS | Project - Oriented Environmental Management System |
| POSMS | Project-Oriented Safety Management System |
| PQAO | Project Quality Assurance Officer |
| PSQM | Perceptual Speech Quality Measurement (PSQM) |
| PUC | The Public Utilities Commission (PUC) |
| PUWER | Provision and Use of Work Equipment Regulations |
| QA | Quality Assurance |
| QMS | Quality Management System |
| QPI | Quality Performance Indicator |
| RSS | Royal School of Signals |
| SAT | Site Acceptance Test |
| SC | Security Check |
| SCIDA | Site Coordinating Installation and Design Authority |
| SECR | Safety and Environmental Case Report |
| SLA | Service Level Agreement |
| SOR | Schedule of Requirements |
| SQEP | Suitably Qualified and Experienced personnel |
| SR | System Requirements |
| SRD | System Requirement Document |
| TAF | Task Acceptance Form |
| T&S | Travel and Subsistence |
| UKAS | United Kingdom Accreditation Service |
| UPS | Uninterruptible Power Supplies |
| URD | User Requirement Document |
| WEEE | The European Waste Electrical and Electronic Equipment Directive |

Installation Maintenance Instruction (IMI).  The IMI details the maintenance tasks required on a particular installation type, in an easy to follow logical sequence.  It is drawn from the maintenance tables contained in the 601 section of the individual equipment AESPs that make up the installation or as detailed in the Interactive Electronic Technical Publication (IETP).  The IMI is produced by the RSIT with the extant version available via the Army Electronic Battle Box (Land Component Handbook (ICS) Section 3.1.4.3.1 Installation Maintenance Instructions).