General

Where specific manufacturers are quoted, the bidder may offer an alternative providing it can be demonstrated that it meets or exceeds the existing requirements (this is to include warranties).  This should be agreed as part of the clarifications.

- External – scope of works

- Strengthen external walls to entrance lobby, and construct new aperture for entrance door on front elevation.

- Overclad all external walls with ‘Cladco’ composite cladding or similar, in Light Grey with (darker) Stone Grey corners and trim, fixed to timber battens and insulated using a ‘YBS Airtec’ insulation membrane. Any weakened patches on the external walls will firstly be repaired.

- Replace all windows and external doors with new double glazed PVC units, wrapped in Anthracite Grey. Windows to front elevation (in main play area) to be full length units, with toughened glass to the lower (fixed) panel.

- To include deeper reveals for the windows to allow for increased wall thickness once external cladding is applied.

- Create aperture for access to new ‘stand-alone’ storage unit to rear, and install a new secure external door.

- Deliver and assemble a new weatherproof external storage unit, and overclad with same material to be used for the external walls.

- Remove existing timber perimeter skirt, and replace using same external cladding product, fitted with vents to allow natural ventilation.

- Remove and dispose of existing access ramp and fire exit platform.

- Provide new fully compliant access ramp, steps and platform with handrails to front elevation, using powder-coated steel and with ‘safetread’ crumb surface.

- Provide new fully compliant fire exit platform and steps with handrails to side elevation, using powder-coatedsteel and with ‘safetread’ crumb surface.

- Create new decking area leading from fire exit to side elevation, to be extended around left-hand corner of front elevation, using similar materials to the external cladding (in matching colour scheme).

- Install a fixed canopy above the decking area (along the front elevation only), allowing protection during sunny conditions.

- Repair and improve timber fencing where necessary to rear of site - provisional sum of £4,000 to be allowed. Improve external lighting around perimeter of building.

- Internal – scope of works

- Remove and relocate internal walls and doors that form the entrance lobby, office, store, kitchen and staff WC to create new layout as drawn – ref: 241-24-03 – including newly-fitted Doc M fitments to form a disabled WC facility.

- Works to also include removing all internal doors and replacing with new oak doors, glazed as appropriate. Install fully glazed partition and door to create new Quiet Zone (also as drawn).

- Remove and dispose of existing kitchen fitments, and replace with new kitchen facility in revised location. To include the installation of base units, wall units, stainless steel sink with drainer, electric cooker, microwave, washing machine, fridge and dishwasher. (Design to involve input from Pre-School team.)

- Over-line existing walls in play area using plastic panels with a ‘wipe clean’ surface.

- Overlay (where possible, otherwise remove and replace) new floor coverings throughout – vinyl or carpet, depending on use of room.

- Remove inadequate heating to play area and office, and replace with wall-mounted comfort cooling units, operated by remote control and with external condensers complete with cages (on rear elevation).

- Replace existing heating to entrance lobby and WCs with energy efficient units.

- Remove ceiling lighting throughout and upgrade to new LED ceiling lighting fitments throughout. Supply and install new wall-mounted ‘Smart’ screen.

- Additionally Requirements

- To include rubbish skips, and disposal of all waste materials to a designated commercial facility.  Waste certificates will be required.

- To also include the provision of a temporary secure storage container (to be sited on the car park), in which all furniture and fitments can be placed while internal works are underway.