- PURPOSE AND SCOPE

The Health and Safety Executive (HSE) define “Working at Height” as work in any place where, if precautions were not taken, a person could fall a distance liable to cause personal injury. An individual is working at height if they:

- are working above ground/floor level

- could fall from an edge, through an opening or fragile surface or

- could fall from ground level into an opening in a floor or a hole in the ground

Work at height does not include a slip or a trip on the level, as a fall from height has to involve a fall from one level to a lower level, nor does it include walking up and down a permanent staircase in a building.

Work at height is the biggest single cause of fatal and serious injury in the construction industry, particularly on smaller projects or tasks.  Over 60% of deaths during work at height involve falls:

- From ladders, scaffolds, working platforms and roof edges

- Through fragile roofs or roof lights

At The Pirbright Institute a number of tasks exist where Working at Height is necessary in order to complete a given task.   This work instruction applies to project and maintenance works performed by EMS personnel who are undertaking activities that will involve working at height, including (but limited to) working on ladders, MEWPs, and scaffolding.

Working at height activities to be carried out on exposed open roofs on site (and on institute owned housing) will be carried out by trained and competent contractors.  Process for activities to be carried out around or involving trenches, pits or opening is detailed in separate work instruction EMS-WI-091.

Contractors must follow the principles of this work instruction, but will ultimately work under their own company’s task and site specific risk assessment and method statement for tasks that involve working at height.

The Health and Safety at Work Act 1974 places a duty on employers to ensure, as far as is reasonably practicable, a safe system of work is in place. Mandatory permits-to-work are an essential component of working in a safe environment. The implementation and correct use of these will ensure that Pirbright Institute is complying with current legislation and ensures that work activities are carried out under a controlled practice, minimising risk to employers.  In addition, the Work at Heights Regulations 2005 requires a task specific assessment to be carried out before starting any work activity at height.

NOTE:  FAILURE TO FOLLOW THIS PROCEDURE MAY RESULT IN INJURY TO PERSONNEL OR DAMAGE TO EQUIPMENT.

- RISK ASSESSMENT

The following types of risk assessment are applicable to this work instruction:

  - A hazard based risk assessment for “Working at Height” is recorded and managed on Oshens (RA000129).  This outlines the control measures to be used by EMS staff when carrying out tasks that involve working at height.

  - A mandatory task specific risk assessment will be required for all activities working at height, with the exception of activities involving a step-ladder/step-up ladder for routine tasks.  This task specific risk assessment will be recorded via Oshens or relevant Risk Assessment and Method Statement (RAMS) form used in the EMS Permit-to-Work process.

  - There is a requirement by the Competent Person to produce a Point of Work Risk Assessment (PoWRA) prior to any and all works.   This will identify and be used to record any additional hazards not already identified in planning stage.

  - All works performed by contractors require a task specific RAMS submitted in advance of the works.

- RESPONSIBILITIES

  - All personnel using this WI are responsible for ensuring that they have read and understood the contents of this WI and that the procedure is followed.

| Senior Capability EMS Leader | Ensures the full implementation of this procedure. Ensures all relevant personnel are trained in the legal requirements of the Health and Safety at Work Act 1974. |
| --- | --- |
| Capability EMS Leader (Engineering & Operations) and Capability EMS Leader Projects and Process | Provide Site Operating Procedures (SOP’s) and Work Instructions to ensure all systems are implemented safely and effectively when used. |
| Capability EMS Health, Safety & Risk Manager | Ensures the Operational safety of site personnel, reviewing documentation as required. Ensure all contractors & EMS/BMS staff fully understand and comply with the Permit to Work system where applicable |
| Senior Maintenance Owner and Senior Specialist Equipment Owner and Senior BMS Owner | Ensures the system is used correctly on all occasions where there is a mandatory, procedural or Risk Assessment requirement. |
| Competent Person | EMS Personnel or Contractors under EMS control carrying out the work task(s). Must understand the conditions and their responsibilities with regards to all Work Instructions & RAMS where required. Only carry out the work in a competent and safe manner. Inform the Supervisor (Responsible/Authorising person) of any training needs or any unsafe working practices. Ensures ALL control measures required to safeguard those who may be affected by the activity, are identified, included on all permits and actioned. |
| Responsible Person | Reviews RAMS for task Ensures ALL control measures required to safeguard those who may be affected by the activity, are identified, included on all permits and actioned. Ensures the competent person undertaking the work is fully conversant with the safety implications and control measures associated with the activity. Ensure ALL contractors & EMS/BMS staff fully understand and comply with their Permit to Work |
| Authorising Person | Person authorising & issuing a Permit to Work. Coordinate all technicians/BMS Engineers activities on site, including EMS related contractors Ensure ALL contractors & EMS Technician/BMS staff fully understand and comply with their Permit to Work Only issues permits after RAMS have been reviewed by the responsible person. |
| Housing and Property Manager | Ensure all housing & property actions / works are performed in line with relevant guidelines. |
| Asset and Compliance Manager | Ensures all documentation obtained from Q-Pulse is correct and available. Ensures the correct documents are being used for the task in hand and where required obtained from Q-Pulse |

- SPECIALIST MATERIALS OR EQUIPMENT

  - N/A

- TRAINING REQUIREMENTS

  - Refer to QA-SOP-29 for general institute training requirements.

  - Personnel who will be working at height must have attended a relevant “Working at Height” toolbox talk, and have completed e-learning online “Working at Height”.   Personnel must also have read and acknowledged this work instruction and the hazard based risk assessment on Oshens.  EMS operations personnel must ensure their competency book is up to date (have attended toolbox talk, completed working at height awareness online course, experience listed).  This is the minimum requirement for personnel working at height using step-ladders or lean-to-ladders, and personnel working on mobile towers and fixed scaffolding (working on but not erecting).

  - Personnel working on Mobile Elevating Working Platforms (MEWPs) must have attended a recognised IPAF (International Powered Access Federation) operator training course and received an IPAF card or PAL (Powered Access Licence) card, listing the categories of MEWP the person is trained to operate.  In addition, personnel must have completed requirements listed under Section 5.2.

  - Personnel who are responsible for assembling, dismantling, moving and inspecting mobile tower scaffold must have attended appropriate recognised training course and hold an in-date PASMA (Prefabricated Access Suppliers and Manufacturers Association) certificate or card licence.  In addition, personnel must have completed requirements listed under Section 5.2.

- PROCESS

  - Planning Work

Personnel planning to carry out work at height are required to identify the risks involved and the precautions to take in order to complete the work safely.  The objective is to make sure work is properly planned, supervised and carried out in a safe manner.  The HSE recommends the following hierarchy principles are applied when planning a working at height task:

For example:

- Avoid work at height where it reasonably practicable to do so, e.g. by assembly at ground level

- Prevent any person falling a distance liable to cause personal injury e.g. by using a scaffold platform with double guard-rail and toe boards, safety restraint harness/lanyard.

- Minimise distance and consequences of a potential fall with fall arrest equipment e.g. fall arrest lanyard (energy absorber), safety nets.

These principles must be applied by all competent EMS personnel when planning an activity involving working at height.

Considering the risks associated with tasks involved and putting in place sensible and proportionate measures to manage them is an important part of working safely.

  - Safe System of Work for WAH activities

Nearly all working at height activities will require a mandatory Permit to Work to be raised.  In order to raise a Permit, a task specific risk assessment and method statement will need to be prepared and in place before work is started.

The use of step-ladders do not require a mandatory Permit to be raised, as these types of ladders are mostly used for low risk short duration tasks.  However, a Permit can be raised for use of step-ladders by the Competent Person carrying out the work, if the risks warrant increased safety protocol.  Refer to EMS-WI-085 for the Permit to Work process.

Table 1: Summary of WAH activities requiring mandatory permit

| Activity | Is Permit to Work required? |
| --- | --- |
| Step ladders (step-ups) | No |
| Lean to ladders | Yes |
| Mobile Elevating Working Platforms | Yes |
| Scaffold tower (Fixed & Mobile) | Yes |
| Work on an open (exposed) roof | Yes |
| Working near opening at ground level, or excavation | Yes |

All activities will require a Point of Work Risk Assessment to be completed before work is started.

For tasks of low risk and short duration, step ladders and step-ups can be a sensible option to complete the activity safely.  However, if the Competent Person deems that the risk is sufficient during completion of PoWRA for use of a stepladder or step-ups, a Permit to Work must be raised in order to complete the work safely.

- Minimum Standards for Safe Use of Working at Height Equipment

  - Use of ladders – General Principles

### Many accidents involving ladders happen during work that lasts less than 30 minutes, and most could have been avoided if work was carried out using alterative means.  Ladders are at the bottom of the WAH hierarchy because they do not prevent or control a fall.  However, if they are used by competent people and are regularly inspected and well maintained then their use may be justified; this is providing it is not reasonably practicable to use other work equipment which will prevent or control a fall (such as MEWPs, scaffold).

### As a general rule, the use of ladders as a working platform at height (particularly lean-to-ladders) should be avoided, and an alternative means used.  The following situations are not appropriate for using a ladder for a work at height task:

- When two hands are needed for task over long duration (see below)

- Where the equipment or materials used are bulky or awkward to handle

- When working at excessive height

- When work will be over long duration (more than 30 mins)

- Where ladder cannot be made secured or made stable

- Where ladder cannot be protected from vehicles

- When working in adverse weather conditions

### If the risk assessment cannot eliminate the use of a ladder, then the following steps must be taken by the Competent Person;

- Pre-use ladder check - this provides the opportunity to pick up any immediate/serious defects before they can cause an accident.

- The manufactures instructions are followed.

- Ladder is in date for inspection and is appropriately tagged* (scaff tag).

- Ladder properly positioned out of way of vehicles and not in way of other access routes.

- Maximum weight is not exceeded and only one person on ladder at any one time.

- Both hands to be kept free to grip the ladder when climbing or descending.

- Their capability to use ladder safely (i.e. physically able, do not suffer from vertigo).

All ladders under control of EMS department are stored and access to the equipment is strictly controlled to authorised personnel.   The majority of ladders on site are stored under the control of a lock and key. The key can be obtained via the Traka cabinet and access to the Traka cabinet will only be granted for personnel that have completed the appropriate training.  Other ladders that are located within biocontainment (restricted) areas on site are secured in areas where there is a physical security access control to area, such as the EMS ‘inside’ workshop.

*All ladders that do not have an in-date inspection tag must be reported to the EMS Supervisors so that inspection records can be checked and issue rectified.

  - Using stepladders and step-ups

    - General Principles

When using step-ladders or step-ups, the Competent Person must apply the following measures:

| - Use the right type of step ladder for task. - The ladder is not damaged and fully functional (see section 7.4.2). - The location to be worked at can be reached easily (see picture 1 showing good and poor practice) - The locking mechanism closed (fixed) - Materials needed for task (on ladder) will not compromise safe use of equipment. - 3 points of contact are maintained with the ladder. If two hands are needed, keep two feet on the same step and use body against the stepladder to maintain these 3 points of contact (see picture 1). | Picture 1: Safe use step ladders |
| --- | --- |

### Step ladders will not withstand any degree of side loading and can overturn very easily if not used correctly.

    - Pre-use checks for step- ladders and step-ups

The following pre-use checks must be made before using step ladders:

| Check the locking bars |  | Do not use the stepladder if locking bars are bent or the fixings are worn or damaged as the ladder could collapse. |
| --- | --- | --- |
| Check the feet |  | Do not use the stepladder if feet are missing, worn or damaged as the ladder could slip. |
| Check the stepladder or step-up platform |  | Do not use the stepladder if the platform is split or buckled, the stepladder could become unstable or collapse. |
| Check the step/treads |  | Do not use the stepladder if the steps/treads are contaminated as they could be slippery. |
| Check the steps for strength and stability |  | Do not use the stepladder if the fixings are loose as they could collapse. |
| Check the stiles |  | Do not use the stepladder if stiles are bent or damaged as the ladder could buckle or collapse. |

The toolbox talk training will cover these points, highlights the things to look out for when using this equipment safely.

  - Using lean-to ladders

    - General

Lean-to-ladders are rarely used for working at height tasks on site.  If the use of a lean-to-ladder is cannot be eliminated in the planning stage, a mandatory Permit-to-work must be raised.

When using lean-to-ladders, the Competent Person must apply the following measures:

| - The right type of ladder is used. - The ladder is not damaged and fully functional. - The surface to be used on is stable and even, and positioned approximately at an angle of 75 degrees (1 unit horizontally to 4 units vertically). - The ladder is secured to fixed structure at top, and at base (footed). - The work location can be reached easily (see picture 2) - The maximum weight (load) is not exceeded. - The materials needed for task (on ladder) will not compromise safe use of equipment. | Picture 2: Safe positioning of lean-to-ladder |
| --- | --- |

If the ladder cannot be suitably restrained at the based, a second competent person must be present and act as a safety sentry supporting the bottom of the ladder at the feet.

    - Pre-use check of lean-to ladders

The following pre-use checks must be made before using lean-to ladders:

| Stiles |  | Do not use the ladder if stiles are bent or split as the ladder could collapse. |
| --- | --- | --- |
| Check the feet |  | Do not use the ladder if feet are missing, worn or damaged as the ladder could slip. |
| Check the rungs |  | Do not use the ladder if rungs are bent, missing or loose as the ladder could become unstable. |

  - Use of Mobile Elevating Working Platforms

    - General Principles

Only personnel who hold a valid and in date IPAF or PAL card are authorised to use MEWPs (refer to Section 5 “Training Requirements”).

MEWPs are designed to provide a safe working platform for temporary work at height.

There are several different types of MEWPs with various rated capacities, working heights, and outreaches. The EMS department owns and maintained two vertical ‘scissor’ lift type MEWPs (one inside containment and one outside containment), and one self-propelled boom type MEWP (commonly referred to as “the cherry picker”).

Factors that may affect safe use of MEWPs include:

- Stability and condition of ground being used on can take weight of MEWP

- Presence of any overhead hazards such as power lines, other buildings, tree branches, etc

- Other traffic in area

- Weather conditions

- Materials to be used on MEWP (falling debris hazard)

- Human factors (duration of task, tiredness, vertigo, etc)

Before and during use of MEWPs, the Competent Person (operator) must apply the following safety measures:

- Equipment pre-use check carried out – these will vary for different types of MEWPs.

- Maximum weight is not exceeded and only one person on platform at any one time (unless otherwise stated in manufactures instructions noted on equipment).

- MEWP never used to transfer people from one level to another or for operator to exit basket at height.

- If using work restraint system, the anchor point outside the MEWP must NOT be used.

- All PPE (e.g. safety harness, lanyards, safety helmet) checked for any damage – any defects must be reported to equipment owner.

- Annual inspection of equipment in date.

### All MEWPs must be inspected every 6 months by appointed Contractor.  If test is out of date, the MEWP must not be used.

The instruction manual specific to each MEWP must be referred to and followed.  The equipment owner should be contacted for this information.

Arrangements for safe use of MEWPs

An appointed person must be present at ground level to supervise work undertaken by operator in MEWP.  They have the following responsibilities during the planned work:

- To notify the permit Authorising Person if there is an issue with planned work

- To notify a second competent operator in the event of MEWP fault, so that they can take control of the MEWP at ground level (refer to emergency situations 2-3)

- To call for a first aider if required (refer to emergency situation 2A).

- To notify an EMS mechanical technician who is familiar with the MEWP functionality, in the event of both normal and auxiliary function failure (refer to emergency situation 4)

Emergency situations

Normal and auxiliary control systems built into a MEWP will allow the operator to bring the platform of the machine safely to ground level under controlled conditions.  It is extremely unusual not to be able to lower the platform using these controls or for all of these systems to fail.

In the unlikely event that the MEWP operator is trapped at height, the operator must not attempt to climb out of the elevated working platform/cage/basket.

Below is an outline of potential situations that could arise, and the action to take if this situation occurs:

| Ref. | Emergency situation | Action to take |
| --- | --- | --- |
| 1 | Failure of upper control functions while elevated. | The operator will use the upper auxiliary controls to lower the platform safely. |
| 2 | Failure of the operator to be able to operate the MEWP functions while elevated due to one of the following reasons: - Operator incapacitated - Auxiliary functions fail to operate from upper control station | A second operator familiarised in the use of the ‘ground’ controls must lower the platform safely using the normal ground controls. In the event that the operator is incapacitated (A), the ground supervisor must follow site emergency procedures (i.e. call first aider). |
| 3 | Failure of normal ground controls | An appointed competent person familiarised in the use of the ‘ground’ controls must use the ground auxiliary controls to safely lower the platform. |
| Ref. | Emergency situation | Action to take |
| 4 | Failure of ALL normal and auxiliary lowering functions | Where all normal and auxiliary functions have failed, an EMS supervisor must be contacted via radio channel 1 by the person supervising the work on the ground. A mid-air rescue carried out by fire emergency services should be considered if normal operation cannot be achieved (see below). |

Any of the above situations must be logged as incidents via the Q-Pulse incident reporting system.

Mid-air rescue

A mid-air, platform to platform rescue should only be considered in exceptional circumstances and only after:

- All normal and auxiliary lowering procedures have been attempted and these are unable to lower the platform.

- The normal and auxiliary lowering systems have been inspected by an EMS mechanical technician familiar with the functions of the MEWP in use.

- If immediate risk exists to Health and Safety of the occupants from remaining in the basket until the MEWP normal operation can be resolved.

If a repair cannot be achieved that will allow the machine to be bought to ground level in a reasonable timeframe, the duty-BSO must be informed by the Authorising Person and the fire emergency services contacted to facilitate mid-air rescue.

Mid-air rescue procedure will be instructed and manged by trained and competent fire service personnel, and therefore all instructions provided to Pirbright site must be followed and adhered to.

  - Mobile Tower Scaffolds

    - General Principles

The type of tower to be used for working at height must be suitable for the planned work and erected and dismantled by trained and competent persons. Staff who erect and work on mobile scaffold towers must hold an in date PASMA licence (refer to Section 5 “Training Requirements”).

As a minimum, scaffolding must be inspected;

- After assembly or significant alteration

- Before use

- Following any circumstances which could jeopardise the safety of the installation such as adverse weather conditions.

Towers must be inspected as often as is necessary to ensure safety. A scaf-tag must be attached to scaffold as a record of the inspection.  If there is no tag, the scaffold must not be used.  The equipment owner must be contacted to arrange inspection of equipment by a competent and qualified person.

Factors that may affect safe use of mobile scaffold towers include:

- The height required for the work

- Access/Egress to/from working platform(s)

- Stability and condition of ground where it will be used

- Traffic in area

- Weather conditions, in particular windy environment

- Materials to be used on scaffolding (falling debris hazard)

- Human factors (duration of task, tiredness, vertigo, error in assembly etc)

The instruction manual specific to each mobile scaffold tower must be referred to and followed.  The equipment owner should be contacted for this information.

Before and during use of mobile scaffold towers, the Competent Person (operator) must apply the following measures:

- Ensure the scaffold suitably secured (stabilisers, to fixed structure).

- Materials and tools to be used on working platform are secured appropriately (avoid falling objects hazard).   Materials and tools to be removed when platform not in use.

- Signage and barriers placed around scaffolding to make pedestrians/other persons aware of work and potential hazards.

- Wheels turned outwards and brakes ‘on’, locked and kept locked when in use. Equipment never moved when people or materials present on equipment.

- Maximum weight is not exceeded (as per manufactures instructions).

- Load (such as bricks) distributed on the working platform to minimise risk of tower tipping over.

- Safe means of access/egress to/from the working platform is used.  Equipment not climbed on outside of structure.

- Guardrails and toeboards in place (if risk of fall over 2 meters) and not damaged.

- PPE (e.g. safety harness, lanyards, helmet) checked for any damage – any defects must be reported to equipment owner.

- If working platform is slippery (e.g. icy / greasy), use material to correct the hazard, such as placing sand or salt on surface.

  - Fixed Scaffolding

    - General Principles

Fixed scaffolding used on site is erected and dismantled by CISRS (Construction Industry Scaffolders Record Scheme) qualified contractors to recognised standard configuration e.g.  NASC (National Access & Scaffolding Confederation & Construction). Personnel must not interfere or alter fixed scaffolding if they are not qualified or competent to do so.

Many of the same principles for mobile scaffolding apply to safe use of fixed scaffolding, including;

- Check that a valid in-date scaf tag is attached to equipment before use.  The scaffolding must be inspected by qualified person at least every 7 days.

- Materials and tools to be used on working platform are secured appropriately (avoid falling objects hazard).   Materials and tools to be removed when platform not in use.

- Load (such as bricks) distributed on the working platform to minimise risk of scaffold collapsing.

- Signage and barriers placed around scaffolding to make pedestrians/other persons aware of work and potential hazards.

- Safe means of access/egress to/from the working platform is used.  Equipment not climbed on outside of structure.  If access route is via secured ladder, the access opening in the working platform must be as small as possible, and must be covered when not in use.

- Guardrails and toeboards in place (if risk of fall over 2 meters) and not damaged.

- PPE (e.g. safety harness, lanyards, helmet) checked for any damage – any defects must be reported to equipment owner.

  - Reporting equipment issues

Any equipment that is damaged or deemed unsafe must not be used.  The equipment must be marked with appropriate “do not use” signage and the equipment taken out of use until it can be repaired or disposal arranged.

Every operative competent to use working at height equipment / machinery has the responsibility to report and highlight equipment that is not safe to use.

Any defects, missing labels/tags, out of date inspection tags must be reported as soon as possible after the issue has been identified.  Issues observed must be reported to the following personnel:

- Lean-to-ladders or step ladders – EMS Mechnical Supervisor

- Mobile Tower Scaffolding – EMS Mechnical Supervisor

- Fixed Scaffolds – EMS Mechnical Supervisor

- MEWPs – Specialist Equipment Owner (Infrastructure)

FLOWCHART – Safe System of Work for WAH Activities

INSTRUCTIONS – Safe System of Work for WAH Activities

| Step by step actions <The numbered Steps correspond with the Flowchart in 6.3> |  |
| --- | --- |
| 1 | Review Scope of Work. The nature and scope of the work / project involved will be reviewed at this stage, using hazards listed in PoWRA booklet. Higher risk hazards will be identified, e.g. work in confined space. |
| 2 | Will task(s) involve working at height? If ‘YES’ go to Step 3. If ‘NO’ go to Step 6. |
| 3 | Will you only need a step-ladder to complete the task working at height? If ‘NO’ go to Step 4. If ‘YES’ go to Step 5. |
| 4 | Will the work planned involve and of the following; - Use of Scaffolding (mobile or fixed)? - Use of a Mobile Elevating Working Platform (MEWP)? - Lean-to-ladder? If the answer is ‘YES’ to any of the above, go to Step 11. If ‘NO’ go to Step 5. |
| 5 | Final Assessment of hazards at work location - PoWRA. EMS staff must go to work location and complete final hazard review of area before starting planned work. This is to ensure no additional hazards are present which were not accounted for in original RAMS. Provide Isolation form to BMS staff if required. SAPO4 Restricted Areas: Visual PoWRA made in lab/animal areas. Complete PoWRA in ‘lower risk’ spaces. Scan and email PoWRA form out of containment. |
| 6 | Any additional hazards identified which require Permit to be raised? If ‘YES’ go to Step 11. If ‘NO’ go to Step 7. |
| 7 | Any other additional hazards identified at point of work? If ‘YES’ go to Step 8. If ‘NO’ go to Step 9. |
| 8 | Ensure additional controls in place and recorded. CP implements and records extra controls needed on PoWRA. Same PoWRA form can be used for jobs that extend over a period of days. |
| 9 | Work started. Competent person(s) starts work under SSoW process, but not under Permit. |
| 10 | Completion of work – return of PoWRA After work has been completed, Competent Person must leave work area in safe condition and return completed PoWRA to Permit Station for archive. Documentation will be subject to EMS internal audit program, external audits and for incident investigation (if required). |
| 11 | Prepare appropriate Permit RAMS / isolation documentation (as required) and request Permit following EMS-WI-085. |

- REFERENCES & RELATED DOCUMENTS

  - Template version EMS-FORM-045v4

  - QA-SOP-29 Training at The Pirbright Institute

  - EMS-WI-085 Permit to Work work instruction

  - Working at Height RA000129 (Oshens Hazard Risk Assessment)

  - HSE Guidance document: INDG401

  - Internal Training: Toolbox Talk: Working at Height

  - Absorb Online Course: Working at Height

  - International Powered Access Federation – Guidance on emergency rescue Re: UKBM 07/14-002