	[image: image2.png]



	Tup Bridge Depot and Marl Pits Garage 




[image: image1.png]



OUTLINE SPECIFICATION 
TO UNDERTAKE BUILDING WORKS TO 
PROVIDE 2 NO. SECURE COMPOUNDS FOR

PARKING AND STORAGE OF THE PARKS

DEPARTMENT VEHICLES AND EQUIPMENT AT:-

TUP BRIDGE DEPOT &
MARL PITS GARAGE AREA RAWTENSTALL
[image: image2.png]
REQUIREMENTS
1.1 This proposal is to invite tenders to undertake building work at these two locations to accommodate the vehicles and machinery to be relocated from Whittaker Park.
1.2 This outline specification sets out the broad requirements and provides sufficient information for a tender submission.
2. THE PROPOSED WORKS
2.1 Tup Bridge Depot
2.1.1 This relates to the area around the ground floor of Tup Bridge Depot  and is accessed from Rawtenstall  Market car park.
2.1.2 Remove all earth and debris, including existing kerbs A-C D-F and rake back the earth E-F to 1:2 gradient.
Excavate from the existing car park level 320 mm.

To the area C-D-E-F supply and installed concrete kerb to match existing in haunched concrete.
2.1.3 To area B-C-D-E supply and lay road plannings and roll to maximum consolidation. Provide 30mm wide concrete edging , flush finish with the tarmac at B-E . 
2.1.4 To Area A-B-E-F supply Terram sheeting to bottom of the area then supply and lay 200 mm Type 1 crush and run stone and roll to maximum consolidation.  Provide then 80mm thick base course, well rolled of dense bitumen macadam and then a 40mm thick wearing course of 10mm dense SMA gritstone (hardstone) all well rolled.
Bitumen overbond joints all joints between new and old tarmac.

2.1.5 Provide 3m high (Ground level )  galvanised steel palisade fencing to N-B-C-D-E-F and G-H-K-J-I and K-L .

At locations C and K fencing to be angled to cover the drop at C, and to the roof slope at K.

2.1.6 At N-B-C and I-J new fence to be located at high level on the cemetery side of the wall.

2.1.7 At F-G 2 No. 2.5m wide palisade gates to be provided with necessary locking facility to use standard padlock.

2.1.8 All fencing posts to be suitably concreated in.

2.1.9 At M provide galvanised steel staircase to upper ground level, and a new external access door and frame.  To include all cutting out and making good.  Final location to be agreed on site.

2.1.10 Include a PC sum of £1,500 for location of movement detector.  Nominated supplier details to follow.

2.1.11 Include a PC sum of £1,500 for additional external lighting.

2.1.12 Include a PC sum of £1,500 for servicing and alterations to the intruder alarm system – details to follow.
2.1.13 Include a PC sum of £1,000 for new road markings to the car park.

Total ……………………

2.2 Marl Pits Garage Area
2.2.1 For area C-D-E-F-G remove all soil, debris, foliage and walling and cart away.

For area C-D-E-F-G-H excavate to 200mm and provide road plannings and roll to maximum consolidation.

2.2.2 For area E-F grade back the soil embankment to c.1:2 gradient.

2.2.3 For C-D-E-F-G provide 4m high steel galvanised palisade fencing.  All posts to be appropriately concreted into the ground.

2.2.4 For A-B provide 3m high steel palisade fencing double gate including all locking/mechanism for a padlock.

2.2.5 Include for the re-siting of steel container units x 3

2.2.6 Include a PC sum of £2,000 for additional high level lighting.

Total ……………………

Tendered sum total 

2.1     

£

2.2 

£

Contingency £2,500
Grand total  £

Spring PCC Limited


213 Chestergate


Stockport


SK3 0AN











JK0110/KD/EJ








November 2019 
JK110/KD/EJ
Page 3
November 2019 

