STATEMENT OF REQUIREMENT FOR THE PROVISION OF TEMPORARY STRENGTH & CONDITIONING SHELTERS (SCS) IN SUPPORT OF FIELD ARMY’S OPTIMISING HUMAN PERFORMANCE (OHP) PROGRAMME

Scope

What is required: 1. Install SCS, complete with the relevant strength and conditioning equipment to military sites* as a proof of concept and a prerequisite for further uplifts in capability across the remaining Fd Army sites. - These sites are: REDACTED - Each SCS should meet the following requirements: - The workout area should have a shelter that allows training with protection from the rain and wind. - The training area should be a minimum of 30m x 20m. - Minimum height of 2.6m above all the floor area. - It should have a solar-powered LED lighting solution. The SCS cannot be connected to any mains power. - It should come with a maintenance contract covering a period of 5 years. - It should be delivered and set up at location(s) identified by the Authority. - It should come with all relevant and applicable Health and Safety Certification. - It should have strength and conditioning flooring for the whole training area. This should consist of Rubber impact tiles (minimum of 30mm deep) capable of managing the impact force from repeated weight impacts across the complete floor area, - It should have an expected lifespan of 15 years with a 5-year warranty. - It should be closed at each end with garage doors and a side door for access at both ends. - It should have internal access to the ISO container for either equipment storage or set up, which can also be secured. - It must comply with BS EN 1991-1-3:2003 (or equivalent) for snow loading. - It must comply with BS EN 1991-1-4:2005 (or equivalent) for wind loading. - Galvanised structural steel must comply with BS4360 or EN 100025 (or equivalent). - It must be fire rated to British or European standards. - Each SCS should be filled with the following equipment: - 16 Olympic lifting bars and collars. - 8 x Full sets of Olympic discs & suitable storage trays. - Hypertrophy rig / wings to facilitate 8 people conducting heaves at any one time. - 4 x Full sets of various weighted dumbbells & holders. - 8 x Free weight training benches. - 4 x Full sets of various weighted Kettle-bells & holders. - 8 x Full sets of various resistant bands. - 4 x Full sets of various weighted Training Bags & holders. - 4 x Concept 2 Rowers. - 4 x Ski-Ergs. - 4 x Air-Bikes. - 4 Sets of various weighted Med-balls & holders. - 8 x Plyo jump-boxes. - 8 x mobile squat stations either built into the rig via adjustable J-Pegs or stand-alone squat stands. - 2-year warranty on all equipment. Security/Access 5. Information. All information provided to tenders should be considered OFFICIAL and should not be used for other activities and should be destroyed after the completion of the project. All information should be considered sensitive and should not be divulged to any other persons or organisation. 6. Contractors should be able to set up the equipment with limited supervision on each of the sites. Security cleared staff would be beneficial but not essential. Access in to any chosen location will be arranged prior to the confirmed delivery date. Sustainability Plan. 7. The initial warranty for external fittings and welding will be 5 years, equipment shall be 2 years. Government Furnished Supplies 8. The contractor will be responsible for the entire installation including all Health & Safety aspects (risk assessments, insurance, workforce insurance etc). * These site details should in no way be construed as a commitment by the Authority to procure SCS for each of the sites listed. The purpose of this procurement exercise is to establish a Framework under which the Authority will procure as many or as few SCS as it deems necessary at its sole discretion. All potential delivery sites are in the UK (inc Northern Ireland). NB The Authority has an aspiration that the first SCS be installed no later than 31 January 2021. Whilst this is neither evaluation criteria, nor a pre-requisite to contract award, bidders should inform the Authority if this date is unachievable..