Part 2 – ITQ – Specification for Car Parks Surfacing and Minor Civils Works

PART TWO
SPECIFICATION: Car Park Surfacing and Minor Civils Works
Kettering Borough Council would like to invite you to quote for the following works:

Car Park Surfacing and Minor Civils Works to following Kettering’s Pay and Display car parks:

•
Commercial Road

•
London Road

•
Municipal Offices

•
Wadcroft

•
School Lane

•
Queen Street
•
Stoneyfield, Mawsley – Footway (surfacing works). 

Please see the enclosed in part 4 - ITQ, the following documents:

•
Bill of Quantities for each site.

•
Site Plans

•
Utility Plans (These will be provided to the successful tenderer)
•
Notification of Known Hazards.
The contractor’s price is to be a ‘lump sum’ that will be agreed to the terms of the I.C.C. Minor Works Contract with the price deemed to include all operations required to carry out the works whether specifically itemised or not. All contractors are required to visit the site and acquaint themselves fully with the site conditions and scope of the works required. The sites may be restricted at times due to each car parks, parking demands. The contractor may be required to work on Saturday or Sunday. 
For Specifications and Standard follow the link:

http://www.standardsforhighways.co.uk/ha/standards/dmrb/vol11/index.htm
Paco Patch - Ironwork Reinstatement 
Paco patch is a polymer-modified mastic asphalt used primarily to permanently reinstate perished surfacing around manholes and other iron works. 

Paco patch requires no compaction therefore no damage is caused to the chamber construction beneath the iron works.  Paco patch comes with a 5 year material warranty and can be trafficked within an hour of completion.  Paco patch is also environmentally friendly through the use of bricks made from recycled aggregates.  Paco patch can be laid any time during the day or night as there is no need to collect materials from quarries or batching plants. 

The revolutionary system comprises a hot applied asphalt grout and Pre-coated or dry chippings are applied to the finished surface to provide a non-skid surfacing complying with UK legislative specifications. 

Pacopatch Reinstatement System 
Pacopatch is a polymer modified mastic asphalt permanent reinstatement system primarily designed for use around manholes and other street ironworks.  

The process is completed by backfilling with additional grout to produce a voidless, impermeable, monolithic and permanent reinstatement which can be completed in a variety of surface finishes. 

Pacopatch requires no compacting, is applied by hand float and enables applicators to economically patch even very small, irregular areas, to a consistent, voidless standard. 

Specification 
A paving grade mastic asphalt, manufactured in accordance with the general requirements of BS 1447:1988 with regard to aggregate grading and binder content. Pacogrout® utilises a  purpose designed polymer modified asphaltic cement manufactured under refinery controlled conditions to a BS EN ISO 9001:2008 Quality System. The coarse aggregate content is in accordance with Clause 5.3.1. of BS 1447. 

Properties of asphaltic cement:  

Penetration @ 25°C 
15 - 25 dmm 

Solubility in trichloroethylene 92% min.  

Pacopatch Application Method 
1. Mark out the area to be excavated. 

2. All perimeter edges of area to be reinstated should be diamond saw cut to ensure a clean vertical face is provided. 
3. Break out the existing material and remove the spoil from the excavation to a minimum depth of gullies 60mm and manholes 100mm. 
4. All faces of the excavation should be coated with an approved jointing compound.
5. Operative (1) should carefully transfer the molten Pacogrout® from the mixer/cauldron into an asphalt bucket and pour into the excavation to an average depth of approximately 20-25mm (see item 11). 

6. If the surface is to receive pre-coated chippings, a head space of 1-2mm should be left to allow for displacement of the Pacogrout® on application of the chippings.  The chippings should be pressed into the Pacogrout® with a wooden float whilst the material is still in a plastic state. 

7. Allow the reinstatement to cool to ambient temperature before opening to traffic.   
8.
Pacogrout® should not be heated to a temperature exceeding 180oC and particular care should be taken to avoid any localised overheating during the melting process. 

Site Access:
Commercial Road – 1no. access/egress via Commercial Road, Kettering, NN16 8DQ

London Road – 1no. separate access/egress London Road, Kettering, NN15 6DT

Municipal Offices – Located within the London Road, Car Park above

Wadcroft – 1no. access and 2no. egress points on Commercial Road, Kettering, NN16 8DQ

School Lane – 1no. access and 1 no. egress Tordoff Place back onto School lane, NN16 0DF

Queen Street – 4 no. access/egress via Queen Street, NN16 0BZ
Stoneyfield, Mawsley – Footway NN14 1SY. Restricted access.  

If a contractor considers the documentation is deficient in any respect, confirmation should be sought from the named officer on the letter head within the following dates. (Thursday 17th December and Friday 24th December 2020)

On accepting the quotation, the following information will be required prior to starting on site.

•            Programme or works after liaison with Client 

•
Method Statements

•
Risk Assessments

•
Public Liability, Employers Liability and any other relevant insurance documents required.

•
The above details for any subcontractors that you intend to employ.
If you would like to discuss any of the works in the specification, please do not hesitate to contact me Kanji Chudasama Email: kanjichudasama@kettering.gov.uk , Tel: 01536534491.

1

