|  | Datchet Parish Council | 1 Allen Way Datchet Berkshire SL3 9HR |
| --- | --- | --- |

Web: www.datchetparishcouncil.gov.uk				       		Clerk to the Council

Email: Clerk@datchetparishcouncil.gov.uk

Tel: 01753 773499

Subject: Recreation Ground Pathways – Rectification

Specification for your company to provide a quotation for rectification of the Recreation ground path, as set out in the attached Drawing No Path 01.

Area A – (Path A Blue)

Works to take place on approximately half of the total length of path A. Fill small cracks within pathway shown in blue. Remaining section of path A shown grey to remain.

Area B – (Path B Red)

Stage 1 - Break out and excavate old tarmac, edging kerbs and soil to a depth of 300mm and allow to remove all surplus excavated materials and dispose off site.

Stage 2 – Supply and lay GEO Tex membrane over the whole specified area to stabilise and strengthen the ground. Supply and lay new edging kerbs to both side of new pathways. All kerbs are to be bedded and backed up with concrete of a suitable and appropriate mix, which will provide a retainer when the new fill material has been laid.

Stage 3 – Supply and lay Type 1 crushed concrete to a depth of 220mm to shape and level area, to be rolled and compacted with a heavy road roller to ensure a sound foundation.

Stage 4 - Supply and lay 20mm DBM base tarmac to a depth of 50mm to shape pathways, to be rolled and compacted with a heavy road roller.

Stage 5 -Supply and lay 6mm black DBM wearing course tarmac to a depth of 30mm for a finish coat and to be rolled and compacted with a heavy road roller to provide the required surface finish.

AREA A (BLUE) – Has been provisionally assessed as crack filling only.

AREA B (RED) – Has been provisionally assessed as 225 square meters, subject to measurements on completion.

Should you have any queries or questions about our specification then contact the Datchet Parish Council on the email above.