ITT No ACT/04518

DCTT FEDERATED NETWORK RAF COSFORD WAN

Table of Contents

Dedicated Internet Access	1

1.0 Introduction	3

2.0 Dedicated Internet access at a glance	4

3.0 The service in detail	6

3.1 Logical Overview	6

3.2 Feature summary table	8

3.3 Global IP Backbone (AS1273)	8

3.4 Access circuit & service bandwidths	10

3.5 Providing service over existing MSP access circuits	11

3.6 Managed customer site router	12

3.7 Optional internet features	12

3.8 Optional Security Features	13

4.0 Technical information	17

4.1 Routing	17

4.1 Managed customer routers	18

4.2 Management traffic	18

4.3 Wires only services	18

4.4 Resilient services	19

4.5 Burstable bandwidth	19

4.6 Providing service over existing MSP access circuits	20

4.7 Domain name services	21

4.8 Use of AS1273 & AS2529	21

4.9 BGP Communities	22

5.0 Service support	23

5.1 Proactive monitoring	23

5.2 Incident & change management	24

5.3 Professional services	24

5.4 Online portal	25

6.0 Pricing & billing	26

# 1.0 Introduction

Vodafone are pleased to respond to the ITT ACT 04518 suggesting the Dedicated Internet Access (DIA) service as being most suitable, and include in this document a description of the standard and optional features, technical information, service and support, together with outline commercial terms.

# 2.0 Dedicated Internet access at a glance

Today’s web applications are essential to business operation and profitability. A poor performing web site or cloud/web application, can quickly impact bottom line profitability, brand reputation and customer dissatisfaction. However, many organisations suffer slow performing web applications and internet browsing. Often they have outgrown their existing internet service. Business internet usage is growing at 20% per annum with increased use video, conferencing, cloud and collaboration services. As organisations begin sending increasingly more traffic, rather than simply downloading, they inevitably require a faster, better performing service. Yet performance and speed are not the only concern, internet security is of paramount importance. Denial of service attacks and cybercrime are increasing, employee mobility is rising. Web sites, content, email and corporate boundaries all need protecting. The challenge facing the IT team is how to address these concerns whilst at the same time delivering lower costs and efficiencies.

Introducing our Dedicated Internet Access Service

A fast, and reliable, dedicated internet access service that connects the customer site directly to our Global IP Backbone (AS1273). Dedicated Internet Access comes with a range of optional internet and security features together with an excellent service experience.

Fast & Reliable

Dedicated Internet Access provides fast and reliable access to the Internet over a dedicated access circuit provided at a range of fixed and burstable bandwidths from 2Mbps to 1Gbps. Relative to business broadband; the service comes with the same upstream and downstream bandwidths, making it ideal for web site connectivity, remote site access & business internet use. Customer sites are connected directly to our leading Global IP Backbone via our extensive UK access network and multi-service platform. Major content distribution networks and millions of internet end points (eyeballs) are directly connected to our Global Backbone which extends across UK, EU, Asia and US and has near 1Terabit of peering with tier 1 ISPs. This ensures faster internet communications with fewer data hops and greater reliability.  What’s more, we offer stringent latency guarantees between our backbone nodes across EU, Asia & US.

Optional Internet & Security Features

We provide a range of security options including: Distributed Denial of Service (DDOS) protection which offers reactive and proactive denial of service mitigation and black-holing options; manage firewall, email and internet proxy/filtering services. A range of redundancy options are supported including dual homing with your second ISP and resilient services which provide protection against last mile connectivity, access POP or internet router failure. We also offer optional managed customer site routers, managed domain name hosting which includes SMTP mail, DNS resolution, primary and secondary DNS. Our service supports IPv6 addressing as well as non-portable (Provider Aggregatable (PA)) IPv4 addressing.

An Excellent Service Experience & Overall Best Value

Customers benefit from an excellent service experience including 30-45 working day lead times for new provisions, 24/7 critical event proactive monitoring & incident management as well as access to our online self-service portal. The MyVodafone portal provides an alternative customer contact media providing access to order tracking, trouble ticketing, e-billing and performance reporting where ordered.

Our Dedicated Internet Access service delivers overall best value. By providing access directly to our Global Backbone we offer fast and reliable access to content and eyeballs reducing internet AS hoping and points of failure. Our optional security and internet features can improve your overall margins. 95% of UK customers are connected using cost effective, higher speed, next generation Ethernet technology. Existing customers, including those with sites in Europe, for whom we provide IPVPN or Ethernet Wireline network services over our Multi-Service Platform, can access the Internet Access service for a lower price. This assumes use of spare bandwidth capacity on last mile Ethernet access circuits supporting the IPVPN WAN or Ethernet Wireline products.  What’s more, for new customers we offer free of charge installation*.

- Subject to survey upon order placement, excludes any excess construction or ancillary charges, 3 year and longer term contracts.

# 3.0 The service in detail

## 3.1 Logical Overview

Internet Access connects the customer site directly to the Internet, via our Global IP Backbone, over a dedicated access 
circuit. The service provides ‘always on’, fixed symmetric bandwidths which can be committed, or burstable, ranging from 
2Mbps to 1Gbps.

The main service components are:

- Access Circuit – this connects the Customer site to the multi-service platform and onwards to an Internet Edge Router on our IP Backbone.

- Service/Port Bandwidth – internet service bandwidth is provisioned over the Access Circuit. This can be throttled to a range of committed, or committed plus burstable, bandwidths allowing the customer to pay only for the service bandwidth which they need.

- Service/Port Bandwidth –internet service bandwidth is provisioned over the Access Circuit. This can be throttled to a range of committed, or committed plus burstable, bandwidths allowing the customer to pay only for the service bandwidth which they need.

- Optional Customer Site Routers – where ordered these will be installed, configured, managed (remotely for changes where possible) and maintained by Vodafone.

- Optional Internet Features - where ordered these include managed Domain Name Services, IP Addresses, and ASN Numbers

- Optional Security Features – where ordered with the Internet Access service, these include DDOS protection, internet proxy services, email security, managed firewalls and resilient service options.

Service support including;

- Proactive monitoring of the Internet service up to the service demarcation point for hard down, alarms and events which map to severity 0-2 faults.

- 24/7 incident management support all year round.

- Service change requests

- Online ‘MyVodafone’ portal

- Comprehensive service level agreement.

## 3.2 Feature summary table

| Feature | Sub- Feature | Standard | Optional |
| --- | --- | --- | --- |
| Access Circuit | 2, 4, 6, 8,10,100,1000Mbps symmetric bandwidths |  |  |
| Service/Port BW | Committed bandwidth options from 2Mbps to 1000Mbps |  |  |
|  | Committed bandwidth plus burst options from 2Mbps burst to 4Mbps to 500Mbps burst to 1000Mbps |  |  |
| Customer Site Routers | Range of Cisco ISRG2 routers come fully installed & configured with remote change management and maintenance |  |  |
| Internet Features | Domain Name Services: transfers/registrations, authoritive DNS, DNS caching, SMTP mail |  |  |
|  | IP Address Blocks: portable & non-transferable IPv4 & IPv6 addresses; IPv4 for Provider Aggregatable internet addressing only IPv6 for Provider Aggregatable and Provider Independent internet addressing |  |  |
|  | Autonomous System Network numbers: registration and routing |  |  |
|  | Performance Reporting |  |  |
| Security Features | DDOS Protection: reactive and proactive options using shared or dedicated cleaning zones |  |  |
|  | Internet Proxy Service |  |  |
|  | Managed Firewall |  |  |
|  | E-Mail Security: sold per mail box |  |  |
|  | Dual Homing & Resilient Services |  |  |
| Service Support | Proactive monitoring for unavailability & 24/7 incident management (requires BGP routing to be used) |  |  |
|  | MyVodafone Self Service Portal |  |  |
|  | Standard change management services |  |  |
| Technical Features | Routing: static, BGP with default route, BGP with full routing table |  |  |

## 3.3 Global IP Backbone (AS1273)

Internet Access is provided over Vodafone Global IP Backbone known as AS1273*. With over 60 points of presence, our AS1273 backbone provides high capacity connectivity at 10s of Gigabits, with assured quality of service, between 17 countries across Asia, Europe and US. Each country is connected, with a minimum of two points of presence, back to our fully meshed optical core using two diverse routes. With multiple protection paths and diverse routing options, AS1273 is engineered to 5 x 9s availability and redundancy at all levels.  In the event of a failure, the backbone automatically deploys the most appropriate protection path, restoring traffic across the network in milliseconds using RSVP fast re-route protocol.

AS1273 handles over 900Gbps of customer traffic at any point in time, and has near 1Terabit of private peering with other leading Tier 1 ISPs and content distribution networks and connectivity to millions of eyeballs where service providers purchase wholesale IP transit from Vodafone. All these factors help to ensures fast and reliable access to content and eyeballs.

We target average Network Latency (measured round trip) and Packet Loss between Primary Global IP Backbone nodes, located in specific geographic regions, of no more than the target monthly average amounts shown in the table below.

| Monthly average Network Latency and Packet Loss targets |  |  |
| --- | --- | --- |
| Region | Parameters | Target |
| Europe | Network Latency (ms) | 35 |
|  | Packet Loss (%) | 0.2 |
| Europe UK | Network Latency (ms) | 20 |
|  | Packet Loss (%) | 0.2 |
| Europe North America | Network Latency (ms) | 90 |
|  | Packet Loss (%) | 0.2 |
| Europe Asia | Network Latency (ms) | 310 |
|  | Packet Loss (%) | 0.2 |
| UK | Network Latency (ms) | 35 |
|  | Packet Loss (%) | 0.2 |
| UK North America | Network Latency (ms) | 90 |
|  | Packet Loss (%) | 0.2 |
| UK Asia | Network Latency (ms) | 210 |
|  | Packet Loss (%) | 0.2 |
| North America | Network Latency (ms) | 65 |
|  | Packet Loss (%) | 0.2 |
| North America Asia | Network Latency (ms) | 165 |
|  | Packet Loss (%) | 0.2 |
| Asia | Network Latency (ms) | 90 |
|  | Packet Loss (%) | 0.2 |

The Network Latency and Packet Loss targets, detailed above, are calculated by taking an average, of the average monthly Latency and Packet Loss performance, for all routes between Primary IP Backbone nodes within the Region or between Regions as applicable.

*Regional aggregation in the UK is typically provided via Vodafone AS2529 that is connected upstream to AS1273.

## 3.4 Access circuit & service bandwidths

The table below outlines the different access circuit and service bandwidth options. New Access Circuits, for Internet Access, are only available in mainland UK:

| Access Circuit BW | Committed Service BWs | Committed + Burst Service BWs | Access Technology | Interface (customer provided router) |
| --- | --- | --- | --- | --- |
| 2Mbps | 2Mbps | N/A | Ethernet or Leased Line | RJ45 or X21 |
| 4Mbps | 2, 4Mbps | 2Mbps burst to 4Mbps | Ethernet | RJ45 |
| 6Mbps | 2, 4, 6Mbps | N/A | Ethernet | RJ45 |
| 8Mbps | 2, 4, 6, 8Mbps | N/A | Ethernet | RJ45 |
| 10Mbps | 2, 4, 6, 8, 10Mbps | 5Mbps burst to 10Mbps | Ethernet | RJ45 |
| 15Mbps | 2, 4, 6, 8, 10, 15Mbps | N/A | Ethernet | RJ45 |
| 20Mbps | 2, 4, 6, 8, 10, 15, 20Mbps | 10Mbps burst to 20Mbps | Ethernet | RJ45 |
| 25Mbps | 2, 4, 6, 8, 10, 15, 20, 25Mbps | N/A | Ethernet | RJ45 |
| 30Mbps | 2, 4, 6, 8, 10, 15, 20, 25, 30Mbps | N/A | Ethernet | RJ45 |
| 35Mbps | 2, 4, 6, 8, 10, 15, 20, 25, 30, 35Mbps | N/A | Ethernet | RJ45 |
| 100Mbps | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100Mbps | 10Mbps burst to 20Mbps 20Mbps burst to 40Mbps 50Mbps burst to 100Mbps | Ethernet | RJ45 |
| 1000Mbps | 100, 200, 250, 300, 400, 500, 600, 700, 800, 900, 1000Mbps | 100Mbps burst to 200Mbps 200Mbps burst to 400Mbps 500Mbps burst to 1000Mbps | Ethernet | 1000BaseSx/Lx |

1) Ethernet may be provided over copper, using Ethernet First Mile technology, or Fibre.

2) Where new access circuits are required, then only the ‘Committed’ and ‘Committed+Burst’ service bandwidths shown above are supported over the respective access circuit bandwidths. Where MSP access circuits already exist at the customer site, and are provided over Fibre Ethernet access technology, then any of the ‘Committed’ and ‘Committed+Burst’ service bandwidths will be supported where the existing access circuit has sufficient spare capacity.

3) Service bandwidths are inclusive of all protocol & access network transmission overheads.

4) Unless agreed otherwise, where managed CPE is required, and considering your bandwidth and feature requirements, we may provide your service at the bandwidths ordered over the access technology of our choice. Where a ‘wires only’ service is required, the technologies and interfaces relevant to each site will be those detailed on the order form and/or agreed during service delivery.

## 3.5 Providing service over existing MSP access circuits

Dedicated Internet Access may be provided, for a lower price, alongside other Vodafone IPVPN or Ethernet Wireline(EWL) network services over the same fibre Ethernet Multi-Service Access Bearer (MSAB).  In this instance, a separate access circuit is not required. The existing MSAB must already connect the customer site directly to our Multi-Service Platform and will currently be used with an IPVPN QoS, W-IPVPN or EWL services:

Please note that it is not possible to provide new access circuits for Dedicated Internet Access outside of the UK. However, where Dedicated Internet Access is provided over an existing MSAB, the customer site may be located in the UK or European Union. Other geographies are not permitted.  Any of the committed, or committed plus burst, service bandwidths, detailed in section 3.4 will be supported over the MSAB where spare capacity exists, to support these bandwidths without introducing bandwidth contention, and the NTE port supports these bandwidths (see section 4 for further details).

Where the existing Network service, provided over the MSAB connecting the customer site, is a Managed IPVPN WAN service then a separate managed customer site router is required to be purchased with the Internet service. The IPVPN WAN managed customer site router cannot be used as this would breach our government accredited security regulations. In all other instances, 
it is not possible to purchase a Managed Customer Site Router for the Internet Access Service where it is provided over an existing MSAB. The table below provides more detail:

| Existing Service on MSAB | Internet Access Conditions Vodafone Managed Customer Site Router Supported |
| --- | --- |
| IPVPN (Wires Only) |  |
| W-IPVPN (Wires Only) |  |
| IPVPN (with our Worldwide Managed Customer Site Router) | (Mandatory) |
| W-IPVPN (with our Worldwide Managed Customer Site Router) | (Mandatory) |

Please take note that if the existing network service, such as an IPVPN WAN, is requested to be ceased at the site then the Dedicated Internet Access service is also required to be ceased.

## 3.6 Managed customer site router

Where ordered, we will install and configure Cisco ISRG2 router equipment in accordance with your service order.  We will manage and maintain the router in life as follows:

- Undertake service change requests, including soft and minor changes which will be remotely configured for example changes to routing, IP addressing or service bandwidths.

- Proactive monitoring, all year round, for critical hard down, unavailability, alarms

- Provide 24/7 fault management, including remote fault diagnosis and fault resolution where possible, and on-site repair or replacement of the Managed Router hardware or software, if necessary, to resolve faults.

## 3.7 Optional internet features

### 3.7.1 Domain name services

Vodafone Domain Name Service (DNS) provides a packaged registration, transfer and hosting service for one or more business, or government, domains. We support a wide range of domain names internationally as well as within the UK subject to terms and conditions. The most popular UK domain names are as follows: .com, .co.uk, .org.uk, .net, .gov.uk, ac.uk.  We provide a high availability domain name hosting service, ensuring that the customer domain names are visible at all times. We provide the following feature options within our Domain Name Service tariff:

- DNS resolution – our DNS resolution caching servers provide a “directory enquiries” type function, for the Internet mapping domain names to individual IP addresses.

- SMTP mail - each name hosted may have an unlimited number of associated computers (and sub-domains) listed as Address (A) records.  Using mail exchange (MX) records in conjunction with corresponding mail systems, our DNS can be used to send and receive e-mail. We can support multiple MX records for email delivery.

- Primary and secondary DNS - we can provide a primary and/or secondary authoritive domain name server service in line with your requirements.

### 3.7.2 Public IP Addresses & ASNS

We are able to assign, IPv4 public IP addresses for PA addressing and IPv6 for PA and PI addressing.IPv6 addresses are provided on a reasonable endeavours SLA basis.

If you require /29 and larger IPv4 address blocks then RIPE approval will be required:

| IP Address Block | Number of IP Addresses | Number of Useable Addresses | RIPE Approval Required |
| --- | --- | --- | --- |
| /30 | 4 | 2 | N |
| /29 | 8 | 6 | Y |
| /28 | 16 | 14 | Y |
| /27 | 32 | 30 | Y |
| /26 | 64 | 62 | Y |
| Larger address blocks may be available upon request |  |  |  |

Where IPv6 address blocks are required, we will assign a /48 address block. This will provide you with ample IP addresses (1,208,925,819,614,629,174,706,176 IP addresses).

A static, public IP address may not always be necessary. Below are some common applications where addresses will be required:

- VPN applications, many of which do not run correctly over NAT or require specialist NAT equipment.

- Hosting of the same service on more than one machine

- Hardware Firewalls, designed to offer better network security.

- Web servers on multiple platforms.

The different classifications of IP addresses are defined below:

- Provider Aggregateable (PA): these addresses are not portable, so should your customer ever want to change provider they will not be able to take the addresses with them. These addresses are not chargeable.

- Provider Independent: (PI) these addresses are fully portable; however they are subject to a small charge.

If your customer has, or is looking to implement, their own Autonomous System Network (ASN), then they will require their own 
PI address space. If your customer already has an ASN and PI addresses, these are required to be indicated on the service order form. Vodafone is able to process qualifying orders for ASNs upon request.

## 3.8 Optional Security Features

### 3.8.1 Resilient Services

Where requested, we will look to provide a second ‘Resilient’ service into a Customer Site and in such instances a higher overall annual service availability SLA can be offered (see service schedule). The following configuration options are available:

- Dual Parent (Fully Diverse)   - with load balancing or active/standby configuration

- Dual Parent (Diverse Internet Edge Routers) – with load balancing or active/standby configuration

#### 3.8.1.2 Dual Parent (Fully Diverse)

The Resilient service will be connected to a separate, MSP access nodes and Provider Edge Routers, as well as IP Backbone nodes and Internet Edge Routers (IER) to that of the Primary service. Reasonable endeavours will be used to provide assured diversity (at a cable and also duct level where possible) between the Resilient and Primary services. At the Customer Site, a common building entrance point will be used as standard. Separate building entrance points may be investigated, upon request, on a bespoke basis.

The Resilient service Internet Port bandwidth must not be less than that of the Primary service. Only committed service/Port bandwidths are permitted on the Primary and Resilient service. It is possible to use the Primary and Resilient services at the same time (i.e. in a ‘Load Balanced’ configuration) or the Resilient Service can be used as a backup should the Primary service fail. Where Managed Routers are provided, we will be responsible for configuring Load Balancing or Failover between the Primary and Resilient services. Where the customer has her own autonomous system network (ASN), we will support BGP resilience between the Primary and Resilient services as standard.

#### 3.8.1.2 Dual Parent (Divers Internet Edge Routers)

The Resilient service will be connected to a separate IP Backbone nodes and Internet Edge Routers (IER) to that of the Primary service. The Resilient service Internet Port bandwidth must not be less than that of the Primary service. Only committed service/Port bandwidths are permitted on the Primary and Resilient service. It is possible to use the Primary and Resilient services at the same time (i.e. in a ‘Load Balanced’ configuration) or the Resilient Service can be used as a backup should the Primary service fail. Where Managed Routers are provided, we will be responsible for configuring Load Balancing, or Failover, between the Primary and Resilient services. Where the customer has her own autonomous system network (ASN), we will support BGP resilience between the Primary and Resilient services as standard.

Fig x: Dual Parent (Diverse Internet Edge Routers)

### 3.8.1 Dual homing

This resilient option allows you to order a separate Internet Access service from another ISP for use alongside the Vodafone service. We will provide a single Internet Access service at the Customer site. The Customer is responsible for managing the resilient configuration (in terms of load-sharing or redundancy) across our service and that of another ISP. The customer is required to have their own ASN. All exchange of routes between Vodafone and the customer will be via BGP.  AS numbers used in BGP sessions and in AS Paths in routes announced by the customer to us shall not be from ranges reserved for private use. For new customers, only IPv6 Provider Independent addresses can now be used with Dual Homing.

Please contact your account team for further information.

### 3.8.2 Distributed denial of service protection

Distributed Denial of service (DDOS) attacks are made by having one or many attacking computers or networks sending a flood of packets every second, with the aim of either saturating the communication link for a certain target network or of keeping its computers busy serving bogus requests, in effect, denying legitimate connections or processes from executing. DDOS Protection aims to stop or diminish the effectiveness of DDOS attacks. It works by stopping packets from reaching their intended destination (the victim network or computer). We use best of breed technology to detect and mitigate against DDOS attacks. Arbor Peakflow SP traffic anomaly detectors are deployed on the edge of our Global IP Backbone (upstream peering points) and Cisco DDOS Guard is used to mitigate attacks.

In the diagram above there are two VLANs shown in red and blue provisioned over the access circuit.  When the customer is not under DDoS protection the traffic to and from the internet will be via the blue VLAN and there should be no traffic across the red VLAN. When under protection, traffic destined for the IP addresses under protection will go via the DDoS guards and "cleaned" traffic returned to the customer via the red VLAN, all other downstream traffic and any upstream traffic will still traverse the blue VLAN.

We offer two service options:

- Service Option 1: an entry level reactive service offering for customers who rely on the internet to conduct business that don’t anticipate regular attacks and can afford a short outage in consequence of an attack. This is a low cost option which is ‘reactive’ in that Vodafone will not be monitoring the customer’s service for DDoS attacks. Customers have access to a 24/7 support line to contact if attacked and there is only a small ongoing charge per IP address block.

- Service Option 2: This is a proactive service which is suited to customers who have a high reliance on the internet as part of their business operations. Typically this suits customers who expect frequent attacks or where any outage is likely to result in reputation damage of significant revenue loss. Key service features:

  - Proactive monitoring – of traffic flows to protected IP addresses against pre-defined thresholds. Vodafone will respond to alarms within 15 minutes, 24/7 all year round. At this point, mitigation steps will be agreed with Vodafone DDOS specialists and the customer

  - 5 Zone Detection configurations – allowing granular monitoring of critical servers and services across your IP address range.

- Service Option 3: This option is used by other service providers that wish to offer DDoS protection to their own customers as well as direct customers that require greater granularity than available under Service Option 2.

The DDoS mitigation service operates from a shared Platform that offers protection for a number of customers. The platform capacity to mitigate DDoS traffic for any given customer is dependent upon a number of factors, including not limited to, capacity in use to mitigate DDoS traffic for other customers of the platform; method of attack used; where the attack enters the Vodafone network.

DDoS is only available to customers that are connected directly to our AS1273 backbone.

### 3.8.3 Wider security portfolio options

Dedicated Internet Access services can be further complemented by our wider Security Product Portfolio. Below is a short summary of some of the options available:

#### 3.8.4.1 Managed internet proxy

Our Managed Internet Proxy product protect the corporate network from incoming internet traffic, it includes integrated anti-x features, and also provide a better user experience for outgoing traffic through web filtering and caching features. We provide fine-grained policy based control to manage traffic by application, protocol, content type or user. Built-in caching and bandwidth management speeds up the delivery of business-critical web traffic while boosting end-users web response. The Anti-Virus (AV) capability scans for malware, viruses, worms, spyware, bots and Trojans entering through Web-based backdoors. For customers choosing either Blue Coat or Microsoft ISA solutions, we offer the ability for customers to view reports on web activity.

#### 3.8.4.2 Email protection

The Vodafone Managed Email Protection service is a cloud based solution for protecting against virus, spam and unwanted content across every inbound and outbound email. To ensure complete email security, we have partnered with Message Labs (part of Symantec). The service does not require any upgrades of email servers or your Internet Access service from Vodafone. Mail is redirected inbound by changes to mail domain records (MX) and outbound by static pointing of outbound mail to the allocated SMTP Mail towers. This allows rapid adoption of email protection across the organisation.

#### 3.8.4.3 Managed firewall

The Managed Firewall Service is designed to offer a passive defence mechanism based on a set of defined Firewall rules. Firewall policies are defined using the principle of everything that is not explicitly allowed is denied. These policies can be defined using a combination of Source IP address (or network), destination IP address (or network) & service port, e.g. HTTP, HTTPS, FTP etc. It is important to note that the function of a firewall is to filter traffic entering/leaving the network (This is sometimes referred to as border protection). A Firewall is part of the defence in depth approach that organisations need to provide comprehensive protection against threats.

# 4.0 Technical information

## 4.1 Routing

### 4.1.1 Static Route

The customer router is configured with a default route via our internet edge router. The internet router advertises the customer prefix range to the internet through a static route(s) redistributed into BGP.

### 4.1.2 BGP with default route

The customer router will be configured to establish a BGP session with our internet edge router using a private ASN assigned by Vodafone. This routing option is the minimum required when a resilient service is used.

- The customer would advertise the ranges assigned to them or their own provider independent IP space to the internet router.

- The internet router will only advertise a single default route back to the customer router.

Note: Default BGP Timers will be used as standard for DIAoMSP services, including existing migrated services that use non-default timers. Default BGP timers are 60 seconds/180 seconds for keep-alive / hold-time. Where customer’s who have dual-homed sites, require increased service fail-over, we can, upon request, tune the BGP timers for the PRIMARY site only, to a maximum of 10 seconds / 30 seconds keepalive/holdtime.

### 4.1.3 BGP Full Table

The same diagram applies to both public and private ASNs as the only difference is the use of private or publicly assigned ASNs.

The customer router will be configured to establish a BGP session with the internet router using their own ASN or one assigned by Vodafone.

- The customer would advertise the ranges assigned to them or their own provider independent IP space to the internet router.

- The internet router would advertise a full internet routing table to the customer.

Note: Default BGP Timers will be used as standard for DIAoMSP services, including existing migrated services that use non-default timers. Default BGP timers are 60 seconds /180 seconds for keep-alive / hold-time. Where customer’s who have dual-homed sites, require increased service fail-over, we are able to tune, upon request, the BGP timers for the PRIMARY site only, to a maximum of 10 seconds / 30 seconds keepalive / holdtime.

## 4.1 Managed customer routers

The latest list of managed routers available with the DIA service can be obtained from your sales representative. We will configure the router, as per your order, for any of the following routing options:

- NAT – Network Address Translation (NAT) does not require a static/fixed IP address to be allocated. It can be achieved over DIA by the customer either purchasing a managed customer site router or a Managed Firewall.  Alternatively the customer can employ their own router or firewall to achieve NAT. NAT is not appropriate where the customer wishes to advertise their web site and/or presence on the Internet.

- STATIC – Using static routing, the customer is provided with a statically configured default route so that traffic can be routed to our Internet Backbone.

- BGP – Using BGP a dynamic routing interface is established between the customer and Vodafone.  BGP allows and provides the customer with control over the routes advertised to Vodafone and ultimately the wider internet.  BGP is especially beneficial when a customer takes more than one link from us and wishes to use the links for Resilient Service fail-over or load-balanced configurations.

- HSRP – will be deployed where we provide multiple managed customer site routers with resilient services.

## 4.2 Management traffic

The Dedicated Internet Access service will be managed in-band. Service bandwidths are inclusive therefore of a small amount of bandwidth which will be used for management purposes. Where sites are connected using third party Fibre Ethernet circuits, we may locate a dedicated Vodafone 1U Adva NTE at the customer site behind the 3rd Party Provider’s NTE. This will function as our service demarcation point and will assist with our remote management of the access circuit.

## 4.3 Wires only services

Where we do not provide Managed Customer Site Routers, the service demarcation point will be the LAN interface of the access circuit NTE at the customer site. Except where 2Mbps access circuits are provided over leased lines, the LAN interface will be configured as an IEEE 802.1Q port to support our management traffic which has an 802.1P QoS marking. The customer traffic will pass over the same VLAN as the management traffic. The VLAN ID is assigned by Vodafone during Provisioning.

Where you order a wires only service, please request a copy of our guide to customer site router configuration.

Where we do not provide Managed Customer Site Routers, then Resilience options, detailed in section 4.4 below, will not be configured by Vodafone. This however does not prevent the Customer from configuring their CPE devices to use a multiple DIA over MSP connections to provide resilience. It does mean that Vodafone will not provide direct support for these resilient scenarios.

## 4.4 Resilient services

### 4.4.1 Dual parent (fully diverse)

The eBGP sessions between our managed customer site router and the Internet Edge Router (IER) ASPATH pre-pend. MED is used to make the backup link less preferred.  Hot Standby Redundancy Protocol (HSRP) is run between to the two managed customer site routers where requested. This enables the routing nomination of the customer site edge router (CE) as Primary or Backup.  In the event of a failure of the Primary CE, traffic would be serviced by the Resilient Service CE.  Upon return of the Primary CE, traffic would then be serviced by the Primary CE. HSRP, if used, is also capable of providing ‘load balancing’ in the form of multiple HSRP groups being used with the different groups using different routers as their primary router.  While this is referred to as “load balancing” it does not guarantee an equal distribution of traffic across both links - that is dependent upon the traffic loads from each of the configured HSRP groups.

### 4.4.12 Dual parent (diverse internet routers)

The Primary and Resilient Internet Access services are connected to a single MSP Edge Router but the end points (internet edge routers) are diverse.  The eBGP sessions between the customer routers and the internet routers and ASPATH prepend, MED would be used make the backup link less preferred.  HSRP would is run between the two customer site routers where requested. As with the fully diverse solution, HSRP is capable of supporting ‘load balancing’ where ordered.

## 4.5 Burstable bandwidth

Customers will also be able to order the ability to exceed their contracted bandwidth, within limits, based on their access circuit and their committed rate. Here the service PIR can be set to up to twice the service CIR. Note, that where access is via TDM leased line (2Mb), there is no burstability option. This burst-ability bandwidth is priced lower due to the fact that out-of-contract (>CIR) traffic will be marked with a lower EXP through the MSP aggregation into our IP/AS backbones. In the internet, the in / out of-contract behaviour does not exist.

## 4.6 Providing service over existing MSP access circuits

Internet Access can only be provided over existing fibre Ethernet access circuits, known as Multi-Service Access Bearers (MSABs) which connect the customer site directly to our Multi-Service Platform.  Internet Access cannot currently be provided, alongside other network services, over access circuits based on Leased Line or Ethernet First Mile (EFM) technology. The MSAB is configured as a.1Q Trunk. This allows Internet Access traffic to be securely partitioned from the network service traffic using VLAN tagging. With wires only services, the VLAN ID for the Internet Access service will be advised during provisioning.

Where the existing Network service, provided over the MSAB connecting the customer site, is a Managed IPVPN WAN service then a separate managed customer site router is required to be purchased with the Internet service. The IPVPN WAN managed customer site router cannot be used as this would breach our government accredited security regulations. In all other instances, it is not possible to purchase a Managed Customer Site Router for the Internet Access Service where it is provided over an existing MSAB.

Where we provide an Adva825 NTE at the customer site with the MSAB (this should be assumed unless advised otherwise), the Internet Access service can be presented on its own port on the LAN side of the NTE or off the same port as the Network service. The table below provides more detail:

| Adva NTE Port Used for Internet Access | Internet Access Service Conditions |
| --- | --- |
| Separate electrical 10/100BaseT Port to Network Service | - Maximum Internet service bandwidth limited to 100Mbps - Distance restrictions apply (typically <100m) re the maximum copper cable distance from the Adva NTE port to router CPE. |
| Same electrical 10/100BaseT Port used as the Network Service | - Maximum Internet service bandwidth limited to 100Mbps less the Network Service Bandwidth - Distance restrictions apply (typically <100m) re the maximum copper cable distance from the Adva NTE port to router CPE - Where the Network Service includes Vodafone managed customer site routers, then the mandated Vodafone managed customer site router for Internet Access must be located in line behind the IPVPN customer site router. |
| Separate 1000BaseSx Port to Network Service | - Maximum Internet service bandwidth limited to 1000Mbps |
| Same 1000BaseSx Port used as the Network Service | - Maximum Internet service bandwidth limited to 1000Mbps less the Network Service Bandwidth. - Where the Network Service includes Vodafone managed customer site routers, then the mandated Vodafone managed customer site router for Internet Access must be located in line behind the IPVPN customer site router. |

## 4.7 Domain name services

Our DNS resolution server addresses are as follows:

Euro-cns3.cw.net -  141.1.1.1

Euro-cns2.cw.net  - 195.27.1.1

Our Global DNS server addresses are as follows:

Ans1.cw.net - 141.1.27.248

Ans2.cw.net – 195.27.1.2

In order to act as a secondary DNS provider for a customer:

- The customer will need to amend the allow-transfer parameter in their DNS configuration to allow zones transfers from 141.1.27.128 for all domains required to be secondary.

- The customer must ensure that each domain we act as secondary for has refresh time value between 1000 and 86400 seconds.

- Once a) & b) have been confirmed, the customer must also notify us of their Public IP address for their DNS server from which the zones transfers are to occur from. Our DNS Hostmaster will configure the required domain(s) on our DNS platform and ensure that the zonefile has been successfully transferred.

- We will then ensure that the name server records have been amended by the registrar for the domain(s) and shows the required authoritative name servers.

## 4.8 Use of AS1273 & AS2529

Internet Access is provided over our Global AS1273 backbone, however in the UK our AS2529 backbone may be used for aggregation purposes. AS2529 is then connected upstream to AS1273. The table below provides more detail:

| Requirement | Default AS |
| --- | --- |
| Existing AS2529 connected customer wishes to make a hard change whilst retaining IP addressing | AS2529 |
| Existing AS1273 connected customer wishes to make a hard change whilst retaining IP addressing | AS1273 |
| New customer service requiring DDOS Protection | AS1273 |
| New customer service requiring IPv6 support (IPv6 is supported on a reasonable endeavours basis at present) | AS1273 |
| Internet Access service provided with an existing network service, over a MSAB, to an EU site which is not located in mainland UK. | AS1273 |
| All other new customer services | AS2529 |

## 4.9 BGP Communities

It is possible to advertise routes between your Autonomous System Network number and Vodafone Worldwide peers and/or transit providers. This is achieved by the use of BGP communities. The latest list of communities available from Vodafone can be provided upon request.

# 5.0 Service support

In this section, we provide guidance on our standard proactive monitoring, incident management and self-service online portal. These are intended to provide you with greater assurance of service continuity, greater control and visibility and operational efficiencies.

## 5.1 Proactive monitoring

As standard, Vodafone undertakes proactive monitoring for a range of alarms which correlate to our definition for severity 0-2 faults as defined in your service schedule. These generally imply that your service is hard down and cannot be used or there is a loss of resilience.

We proactively monitor our Internet Backbones, Multi-Service Platform. We will also monitor access circuits connecting your sites and any Managed Routers so long as you are using the BGP routing option. We use reasonable endeavours to identify and react to alarms, open trouble tickets and commencing service restoration activity, ahead of you raising a trouble ticket, and notify you in accordance with your contact plan within 15 minutes of us opening a trouble ticket

The table below specifies the alarms for which we proactively monitor as standard*:

| Impact Grouping | Event Type | Description | Internet Access |
| --- | --- | --- | --- |
| Access Circuit** | Interface Down | The circuit is unavailable to send or receive IP packets in either direction. Applicable to access circuits provided over leased line technology. | Y |
|  | Loss of Resilience | Traffic has failed to a resilient service provided at the customer site. | Y |
|  | BGP Routing Failure | BGP routing protocol is not functioning. Applicable to Access Circuits provided using Ethernet technology. | Y |
|  | Bouncing Interfaces | An Access Circuit is subject to brief instances of unavailability. If there are more than eight of these instances over an 8 minute period and alarm will be triggered. | Y |
| Managed Customer Site Router | Device unreachable | Connectivity is lost to the Vodafone Managed CE device from the Internet Edge Router. | Y |

Where access circuits are provided using Ethernet technology, we will only be able to identify service unavailability through loss of BGP routing alerts rather than interface down alarms. This requires the customer to use BGP routing. Where Ethernet First Mile technology is used, we are not able to proactively identify loss of capacity alarms, only full circuit failure upon loss of BGP. In order to avoid unnecessary impact to the Customer Experience, Vodafone proactive monitoring policy (for the alarms detailed above) accounts for the suppression and buffering rules detailed below:

### Suppression

The following events are suppressed from being presented to our Network Management Centres for validation and generation of a trouble ticket:

- Planned Outages & Emergency Maintenance: alarms in consequence of these events will be fully suppressed.

- Restricted Monitoring: for certain customer sites &/or services, proactive monitoring may be withdrawn where requested by you  for example where sites are powered down at night, or in consequence of  frequent occurrence (3 or more in a calendar year) of Customer caused events such as powering down.

### Buffering

Alarms which are not suppressed will be subject to the following buffering policies prior to presentation to the Network Management Centres:

- Basic Buffering (all alarms) – all alarms will be buffered for 10 minutes from first occurrence.

- Bouncing Interfaces – except where there are more than 8 bounces over an 8 minute period, alarms relating to bouncing interfaces will be buffered. The following exceptions apply:

- All other access circuit interfaces - if there are more than 5 occurrences over a two hour period, an alarm will be triggered on the 6th occurrence for proactive monitoring following the defined 10 minute buffering period.

- Proactive Monitoring excludes all service features and components not specified.

## 5.2 Incident & change management

As standard, Vodafone undertakes proactive monitoring for a range of alarms which correlate to our definition for severity 0-2 faults as defined in your service schedule. These generally imply that your service is hard down and cannot be used or there is a loss of resilience.

All Vodafone customers benefit from 24/7 year round fault support. Please refer to our Service Schedule for specific details. It is possible to raise faults in a variety of ways ranging from telephone to using the online ‘MyVodafone’ self-service portal which is summarised in the next section. We will also undertake soft changes (including to managed customer site routers, service bandwidths, routing and IP addressing, optional internet and security features) as well as hard changes to your service where requested and in accordance with the Service Schedule.

## 5.3 Professional services

Dependant on your requirements, we can provide a range of professional services which help you to ensure an excellent user experience, assure service continuity and reduce costs. Below is a list of some of the more common professional services which can be provided with Internet Access solutions:

- Project Management (particularly important to ensure a seamless transition and implementation)

- Service Management

- Network & solution design

- Capacity & Performance Management

- Lifecycle management

- Dedicated Customer Change management

- Security and compliance management

## 5.4 Online portal

Where ordered, we are able to provide secure access to our MyVodafone portal. The portal provides a range of self service applications as well as supporting information making it easier to interface with us and providing you with greater visibility and control. Below is summary of some of the key applications relevant to Internet Access:

- Supporting information: we provide product and support materials and, dependant on your requirements, it may be that service and operational manuals are located on your instance of the portal together with your contact plan for faults and issues

- Order tracking: track your order through our Provide milestones from order acceptance through to commissioning and delivery

- Trouble ticketing: raise and track trouble tickets, add queries and comments

- E-billing: view your current and historical bills

- Resellers are also given access to online ordering and pricing tools.

# 6.0 Pricing & billing

Internet Access is billed monthly in advance. The pricing structure for service components and additional service features is summarised below:

| Feature | Sub-feature | One off charge | Monthly recurring charge | Additional info |
| --- | --- | --- | --- | --- |
| Access Circuit |  |  |  |  |
| Committed Service BW Committed |  |  |  |  |
| Committed Plus Burst BW |  |  |  |  |
| Customer Site Routers |  |  |  | One off installation charge & optionally a purchase charge. Ongoing maintenance and management charge & optionally a router rental charge. |
| Internet Features | Domain Name Services |  |  |  |
|  | Public IP Addresses |  |  | Only applies to Provider Independent IP Address ranges. |
|  | ASN |  |  |  |
|  | Performance Reporting |  |  |  |
| Security Features | Resilient Services (Any Dual Parent Options -Active/Backup Configuration) |  |  | The Service/Port BW on the resilient service will be subject to a reduced charge. |
|  | Resilient Services (Any Dual Parent Options – Load Balancing Configuration) |  |  | The Service/Port BW on the resilient service will be fully chargeable where a load balancing configuration is required. |
|  | Resilient Services (Dual Homing) |  |  |  |
|  | DDOS |  |  |  |
|  | Wider Security Service Options | POA | POA |  |
| Service Support | Professional Services | POA | POA |  |