3 : THE WORKS

3.1 : GENERAL CLAUSES

Services

| 3.1.1 | The Contractor is to take particular care to ensure that all Services are maintained to the property and surrounding properties during the course of the work. The Contractor is to ascertain the position of such Services and to arrange for their protection at commencement and during progress of the work. Any temporary disconnections required to be carried out shall be done at times convenient to the Employer and surrounding occupiers and with their prior knowledge. |  |
| --- | --- | --- |
| Occupation |  |  |
| 3.1.2 | The Old School Hall will not be occupied during the course of the work. The Client wishes to discuss with the Contractor the potential for using at least the front section of the building and possibly other parts while the work is in progress. |  |
| Protection |  |  |
| 3.1.3 | The Contractor is to allow for protection to the existing property during the course of the work. Any damage arising from the works will be made good at the Contractor’s expense. |  |
| Rubbish |  |  |
| 3.1.4 | Remove all rubbish and debris from the site from time to time as it accumulates and at the completion of the Contract. Leave the site neat and tidy to the satisfaction of the Contract Administrator. |  |
| Materials Storage |  |  |
| 3.1.5 | The Contractor is to agree the location of materials and plant storage with the Employer and the Contract Administrator prior to commencement of the project. |  |
| Cleaning |  |  |
| 3.1.6 | The Contractor is to allow for thorough cleaning at the end of each working period and at completion of the project to the approval of the Contract Administrator. |  |

| Drawings |  |  |
| --- | --- | --- |
| 3.1.7 | This Specification is to be read in conjunction with Drawing Nos 233/100, 101C, 102A, 151, 202, 203, 204 and 206. |  |

| Access Equipment |  |  |
| --- | --- | --- |
| 3.1.8 | Allow for full scaffolding externally to undertake all high level work. |  |

| 3.2 : EXTERNAL |  |  |
| --- | --- | --- |
| Chimney Stacks |  |  |
| 3.2.1 | Remove all moss growth and clean down 3 No chimney stacks. |  |
| 3.2.2 | Allow for minor repointing to chimney stacks in isolated areas (provisional area 10 square metres) |  |

| Roof Coverings |  |  |
| --- | --- | --- |
| 3.2.3 | Strip off existing roof tiles to all areas excluding front Hall, including removal of all lead flashings, timber battens and any underfelt to include ridge and valley tiles and bonnet tiles and cart away all debris from site. |  |
| 3.2.4 | Allow for salvaging 40% of tiles, or such other greater quantity as may be deemed reusable as the roof tiles are stripped from the roof, set aside and allow for later re-use. |  |
| 3.2.5 | Supply and lay new breathable membrane under new 50mm x 25mm counter battens to tops of rafters and new 38mm x 25mm treated softwood battens fixed with 65mm galvanised nails, set to a gauge to suit new roof coverings. |  |
| 3.2.6 | Provide new roof tiles to match exactly the existing and lay new and salvaged tiles to roof slopes to include ridge tiles and valley tiles to match existing. |  |
| 3.2.7 | Supply and fit 300mm 5u felt support at eaves. |  |
| 3.2.8 | Provide and lay double course of tiles at eaves position. |  |
| 3.2.9 | Allow for straight cuts to both sides of valleys and parapet wall abutments. |  |
| 3.2.10 | Ridges and verges to be bedded in cement sand mortar 1:3 and pointed in same. |  |
| 3.2.11 | Supply and fit new Code 4 lead flashings to 2 No chimney stacks, including all welding and fabrication of lead trays. |  |
| 3.2.12 | Provide and install Code 3 lead soakers at parapet wall abutments. |  |
| 3.2.13 | Provide and install Code 4 lead flashings at all parapet wall abutments. |  |
| 3.2.14 | All leadwork to be installed in accordance with Lead Association requirements and treated with patination oil before and after installation. |  |

| Eaves and Rainwater Goods |  |  |
| --- | --- | --- |
| 3.2.15 | Strip out all timber fascias and bargeboards, together with existing gutters and downpipes and cart away all debris from site. |  |
| 3.2.16 | Provide and fix new deep flow Aqualine extruded aluminium gutters and Heritage cast aluminium downpipes to existing locations as produced by AWMS, White House Works, Bold Road, Sutton, St Helens WA9 4JG, Telephone : 01744 648400. |  |
| 3.2.17 | Rainwater goods to be fixed strictly in accordance with manufacturer’s instructions. |  |
| 3.2.18 | Provide and install new softwood fascias and bargeboards where previously fixed to a section to match the existing. Timber fascias and bargeboards to be pre-primed prior to installation. |  |
| Decorator |  |  |
| 3.2.19 | To Old School Hall fascias and soffits, properly prepare, well rub down and apply two oil based undercoats and one coat oil based gloss finishing paint. |  |

| 3:3 : PROVISIONS AND CONTINGENCIES |  |  |
| --- | --- | --- |
| Provisional Sums |  |  |
| 3.3.1 | Allow the Provisional Sum of £500.00 for investigation of the support to the double chimney stack adjacent to the Dining area. | £500.00 |
| 3.3.2 | Allow the Provisional Sum of £100.00 to check the condition of the leadwork to 2 No roof vents. | £100.00 |
| Contingency Fund |  |  |
| 3.3.3 | Allow the Contingency Fund of £3000.00 to be expended or deducted in whole or in part as directed by the Contract Administrator. | £3000.00 |

SUMMARY

GENERAL PRELIMINARIES

PREAMBLES

THE WORKS

GENERAL CLAUSES

CHIMNEY STACKS

ROOF COVERINGS

EAVES AND RAINWATER GOODS

DECORATOR

PROVISIONS AND CONTINGENCIES

TOTAL