`

[image: image1.jpg]Kettering
Borough Council




FIRE RISK ASSESSMENT WORKS 
SCHEDULE OF WORKS.
Installation of 

Emergency lights 

To communal areas
Of 

General needs blocks

Sited in Kettering 
	PROJECT
	FIRE RISK ASSESSMENT REPAIRS

	BILL NO
	2
	SECTION
	WORK SPECIFICATIONS
	PAGE NO
	1

	ITEM
	
	QTY
Per

block
	unit
£
	d
	Total

£
	d

	1
2

3

4

5
6
7
    8
     9
	SCOPE OF WORKS 

Remove existing communal lighting throughout the blocks listed below and replace and/or reposition with new bulkhead infra-red microwave sensor emergency light. 

Product code: LBH154KMW3M, 
Product - supply and install Luceco, Atlas bulkhead light fitting, 20W, Microwave sensor, 3hr emergency, white in colour, inc metal alloy clad, anti-tamper body, polycarbonate diffuser with anti-tamper screw
Schedule of Electrical Works for:

Blocks 
Arthurs way – 1-3B, 2–4B, 5–7B, 9–11B & 13–15B 
Gloucester Close – 1-3B, 9-11B & 13-15B
Thorngate St – 2-4B
Alexandra St – 61-63B, 70-72B & 74-76B
New Light Fitting Position

Ground Floor – on wall as existing, per block.
1st Floor – on ceiling as existing, per block
2nd floor - on ceiling as existing, per block

Outside Front – new position to the side wall opposite the front door entry panel, height to be aprox 2.3m from floor per block 
Outside rear – new position to the side wall opposite side of the door entry panel, height to be aprox 2.3m from floor per block 
Supply fed from landlords meter sited in main electrics cupboard under the stairs 
Works Required per block
Existing Light Fitting: Isolate supply and remove existing bulkhead light fittings from wall/ceiling position dispose of from site.
New Bulkhead Fitting: Supply and fit new Luceco  20W bulkhead light fitting, mounted on the ceiling/wall as per existing or as instructed as specified above,  test and make good where required.
Conduit: where required only to outside positions, supply and fix new 25mm diameter heavy gauge galvanised steel conduit including all necessary fittings, galvanised steel spacer saddles at 1200mm centres to any surface, make good. Per block.
Block 

Thorngate St – 46-54
New Light Fitting Position

Ground Floor – on lobby ceiling as per existing, 

1st Floor half landing – on ceiling as existing, 

1st Floor - on ceiling as existing
2nd floor half landing - on ceiling as existing
2nd floor - on ceiling as existing
Outside Front – position to the side wall as per existing 

Total no new light fittings

Lineal meterage galvanised conduit 


	2no
1no
1no
1no
1no
5no

5no
5Lm

1no

1no

1no

1no

1no

1no

66no

60m
	
	
	
	

	

	

	
	

	
	Total costs

	
	
	
	
	


