SPECIFICATION FOR THE REPAIR OF CHALLENGER 2 (CR2) ROAD WHEELS

SPECIFICATION FOR THE REPAIR OF CHALLENGER 2 (CR2) ROAD WHEELS

The contents of this specification must not be communicated to a third party or used for any other work than that for which the specification is issued without the written agreement of the Babcock DSG Repair Manager

The contents of this specification must not be communicated to a third party or used for any other work than that for which the specification is issued without the written agreement of the Babcock DSG Repair Manager

Airports  |  Communications  |  Defence  |  Education  |  Emergency Services | Energy  |  Mining and Construction  |  Property  |  Rail  |  Training

Airports  |  Communications  |  Defence  |  Education  |  Emergency Services | Energy  |  Mining and Construction  |  Property  |  Rail  |  Training

# Contents

Cover Sheet

Contents	2

List of Tables	2

Amendment Record	3

1.0 Introduction	4

2.0 Publications	4

3.0 Documentation	5

4.0 Repair Policy	6

5.0 Repair Requirement	7

6.0 Performance and Test Acceptance	8

7.0 Preservation & Packing	8

# List of Tables

Table 1 – Equipment Details	4

Table 2 – Support Publications	5

## Amendment Record

| Issue/ Amdt No | Identification of Change Page & Para No | Date Entered | Date Effective | Authorisation |
| --- | --- | --- | --- | --- |

## Introduction

1.1    The equipment covered by this Statement of Work belongs to the Armoured Tracks Portfolio Project Team (ATP). The wheels scheduled for repair under this contract are part of the Challenger Tank used by UK military.

Table 1 – Equipment Details

| DMC | NSN | DESCRIPTION |
| --- | --- | --- |
| 4CR2A | 2530996659014 | Road Wheel (with sand holes) |
| 4CR2A | 2530999022293 | Road Wheel |

  - This document is intended as an outline specification detailing the engineering requirement to enable a company to apply their expertise to produce a compliant product that meets the in-service user requirements, which shall be acceptable to the Authority and for which a warranty shall be provided.  The performance of completed assemblies shall meet or exceed that of the original equipment manufacturers (OEM) specification and the criteria contained in this specification.  Should any differences exist between the OEM and MoD specifications, either in build or test criteria, the MoD will generally take precedence.  Clarification should be sought from the Babcock DSG Repair Manager.

  - The demanding operational role of Armed Service equipment is significantly different to that of equivalent commercial equipment.  It is essential to ensure that this equipment proves reliable when used and that the end user has the necessary confidence that it will survive the rigours of Service application.

  - It is MoD Policy that contractors hold a current recognised third party Quality Accreditation Certification (UKAS or International equivalent).  For the work requirement of this specification, the contractor shall be registered in accordance with the requirement of ISO 9001:2015 suitably scoped as a minimum.

  - There may be circumstances, such as urgent operational requirements (UOR) where it will be to the Authority’s benefit to accept delivery of products that do not conform to contract requirements, as detailed in Defence Standard 05-061 (Part 1) (Concessions), but there must be a clear and demonstrable benefit to the Authority and approval must be given by the Babcock DSG Repair Manager, (in writing), before this takes place.

  - Any quantities referred to are estimated quantities only. The Authority may order more or less than those referred to. Any figures are for guidance only and no guarantee can be given that any specific quantities of repairable items will become available.

## Publications

2.1	Contractors are responsible for obtaining the latest OEM publications, parts lists and supersession lists for the equipment.

  - Publications produced by the MoD for service use are, in general, based upon the commercial publications but the format is specific to the service user. Contractors are responsible for obtaining and maintaining the latest issues of these publications.

  - Publications currently applicable to the assemblies within this requirement are covered in Table 2 below:

#### Table 2 – Support Publications

| Publication | Title |
| --- | --- |
| JSP 886 | Defence Logistics Support Chain Manual |
| AESP 0200-A-220-013 | Preservation, Identification and Packaging of Assemblies |
| DEF STAN: 05-57 | Configuration Management of Defence Material (issue no 7 dated 28/07/18) |
| DEF STAN: 05-99 | Managing Government Furnished Equipment in Industry. Part No1 (Issue no 1 dated 14/07/17) |
| DEF STAN: 05-61 | Quality Assurance Procedural Requirements Part 1 – Concessions (Issue 6 dated 31/03/16 Part 4 – Contractor Working Parties (Issue no 3 dated 25/10/02, amendment No 1 dated 28/01/11) |
| DEF STAN:00-056 | Safety Management Requirements for Defence Systems Part 2 – (Issue no 5 dated 28/02/17) |
| DEF STAN: 05-135 | Avoidance of Counterfeit Material (Issue no 1 dated 10/07/14) |
| DEF STAN: 81-41 | Packaging of Defence Material Part 1 – Introduction to Defence Packaging Issue 9 Part 6 – Package Marking (Issue 10 dated 14/02/18) |
| FV2301008 | NSN 2530-99-665-9014 Drawing |

## Documentation

3.1	 A draft quality plan (QP) will be required at the ITT stage to demonstrate how equipment is to be managed.  The completed QP shall be submitted within three months of the commencement of the contract.  The QP should identify all risk areas and detail how they will be mitigated and managed throughout the duration of the contract.  The QP shall reference procedures, developed in accordance with the Contractors Quality Registration, which detail how control of the repair relating to the Company Quality Assurance processes will be achieved.  Inspection and test points shall be clearly indicated making reference to any Authority requirements included in any Final Contract award.  Documentation relating to critical or safety related items and assemblies shall be highlighted.

  - Following the provision of equipment to repair under this contract, the Contractor is to take all necessary action to sentence the equipment as Repairable or Non Repairable (BR).

  - At the commencement of the Contract, and thereafter at reasonable intervals depending upon need arising and priorities, the Babcock DSG Repair Manager and Contractor shall agree a "production plan" for the repair.  The Contractor shall provide a monthly report on the progress of the repair work against the plan to the Babcock DSG Repair Manager.  This report must include expected delivery dates, financial accrual information and any mitigating factors to support repair and/or delivery variations.

  - Records, comprising repair, calibration, inspection, spares and test reports as applicable and defined in this specification, shall be maintained by the Contractor.  Additionally the Contractor is to keep records of all visits/survey reports, approvals and costs incurred in the repair/manufacture of the Contractor deliverables.  Where there is a legislative requirement, documents are to be kept for the period specified in that legal requirement. All records must be made available to the Authority as required.

## Repair Policy

4.1	 Repair. Assemblies submitted for repair will have been removed from service for a multitude of reasons.  The requirement for the remanufacture of these assemblies is to give an expected life of not less than eighty percent of that of a new assembly.  This specification is not to be considered as comprehensive for the work requirement and is not to be used as a reason to limit any work on the assembly.  It is the Contractor's responsibility to produce a comprehensive repair specification and to ensure that the quality of the assembly returned after refurbishment shall meet the requirement of "as new" with the stated life requirement.

It is understood that the Prime Contractor will need to sub-contract elements of this repair to Third Parties. It is the responsibility of the Prime Contractor to ensure that the flow of work between all parties is in the best interests and provides the best value for money for the Authority and that their suppliers have the correct quality processes and relevant approvals in place to maintain the standard of the end product.

Wheels found not suitable for repair are to be disposed of by the Contractor once authorised by the Babcock DSG Repair Manager, following declaration of the quantity, scrap weight & scrap value

4.2	 Beyond Economical repair (BER). Fully priced cost shall be required for any assembly not considered by the repairer as economic to repair.  These must be submitted at the survey stage (before repair work commences). The Authority will only agree Beyond Economic Repair (BER) classification where the Contractor has been able to demonstrate that their cost to repair is greater than the eighty percent of new cost as supplied to the MoD, or the wheel has exceeded its repairable life. Once BER has been agreed the Authority will issue disposal instructions and requirements for the scrapped wheel accordingly.

4.3 	Remanufacture.  The repaired wheel will be to the latest MoD approved OEM specification and modification state.  The performance and quality standard of the assembly shall meet or exceed the requirement of the OEM specification.  Records of performance tests and results as applicable shall be supplied as stated in this specification.  All wheels shall be covered by a warranty as defined in the terms and conditions of the Contract.

4.4	Replacement Parts. Procurement of all replacement materials used in the repair shall be the responsibility of the Contractor.  All materials shall meet or exceed the OEM specification and shall be purchased from approved suppliers.  Certificates of conformity (COC) shall be obtained for all parts which have not been sourced through the OEM, and shall be made available to the Babcock DSG Repair Manager or a nominated representative when requested.

  - Safety. The Contractor has an obligation towards safety.  Any failures or incidents in relation to the equipment which affects safety shall be reported to the Babcock DSG Repair Manager without delay.  The Babcock DSG Repair Manager shall be entitled to require action to be taken to correct the failure and to prevent reoccurrence.

  - Modifications. All modifications approved by the OEM & MoD as defined in the latest technical documentation shall be incorporated as part of the repair. Unauthorised modifications shall not be incorporated.

## 5.0 Repair Requirement

5.1	Repair Inspection. Wheels received for repair are to be checked for correct nomenclature and part number and a report produced detailing the modification/re-life status (if applicable).

5.2      MOD Form 445 (Discrepancy Report).  Any discrepancies in the items delivered should be reported using MoD Form 445 (Discrepancy Report).  These reports shall be completed in accordance with the criteria laid down in the relevant JSP 886 and distributed as required by the Contract with two copies to the issuing depot and one to the Babcock DSG Repair Manager.

5.3	Repair Requirement. The scope of the repairs to be carried out shall be determined from the survey against OEM specification.

5.4	 Rebuild. Assemblies are to be rebuilt in accordance with the latest OEM specification using reclaimed and new materials, incorporating all approved modifications where applicable. In the case of 2530999022293 being supplied as part of this contract these are to be drilled with sand holes and returned as per specification 2530996659014.

5.5	Identification and Marking. The Contractor shall permanently identify the reclamation status in accordance with drawing FV2301008.

## 6.0 Performance and Test Acceptance

6.1		On completion of repair the wheels shall be subjected to suitable inspection and acceptance by the Contractor.

6.2	Where discrepancy exists between the OEM and MoD test specification the MoD specification will generally take precedence, but the Contractor shall ultimately seek clarification from the Babcock DSG Repair Manager.  It is the responsibility of the Contractor to ensure that all test equipment is maintained and calibrated.

6.3	Inspection/test records shall be retained for all assemblies for a period of four years in accordance with DEFCON 609 and made available for the Babcock DSG Repair Manager or nominated representatives of the Authority upon request.

## 7.0 Preservation & Packing

7.1     Where appropriate, completed assemblies are to be painted to OEM specification.

7.2     Completed assemblies are to be packed in accordance with the relevant Service Packaging Instruction Sheet (SPIS) and to the level shown in the contract or order.

7.3     Any replacement wood used in packaging must be ISPM 15 compliant and carry the Forestry Commission, Heat Treated, mark (see below) (DEFCON 129 refers).

Charles Warner

Repair Manager (ATP)

Bldg. B15,

Donnington,

Telford.

Shropshire

TF2 8JT

Charles Warner

Repair Manager (ATP)

Bldg. B15,

Donnington,

Telford.

Shropshire

TF2 8JT

Army Equipment Support Publication

For the purposes of DEFCON 611 all contractor deliverables issued under the contract will be issued on as a Contract work Item (formerly Contract Loan) basis.

The contractor is advised that no guarantee can be given or responsibilities accepted by the Authority regarding the completeness or correctness of equipment issued for repair, or give any indications of the level of repair required.

Once value is declared the Authority reserves the rights to recover any excess revenues generated by the scrap value revenue stream.

This is the general guide criteria but can be varied depending upon stock levels, new buy time limits and supply urgency, the Babcock DSG Repair Manager will advise in all BER requests.