Building Services Electrical Specification

Footpath Lighting in

Upchurch Parish

CONTENTS

- Column Lighting [C]

- Pole mounted Lighting [P]

- Column Lighting [CP]

- Electrical Work

- Ground Works

- Removing & making good

- Demonstration

- Handover

Scope of works

The footpath lighting assets comprise of 48 lighting units of various types of lamp and mounting, the arrival of LED lighting gives an opportunity to replace the ageing and obsolete units for LED units reducing energy, and maintenance costs.

1.0 Column Lighting [C]

The existing columns are to be replaced with new Tubular steel lighting columns with a mounting height of 5 metres with demountable 0.5 metre bracket arm arrangement.

Corrosion Protection

The corrosion protection system is to be hot dip galvanised to BS EN 1461. In addition, a full paint specification is to be provided.

The luminaire is to be

Ansell lighting AONIXLED/3/PC Die-cast aluminium construction for optimal heat dissipation for optimal performance

UV stabilized polycarbonate cover

Toolless gear tray removal for ease of maintenance

Built in anti-tamper isolation facility

IK10 Rated

Surge protection

Compatible with post top or side entry utility 76mm spigot

Available in standard, complete with photocell option

LED lifespan L70 50,000 hours

G3 Glare Rating

All to be with secondary isolation, fused cut-out’s and new connections to the public supply.

2.0 Pole mounted Lighting [P]

The existing utility pole mounted luminaires are to be replaced with new luminaires, utility pole mount brackets and control boxes.

The luminaire is to be:

Ansell lighting AONIXLED/3/PC Die-cast aluminium construction for optimal heat dissipation for optimal performance

UV stabilized polycarbonate cover

Toolless gear tray removal for ease of maintenance

Built in anti-tamper isolation facility

IK10 Rated

Surge protection

Compatible with post top or side entry utility 76mm spigot

Available in standard, complete with photocell option

LED lifespan L70 50,000 hours

G3 Glare Rating

The pole mount brackets are to be:

A.C. Ford Limited, Type 12- pole mounting 0.5 – 1.0 metre projection dependant on the location.

All to be with secondary isolation, fused cut-out’s and new connections to the public supply.

3.0 Column Lighting [CP]

The existing columns are to be replaced with new Tubular steel lighting columns with a mounting height of 5 metres.

Corrosion Protection

The corrosion protection system is to be hot dip galvanised to BS EN 1461. In addition, a full paint specification is to be provided.

The luminaire is to be

Ansell lighting ASULED/2/CW/PC

Modern die-cast aluminium post top lantern

Electrostatic paint finish for enhanced corrosion resistance

IK10 Rated

Suitable for residential and commercial use

Ideal for large carparks and large open areas

Adjustable spigot bracket 50-76mm supplied as standard

Non-dimmable

LED lifespan L70 50,000 hours

Powered by Philips

All to be with secondary isolation, fused cut-out’s and new connections to the public supply.

4.0 Electrical Work

The electrical installation work detailed in this specification is to comply with the current edition of BS7671 as amended and all other relevant B.S.

All contractors must be enrolled in one of the UKAS third party certified bodies NICEIC, ELECSA, etc...

5.0 Ground Works

The project will require some substantial grounds works to securely fix all columns into a secure base.

6.0 Removing & Making good

Within the parish all columns to be removed and replaced, the area made good, with UKPN being in attendance for the supply to be utilized for the new lighting columns and utility pole mounts.

All traffic management will be the responsibility of the contractor.

All ground works to be made good and area re-instated to the approval of the council.

7.0 Demonstration

Following the satisfactory completion of installation, testing and start up, the Contractor shall demonstrate to Upchurch Parish Council that the Luminaires operate as required.

The council shall also be shown how to carry out routine maintenance and testing procedures, and how to keep the Log Book.

The Contractor shall present an Acceptance Certificate for signature by Upchurch Parish Council.

8.0 Handover

The Contractor, upon completion of the commissioning activity, shall hand over the installation to Upchurch Parish Council.

All works to come with twelve months warranty on all new installations & fittings.

At the time of hand over, the Contractor shall provide Upchurch Parish Council with the following documentation:

1.	Component and equipment list

2.	Product description sheets

3.	Operating and service manuals

4.	Certificate of commissioning

5.     	Electrical Installation Certificate

# TENDER CERTIFICATE

# to be returned by 12 noon on TUESDAY 22nd MARCH 2019

Name / Position:	[INSERT NAME & POSITION]

Company:	[INSERT COMPANY NAME]

Address:		[INSERT FULL COMPANY ADDRESS]

Date:		[INSERT DATE]

RE: PROJECT NO. 2602 Footpath Lighting in UPCHURCH PARISH

| Section: | £ | p |
| --- | --- | --- |
| A Preliminaries/General Conditions (General) Sum : |  |  |
| Health and Safety File |  |  |
| Contingency | 5000 | 00 |
| B Schedule of Works Sum : |  |  |
| - Luminaire Cost |  |  |
| - Utility Pole mount Cost |  |  |
| - Column Cost |  |  |
| - UKPN Costs |  |  |
| - Any items not specifically detailed or included in the specification, but in the opinion of the Tenderer, required for a good and proper, complete and safe installation (list below) |  |  |
| C Total Sum Extended to Form of Tender: |  |  |