Background

Council of Isles of Scilly along with its partners including Isles of Scilly Steamship Group were successful in securing Levelling Up Funds for a vessel replacement project to introduce new vessels to provide lifeline passenger and freight services between Penzance and Isles of Scilly.

Isles of Scilly Steamship Group were already well progressed with the design of three replacement vessels working with their appointed naval architects.

The vessel designs had resulted in three vessels:

- A passenger ferry, also capable of carrying freight to sail between Penzance and St. Mary’s harbour at Isles of Scilly.

- A freight vessel sailing between Penzance and St. Mary’s harbour at Isles of Scilly.

- An inter-island freight vessel capable of transporting freight between the islands of Scilly.

As the next stage of the project, the public sector must undertake a due diligence review of the vessels to confirm they are suitable to deliver the lifeline sea link services prior to competitive procurement.

Scope of Work

For each of the three vessels we wish to undertake a review of its suitability to the intended service covering:

- Overall concept (principal particulars and arrangement)

- Certification and safety

- Passenger and cargo capacity

- Passenger and cargo handling

- Suitability of materials selected for the construction, operations, maintenance and repair

- Seakeeping

- Speed and range

- Power and propulsion including ability to meet net zero by 2050

- Operations

- Reliability

- Maintenance include access to specialist facilities such as aluminium welding and access to local dry dock facilities

- Suitability for the intended route

- Maturity of the concept designs to allow procurement to be undertaken and for shipyards to be able to convert the concept designs into final designs

As the vessels will be providing lifeline services, the design assurance must take into account the target route for the vessels and the specific challenges this presents, including the harbours of St Mary’s and Penzance.

Method

We intend to hold a kick-off meeting with our partner, Isles of Scilly Steamship Group, who are the operator of the current vessels serving the route and their appointed naval architects to understand the statement of requirements for each vessel and the factors which have led to the current designs.

The following documents shall be made available for all three vessels:

- Statement of requirements

- General Arrangement

- Overview of typical cargoes carried

- Build specification or tender specification

- Stability documentation including weight estimate

- Midship section and structural drawings

- Systems drawings or descriptions

- Overview of the ship build contracting strategy.

Access to Isles of Scilly Steamship Group resources and their naval architects shall be made available to clarify any points.

Observations and recommendations arising as a result of our review of the above will be collated and summarised in a draft report for comments and provided to Council of Isles of Scilly. Any comments shall then be addressed and resolved in a formal final issue to Council of Isles of Scilly.

During the execution of this scope we would propose to meet weekly to discuss progress, outstanding information, and ensure alignment with your requirements throughout.

Deliverables and Timescales

The draft report shall be delivered no later than three months from instruction to proceed, assuming all required information is provided at that time.

It is scheduled that the design assurance needs to be concluded before end of January 2023.