Pre-construction Information

for tender purposes only

Community Hub and Office Extension to the John Lampon Hall

of the Orpen Memorial Hall Complex, Lexden Road, West Bergholt

Description of project

Project description and programme details:

- Anticipated dates (start and finish dates of the construction phase)

Start date: 01 September 2023, completion date: 30 November 2023

- Details of WBPC, McLean Architectural, and other consultants

Client: West Bergholt Parish Council, 80 Chapel Road, West Bergholt, Colchester, Essex, CO6 3HB

clerk@westbergholt-pc.gov.uk

07726 424419

- Designer: McLean Architectural, Suite 1, The Treble Tile, Colchester Road, West Bergholt, Essex, CO6 3JQ

studio@mcleanarchitectural.co.uk

01206 241809

- Other consultants: TBC

- Extent and location of existing records and plans

There are no existing records or plans currently available.

- Project description: The works involve the construction of a single storey pitched roof extension of cavity wall construction to the John Lampon Hall, Lexden Road, West Bergholt, Colchester, Essex for use as a Community Hub and Parish Office, which will be self-contained in respect of its own welfare facilities and access.

Client’s considerations and management requirements

Arrangements for:

- Planning for and managing the construction work, including health and safety goals

The management of the project will be by McLean Architectural, who will liaise with the Principal Contractor on all aspects of planning the construction. The goal of West Bergholt Parish Council is for no reportable accidents to take place during the course of the construction.

- Communication and liaison between WBPC and others

Communication and liaison will be via McLean Architectural; with the Parish Clerk being the Council’s primary point of contact.

- Security of the site

Security of the site of the works, and the works, will be the sole responsibility of the Principal Contractor.

- Welfare provisions

Adjacent to the site is an externally accessible disabled access toilet, which will be shared with users of the adjacent playing, but which during the course  of the works must be kept clean by the Principal Contractor whilst on site. There is no shelter from inclement weather  or facilities for making of hot drinks, etc.

Requirements relating to the health and safety of WBPC’s employees, Councillors or those involved in the project

- Site hoarding: No site hoarding is required but the works area and Principal Contractor’s compound will be required to be fenced off.

- Traffic Management Plan and vehicle movement restrictions: The adjacent carpark has a clear entrance and exit that the Principal Contractor is to be adopt SAVE if delivery vehicles are unable to use, in which case banks men will be required to be employed by the Principal Contractor to ensure safe ingress and agree form the car park. The car park is used to a varying extent every day depending upon the use of the Hall. No temporary access will need to be formed.

- WBPC’s permit-to-work arrangements: Currently the Parish Council does not have any permit to work arrangements.

- Fire precautions: The Principal Contractor is to develop and agree with McLean Architectural the extent of fire precautions, emergency procedures and means of escape needed to be adopted during the course of the works.

- Restricted areas or other authorisation requirements: The Orpen Hall and John Lampon Hall are in all respects off limits to the Principal Contractor save for service connections by prior arrangement with McLean Architectural.

- Any areas WBPC has designated as confined spaces: Currently the Parish Council has not designated any space as confined.

- Smoking and parking restrictions: No smoking will be allowed on site. Access to the site is via the Hall car park, the extent to which such may be used for parking their vehicles is to be in designated areas as required to be agreed in advance with McLean Architectural.

Environmental restrictions and existing on-site risks

- Restrictions on deliveries, waste collection and storage: Other than ensuring the safe delivery of materials and waste collection, with suitable protection of the surface of the existing car park as deemed necessary, there are no restrictions SAVE ensuring the safe use of the carpark at school dropping off and collection times of children aged from one to eleven.

- Adjacent land/building uses: The Orpen and John Lampon Halls are used five days a week during term time as a nursery run by Bluebells.  The Lorkin Daniell field is used for sporting purposes, as a children’s playground and has two tennis courts, with the use of all facilities varying from day to day, etc. There are no height restrictions, save overhead services.

- Existing storage of hazardous materials: There are currently no known hazardous materials stored in the vicinity of the proposed Community Hub/Parish Office extension.

- Location of existing services – water, electricity, gas, etc.: Foul water drainage runs parallel to the proposed Lexden Road facing elevation, beneath the glazed link between the Orpen Memorial Hall and John Lampon Hall. All services are present in the boiler room at the corner of the Orpen Hall nearest to the site of the proposed work. The routes of the services below ground are not known.

- Ground conditions, underground structures etc. At the time of tender, ground conditions that might affect the safe use of plant, or the safety of groundworks are unknown, however, the foundations of the John Lampon Hall are said to be trench fill/strip and relatively shallow on a matrix of clay, silt sand and gravel. No contamination survey has been undertaken.

- Previous structural modifications, etc.: The Orpen Hall and John Lampon Hall are considered stable of cavity wall construction, with tiled roof and to be partly framed. There are limited locations at which asbestos has been identified. One such location is the ceiling of the aforementioned boiler room.  The lean-to sections to the Orpen Hall are extensions to the original hall, as is the John Lampon Hall. The Hall is well maintained and is not known to have suffered fire damage or similar at any stage.

- Health and safety information contained in earlier construction or ‘as-built’ drawings: Currently there are no as built drawings available, save for the refurbishment this year of the toilets used by the Social Club, facing the tennis courts and the external w c, which will be made available for inspection to the Principal Contractor.

Significant design and construction hazards

- Significant design assumptions and suggested work methods, sequences, etc.:  There are not considered any unusual aspects to the design, nor any specific methods or sequencing of construction proposed.  Similarly, there are not considered any materials that will require any particular precautions that a competent Principal Contractor would not have identified before commencing work.

The health and safety file

  - The format and conditions relating to the H & S file: The nature and content of the health and safety file is to be agreed in advance with McLean Architectural.