Asset Defect Data Standard for ASC

Network Delivery and Development Directorate

- Document Control

| Document Title | Asset Defect Data Standard for ASC |
| --- | --- |
| Author | Nigel Dowden |
| Owner | Janet Sivorn, National Performance Unit |
| Distribution |  |
| Document Status | Revision v1.0 |

- Revision History

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| Draft – v0.0 | 26-Jan-2012 | Draft – for discussion with Area 9 sample SP | Nigel Dowden |
| Draft – v0.1 | 28-Mar-2012 | Draft – incorporating IAMIS metadata | Nigel Dowden |
| Initial Release – v1.0 | 29-Mar-2012 | Incorporating stakeholder input | Nigel Dowden |

- Approvals

| Name | Document Version | Approval Date |
| --- | --- | --- |
| Jan Sivorn | 1.0 – Initial Release | 29-Mar-2012 |

- Contents

Document Control	2

Revision History	2

Approvals	2

Contents	3

1.	Purpose	3

2.	Interface Overview	3

2.1	Data Sources	3

2.2	Content	3

3.	Data Dictionary Overview	3

4.	Monthly Data File – Structure and Naming Convention	3

4.1	Asset Defect Data – ASC Contract Types and Retro-fit MAC Contracts	3

5.	Asset Defect Data Standard Data Dictionary	3

5.1	Asset Defect Data – ASC Contract Types and Retro-fit MAC Contracts	3

6.	HA Reference Data	3

# Purpose

The purpose of this document is to define the Asset Defect Data that the Highways Agency (HA) requires each Supply Chain contractor to provide on a monthly basis in order to enable AMOR metrics to be reported.

This data is mandated within the Asset Support Contract (ASC) and its associated Asset Maintenance and Operational Requirements (AMOR). The data standard within caters for Areas operated under ASC and MAC with retro-fit contract types by the Service Provider (SP).

# Interface Overview

## Data Sources

The latest AMOR (v1.7) contains Performance Metrics which will be derived from one of three sources:

- SP systems providing operational data – e.g. Time Taken to Make Safe Defects – from the SP’s Routine and Planned Maintenance System (RPMS).

- HA systems providing constructed data – e.g. Number of Flood Events on Trafficked Surfaces – from the HADDMS system, and existing monthly reports.

- HA Service Managers monthly scoring – e.g. Number of Occupancies that caused unacceptable additional delay – these will be collected alongside Compliance Metric data on a monthly basis, using a structured return similar to that used to score PMF

This Data Standard is concerned with the data that should be derived from the SP’s RPMS i.e. operational data.

## Content

This data standard contains details that are common to each asset such as ASSET_GROUP, INVENTORY_CODE and DEFECT_CODE. These are referred to as ‘common asset data’. There is then a number of data elements that are only relevant to the specific defect, referred to as ‘defect details’.

Each record should be populated with two sets of data – the common asset data and the defect details.

This necessarily results in each row of data having a number of unpopulated data elements, in the defect detail sections of unrelated asset groups. This is intentional, in order that the monthly data extract from the SP’s RPMS can be limited to a single ‘de-normalised’ file, instead of requiring multiple header and detail records, which is a more complex process.

# Data Dictionary Overview

The Asset defect Data Standard below is in ‘Data Dictionary’ format.

Each data column that needs to be supplied by the SP on a monthly basis is described as an individual row containing the following information:

| Information | Description |
| --- | --- |
| Name | This is the column name required in the monthly data file |
| Data type | Describes the data type each column should be presented as within the monthly data file |
| Mandatory | When this is Y then the column must be populated in accordance with the described rules. If it is blank then the data load will not fail, however the MAC should follow the guidance in relation to how to populate the field – in some cases the field should be left blank, whilst in others a default value is defined. |
| Comment | Describes any rules pertaining to population of the column. |

# Monthly Data File – Structure and Naming Convention

The monthly data file should be produced as a number of rows (one per Incident Response) within an Excel Spreadsheets (Excel 2003).

## Asset Defect Data – ASC Contract Types and Retro-fit MAC Contracts

The Performance Data file should contain 47 columns of data – starting with HA_AREA, then REPORTING_PERIOD and so on, with the final column being FLEXIBLE_ATTRIBUTE_10.

The file naming convention should be as follows:

- Asset_Defect_AA_YYYYMM.xls where AA is the Area including a leading-edge zero if necessary, YYYY is the year, and MM is the Reporting Month

- E.g. File name for Area 9 reporting in May 2012 on April 2012 defects would be Asset_Defect_06_201205.xls

# Asset Defect Data Standard Data Dictionary

## Asset Defect Data – ASC Contract Types and Retro-fit MAC Contracts

| Name | Data Type | Mandatory | Comment |
| --- | --- | --- | --- |
| HA_AREA | Characters (2) | Y | E.g. 03 or 12. Must contain leading zero |
| REPORTING_PERIOD | Number (6) | Y | Must be of format YYYYMM |
| DEFECT_ID | Characters (8) | Y | Unique ID from RPMS |
| ASSET_ID | Character(25) | Y | Unique ID for Asset |
| ASSET_GROUP | Characters(255) | Y | Constrained via REF DATA – e.g. DRAINAGE, PAVEMENT |
| ASSET_ELEMENT | Character(255) | Y | Constrained via REF DATA – e.g. CARRIAGEWAY, GULLIES |
| ASSET_SUB_ELEMENT | Character(255) | Y | Constrained via REF DATA – e.g. HARD SHOULDER, RUNNING LANES |
| ASSET_DESCRIPTION | Character(255) | C | Mandatory where Asset = Sweeping and Cleaning and where DEFECT_EVENT_TYPE is the code for Amenity Site Cleaning |
| INVENTORY_CODE | Character(2) | Y | Constrained via REF DATA – e.g. CW, HS, GY |
| DEFECT_CODE | Character(4) | Y | Constrained via REF DATA – e.g. POTH, CRCK |
| DEFECT_DESCRIPTION | Character(255) | Y | Textual description of defect |
| SECTION_REF | Character(255) | Y | Reference in accordance with NMM Part 2, Annex 2.3.1 and Annex 2.4.1 e.g. 1900A1M/25, 4720A6210/103, 1900M62/105 |
| ROAD_CODE | Character(4) | Y | Reference data pertaining to the HAPMS Section - E.g. S2/D2/D4M |
| SECTION_FUNCTION . | Character(25) | Y | Reference data pertaining to functions - E.g. MAINLINE, ENTRY SLIP, EXIT SLIP, JUNCTION, ROUNDABOUT |
| DIRECTION_CODE | Character(2) | Y | Constrained values from REFERENCE DATA – e.g. N, S, E, W, CW, AC, RB |
| DEFECT_XSP | Character(3) | Y | Constrained by valid XSP codes – E.g. –R2, LB, CL1, OL3 |
| DEFECT_NORTHING | Number(6) | Y | E.g. 123456 |
| DEFECT_EASTING | Number(6) | Y | E.g. 234567 |
| DEFECT_LOCATION_DESCRIPTION | Character(255) | N | Description |
| DEFECT_EVENT | Character(12) | Y | Constrained to one of NOTIFICATION, INSPECTION |
| DEFECT_EVENT_TYPE | Character(10) | Y | Constrained to valid event types describing the detail below NOTIFICATION/INSPECTION e.g. BP (stands for Balanced Pond) |
| DEFECT_NOTIFICATION_DATE_TIME | Date & Time | C | DDMMYY HH:MM – mandatory when DEFECT_EVENT = NOTIFICATION |
| DEFECT_INSPECTION_DATE_TIME | Date & Time | C | DDMMYY HH:MM – mandatory when DEFECT_EVENT = INSPECTION |
| MAKE_SAFE_DATE_TIME | Date & Time | C | DDMMYY HH:MM – one of MAKE_SAFE_DATE_TIME or PERMANENT_REPAIR_DATE+TIME is required |
| PERMANENT_REPAIR_DATE_TIME | Date & Time | C | DDMMYY HH:MM – one of MAKE_SAFE_DATE_TIME or PERMANENT_REPAIR_DATE+TIME is required |
| REPAIR_TYPE | Character(9) | Y | Constrained: MAKE_SAFE or PERMANENT |
| DEFECT_PRIORITY | Character(4) | Y | Priority allocated to DEFECT. Normally numeric, but will still include CAT1 and CAT2 where specified in AMOR. |
| DEFECT_TYPE | Character(10) | Y | Constrained to either SAFETY or NON-SAFETY |
| FLOOD_NOTIFICATION_DATE_TIME | Date & Time | C | DDMMYY HH:MM – mandatory where ASSET_GROUP, ASSET_ELEMENT and DEFECT_CODE indicate a flood on trafficked suface |
| FENCES_DEFECT_TYPE | Character(15) | C | Mandatory where ASSET_GROUP etc. is Fences, Screens, Env Barriers. Constrained to one of: PERFORMANCE or SAFETY_STABILITY |
| PAVEMENT_WARNING_TIME | Time | C | Mandatory where ASSET_GROUP etc. is Pavements. HH:MM |
| PAVEMENT_DEFECT_TYPE | Character (15) | C | Mandatory where ASSET_GROUP etc. is Pavements. Constrained: POTHOLE, DEFORMATION, IRONWORK, TRIP_HAZARD |
| PAVEMENT_DEFECT_DIAMETER | Number(3) | C | Mandatory where ASSET_GROUP etc. is Pavements and DEFECT_TYPE = POTH. Number in mm. |
| PAVEMENT_DEFECT_DEPTH | Number(3) | C | Mandatory where ASSET_GROUP etc. is Pavements and DEFECT_TYPE = POTH. Number in mm. |
| PAVEMENT_DEFECT_LEVEL | Number(3) | C | Mandatory where ASSET_GROUP etc. is Pavements and DEFECT_TYPE <> POTH. Number in mm. |
| RESTRAINT_IMPACT_DAMAGE | Character(1) | C | Mandatory where ASSET_GROUP etc. is Road Restraint Systems. Y/N |
| CLEANING_AMENITY_DATE_TIME | Date & Time | C | Mandatory where ASSET_GROUP etc. is Sweeping and Cleaning |
| FLEXIBLE_ATTRIBUTE_1 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_2 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_3 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_4 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_5 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_6 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_7 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_8 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_9 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_10 | Variable |  | This fields is not yet defined |

# HA Reference Data

There will be a number of Reference Data which need to be maintained in order to make constrained value checks including but not limited to:

- ASSET GROUP | ASSET ELEMENT | ASSET SUB-ELEMENT | INVENTORY CODE | ASC OPERATIONAL REQUIREMENT

- DEFECT CODES

- ROAD CODES

- SECTION FUNCTIONS

- DIRECTION CODES

- SECTION AUTHORITY

- SECTION XSP

- DEFECT EVENT | DEFECT EVENT TYPE

- REPAIR TYPE | DEFECT CATEGORY

- FENCES DEFECT TYPE

- PAVEMENT DEFECT TYPE