PRE-CONSTRUCTION INFORMATION PACK

FOR

HV Cable Oil Project.

AT

South Tees Site.

| Document Reference No.: | HVCO00001 |
| --- | --- |

|  | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared by: | Rod Lavan |  | 19/08/2020 |
| Reviewed by: | John Noteyoung |  | 08/10/2020 |
| Approved by: | John Noteyoung |  | 08/10/2020 |

Revision Record

| Revision No. | Date of Revision | Reason |
| --- | --- | --- |

South Tees Site Company

Teesside Management Offices

Redcar

TS10 5QW

Document ‘hold’ Register

All ‘HOLDS’ in this document shall be recorded here. The purpose of the ‘HOLD’ register is to ensure that there are no data that could potentially threat the progress of the project.

| HOLD No. | DESCRIPTION | REFERENCE |
| --- | --- | --- |

- Contents

MAP/LOCATION	5

ABBREVIATIONS	6

INTRODUCTION	7

1	Description	8

1.1	Project Details	8

1.2	Key Dates	8

1.3	Project Location	8

1.4	Nature of work - Brief Overview	8

1.5	Project Duration	8

2	Existing Environment	9

2.1	Utilities	9

2.2	Surrounding Land Uses, Processes and Related Restrictions	9

2.3	Residual Risks	9

2.4	Sumps	9

2.5	Existing Underground Services / Status of Isolations	9

2.6	Access and Egress, Traffic Movements	9

2.7	Ground Conditions	9

2.8	Chemical Hazards	9

2.9	Asbestos / Ceramic Fibre	9

2.10	Biological	9

2.11	Previous Contaminants	9

2.12	Ecological / Environment	9

2.13	Electrical	10

2.14	Decommissioning	10

3	Existing Drawings	10

4	Design	10

4.1	Risk of Fire / Explosion	10

4.2	Accumulation of Explosive Vapours	10

4.3	Excavations	10

4.4	Working at Height	10

4.5	Manual Handling	10

4.6	Ionising Radiation	10

4.7	Demolition	10

4.8	Housekeeping	10

4.9	Risk Assessments / Method Statements (RAMS)	10

5	Materials of Construction	10

6	Site-Wide Elements	11

6.1	Provision of Welfare Facilities	11

6.2	Provided by Client	11

6.3	Provided by the Principal Contractor	11

6.4	Loading / Unloading, Layout & Storage Areas	11

6.5	Site Security	11

6.6	Noise on Site	11

6.7	Health Monitoring	11

6.8	Records	11

6.9	Environmental Requirements	11

7	Site Rules	11

7.1	General	11

7.2	Permits to Work System	11

7.3	Hot Work Permit	11

7.4	Confined Space Entry Permits	11

7.5	Signing In & Out	12

7.6	Smoking, Alcohol & Drugs	12

7.7	Personnel Protection	12

7.8	Accident Reporting and First Aid	12

7.9	Fire Regulations	12

7.10	Security General	12

7.11	SHE Audits	12

7.12	Competence	12

7.13	Plant and Equipment	12

8	CONTINUING LIAISON	12

8.1	Liaison Meetings	12

8.2	Amendments and Revisions	12

8.3	Relevant Legislation	13

9	SITE PLANS	13

9.1	Key Plans	13

9.2	Process Plans	13

9.3	Utility System Plans	13

10	HEALTH & SAFETY FILE	14

11	Appendices	14

Appendix A	DRAWINGS	14

Appendix B	Previous Repair Images.	14

# MAP/LOCATION

###### South Tees Site Company. Teesworks.

###### Figure 1: STSC Redcar Works.

Figure 2: Ariel View of the South Tees Site.

# ABBREVIATIONS

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| CDM | Construction Management Design [Regulations] |
| COMAH | Control of Major Accident Hazards Regulations. |
| EA | Environmental Agency |
| EIA | Environmental Impact Assessment |
| HAV | Hand Arm Vibration |
| HSE | Health & Safety Executive (www.hse.gov.uk) |
| HSEQ | Health, Safety, Environment and Quality |
| LEL | Lower Explosive Limits |
| MSDS | Material Safety Data Sheets |
| N/A (n/a) | Not Applicable |
| PPE | Personal Protective Equipment |
| RAMS | Risk Assessments and Method Statements |
| SSoW | Safe System of Work |
| WIP | Work In Progress |

# INTRODUCTION

This project falls within the requirements of the Construction (Design & Management) Regulations 2015

The purpose of this document is to provide a clear understanding of the site’s state and condition.

The information provided will enable the Principal Contractor to develop a Construction Phase Plan (CPP) including within it the design and developed safe systems of work and control measures. The Principal Contractor must also maintain a Health and Safety File which on completion is passed to the Principal Designer. This should be undertaken along with all the other co-ordinating and safety roles associated with the role of Principal Contractor and as required by the Construction (Design & Management) Regulations 2015.

The Principal Contactor and other sub-contractors will need to familiarise themselves with relevant legislation as listed in 8.3 of this document. Any information within this Pre-Construction Information Pack including appendices / attachment's which have been obtained from third parties i.e. sub-scan drawings, handover reports, asbestos surveys etc., have not been independently verified by Client / Principal Designer unless otherwise stated within the Pre-Construction Information Pack.

PROJECT BACKGROUND INFORMATION.

South Tees Site Company Limited has been appointed to maintain the safety of the Redcar site (formally SSI Steel).

On 2 October 2015 the SSI steel works in Redcar was placed into compulsory liquidation and an official receiver (OR) was appointed as liquidator.  On 12 October, following no buyer for the steel works being found, the decision was taken by the official receiver to set about the hard closure of the site.  Since that time the official receiver undertook a protracted liquidation of SSI and, in the absence of a new owner, he has been overseeing the safe and secure closure of the former SSI site whilst maintaining the delivery of services to other site residents.  Government, through the Department for Business, Energy and Industrial Strategy, provided an indemnity to the OR so that he could carry out his duties as liquidator of the company and ensure its ongoing safety and security.

On the 1st December 2016, The Department established a Government company, known as the South Tees Site Company Limited (STSC), in order to take forward the safety and security of the site from the OR.  STSC have a management team as well as a board of directors, accountable to the BEIS Secretary of State.  In order to allow the board of directors and management team to carry out their duties, as well as funding the operation of the Company, BEIS has agreed to indemnify them against all claims, proceedings, costs - including the cost of defending proceedings - and expenses.

On the 8th October 2020 STSC became a wholly owned subsidiary of South Tees Development Corporation and part of the wider Tees Valley Combined Authority Group.

# Description

Electricity physically enters the site via National Grid 275kV overhead cables into two substations (Grid Connection Points).  At each substation there are two super-grid transformers (SGT) where the voltage is reduced from 275kV to 66kV: -

- Corridor	(SGT1 & SGT2)

- Grangetown	(SGT1B & SGT2B)

From these substations, at each of the grid connection compounds, power is transmitted throughout the site via an underground 66kV oil-filled cable network.

The construction of these cables involves the use of electrical insulating oil, maintained at a pressure of c.25psi.  Cable oil pressures are routinely monitored, with repeated oil pressure reduction being a clear indication of a cable leak.  Low oil pressure can result in reduced insulating properties, and ingress of moisture, leading to a catastrophic cable failure.

During the life of a cable it is not uncommon for leaks to occur which breach the outer insulating layer of the cable leading to a gradual release of the oil into the surrounding ground. Such leaks can result from a number of sources, including but not limited to:-

- Poorly installed cable resulting in gradual damage to the outer insulation

- Ground movement, leading to damage to outer insulation

- Cable joint location. Natural weak point along the cable.

The 66kV network is operated and maintained by the Power Distribution team working for South Tees Site Company.  Recently, two cables have shown significant signs of leakage: -

- Dock Road Indoor 66kV Substation to Transformer 67 at South Bank Coke Ovens

- Corridor 66kV Substation to REF Mill 66kV Substation.

Attempts have been made to locate these leaks at previous repairs, but to no avail.  The leaks remain, and a more efficient method of locating and identifying the leaks is now required.  The use of Perfluorocarbon Tracers (PFT) has been identified as such a method.

Project Details

Client:			South Tees Site Company Ltd

Management Offices

Redcar

Teesside

TS10 5QW.

Principal Contact:	Mr John Noteyoung.

Principal Designer:	TBC

Principal Contact:	[TBC]

Principal Contractor:	TBA

Principal Contact:		TBA

Designers:			TBA

Principal Contact:		TBA

## Key Dates

- Proposed start date for the works is 	[TBC]

- Site Visit 				[TBC]

- Tender returns			[TBC]

- Mobilisation 				[TBC]

## Project Location

The possible work areas are somewhere along the length of each cable. The exact point of the oil leak on both cables is to be identified as part of the project repair.

At present all that is known is that the Corridor – Ref Mill cable is leaking somewhere between Corridor 66kV substation on Redcar site and the REF Mill 66kV sub station on Lackenby site. This cable is approximately 2.5Km’s in length

The second cable runs from Dock Road Indoor 66kV substation at Cleveland (Dorman Point) to Transformer 67 at the South Bank Coke Ovens plant. This cable is approximately 1.3Km’s in length.

The Corridor – REF Mill cable is buried underground for its complete length. The cable route is covered in heavy vegetation. There are some cable joint markers along the works fence line close to the A1085 Trunk Road footpath.

The Dock Road Cable is buried underground for approximately 80% of its route. Again, this route is covered in heavy vegetation and runs parallel with the Saltburn to Darlington Network Rail line. The remaining approximate 20% of the cable is enclosed inside metal trunking which is above ground.

## Nature of work - Brief Overview

The nature of the work covered by this PCIP is:-

To identify the exact location of the oil leak/leaks on both cables.

The leak/leaks will be identified by the use of Perfluorocarbon Tracer (PFT) tagged

Cable oil. The leaks will then be repaired either by a new section of cable will be jointed in or the cable can be sleeved. The type of repair will depend on how bad the oil leak/leaks are on each cable. An excavation will be required to carry out a repair to the Corridor cable but the Dock Road cable may require scaffold access if the damage is found to be above ground.

## Project Duration

The proposed start date for the works is [TBC].

The duration of works is estimated at ten (10) working days, however this will be confirmed on appointment of the Principal Contractor.

# Existing Environment

## Utilities

As per Appendices

## Surrounding Land Uses, Processes and Related Restrictions

Due to the proximity of both cable routes to Public footpath and public roadways. Redcar and Cleveland Borough Council may have to be informed of the planned work front. Also, Network Rail may need to be made aware of possible work being carried out adjacent to the Railway line.

## Residual Risks

During each cable repair there is a risk to site supplies. This is because the REF Mill will be supplied by one 66kV feeder during the Corridor -Ref 2 cable repair and a similar risk applies to the Dock Road Indoor - T67 cable repair at South Bank, as this will only be supplied by T68 during that cable repair.

## Sumps

N/A.

## Existing Underground Services / Status of Isolations

Existing underground services do exist on site but there is segregation between water, gases, steam, and electricity. Only the cables that are going to be repaired will be isolated. Any excavation work will be carried out by trained and

## Access and Egress, Traffic Movements

All access and egress to site and the work area will be only be recognised roadways and footpaths. There will be no large traffic movements. All traffic will follow the site road rules and speed limits.

## Ground Conditions

Ground conditions will be uneven in many areas. Underfoot conditions could be hazardous. Care should be taken when walking along the footpaths and cables routes.

## Chemical Hazards

There are no known chemical hazards in the work areas however contaminated spoil will have to be assessed to ascertain its hazardous properties and then removed from site.

Hazard Id sheet & COSHH’s reports required for the Perfluorocarbon Tracer (PFT) tagged cable oil and any cable jointing chemicals.

## Asbestos / Ceramic Fibre

There are no known asbestos / ceramic fibres in the work areas however contaminated spoil will have to be assessed to ascertain its hazardous properties and then removed from site.

## Biological

There are no known biological elements in the work/excavation areas, however contaminated spoil will have to be assessed to ascertain its hazardous properties and then removed from site.

## Previous Contaminants

Previous contaminated spoil was contained on a ground sheet and was assessed to ascertain its hazardous properties and then removed from site.

## Ecological / Environment

Depending on location we may be able to reference the 2004 Enviros study for an assessment of ground conditions. The more recent CH2M material may also be available.

## Electrical

Any live systems adjacent to work area will be identified prior to commencement of task.

## Decommissioning

N/A.

# Existing Drawings

A selection of available drawings showing cable routes and services can be

found in Appendix A.

.

# Design

## Risk of Fire / Explosion.

Risk Assessment to identify any work that may increase fire risk and necessary mitigations to be in place.

Hot Works will be required and should be suitably detailed within task specific risk assessment. A Hot Works Certificate will be issued. The PC to supply Fire Extinguishing equipment suitable to cover the fire risk

## Accumulation of Explosive Vapours

N/A.

## Excavations

STSC Excavation Policy must always be adhered to. Contaminated spoil will have to be assessed to ascertain its hazardous properties and then removed from site.

## Working at Height

Scaffold/MEWP may be required at some points on cable route. Safety paperwork covering the use of a safety harness will be required if work is required from a MEWP.

## Manual Handling

Any required lifting should be mitigated in task specific risk assessment

## Ionising Radiation

There are no known Ionising radiation instruments within the work areas.

## Demolition

N/A.

## Housekeeping

Work area to be kept clean and free from debris. All waste material is to be removed fron site by the PC and disposed of correctly. Contaminated Spoil should be held in a suitable location (ie on tarpaulin or in a skip). General waste management should be identified within construction phase plan

## Risk Assessments / Method Statements (RAMS)

Risk Assessments to be provided by PC and reviewed and approved by STSC

# Materials of Construction

N/A

# Site-Wide Elements

## Provision of Welfare Facilities

WC & wash facilities available at AWS offices. No provisions in remote areas of plant. PC to arrange mobile welfare facilities where necessary.

## Provided by Client

Non provided.

## Provided by the Principal Contractor

PC to arrange mobile welfare facilities where necessary.

## Loading / Unloading, Layout & Storage Areas

Loading, laydown, and storage area’s to be agreed between STSC and PC prior to any equipment arriving on site.

## Site Security

The PC will have to ensure all equipment including mobile plant, cabling and tools are to be made safe and secure outside of working hours.

## Noise on Site

No populated areas close to cable routes. Hearing protection to be part of basic PPE and to be used when required.

## Health Monitoring

STSC Emergency Medical/Security Procedures to be identified to PC in PCIP

## Records

As per appendices

## Environmental Requirements

Assessment of area to be made before work commences in an area that may disturb wildlife or encounter hazardous flora/fauna. Depending on location we may be able to reference the 2004 Enviros study for an assessment of ground conditions. The more recent CH2M material may also be available.

# Site Rules

## General

Site HV Rules (EIS5), Site LV Rules (EIS7), Inductions, Excavation Policy, Confined Space Policy, WAH Policy.

## Permits to Work System

Site policies and procedures to be followed as required.

## Hot Work Permit

Risk Assessment to identify any work that may increase fire risk and necessary mitigations to be in place.

Hot Works will be required and should be suitably detailed within task specific risk assessment. Refer also to section 4.1.

## Confined Space Entry Permits

Site requirements, policies, and procedures to be followed as required.

## Signing In & Out

Site requirements, policies, and procedures to be followed as required.

## Smoking, Alcohol & Drugs

Site policies and procedures to be followed as required.

## Personnel Protection

PPE required as detailed in job specific R/A.

## Accident Reporting and First Aid

Site policies and procedures to be followed as required.

## Fire Regulations

Site policies and procedures to be followed as required.

## Security General

Site policies and procedures to be followed as required.

## SHE Audits

Site Shift Management team to carry out daily audit of work area. Working Party to

Take “2” and “POWRA” prior to commencing daily work.

## Competence

Competency of contractor personnel to be confirmed by the PC. Training records and qualifications to be produced to Facilities Dept as part of the Contract approval.

## Plant and Equipment

A comprehensive list of Plant and equipment to be provided by the PC prior to commencing work.

# CONTINUING LIAISON

## Liaison Meetings

Daily update meetings to be held between PC and STSC project manager. Weekly meeting to beheld with PC and senior management team.

## Amendments and Revisions

N/A

## Relevant Legislation

Relevant legislation shall be referenced in the Construction Phase Health and Safety Plan; this may include but not be limited to the following:

- Health & Safety at Work Act 1974

- Management of Health & Safety at Work Regulations 1999

- Reporting of Injuries, Diseases and Dangerous Occurrences Regulations

2013(RIDDOR)

- The Work at Height Regulations 2005

- Control of Asbestos Regulations 2012

- Lifting Operations Lifting Equipment Regs (LOLER)

- The Confined Space Regulations 1997

- Provision & Use of Work Equipment Regulations 1998 (PUWER)

- Construction (Design & Management) Regulations 2015

- Workplace (Health, Safety and Welfare) Regulations 1992

- Personal Protective Equipment Regulations 2002

- Manual Handling Operations Regulations 1992 as amended in 2002

- COSHH Regulations 2002

- Noise at Work Regulations 2005

- Health & Safety (First-Aid) Regulations 1981as amended 2013

- HSE Guidance Note HS(G) 150 Healthy and Safety in construction (Third Edition)

- HSE Guidance Note HS(G) 47 Avoiding danger from underground services

- CS15 The Cleaning and Gas Freeing of Tanks Containing Flammable Residues

- Rendering safe and demolition of Storage tanks (LFEPA)

- BS 6187 Code of practice for Demolition

- Duty of Care (EPA 1990)

- Electricity at work HSG85 (Third edition) 2013

- Hand-arm vibration The Control of Vibration at Work Regulations 2005 Guidance on Regulations L140

Including any updates to these regulations and compliance with associated guidance.

# SITE PLANS

## Key Plans

N/A

## Process Plans

N/A

## Utility System Plans

See Appendix A

# HEALTH & SAFETY FILE

The following information shall be provided on completion of the work, this shall be in the form of a signed and dated ‘Completion Statement’.

Sections shall include but not be limited to the following:

- Introduction

- Parties to the project including contact details

- Works completed (specific details)

- Durations

- Services (Services isolated and those remaining, including status of any interceptors)

- Structures (Details of remaining structures, asbestos surveys (for remaining structures only), ground coverings, remaining enabling works etc.)

- Drawings (Relevant drawings only, to support the above)

- SHE (Description of any events or conditions which may be useful to any owner of the land.)

- Imported Fill (Description and Quantity of fill material including certificate of analysis / origin.)

- Security (Current Status.)

- Hazard Data Sheets

- Other (Any additional information, which may be useful to any occupier, at a future date.)

In addition to the above, the following shall be provided under separate cover for [Insert Client] retention.

- Waste.
Copies of carrier and waste disposal site / transfer station licences.

- Copies of pre-notification and copies of consignment notes / Copies of Duty of Care notes etc.

Asbestos Monitoring and Clearance Certificates, (if applicable).

# Appendices

## Appendix A	Drawings and Specifications

## Appendix B	Previous Repair