SYSTEM REQUIREMENT DOCUMENT (SRD)

FOR EMBODIMENT OF FASGW CAPABILITY ONTO THE WILDCAT

SYNTHETIC TRAINING EQUIPMENT (STE)

| Issue Status: |  | Version 1.1 |
| --- | --- | --- |
| Date of Issue: |  | 09 Mar 2018 |
| Author: | FOIA 2000 section 40 Personal Information | DES Lynx-Wildcat Training Configuration Manager |
| Authorised by: | FOIA 2000 section 40 Personal Information | DES Lynx-Wildcat Projects Manager |
| Endorsed by: | FOIA 2000 section 40 Personal Information | DES Lynx-Wildcat Requirements Manager |

THIS DOCUMENT IS THE PROPERTY OF HER BRITANNIC MAJESTY'S GOVERNMENT, and is issued for the information of such persons only as need to know its contents in the course of their official duties. Any person finding this document should hand it in to a British forces unit or to a police station for its safe return to the MINISTRY OF DEFENCE, (DSy(Pol)), MAIN BUILDING, WHITEHALL, LONDON, SW1A 2HB, with the particulars of how and where found.

THE UNAUTHORISED RETENTION OR DESTRUCTION OF THE DOCUMENT MAY BE AN OFFENCE UNDER THE OFFICIAL SECRETS ACT OF 1911 - 1989.

When released to persons outside Government service, this document is issued on a personal basis and the recipient to whom it is entrusted in confidence, within the provisions of the Official Secrets Acts 1911-89, is personally responsible for its safe custody and for seeing that its contents are disclosed only to authorized persons.

CONTENTS

REFERENCES	iii

Acronyms	iii

PART ONE - INTRODUCTION	1

2	Background	1

3	Objective	1

4	Document structure	1

5	Requirement Descriptor Definitions	1

PART TWO - AIRCREW TRAINING EQUIPMENT	3

6	ATE System Requirements Purpose	3

7	ATE System Requirements Scope	3

8	Training Objectives	3

9	ATE Key User Requirements	6

10	ATE System Requirements	7

11	ATE Requirements to be provided as Options	12

PART THREE - MAINTAINER TRAINING EQUIPMENT	14

12	MTE System Requirements Purpose	14

13	MTE System Requirements Scope	14

14	MTE System Requirements Overview	14

15	MTE Training Objectives	14

16	MTE Key User Requirements	16

17	MTE System Requirements	16

18	MTE Requirements to be provided as Options	19

PART FOUR - NON-FUNCTIONAL REQUIREMENTS	22

19	Non-Functional STE Requirements	22

PART FIVE – FACTORY DELIVERED TRAINING	25

20	Factory Delivered Training Baseline Requirement	25

21	Factory Delivered Training Non-baseline Options	28

LIST OF TABLES

Table 1 - Requirements Descripter Definitions	2

Table 2 - ATE Training Objectives	6

Table 3 - ATE Key User Requirements	7

Table 4 - ATE System Requirements	12

Table 5 - ATE Requirements to be provided as Options	13

Table 6 - MTE Training Objectives	16

Table 7 - MTE Key User Requirements	16

Table 8 - MTE System Requirements	19

Table 9 - MTE Requirements to be provided as Options	20

Table 10 - Non-Functional STE Requirements	24

Table 11 - FDT Course 1A (Aircrew)	25

Table 12 - FDT Course 1B (Maintainers)	26

Table 13 - FDT Course 2A (Aircrew)	26

Table 14 - FDT Course 2B (Maintainers)	27

Table 15 - FDT Course 3A (Aircrew)	28

Table 16 - FDT Course 3B (Maintainers)	28

Table 17 - Non-baseline FDT Course 4A (Aircrew)	29

Table 18 - Non-baseline FDT Course 4B (Aircrew)	29

Table 19 - Non-baseline FDT Course 4C (Maintainers)	29

## REFERENCES

A.	Comprehensive Core Requirement (CCR) Iss 3, dated 25 May 2010.

B.	Contractual System Requirements Document (cSRD) for Wildcat TCE, dated 25 Nov 2011.

C.	FASGW Training Need Analysis Final Report, dated 03 Jun 2013.

## Acronyms

| A/C | Aircraft |
| --- | --- |
| AAR | After Action Review |
| ALGU | Airborne Laser Guidance Unit |
| AMS | Avionics Management System |
| AOR | Area of Responsibility |
| ASG | Acquisition System Guidance |
| ASM | Air to Surface Missile |
| ATE | Aircrew Training Equipment |
| ATM | Acceptance Test Manual |
| ATP | Acceptance Test Plan/Procedure |
| AW | AgustaWestland Limited |
| AWW | Above Water Warfare |
| BDA | Battle Damage Assessment |
| BFE | Buyer Furnished Equipment |
| BFX | Buyer Furnished Items |
| BIT | Built in Test |
| CIETP | Compound Interactive Electronic Technical Publication |
| CBIT | Continuous Built in Test |
| CCD | Cursor Control Device |
| CCR | Comprehensive Core Requirement |
| CDR | Critical Design Review |
| CDS | Cockpit Display System |
| CGF | Computer Generated Forces |
| CiC | Captured in Contract |
| CMDS | Counter Measures Dispensing System |
| COTS | Commercial of the Shelf |
| CPS | Cardinal Point Specification |
| cSRD | Contractual System Requirements Document |
| D&M | Design and Manufacture |
| DAMASS | Damage Assessment |
| DAR | Design Assessment Report |
| DAS | Defensive Aids Suite |
| DE&S | Defence Equipment and Support |
| DEGR | Degraded |
| DMSP | Defence Modelling & Simulation Standards Profile |
| DSAT | Defence Systems Approach to Training |
| DSC | Defence Simulation Centre |
| DSTL | Defence Science and Technology Laboratory |
| DTEC | Defence Training and Education Coherence |
| EODS | Electro Optical Designation System |
| EMRU | Electro Magnetic Release Unit |
| EW | Electronic Warfare |
| FAC | Fast Attack Craft |
| FASGW(H/L) | Future Air to Surface Guided Weapon (Heavy/Light) |
| FAT | Factory Acceptance Testing |
| FDT | Factory Delivered Training |
| FMS | Full Mission Simulator |
| FTD | Flight Training Device |
| GATM | Ground Air Training Missiles |
| GFE | Government Furnished Equipment |
| GPMG | General Purpose Machine Gun |
| HMG | Heavy Machine Gun |
| HMI | Human Machine Interface |
| HUMS | Health and Usage Monitoring System |
| HW | Hardware |
| ICBL | IR Calibration Before Launch |
| ICD | Interface Control Document |
| IDU | Integrated Display Unit |
| IGI | Integrated GPS/INS |
| iMMM | Improved Massive Memory Module |
| IOS | Instructor Operating Station |
| IR | Infra-Red |
| IRS | Infra-Red Sensor |
| IRT | Internal Run Test |
| ITT | Invitation to tender |
| KUR | Key User Requirement |
| LHD | Leonardo Helicopters Division |
| LOS | Line of Sight |
| LWPT | Lynx Wildcat Project Team |
| M&S | Modelling & Simulation |
| MAA | Military Aviation Authority |
| MDTL | Missile Data Link Terminal |
| MITL | Man in the Loop |
| MMR | Moving Model Review |
| MOD | Ministry of Defence |
| mQTG | Master Qualification Test Guide |
| MSM | Missile Status Message |
| MTE | Maintainer Training Equipment |
| OEM | Original Equipment Manufacturer |
| OTW | Out of The Window |
| PDR | Preliminary Design Review |
| PIE | Permanently Installed Equipment |
| PSA | Pilot Subjective Assessment |
| PTSL | Pennant Training Systems Limited |
| RA2375 | [MAA] Regulatory Article 2375 [Approval for Use] |
| RFP | Request for Proposal |
| RH | Relative Humidity |
| RN | Royal Navy |
| RNAS | Royal Naval Air Station |
| ROE | Rules of Engagement |
| RTS | Release to Service |
| SAM | Surface to Air Missile |
| SEDTU | Security Enhanced Data Transfer Unit |
| SMC | Stores Management Computer |
| SMS | Stores Management System |
| SoW | Statement of Work |
| SRD | System Requirements Document |
| SRR | System Requirements Review |
| SRS | Software Requirements Specification |
| SRU | Stores Release Unit |
| SSDD | Sub-System Definition Document |
| SSON | Single Statement of Need |
| STE | Synthetic Training Equipment |
| STOT | Simultaneous Time on Target |
| StRS | Stakeholder Requirements Specification |
| SW | Software |
| SyRS | System Requirements Specification |
| TA(UK) | Training Academy United Kingdom |
| TCE | Training Capital Equipment |
| TEM | Training Equipment Manufacturer |
| TNA | Training Needs Analysis |
| TO | Training Objective |
| TP | Tactical Processor |
| TRR | Test Readiness Review |
| TRT | Trial Run Testing |
| TSD | Technical Specification Document |
| URD | User Requirements Document |
| VDB | Visual Databases |
| VDD | Version Definition Document |
| VDR | Visual Database Review |
| W&B | Weight and Balance |
| WCA | Warning Caution Advisory |
| WCP | Weapon Control Panel |
| WIST | Wildcat Integrated Support and Training |
| WLST | Weapon Loading and System Trainer |
| WMSS | Wildcat Mission Support System |
| WTC | Wildcat Training Centre |
| WTS | Wildcat Training Service |

## PART ONE - INTRODUCTION

- Issue

  - Provision of a full Wildcat Training Service (TS), incorporating courseware, training delivery and Synthetic Training Equipment (STE), to train RN aircrew and maintainers in relation to the Future Anti-Surface Guided Weapon (FASGW) capability shall be provided.

- Background

  - The Comprehensive Core Requirement (CCR) document, Reference A, articulated the endorsed Customer Requirements for the Wildcat TS to be delivered by Industry, which formed a key element of the overall training capability for Wildcat Mk1 and Mk2.

  - The CCR was used by Defence Equipment & Support (DE&S) and Industry to inform the Training Service Contract Specification and formed the key measures of success in the subsequent customer acceptance process detailed in the Contractual System Requirements Document (cSRD), Reference B.

  - The cSRD captured the requirements for the Media, Devices and Facilities (MDF) to support Wildcat training. The capabilities and scope of the MDF were fundamentally derived from, and linked to, the Wildcat Training Needs Analysis (TNA), with additional detail provided where considered necessary by the Authority.  Acceptance of the requirements, where appropriate, were linked to the Wildcat training tasks as identified in the TNA, whether for ab-initio, conversion or continuation and currency training.

- Objective

  - This System Requirement Document (SRD) captures the Customer Requirements to support the introduction of the FASGW capability into the Wildcat TS, Aircrew Training Equipment (ATE) and Maintainer Training Equipment (MTE).

- Document structure

  - This requirement is divided into the following parts:

    - Part One.  Introduction.

    - Part Two.  System Requirements for the introduction of FASGW capability into the Wildcat ATE.

    - Part Three.  System Requirements for the introduction of FASGW capability into the Wildcat MTE.

    - Part Four.  Non-Functional Requirements for the introduction of FASGW capability into the Wildcat STE.

    - Part Five.  Factory Delivered Training Requirements.

- Requirement Descriptor Definitions

| Descriptor | Requirement Definition |
| --- | --- |
| Shall | Requirements are mandatory binding provisions, synonymous with “Must”. |
| Could | Non-requirements, which shall be provided as a costed Option. Statement is intended to facilitate system requirement derivation. |
| Shall Not | A mandatory requirement to exclude a feature or capability. |
| Heading | An introductory title describing the following group of requirements |
| Information | A statement providing background information or context for subsequent requirement decomposition |

Table 1 - Requirements Descripter Definitions

## PART TWO - AIRCREW TRAINING EQUIPMENT

SYSTEM REQUIREMENTS FOR THE INTRODUCTION OF FASGW CAPABILITY INTO THE WILDCAT

- ATE System Requirements Purpose

  - There is a requirement to upgrade to the existing ATE in order to address, through hardware and software modifications, the introduction of the FASGW systems functionality, Human Machine Interface (HMI) and Synthetic operating environment (including Computer Generated Forces and all visual special effects) to fully support the training objectives associated with the FASGW weapon systems.

- ATE System Requirements Scope

  - This ATE Systems Requirements covers the embodiment of the training capability of the two weapon systems; FASGW (L), Martlet and FASGW (H), Sea Venom onto the Wildcat ATE. The capabilities already provided through the existing ATE shall not be affected.

- Training Objectives

  - The FASGW Training Need Analysis, Reference C, lists a series of high level Training Objectives (TOs).  These identified TOs have been decomposed into training tasks in order to inform the equipment functional requirement derivation. In addition, proposed training tasks are also captured in the table below to assist in the definition of the required fidelity of the ATE in order to deliver the required training.

  - These TOs are included to provide definition to the purpose of the target system.  The ATE System Requirements defined in Section 10, below, provides the requirement of the desired system capability or performance that, once verified, will enable the delivery of these Training Objectives.

| Serial No | Preamble | Descriptor | Requirement | TO Amplifying Information |
| --- | --- | --- | --- | --- |
| ATE-100.0.0 | - | Heading | Training Requirements |  |
| ATE-102.0.0 | The Aircrew | Shall | Perform internal checks and inspections (Pilot, Obs) | Objective 2 - excludes external checks |
| ATE-102.0.1 | - | Shall | Perform Internal Pre-Flight Checks | Task Ref 2.3 |
| ATE-102.0.2 | - | Shall | Perform Inventory Management |  |
| ATE-102.0.3 | - | Shall | Perform in-flight checks | Task Ref 2.8 |
| ATE-102.0.4 | - | Shall | Conduct Post Take Off Checks |  |
| ATE-102.0.5 | - | Shall | Perform Pre-Landing Checks |  |
| ATE-102.0.6 | - | Shall | Perform Internal Post-Flight Checks | Task Ref 2.4 |
| ATE-104.0.0 | The Aircrew | Shall | Respond to aircraft emergencies, system faults, failures and malfunctions procedures (Pilot, Obs) | Objective 4 |
| ATE-104.0.1 | - | Shall | Respond to mission system sensor failures | Task 4.10 (Identify aircraft system malfunctions affecting FASGW, Assess effects on aircraft / weapon system, Identify FASGW weapon emergencies, Perform actions in the event of a FASGW weapon emergency) |
| ATE-104.0.2 | - | Shall | Respond to other emergencies contained in the Flight Reference Cards | Task 4.12 (Identify aircraft system malfunctions affecting FASGW, Assess effects on aircraft / weapon system, Identify FASGW weapon emergencies, Perform actions in the event of a FASGW weapon emergency) |
| ATE-104.0.3 | - | Shall | Respond to FASGW system malfunctions |  |
| ATE-106.0.0 | The Aircrew | Shall | Perform picture compilation (Obs) | Objective 6 |
| ATE-106.0.1 | - | Shall | Perform EODS target tracking | Task 6.3 (Operate EODS during FASGW engagements, especially tracking small, fast moving contacts during operation of FASGW (L)) |
| ATE-106.0.2 | - | Shall | Perform Battle Damage Assessment (BDA) [DAMASS] | Task 6.6 (Perform target battle damage assessment during FASGW (L) engagements to assess if immediate re-attack necessary) |
| ATE-108.0.0 | The Observer | Shall | Operate aircraft systems (Obs) | Objective 11 |
| ATE-108.0.1 | - | Shall | Operate controls and switches located on the mission control panel and HCU | Task 11.15 |
| ATE-108.0.2 | - | Shall | Operate the controls and switches located on the master armament panel and manage the weapon control and release functions | Task 11.16 |
| ATE-108.0.3 | - | Shall | Operate Cursor Control Device (CCD) | Task 11.17 |
| ATE-108.0.4 | - | Shall | Operate Electro-Optical Designating System (EODS) | Task 11.21 |
| ATE-108.0.5 | - | Shall | Operate Weapons Datalink (PIE) | Task 11.24 |
| ATE-110.0.0 | The Pilot | Shall | Operate aircraft systems (Pilot) | Objective 13 |
| ATE-110.0.1 | - | Shall | Operate controls and switches located on the cyclic and collective grips | Task 13.16 |
| ATE-110.0.2 | - | Shall | Operate controls and switches located on the mission control panel and HCU | Task 13.17 |
| ATE-110.0.3 | - | Shall | Operate the controls and switches located on the master armament panel | Task 13.18 |
| ATE-110.0.4 | - | Shall | Operate Weapons Datalink (PIE) | Task 13.20 |
| ATE-112.0.0 | The Aircrew | Shall | Maintain currency (Obs & Pilot) | Task 13 & 15 |
| ATE-114.0.0 | The Aircrew | Shall | Recognise and Classify Land, Sea and Air Units (Pilot, Obs) | Task 20.1 (Potential expansion of EO/radar devices to achieve higher level of identification capability, No required increase in EODS capability) |
| ATE-116.0.0 | The Aircrew | Shall | Support Power Projection including Littoral Warfare & Troop Insertion (Pilot, Obs) | Objective 22 |
| ATE-116.0.0 | The Aircrew | Shall | Conduct AWW Missions: | Task 22.1 |
| ATE-116.0.1 | - | Shall | Conduct Target Prioritisation | Task 22.1 |
| ATE-116.0.2 | - | Shall | Select Target(s) | Task 22.1 |
| ATE-116.0.3 | - | Shall | Select Weapon System | Task 22.1 |
| ATE-116.0.4 | - | Shall | Assign, Manipulate and Confirm Package(s) to Target(s) | Task 22.1 |
| ATE-116.0.5 | - | Shall | Confirm ROE | Task 22.1 |
| ATE-116.0.6 | - | Shall | Order Engagement | Task 22.1 |
| ATE-116.0.7 | - | Shall | Operate Mission Recorder | Task 22.1 |
| ATE-116.0.8 | - | Shall | Perform Weapon Pre-Attack Checks | Task 22.1 |
| ATE-116.0.9 | - | Shall | Perform FASGW (H) Attack Phase | Task 22.1 |
| ATE-116.0.10 | - | Shall | Perform FASGW (H) Weapon Abort | Task 22.1 |
| ATE-116.0.11 | - | Shall | Perform FASGW (L) Attack Phase | Task 22.1 |
| ATE-116.0.12 | - | Shall | Perform FASGW (L) Weapon Abort | Task 22.1 |
| ATE-118.0.0 | The Aircrew | Shall | Conduct Multiple A/C Co-operative Engagements against a Single Target |  |
| ATE-120.0.0 | The Aircrew | Shall | Conduct Multiple A/C Co-operative Engagements against Multiple Targets |  |
| ATE-122.0.0 | The Aircrew | Shall | Conduct Deck Landing Operations (Pilot, Obs) | Objective 30 |
| ATE-122.0.1 | - | Shall | Perform Deck Landings [and Take Offs, when armed with Weapons] | Task 30.6 All weapon load out combinations |
| ATE-130.0.0 | The Aircrew | Shall | Conduct Additional or Expanded training tasks as below: |  |
| ATE-130.1.0 | The Aircrew | Shall | operate the Martlet (FASGW (L)) missile system to conduct the following a/c carriage phase tasks: |  |
| ATE-130.1.1 | - | Shall | Inventory Management |  |
| ATE-130.1.2 | - | Shall | Launcher Jettison |  |
| ATE-130.2.0 | The Aircrew | Shall | operate the Martlet (FASGW (L)) missile system to conduct the following pre-launch phase tasks: |  |
| ATE-130.2.1 | - | Shall | Target Tracking |  |
| ATE-130.2.2 | - | Shall | Target Acquisition |  |
| ATE-130.2.3 | - | Shall | Pre-Attack Checks |  |
| ATE-130.2.4 | - | Shall | Establish A/C in Launch Envelope |  |
| ATE-130.3.0 | The Aircrew | Shall | operate the Martlet (FASGW (L)) missile system to conduct the following launch phase tasks: |  |
| ATE-130.3.1 | - | Shall | Maintain A/C within Missile Launch Envelope |  |
| ATE-130.3.2 | - | Shall | Conduct Pre-Launch Checks |  |
| ATE-130.3.3 | - | Shall | Release Weapon | Individually only |
| ATE-130.3.4 | - | Shall | Establish Weapon Guidance |  |
| ATE-130.3.5 | - | Shall | Establish A/C within Missile Post-Launch Envelope |  |
| ATE-130.4.0 | The Aircrew | Shall | operate the Martlet (FASGW (L)) missile system to conduct the following guidance phase tasks: |  |
| ATE-130.4.1 | - | Shall | Maintain A/C within Missile Post-Launch Envelope |  |
| ATE-130.4.2 | - | Shall | Maintain Missile Guidance to Target Impact |  |
| ATE-130.4.3 | - | Shall | Conduct Attack Abort Procedure |  |
| ATE-130.4.0 | - | Shall | operation of the Martlet (FASGW (L)) missile system to conduct the following post impact phase tasks: |  |
| ATE-130.4.1 | - | Shall | Conduct Post-Attack Checks |  |
| ATE-132.1.0 | The Aircrew | Shall | operate the Sea Venom (FASGW (H)) missile system to conduct the following a/c carriage phase tasks: |  |
| ATE-132.1.1 | - | Shall | Inventory Management |  |
| ATE-132.1.2 | - | Shall | Weapon Jettison |  |
| ATE-132.2.0 | The Aircrew | Shall | operate of the Sea Venom (FASGW (H)) missile system to conduct the following pre-launch phase tasks: |  |
| ATE-132.2.1 | - | Shall | Target Acquisition |  |
| ATE-132.2.2 | - | Shall | Conduct Weapon Salvo Planning |  |
| ATE-132.2.3 | - | Shall | Pre-Attack Checks |  |
| ATE-132.2.4 | - | Shall | Establish A/C in Launch Envelope |  |
| ATE-132.3.0 | The Aircrew | Shall | operate of the Sea Venom (FASGW (H)) missile system to conduct the following launch phase tasks: |  |
| ATE-132.3.1 | - | Shall | Maintain A/C within Missile Launch Envelope |  |
| ATE-132.3.2 | - | Shall | Conduct Pre-Launch Checks |  |
| ATE-132.3.3 | - | Shall | Release Weapon/s | Individually & in Salvo |
| ATE-132.4.0 | The Aircrew | Shall | operation of the Sea Venom (FASGW (H)) missile system to conduct the following post-launch phase tasks: |  |
| ATE-132.4.1 | - | Shall | Maintain Target Tracking |  |
| ATE-132.4.2 | - | Shall | Conduct Mid-Course Target Update/s |  |
| ATE-132.4.3 | - | Shall | Monitor Missile Condition |  |
| ATE-132.4.4 | - | Shall | conduct Missile Abort |  |
| ATE-132.5.0 | The Aircrew | Shall | operation of the Sea Venom (FASGW (H)) missile system to conduct the following terminal phase targeting adjustments: |  |
| ATE-132.5.1 | - | Shall | confirmation of correct target and monitor to impact |  |
| ATE-132.5.2 | - | Shall | confirmation of correct target and Aim Point Refinement |  |
| ATE-132.5.3 | - | Shall | determination of Incorrect target followed by re-targeting to valid target |  |
| ATE-132.5.4 | - | Shall | determination of Incorrect target followed by abort selection |  |
| ATE-132.5.5 | - | Shall | conduct Missile Abort |  |

Table 2 - ATE Training Objectives

- ATE Key User Requirements

| Serial No | Preamble | Descriptor | Requirement | Amplifying Information |
| --- | --- | --- | --- | --- |
| ATE-0.0.0 | - | Heading | Key User Requirements (KUR) & General Context |  |
| ATE-0.1.0 | The ATE | Shall | address, through hardware and software modifications, the introduction of the FASGW systems functionality, Human Machine Interface (HMI) and Synthetic operating environment (including Computer Generated Forces and all visual special effects) to fully support the training objectives |  |
| ATE-0.2.0 | The ATE | Shall | enable the delivery of training to operate the FASGW systems in order that Wildcat HMA-FASGW Aircrews can operate the FASGW systems within representative tactical and physical environments | By Day and Night (Conventional and Reversionary), VMC & IMC |
| ATE-0.3.0 | The ATE | Shall | retain all Wildcat Mk1 AH and Mk2 HMA capabilities as provided under the preceding D&M TCE Contracts | Procurement Specifications: PJT-078576, PJT-010033, PJT-078575 |
| ATE-0.4.0 | The ATE upgrade | Shall Not | include the embodiment of the FASGW capability onto the FTD |  |

Table 3 - ATE Key User Requirements

- ATE System Requirements

| Serial No | Preamble | Descriptor | Requirement | Amplifying Information |
| --- | --- | --- | --- | --- |
| ATE-200.0.0 | - | Heading | Aircraft Systems Simulation |  |
| ATE-200.1.0 | The ATE | Shall | be designed such that the presentation of specified data and the execution of specified functions, as perceived by the student, is representative of the aircraft Human Machine Interface (HMI) |  |
| ATE-200.2.0 | The FTD | Shall Not | be modified to include FASGW capability |  |
| ATE-210.0.0 | - | Heading | Airframe Systems |  |
| ATE-212.0.0 | - | Heading | Electrical |  |
| ATE-212.1.0 | The ATE | Shall | provide a representative simulation of the controls, indications and operation of the AC and DC (including external power) electrical systems in both: |  |
| ATE-212.1.1 | - | Shall | Normal Operation |  |
| ATE-212.1.2 | - | Shall | Malfunction Conditions | as contained within the Malfunction List from the previously supplied STE |
| ATE-214.0.0 | - | Heading | HUMS |  |
| ATE-214.1.0 | The ATE | Shall | provide a simplified generic HUMS simulation to replicate the HMI and top level menu structure |  |
| ATE-250.0.0 | - | Heading | Avionic Systems |  |
| ATE-252.0.0 | - | Heading | CDS |  |
| ATE-252.2.0 | The ATE | Shall | provide a specific representation of the CDS associated with the FASGW system including all reversionary modes and failure/cautionary indications |  |
| ATE-254.0.0 | - | Heading | AMS |  |
| ATE-254.1.0 |  | Shall | provide a representative simulation of the controls, indications and operation of the AMS in both: | associated with the FASGW weapon systems |
| ATE-254.1.1 | - | Shall | Normal Operation |  |
| ATE-254.1.2 | - | Shall | Malfunction Conditions | as contained within the Malfunction List from the previously supplied STE |
| ATE-300.0.0 | - | Heading | Mission Systems |  |
| ATE-302.0.0 | - | Heading | TP |  |
| ATE-302.1.0 | The ATE | Shall | provide a specific representation of the TP HMI and displays | rehosted a/c SW |
| ATE-302.2.0 | The ATE | Shall | support the upload of pre-Mission TP data using a representation of the iMMM/KSU/SEDTU Combination: | existing capability using COTS USB device |
| ATE-302.2.1 | - | Shall | Maps | existing capability |
| ATE-302.2.2 | - | Shall | Communications | existing capability |
| ATE-302.2.3 | - | Shall | Flight plans | existing capability |
| ATE-302.2.4 | - | Shall | Performance | existing capability |
| ATE-302.2.5 | - | Shall | Navigation | existing capability |
| ATE-302.2.6 | - | Shall | IDM initialisation | existing capability |
| ATE-302.2.7 | - | Shall | SMS | existing capability |
| ATE-302.2.8 | - | Shall | FASGW pre-mission planning data |  |
| ATE-302.3.0 | The ATE | Shall | support the download of post-Mission TP data using a using a representation of the iMMM/KSU/SEDTU Combination: | existing capability using COTS USB device |
| ATE-302.3.1 | - | Shall | Mission Recorder |  |
| ATE-302.3.2 | - | Shall | Engagement Record |  |
| ATE-302.4.0 | The ATE | Shall Not | provide a simulation of the Training Mode functionality as provided within the Live a/c | if present within the rehosted TP, functionality is not required to be tested |
| ATE-304.0.0 | - | Heading | EODS |  |
| ATE-304.1.0 | The ATE | Shall | provide a representative simulation of the controls, indications and operation of the EODS in both: |  |
| ATE-304.1.1 | - | Shall | Normal Operation |  |
| ATE-304.1.2 | - | Shall | Malfunction Conditions | as contained within the Malfunction List from the previously supplied STE |
| ATE-306.0.0 | - | Heading | SMS |  |
| ATE-306.1.0 | The ATE | Shall | provide a representative simulation of the controls, indications and operation of the SMS in both: |  |
| ATE-306.1.1 | - | Shall | Normal Operation |  |
| ATE-306.1.2 | - | Shall | Malfunction Conditions | as contained within the Malfunction List from the previously supplied STE |
| ATE-350.0.0 | - | Heading | Weapons Systems |  |
| ATE-350.1.0 | The ATE | Shall | representatively simulate Weapon performance for the FASGW systems | utilising BFE provided missile flyout model |
| ATE-350.2.0 | The ATE | Shall | representatively simulate Weapon release and jettison |  |
| ATE-350.3.0 | The ATE | Shall | representatively simulate the controls, indications and operation of the aircraft FASGW systems |  |
| ATE-350.4.0 | The ATE | Shall | replicate all normal modes of operation for the installed systems |  |
| ATE-350.5.0 | The ATE | Shall | replicate all degraded modes of operation for the installed systems | iaw the Malfunction List |
| ATE-352.0.0 | - | Heading | Martlet - FASGW (L) |  |
| ATE-352.1.0 | The ATE | Shall | provide representative behaviour as viewed through the OTW of the following: | utilising BFE provided missile flyout model |
| ATE-352.1.1 | - | Shall | Motor Burn, exhaust plume and debris |  |
| ATE-352.1.2 | - | Shall | Physical Appearance, both when in the launcher and when airborne |  |
| ATE-352.1.3 | - | Shall | Flight Dynamics | utilising BFE provided missile flyout model |
| ATE-352.1.4 | - | Shall | Missile Impact | with either designated target or as a 'miss' Explosion, Debris, Smoke |
| ATE-352.1.5 | - | Shall | Missile Ditching into Sea | Splash, Debris |
| ATE-352.2.0 | The ATE | Shall | Support all Missile Modes and Target Types: | as applicable to the Wildcat-FASGW |
| ATE-352.2.1 | - | Shall | Impact Fusing |  |
| ATE-352.2.2 | - | Shall | Proximity Fusing | variable adjustable at the LPSG |
| ATE-352.3.0 | - | Heading | Representative behaviour as viewed through the EODS of the following: |  |
| ATE-352.3.1 | The ATE | Shall | Motor Ignition and Burn, exhaust plume and debris |  |
| ATE-352.3.2 | - | Shall | Physical Appearance, both when in the launcher and when airborne |  |
| ATE-352.3.3 | - | Shall | Flight Dynamics | utilising BFE provided missile flyout model |
| ATE-352.3.4 | - | Shall | Missile Impact | with either designated target or as a 'miss' Explosion, Debris, Smoke |
| ATE-352.3.5 | - | Shall | Missile Ditching into Sea | Splash, Debris |
| ATE-352.4.0 | The ATE | Shall | have a representative Time of Flight for the FASGW(L) missile | tuneable acceleration and velocity parameters within the LPSG |
| ATE-354.0.0 | - | Heading | Martlet - FASGW (L) Malfunctions: |  |
| ATE-354.1.0 | The ATE | Shall | Support the following missile or targeting system malfunctions during the engagement phase: |  |
| ATE-354.1.1 | - | Shall | Launcher fails to Initialise on Power Up (Launchers 1-4) - NOGO |  |
| ATE-354.1.2 | - | Shall | Launcher Degraded (DEGR) |  |
| ATE-354.1.3 | - | Shall | ALGU Thermal Shutdown |  |
| ATE-354.1.4 | - | Shall | Missile - Store Battery or Motor Misfired |  |
| ATE-354.1.5 | - | Shall | Hang Up (Jettison Fail) |  |
| ATE-354.1.6 | - | Shall | Guidance Failure |  |
| ATE-356.0.0 | - | Heading | Sea Venom - FASGW (H) |  |
| ATE-356.1.0 | The ATE | Shall | provide representative behaviour as viewed through the OTW of the following: | utilising GFE provided missile flyout model |
| ATE-356.1.1 | - | Shall | Motor Burn, exhaust plume and debris |  |
| ATE-356.1.2 | - | Shall | Physical Appearance, both when on the launcher and when airborne |  |
| ATE-356.1.3 | - | Shall | Flight Dynamics | utilising BFE provided missile flyout model |
| ATE-356.1.4 | - | Shall | Missile Impact | with either designated target or as a 'miss' Explosion, Debris, Smoke |
| ATE-356.1.5 | - | Shall | Missile Ditching into Sea | Splash, Debris |
| ATE-356.2.0 | The ATE | Shall | provide representative behaviour as viewed through the EODS of the following: |  |
| ATE-356.2.1 | - | Shall | Motor Ignition, Burn, exhaust plume and debris |  |
| ATE-356.2.2 | - | Shall | Physical Appearance, both when on the launcher and when airborne |  |
| ATE-356.2.3 | - | Shall | Representative Flight Dynamics | only as required for the Training Task |
| ATE-356.2.4 | - | Shall | Terminal Approach | if within EODS range |
| ATE-356.2.5 | - | Shall | Missile Impact | with either designated target or as a 'miss' Explosion, Debris, Smoke |
| ATE-356.2.6 | - | Shall | Missile Ditching into Sea | Splash, Debris |
| ATE-356.3.0 | The ATE | Shall | provide representative behaviour as viewed through the IRS of the following: |  |
| ATE-356.3.1 | - | Shall | Representative Flight Dynamics | only as required for the Training Task |
| ATE-356.3.2 | - | Shall | Terminal Approach |  |
| ATE-356.3.3 | - | Shall | Missile Impact |  |
| ATE-356.4.0 | The ATE | Shall | have a representative Time of Flight for the following: | tuneable acceleration and velocity parameters within the LPSG |
| ATE-356.4.1 | - | Shall | To Final Approach Point |  |
| ATE-356.4.2 | - | Shall | During Terminal Phase |  |
| ATE-356.5.0 | The ATE | Shall | Provide a minimum of 4x representative FASGW(H) missile IRS Views across the ATE devices | 2x missile views for FMS#1, 1x view each for FMS#2 & CPT display to be tuned during SW development |
| ATE-356.8.0 | The ATE | Shall | provide missile IRS views that responds representatively to changes to Meteorological Conditions: | 3 cases to be agreed: Normal, Reduced, Poor Conditions to include: Absolute Humidity, Air Temperature, Visibility, Sea Surface Temperature, Precipitation |
| ATE-356.9.0 | The ATE | Shall | include representative System Latency between Missile and A/C |  |
| ATE-356.10.0 | The ATE | Shall | Support all Missile Modes and Target Types: | as applicable to the Wildcat-FASGW |
| ATE-356.10.1 | - | Shall | Normal Launch Mode |  |
| ATE-356.10.2 | - | Shall | ICBL Launch Mode |  |
| ATE-356.10.3 | - | Shall | STOT Mode |  |
| ATE-356.10.4 | - | Shall | Targeting Variables - Sea Skimming Mode, Cruise Height, Acquisition Height, Split Angle, Fuse Delay, Target Length, Aim Point Location, Above Land & Missile Position within a Salvo |  |
| ATE-356.10.5 | - | Shall | Target Hardkill |  |
| ATE-356.10.6 | - | Shall | Target Type (Sea); Ship at Sea | abbreviated to Sea |
| ATE-356.10.7 | - | Shall | Target Type (Harbour); Ship at Harbour | abbreviated to Harbour |
| ATE-356.10.8 | - | Shall | Target Type Land | abbreviated to Land |
| ATE-356.11.0 | The ATE | Shall | Support the following Missile Target Adjustments in Flight: |  |
| ATE-356.11.1 | - | Shall | Re-Targeting | In Sea mode |
| ATE-356.11.2 | - | Shall | Aim Point Adjustment | In Sea mode |
| ATE-356.11.3 | - | Shall | Operator Guidance | In Harbour or Land modes |
| ATE-356.12.0 | The ATE | Shall | Provide a representative missile simulation when targets are engaged using an inappropriate mode | i.e. Sea target in Land/Harbour mode or Land/Harbour Target in Sea mode |
| ATE-356.13.0 | The ATE | Shall | Support single weapon target engagements for 1 missile per Ownship | for all ATE devices |
| ATE-356.14.0 | The ATE | Shall | Support multiple weapon target engagements for 2 missiles for a single Ownship | FMS#1 |
| ATE-356.15.0 | The ATE | Shall | Support collaborative engagements with HLA networked playmate aircraft | FMS#1 & FMS#2; FTD excluded |
| ATE-356.16.0 | The ATE | Shall | Support collaborative engagements with CGF aircraft |  |
| ATE-358.0.0 | - | Heading | Sea Venom - FASGW (H) Malfunctions |  |
| ATE-358.1.1 | The ATE | Shall | Replicate Malfunctions representing degraded or lost conditions for the missile IR sensor and Datalink available during the missile time of flight |  |
| ATE-358.2.0 | The ATE | Shall | Support the following missile or targeting system malfunctions during the engagement phase: |  |
| ATE-358.2.1 | - | Shall | PIE Failure - NOGO |  |
| ATE-358.2.2 | - | Shall | Missile BIT - DEGR (No MDLT Capability) |  |
| ATE-358.2.3 | - | Shall | Missile BIT - NOGO |  |
| ATE-358.2.4 | - | Shall | Launch Fail - Hang Up |  |
| ATE-358.2.5 | - | Shall | Launch Fail - Misfire |  |
| ATE-358.2.6 | - | Shall | Hang Up (Jettison Fail) |  |
| ATE-358.2.7 | - | Shall | Sustain Motor Fail |  |
| ATE-358.2.8 | - | Shall | IRS Degraded/Damaged | by icing or other contamination |
| ATE-358.2.9 | - | Shall | Datalink Fail (post Launch) |  |
| ATE-358.2.10 | - | Shall | Abort Message Fail |  |
| ATE-500.0.0 | - | Heading | Cockpit Structure |  |
| ATE-500.1.0 | The ATE | Shall | provide replica cockpit environment with accurate tactile and visual feedback from all cockpit controls and indicators |  |
| ATE-500.2.0 | The ATE | Shall | provide tactile, functional replicas of the IDUs | to include FASGW specific modification |
| ATE-500.3.0 | The ATE | Shall | provide a tactile, functional replica of the WCP | to include FASGW specific modification |
| ATE-600.0.0 | - | Heading | Cueing Environment |  |
| ATE-602.0.0 | - | Heading | Flight Model |  |
| ATE-602.1.0 | The ATE | Shall | provide a representative simulation of the effects on flight performance with FASGW role equipment and weapons loaded |  |
| ATE-602.2.0 | The ATE | Shall | represent any latency, or delay, between the operation of a control and the associated display or indication being activated |  |
| ATE-604.0.0 | - | Heading | CLS/Flying Controls |  |
| ATE-604.1.0 | The ATE | Shall | provide specific physical and functional representation of the flying controls grips and sticks | to include FASGW specific modification |
| ATE-604.2.0 | The ATE | Shall | provide a specific representation of the FCMC, or control travel and forces, as per the prototype aircraft |  |
| ATE-606.0.0 | - | Heading | Sound |  |
| ATE-606.1.0 | The ATE | Shall | provide representative sounds of aircraft systems and equipment as audible to the aircrew | e.g engines, transmission, cockpit instrument display fan, avionic fan noises and CWS warnings & cautions |
| ATE-606.2.0 | The ATE | Shall | provide representative sound of launch, boost and sustain motor ignition and burn for: | as audible to the Ownship Aircrew |
| ATE-606.2.1 | - | Shall | Martlet - FASGW (L) | Weapon Launch |
| ATE-606.2.2 | - | Shall | Sea Venom - FASGW (H) | Weapon Launch |
| ATE-606.3.0 | The ATE | Shall | provide representative sound of launch, boost and sustain motor ignition and burn for missles fired from companion aircraft | as audible to the Ownship Aircrew when in sufficiently close company |
| ATE-500.0.0 | - | Heading | Visual Display System |  |
| ATE-500.1.0 | The ATE | Shall | provide a visual representation of FASGW munitions release and in flight with all appropriate weapon visual special effects | explosion on missile self-destruct (FASGW (L)) or FASGW (H) crash into sea on abort |
| ATE-500.2.0 | The ATE | Shall Not | cause student distraction from visual scene artefacts including display image popping, halting, blurring, stepping or jumping | visual, EODS or IRS views |
| ATE-700.0.0 | - | Heading | Visual Database & Moving Models |  |
| ATE-702.0.0 | - | Heading | Moving 3D Models |  |
| ATE-702.1.0 | The ATE | Shall | provide accurate 3D moving model representations of Wildcat-FASGW fitted with every combination of FASGW loads including mixed loads with existing weapons of Stingray torpedo and Mk11 Depth Charge | Cleared loadout combinations only |
| ATE-702.2.0 | The ATE | Shall | provide a 3D model of the Sea Venom - FASGW (H) in its launch, cruise and terminal phases | High fidelity missile flight profile not required, representative velocities and altitudes are required, parameters tuneable at the IOS |
| ATE-702.3.0 | The ATE | Shall | shall provide a 3D model of the Martlet - FASGW (L) in its launch, cruise and terminal phases | High fidelity missile flight profile not required, representative velocities and altitudes are required, , parameters tuneable at the IOS |
| ATE-702.4.0 | The ATE | Shall | Provide the ability for CGF to deploy the following decoys, countermeasures or tactics to simulate ASMD reactions: | Naval CGF only |
| ATE-702.4.1 | - | Shall | Flares | Existing Capability |
| ATE-702.4.2 | - | Shall | Chaff | Existing Capability |
| ATE-702.4.3 | - | Shall | Floating Decoy/s | Existing Capability |
| ATE-702.4.4 | - | Shall | Laser Dazzler | Existing Capability |
| ATE-702.4.6 | - | Shall | Smoke | Target Obscuration |
| ATE-800.0.0 | - | Heading | Instructor Operating Station |  |
| ATE-800.1.0 | The ATE | Shall | display the system state/mode selections and associated functions controlling the FASGW missile system |  |
| ATE-800.2.0 | The ATE | Shall | have the ability to record the seeker head return picture within the AAR system | in addition to the IDU display |
| ATE-800.3.0 | The ATE | Shall | have the ability to record the fall of shot or missile hit location/s with reference to the designated target | in addition to the IDU display |
| ATE-800.4.0 | The ATE | Shall | provide automatic monitoring, and recording of pre and post release aircraft attitude conditions with reference to the relevant limitations with an Instructor alert if exceeded |  |
| ATE-900.0.0 | - | Heading | Interoperability |  |
| ATE-900.1.0 | The ATE | Shall | support collaborative FASGW engagements within the WTC LAN |  |
| ATE-900.2.0 | The ATE | Shall | support collaborative FASGW engagements over a HLA configured WAN |  |

Table 4 - ATE System Requirements

- ATE Requirements to be provided as Options

Table 5 contains a number of ATE options to be potentially provided above the baseline.

| Option No | Serial No | Preamble | Descriptor | Requirement | Amplifying Information |
| --- | --- | --- | --- | --- | --- |
| ATE-1 | GEN-130.2.0 | The STE | Could | include a spares package to cover the remaining duration of PP2 | PP2 completes March 2022 - notionally a 30-month duration |
| ATE-2 | ATE-200.2.0 | The FTD | Could | be modified to include FASGW capability |  |
| ATE-3 | ATE-356.5.1 | The ATE | Could | support the selection of the 3rd and 4th missile IRS views by repositioning the 1st and 2nd missile IGs | Non-representative IRS view switching latency is recognised |
| ATE-4 | ATE-356.6.0 | The ATE | Could | provide 6x representative FASGW(H) missile IRS Views across the ATE devices; excluding FTD | 2x missile views each for FMS#1, FMS#2 & CPT display to be tuned during SW development |
| ATE-5 | ATE-356.7.0 | The ATE | Could | provide 8x representative FASGW(H) missile IRS Views across the ATE devices | 2x missile views for each device display to be tuned during SW development |
| ATE-6 | ATE-702.5.0 | The ATE | Could | provide the ability for CGF to deploy the following decoys, countermeasures or tactics to simulate ASMD reactions: | Naval CGF only |
|  | ATE-702.5.1 | - | Could | Multiple Flares | 5x Preset patterns editable at the LPSG, triggered by either automatic detection based upon preset conditions - including range and target sensor status, or by Instructor intervention at the IOS |
|  | ATE-702.5.2 | - | Could | Manoeuvre | 5x Preset manoeuvre patterns editable at the LPSG, triggered by either automatic detection based upon preset conditions - including range and target sensor status, or by Instructor intervention at the IOS |
| ATE-7 | N/A | The ATE | Could | be modified to include the additional WOANS included within JMS LH18331 TP Uplift (cost delta) | JMS LH18331 TP Uplift to be considered as FASGW baseline (ATE-302.1.0), minus the WOANS |

Table 5 - ATE Requirements to be provided as Options

## PART THREE - MAINTAINER TRAINING EQUIPMENT

## SYSTEM REQUIREMENTS FOR THE INTRODUCTION OF FASGW CAPABILITY INTO THE WILDCAT

- MTE System Requirements Purpose

  - The MTE shall be augmented with a new Weapon Loading System Trainer (WLST) in order to expand the STE capability of the WTC, to include FASGW  a/c systems and the loading and unloading of the weapons, to fully support the TOs associated with the introduction of FASGW weapon systems.

- MTE System Requirements Scope

  - This MTE Systems Requirements covers the embodiment of the training capability of the two weapon systems; FASGW (L), Martlet and FASGW (H), Sea Venom into the Wildcat MTE suite, through the introduction of a new WLST.  The WLST shall also have the capability to support training of existing weapons – HMG, Stingray Torpedo and Mk11 Depth Charge. The capabilities already provided through the existing MTE shall not be affected.

- MTE System Requirements Overview

  - The training design concept for the MTE is not to provide a direct replica a/c that would be capable of simulating systems as such high fidelity as to be able to allow ‘freeplay’ or ad-hoc training delivery.  The MTE have been designed to allow the training of the conduct of specific maintenance procedures.  These procedures are as defined within the CIETP. The WLST design shall follow the same concept and shall also minimise the duplication of maintenance tasks between devices.

- MTE Training Objectives

  - The FASGW Training Need Analysis, Reference C, lists a series of high level TOs.  These identified TOs have been decomposed into training tasks in order to inform the equipment functional requirement derivation. In addition, proposed training tasks are also captured in the table below to assist in the definition of the required fidelity of the MTE in order to deliver the required training.

  - These TOs are included to provide definition to the purpose of the target system.  The MTE System Requirements defined in Section 17, below, provides the requirement of the desired system capability or performance that, once verified, will enable the delivery of these Training Objectives.

| Serial No | Preamble | Descriptor | Requirement | TO Amplifying Information |
| --- | --- | --- | --- | --- |
| MTE-100.0.0 | - | Heading | Training Requirements |  |
| MTE-100.1.0 | The WLST | Shall | Be designed to support the training of: | As applicable to physical layout, location and accessibility of FASGW system components and LRUs |
| MTE-100.1.1 | - | Shall | a) Component location | for FASGW Permanent and Temporary Role equipment components |
| MTE-100.1.2 | - | Shall | b) System operation for functional items | as required to meet the training objectives |
| MTE-100.1.3 | - | Shall | c) Fault Diagnosis and Rectification | rectification limited to FASGW temporary role equipment exempt PIE |
| MTE-100.1.4 | - | Shall | d) Removal and Installation of Components | remove and installation limited to FASGW temporary role equipment exempt PIE |
| MTE-100.1.5 | - | Shall | e) Pre and Post installation checks | for the designated maintenance tasks |
| MTE-100.2.0 | The WLST | Shall Not | Support training of Flight Servicing |  |
| MTE-100.3.0 | The WLST | Shall | support the following FASGW training tasks: | as identified within PJT-339440 |
| MTE-100.3.1 | - | Shall | Conduct Arming/De-Arming Operations with a FASGW(H) |  |
| MTE-100.3.2 | - | Shall | Conduct Arming/De-Arming Operations with a FASGW(L) |  |
| MTE-100.3.3 | - | Shall | Conduct FASGW/Wildcat Integration Fault Diagnosis |  |
| MTE-100.3.4 | - | Shall | Fit/remove FASGW H role equipment |  |
| MTE-100.3.5 | - | Shall | Fit/remove FASGW L role equipment |  |
| MTE-100.3.6 | - | Shall | React to a FASGW H incident |  |
| MTE-100.3.7 | - | Shall | React to a FASGW L incident |  |
| MTE-100.3.8 | - | Shall | Carry out Scheduled maintenance on the Wildcat FASGW system | to satisfy the required EO/TOs |
| MTE-100.3.9 | - | Shall | Carry out Unscheduled maintenance on the Wildcat FASGW system | to satisfy the required EO/TOs |
| MTE-100.4.0 | The WLST | Shall | Support training of the Handling of FASGW incidents: | post a/c recovery both FASGW variants |
| MTE-100.4.1 | - | Shall | Hung Store |  |
| MTE-100.4.2 | - | Shall | Misfire |  |
| MTE-102.5.0 | The WTC | Information | The WTC shall deliver the following FASGW related training tasks, however the MTE will not be used to support them: | as identified within PJT-339440 |
| MTE-102.5.1 | - | Shall | Carry out FASGW Acceptance Inspection |  |
| MTE-102.5.2 | - | Shall | Prepare FASGW(H) for flight |  |
| MTE-102.5.3 | - | Shall | Prepare FASGW(L) for flight |  |
| MTE-102.5.4 | - | Shall | Deprep a FASGW (H) |  |
| MTE-102.5.5 | - | Shall | Deprep a FASGW (H) |  |
| MTE-102.5.6 | - | Shall | FASGW weapons handover procedures | at the EDP |
| MTE-102.5.7 | - | Shall | Existing Weapon handover procedures | at the EDP |
| MTE-102.5.8 | - | Shall | Carry Out FASGW Ground Support Equipment Maintenance | synonymous with ASSE Limited to Pre and Post Use Checks only |
| MTE-102.5.9 | - | Shall | Carry Out FASGW Role Equipment Maintenance | Limited to Pre and Post Use Checks only |
| MTE-102.6.0 | The WLST | Shall | Support the training of the installation/removal of the FASGW Temporary Role Equipment items except the PIE | iaw PJT-320113 |
| MTE-102.7.0 | The WLST | Shall Not | Support the training of the installation/removal of the FASGW Permanent Role Equipment items: | iaw PJT-320113 |
| MTE-102.8.0 | The WLST | Shall Not | Support weapon loading or system training associated with pre-FASGW modified a/c | iaw PJT-320113 |
| MTE-104.1.0 | The WLST | Shall | support the following Existing Weapon training tasks: |  |
| MTE-104.1.1 | - | Shall | Conduct Arming/De-Arming Operations with a Existing Weapon |  |
| MTE-104.1.2 | - | Shall | Conduct Existing Weapon/Wildcat Integration Fault Diagnosis |  |
| MTE-104.1.3 | - | Shall | Fit/remove Existing Weapon role equipment |  |
| MTE-104.1.4 | - | Shall | React to a Existing Weapon incident |  |
| MTE-104.1.5 | - | Shall | Carry out Scheduled maintenance on the Wildcat Existing Weapon system | to satisfy the required EO/TOs |
| MTE-104.1.6 | - | Shall | Carry out Unscheduled maintenance on the Wildcat Existing Weapon system | to satisfy the required EO/TOs |
| MTE-104.2.0 | The WLST | Shall | Support training of the Handling of Existing Weapon incidents: | post a/c recovery & shutdown |
| MTE-104.2.1 | - | Shall | Hung Store |  |
| MTE-104.2.2 | - | Shall | Misfire |  |
| MTE-106.1.0 | The WLST | Shall | support the following M3M 12.7MM (0.5”) HMG training tasks: |  |
| MTE-106.1.1 | - | Shall | Fit/remove Existing Weapon role equipment |  |
| MTE-106.1.2 | - | Shall | Fit/remove HMG |  |
| MTE-108.1.0 | The WLST | Shall Not | support 7.62mm GPMG training tasks |  |

Table 6 - MTE Training Objectives

- MTE Key User Requirements

| Serial No | Preamble | Descriptor | Requirement | Amplifying Information |
| --- | --- | --- | --- | --- |
| MTE-0.0.0 | - | Heading | Key User Requirements (KUR) & General Context |  |
| MTE-0.1.0 | The WLST | Shall | support Weapon loading and system training on both Wildcat HMA-FASGW weapon variants, Existing weapons, HMG door gun and mixed weapon loads as cleared for FASGW-modified Wildcat aircraft |  |
| MTE-0.2.0 | The WLST | Shall | support trainee maintainer activities in accordance with technical procedures derived from data modules from within the Compound Interactive Electronic Technical Publications (CIETP) |  |
| MTE-0.3.0 | The WLST | Shall | be an additional MTE device to support weapon loading of all Wildcat HMA-FASGW to a sufficient fidelity to achieve the training tasks | Weapon Loading System Trainer (WLST) |
| MTE-0.4.0 | The WLST | Shall | utilise the donor GFE provided Wildcat TI#2 airframe as the training device structure |  |

Table 7 - MTE Key User Requirements

- MTE System Requirements

| Serial No | Preamble | Descriptor | Requirement | Amplifying Information |  |
| --- | --- | --- | --- | --- | --- |
| MTE-200.0.0 | - | Heading | Aircraft Systems Simulation |  |
| MTE-200.1.0 | The fidelity | Shall | be sufficient such that the trainee has the impression of interacting with the real aircraft in terms of hazard avoidance and obstructions |  |
| MTE-200.2.0 | The WLST | Shall | accurately represent FASGW and aircraft FASGW–related systems such that the EOs/TOS can be fulfilled in support of scheduled and non-scheduled maintenance training, including: |  |
| MTE-200.3.0 | The WLST | Shall | Support training of Normal modes of operation |  |
| MTE-200.4.1 | The WLST | Shall Not | Support training of Weapon and FASGW system malfunctions |  |
| MTE-200.6.0 | The WLST | Shall Not | Support training of Abnormal modes of operation |  |
| MTE-200.7.0 | The WLST | Shall | Support the installation/removal of the FASGW Temporary Role Equipment items | iaw PJT-320113, excluding the PIE |
| MTE-200.7.1 | - | Shall | LH & RH Weapon Wings |  |
| MTE-200.7.2 | - | Shall | Stores Release Units |  |
| MTE-200.7.3 | - | Shall | LMM Launcher & Wiring |  |
| MTE-200.8.0 | The WLST | Shall Not | include any representation of the PIE LRUs |  |
| MTE-200.9.0 | The WLST | Shall | include any tactile, non-functional representation of the PIE LRUs | space model only, to be provided as an Option |
| MTE-200.10.0 | The WLST | Shall Not | Support the training of the maintenance, loading or unloading of the CMDS |  |
| MTE-200.13.0 | The WLST | Shall Not | Support the installation/removal of the FASGW Permanent Role Equipment items: | iaw PJT-320113 |
| MTE-202.1.0 | The WLST | Shall | Support the installation/removal of the Existing Weapon Role Equipment items; | iaw PJT-320113 |
| MTE-202.1.1 | - | Shall | HMG Mounting Beam |  |
| MTE-202.1.2 | - | Shall | EMRU |  |
| MTE-204.1.0 | The WLST | Shall | support the training of the following Maintenance Procedures from the CIETP: | to be finalised no later than SRR |
| MTE-204.1.1 | - | Shall | C93-51-xx-xx-xxxA-A Electro-optical designating system ALGU - General - Calibrate (not yet issued) | see LR91T0005W excluding fitment of the ALGU BAT tool (equivalent to the standard BAT training task) |
| MTE-204.1.2 | - | Shall | C94-11-03-00-340A-A Normal weapon release - Armament indicators - Function tests |  |
| MTE-204.1.3 | - | Shall | C94-13-00-00-340A-A Stores management system - General - Function tests |  |
| MTE-204.1.4 | - | Shall | C94-51-01-01-920A-A Left medium pintle head - Remove and install | HMG - and/or RHS equivalent |
| MTE-204.1.5 | - | Shall | C94-51-01-00-920A-A Left link basket assembly - Remove and install | HMG - and/or RHS equivalent |
| MTE-204.1.6 | - | Shall | C94-xx-xx-xx-xxxA-A PIE - General - Function tests (not yet issued) |  |
| MTE-204.1.7 | - | Shall | C94-xx-xx-xx-xxxA-A Weapon carriers - Left Weapon Wing - Remove and install (not yet issued) |  |
| MTE-204.1.8 | - | Shall | C94-xx-xx-xx-xxxA-A Weapon carriers - Right Weapon Wing - Remove and install (not yet issued) |  |
| MTE-204.1.9 | - | Shall | C94-31-07-00-920A-A Weapon carriers - EMRU 22 Mk 1 - Remove and install | for Existing weapons |
| MTE-204.1.10 | - | Shall | C94-xx-xx-xx-xxxA-A Left SRU - Remove and install (not yet issued) | replacement for EMRU Mk22 for FASGW |
| MTE-204.1.11 | - | Shall | C94-xx-xx-xx-xxxA-A Right SRU - Remove and install (not yet issued) | replacement for EMRU Mk22 for FASGW |
| MTE-204.1.12 | - | Shall | C94-xx-xx-xx-xxxA-A Left Umbilical Cable - Remove and install (not yet issued) | replacement for EMRU Mk22 for FASGW |
| MTE-204.1.13 | - | Shall | C94-xx-xx-xx-xxxA-A Right Umbilical Cable - Remove and install (not yet issued) | replacement for EMRU Mk22 for FASGW |
| MTE-204.1.14 | - | Shall | C94-xx-xx-xx-xxxA-A Martlet - Launcher - Remove and Install (not yet issued) |  |
| MTE-204.1.15 | - | Shall | C94-xx-xx-xx-xxxA-A Martlet - Launcher - Function Test (not yet issued) |  |
| MTE-204.1.16 | - | Shall | C94-xx-xx-xx-xxxA-A Martlet - Launcher - No Volts Safety Test (not yet issued) |  |
| MTE-204.1.17 | - | Shall | C94-xx-xx-xx-xxxA-A Martlet - Launcher - General fault isolation procedures (not yet issued) |  |
| MTE-204.1.18 | - | Shall | C94-51-01-02-920A-A Left M3M machine gun - Remove and install | HMG - and/or RHS equivalent |
| MTE-206.1.0 | The WLST | Shall | support the Weapon Loading & Unloading Procedures for the following weapon systems; | iaw Wildcat HMA-FASGW Weapons Loading/Unloading Procedures; including approved mixed loads |
| MTE-206.1.1 | - | Shall | FASGW(L) |  |
| MTE-206.1.2 | - | Shall | FASGW(H) |  |
| MTE-206.1.3 | - | Shall | Stingray Torpedo |  |
| MTE-206.1.4 | - | Shall | Mk 11 Depth Charge |  |
| MTE-206.1.6 | - | Shall | M3M 12.7MM (0.5”) HMG |  |
| MTE-206.3.0 | The WLST | Shall Not | support the Weapon Loading Procedures for the 7.62mm GPMG |  |
| MTE-300.0.0 | - | Heading | Airframe Structure |  |
| MTE-300.2.0 | The WLST | Shall | have a physical appearance of the relevant Wildcat aircraft structure and weapon systems, including (but not be limited to) the simulation of: | utilising the GFE provided TI#2 airframe |
| MTE-300.2.1 | - | Shall | FASGW Temporary Role Equipment Items |  |
| MTE-300.2.2 | - | Shall | WCP |  |
| MTE-300.2.3 | - | Shall | Cable Breaks/Sockets |  |
| MTE-300.2.4 | - | Shall | reconfigurable CMDS dispenser brackets |  |
| MTE-300.2.5 | - | Shall | sponsons |  |
| MTE-300.2.6 | - | Shall | undercarriage |  |
| MTE-300.2.7 | - | Shall | relevant airframe obstructions and impediments to maintenance and weapon loading tasks |  |
| MTE-300.2.8 | - | Shall | Pre-FASGW Mod AIS Antenna Removal | AIS antenna not present on TI#2 a/c |
| MTE-400.0.0 | - | Heading | Avionic Systems |  |
| MTE-402.0.0 | - | Heading | Cockpit Display System (CDS) | associated with the FASGW weapon systems |
| MTE-402.1.0 | The WLST | Shall | provide a representative CDS HMI such that the EOs/TOS can be fulfilled in support of the specified maintenance tasks. | Minimum requirement for 2x IDU & 1x CDNU |
| MTE-500.0.0 | - | Heading | Mission Systems |  |
| MTE-502.0.0 | - | Heading | Tactical Processor (TP) |  |
| MTE-502.1.0 | The WLST | Shall | provide a representative TP HMI such that the EOs/TOS can be fulfilled in support of the specified maintenance tasks. |  |
| MTE-504.0.0 | - | Heading | Stores Management Computer (SMC) |  |
| MTE-504.1.0 | The WLST | Shall | provide a representative SMC HMI such that the EOs/TOS can be fulfilled in support of the specified maintenance tasks. |  |
| MTE-504.2.0 | The SMC | Shall | provide a representative simulation of the SMC PBIT functionality including malfunction conditions | to support the maintenance tasks and defect investigation and diagnostics |
| MTE-504.3.0 | The SMC | Shall | provide a representative simulation of the SMC IBIT functionality including malfunction conditions |  |
| MTE-506.0.0 | - | Heading | Electro-Optical Designation System (EODS) |  |
| MTE-506.1.0 | The WLST | Shall | provide a representative EODS HMI such that the EOs/TOS can be fulfilled in support of the specified maintenance tasks. |  |
| MTE-506.2.0 | The WLST | Shall Not | Support training of the ALGU BAT fitting elements of the ALGU calibration maintenance task |  |
| MTE-506.3.0 | The WAvT | Shall Not | be updated with ALGU functionality within its EODS simulation |  |
| MTE-506.4.0 | The WLST | Shall | Provide a representative simulation of the Armament Laser Guidance Unit (ALGU) HMI to complete the required training task/s | FASGW(L) |
| MTE-506.5.0 | The WLST | Shall | simulate the function of the ALGU Boresight Alignment Tool (BAT) | cockpit HMI only, ALGU BAT not to be fitted |
| MTE-506.6.0 | The WLST | Shall | simulate the following EODS/ALGU Malfunctions: |  |
| MTE-506.6.1 | - | Shall | ALGU Thermal Shutdown |  |
| MTE-506.6.2 | - | Shall | Turret boresight position error |  |
| MTE-506.6.3 | - | Shall | Stale SWLS boresight |  |
| MTE-506.6.4 | - | Shall | EON reported error |  |
| MTE-506.6.5 | - | Shall | ATT reported error |  |
| MTE-506.6.6 | - | Shall | Insufficient signal |  |
| MTE-506.6.7 | - | Shall | Excessive signal/scatter |  |
| MTE-600.0.0 | - | Heading | Weapons Systems |  |
| MTE-602.0.0 | - | Heading | FASGW(H) - Sea Venom |  |
| MTE-602.1.0 | The WLST | Shall | provide a representative Sea Venom/FASGW(H) HMI such that the EOs/TOS can be fulfilled in support of the specified maintenance tasks. |  |
| MTE-602.2.0 | The WLST | Shall | provide a tactile, representative simulation of the missile Isolation Safety Plug |  |
| MTE-604.0.0 | - | Heading | FASGW(L) - Martlet |  |
| MTE-604.1.0 | The WLST | Shall | provide a representative Martlet/FASGW(L) HMI such that the EOs/TOS can be fulfilled in support of the specified maintenance tasks. |  |
| MTE-604.2.0 | The WLST | Shall | include a tactile, representative simulation of the Store Enable Interlock Device plugs: |  |
| MTE-604.3.0 | The WLST | Shall | include a tactile, representative simulation of the Launcher Arming Enable Plug functionality |  |
| MTE-604.4.0 | The WLST | Shall | include a tactile, representative simulation of the Launcher Safety Plug functionality |  |
| MTE-700.0.0 | - | Heading | Aircraft System Support Equipment (ASSE) |  |
| MTE-700.1.0 | The WLST | Shall | include tactile, functional representations of the following ASSE, as required to conduct the maintenance tasks: | assumed to supplied as GFE or modified GFE/BFE |
| MTE-700.1.1 | - | Shall | Weapon Trolley | assumed to supplied as GFE |
| MTE-700.1.2 | - | Shall | Weapon Loading Boxes | assumed to supplied as GFE |
| MTE-700.1.3 | - | Shall | Weapon Hoist FASGW(H) Weapon Sling | assumed to supplied as GFE |
| MTE-700.1.4 | - | Shall | Quad Pod GSE to connect to Weapon Hoist | assumed to supplied as GFE |
| MTE-700.1.5 | - | Shall | Weapon Hoist LMM Interface | assumed to supplied as GFE |
| MTE-700.1.6 | - | Shall | FASGW (H) Weapon to Hoist interface | assumed to supplied as GFE |
| MTE-700.1.7 | - | Shall | AEREA EMRU/SRS test set (off Aircraft) | assumed to supplied as GFE |
| MTE-700.1.8 | - | Shall | AEREA Stores Loading/Unloading Torque Tool | assumed to supplied as GFE |
| MTE-700.1.9 | - | Shall | 1760 Databus Test Set | assumed to be simulated or modified GFE |
| MTE-700.1.10 | - | Shall | Launcher Continuity Test Set (LCTS) | assumed to be simulated or modified GFE |
| MTE-700.1.11 | - | Shall | GBIT Aircraft System Test Set | assumed to be simulated or modified GFE |
| MTE-800.0.0 | - | Heading | Instructor Operating Station |  |
| MTE-800.1.0 | The IOS | Shall | support injection of malfunctions using existing HMI |  |
| MTE-800.2.0 | The IOS | Shall | have the ability for instructors to set and remove malfunctions as listed elsewhere |  |

Table 8 - MTE System Requirements

- MTE Requirements to be provided as Options

Tables 9 contains a number of MTE options to be potentially provided above the baseline.

| Option No | Serial No | Preamble | Level | Requirement | Amplifying Information |
| --- | --- | --- | --- | --- | --- |
| See Option ATE-1 | GEN-130.2.0 | The STE | Could | include a spares package to cover the remaining duration of PP2 | PP2 completes April 2022 - notionally a 30-month duration |
| MTE-1 | MTE-200.5.1 | The WLST | Could | Support training of Weapon and FASGW system malfunctions |  |
| MTE-2 | MTE-200.9.0 | The WLST | Could | include tactile, non-functional representations of the PIE LRUs | space model only, to be provided as an Option |
| MTE-3 | MTE-200.11.0 | The WLST | Could | Support the re-alignment of the CMDS dispensers |  |
|  | MTE-200.12.0 | The WLST | Could | Support the training of the maintenance, loading or unloading of the CMDS |  |
| MTE-4 | MTE-204.2.0 | The WLST | Could | support the training of the following Maintenance Procedures from the CIETP: | Option iaw MTE-200.12.0 |
|  | MTE-204.2.1 | - | Could | C99-31-00-00-340A-A Countermeasures dispensing system - Function tests | Option iaw MTE-200.12.0 |
|  | MTE-204.2.2 | - | Could | C99-xx-xx-xx-xxx-A Countermeasures dispensing system - LH Sponson Dispenser Realignment (not yet issued) | Option iaw MTE-200.12.0 |
|  | MTE-204.2.3 | - | Could | C99-xx-xx-xx-xxx-A Countermeasures dispensing system - RH Sponson Dispenser Realignment (not yet issued) | Option iaw MTE-200.12.0 |
| MTE-5 | MTE-206.2.0 | The WLST | Could | support the Weapon Loading & Unloading Procedures for the following weapon systems; | to be provided as an Option iaw Wildcat AH Mk1 and HMA-FASGW Weapons Loading/Unloading Procedures; including approved mixed loads |
|  | MTE-206.2.1 | - | Could | VICON 78 Series 455 CMDS ARI 18254 | Option iaw MTE-200.12.0 |
| MTE-6 | MTE-602.3.0 | The WLST | Could | simulate the following Sea Venom - FASGW(H) Malfunctions; |  |
|  | MTE-602.3.1 | - | Could | PIE NOGO Status |  |
|  | MTE-602.3.2 | - | Could | Missile DEGR (No MDLT) |  |
|  | MTE-602.3.3 | - | Could | Missile NOGO |  |
|  | MTE-602.3.4 | - | Could | Misfire - missile did not report committed to store separation - [STORE HAZARD] |  |
|  | MTE-602.3.5 | - | Could | Missile Hung - missile reported committed to store separation - [STORE HAZARD] |  |
|  | MTE-602.3.6 | - | Could | Missile Hung - unsuccessful Jettison |  |
| MTE-7 | MTE-604.5.0 | The WLST | Could | simulate the following Martlet - FASGW(L) Malfunctions; |  |
|  | MTE-604.5.1 | - | Could | Launcher DEGR |  |
|  | MTE-604.5.2 | - | Could | Launcher NOGO |  |
|  | MTE-604.5.3 | - | Could | Missile Aborted Message |  |
|  | MTE-604.5.4 | - | Could | Store Battery Misfired - [STORE HAZARD] |  |
|  | MTE-604.5.5 | - | Could | Store Motor Misfired - [STORE HAZARD] |  |
| MTE-8 | N/A | The MTE | Could | be modified to include the additional WOANS included within JMS LH18331 TP Uplift (cost delta) | JMS LH18331 TP Uplift to be considered as FASGW baseline (MTE-502.1.0), minus the WOANS |

Table 9 - MTE Requirements to be provided as Options

## PART FOUR - NON-FUNCTIONAL REQUIREMENTS

FOR THE INTRODUCTION OF FASGW CAPABILITY INTO THE WILDCAT STE

- Non-Functional STE Requirements

| Serial No | Preamble | Descriptor | Requirement | Amplifying Information |
| --- | --- | --- | --- | --- |
| GEN-10.0.0 |  | Heading | General Context |  |
| GEN-10.1.0 | The design | Shall | be based upon an enhancement to the existing STE system as delivered to the UK MOD under the AW159 Wildcat TCE contract |  |
| GEN-10.2.0 | The design | Shall | incorporate technological advances and lessons identified from previous programmes |  |
| GEN-10.3.0 | The design | Shall | be designed to minimise the through life cost of change by maximising the commonality of design between all the STE |  |
| GEN-10.4.0 | The design | Shall | align with the latest FASGW schedule and include FASGW programme changes up to Contract Amendment 109 |  |
| GEN-20.0.0 |  | Heading | Usability requirements |  |
| GEN-20.1.0 | The Systems | Shall | utilise a Microsoft Windows based Software Environment |  |
| GEN-20.2.0 | The Systems | Shall | utilise an Operating system that on delivery is within the extended support period of the SW lifecycle | optimally not less than 2-years of support remaining |
| GEN-30.0.0 |  | Heading | Performance requirements |  |
| GEN-30.1.0 | The ATE | Shall | comply with the requirements of ICAO 9625, unless specifically addressed within the contracted requirement set |  |
| GEN-40.0.0 |  | Heading | System interfaces |  |
| GEN-40.1.0 | The Systems | Shall | employ industry standard protocols for network communication whether internally via LAN or externally via a WAN | i.e. HLA |
| GEN-45.0.0 |  | Heading | Interoperability |  |
| GEN-45.1.0 | The ATE | Shall | support the use of international open standards for simulation interoperability |  |
| GEN-45.2.0 | The ATE | Shall | Provide, for future UK MOD use, a copy of any updated data or databases and 3D models to the UK MOD Defence Simulation Centre (DSC) repository in a format consistent with the DTEC Modelling & Simulation (M&S) Standards Profile (DMSP) v5 dated 13 Nov 15. | iaw DTEC MODELLING & SIMULATION (M&S) STANDARDS PROFILE (DMSP) |
| GEN-50.0.0 |  | Heading | System operations |  |
| GEN-50.1.0 | The Systems | Shall | utilise the same operating HMI for all devices of the same grouping; ATE or MTE |  |
| GEN-50.2.0 | The ATE Supplier | Shall | provide for and support 2x test bed environments for SW development and initial integration | identical specifications as defined by the Supplier located in the Supplier's facility and LH Yeovil Site |
| GEN-60.0.0 |  | Heading | System modes and states |  |
| GEN-60.1.0 | The STE | Shall | provide self-diagnostic testing of the STE to the instructor station in order to determine the integrity of hardware and software operation whilst in operation |  |
| GEN-60.2.0 | The STE | Shall | provide a means for quickly and effectively conducting daily testing of the STE software and hardware. |  |
| GEN-90.0.0 |  | Heading | System security |  |
| GEN-90.1.0 | The STE | Shall | conform to the requirements of JSP 604 for installation | replaces JSP 480 |
| GEN-90.2.0 | The ATE | Shall | will maintain Security Classification of the STE at Official-Sensitive | Operation at Security Classifications above OS to be managed procedurally to prevent escalation of ATE classification. |
| GEN-110.0.0 |  | Heading | Information management |  |
| GEN-110.1.0 | The Supplier | Shall | submit data for information or approval iaw with the CDRL and corresponding DID | iaw the contracted SOW |
| GEN-110.2.0 | The Supplier | Shall | implement a DRACAS process through the utilisation of a shared Snag/Bug/Discrepancy Tracking Tool |  |
| GEN-120.0.0 |  | Heading | Policies and Regulations |  |
| GEN-120.1.0 | The Supplier | Shall | Comply with the requirements of QRS01 - AgustaWestland - Quality Requirements for Suppliers |  |
| GEN-120.2.0 | The Systems | Shall | Qualify for CE Marking; confirming, as a minimum, compliance with the relevant European health, safety and environmental protection legislation plus the Low Voltage and the Electro Magnetic Compatibility (EMC) Directives | Suppliers will provide the required Declaration of Conformity Regulation (EC) No 765/2008 |
| GEN-120.3.0 | The ATE | Shall | be developed and tested consistently to the guidance within SISO-GUIDE-001.1-2012; Guide for Generic Methodology for Verification and Validation (GM-VV) to Support Acceptance of Models, Simulations, and Data |  |
| GEN-120.4.0 | The Supplier | Shall | conduct Configuration Management activities in accordance with DEF STAN 05-57; Configuration Management of Defence Materiel and ISO 10007:2017; Quality management -- Guidelines for configuration management |  |
| GEN-120.5.0 | The Supplier | Shall | conduct Configuration Management activities in accordance with the AgustaWestland document NA00G0004W |  |
| GEN-130.0.0 |  | Heading | System life cycle sustainment |  |
| GEN-130.1.0 | The Design | Shall | use, as far as possible COTS components and SW |  |
| GEN-130.5.0 | The Design | Shall | minimise the demand on BFE/GFE as far as is possible through either the external sourcing of real a/c parts or by manufacture | BFX/GFX requirement to be identified as early as possible recognising variation to the list may change |
| GEN-130.6.0 | The ATE | Shall | comprise a capability enhancement to the devices contracted under the Wildcat D&M TCE contracts | The requirements contained within the associated contracts will remain compliant to the extent determined at acceptance of those devices. |
| GEN-130.7.0 | The Supplier | Shall | Minimize the impact upon the availability for training of the existing devices during the upgrade process | disruption within the MTE hall should be minimised |
| GEN-130.8.0 | The Supplier | Shall | Identify any impact associated with this upgrade in respect of the Customer Logistic Support (CLS) contract and spares inventory. |  |
| GEN-140.0.0 |  | Heading | Packaging, handling, shipping and transportation |  |
| GEN-140.1.0 | The Systems | Shall | Be delivered to and installed at the WTC, RNAS Yeovilton |  |

Table 10 - Non-Functional STE Requirements

## PART FIVE – FACTORY DELIVERED TRAINING

- Factory Delivered Training Baseline Requirement

The ITT proposal response shall support Factory Delivered Training (FDT) courses as detailed in Tables 11 through 16. FDT shall require the training of a number of different Military roles, including Train-the-Trainers and RN IOC Ships Flight Crews.

| FDT Course 1A |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4 Aircrew |  | 2 x WTC RN Pilot/Observer crew |  |  |
| Serial | Lesson Phase | Delivered By | Location | Remarks |
| 1.1 | Classroom | TA(UK) supported by OEM FASGW(H&L) personnel | TA(UK) LH Yeovil | Lesson to include: Component location & function, armed aircraft safety, role change and restrictions on role, FASGW HMI (SMS, WCP, EODS Wpn page). |
| 1.2a OR | Practical - using live aircraft (Depending on delivery of WLST) | TA(UK) supported by RN ATEC/LH Test &Trials Weapons Team | LH Yeovil | Safety pin routine and component location. Witness FASGW H&L, Depth Charge and Stingray loading/unloading procedures. |
| 1.2b | Practical – using WLST (Depending on delivery of WLST) | TA(UK) supported by RN ATEC/LH Test &Trials Weapons Team | WTC Yeovilton | Safety pin routine and component location. Witness FASGW H&L, Depth Charge and Stingray loading/unloading procedures. |
| 1.3 | Engineering Simulator | RN ATEC/LH flight/weapons trials test crew | LH Yeovil | Crew-based FASGW H&L attack operations, including malfunctions and hang-up drills. 2 x 2 hour sorties per crew. |
| 1.4 | Live flying serial – Pilot student | RN ATEC/LH flight/weapons trials test crew | Merryfield | Dummy weapon flying serial to include: normal carriage load handling, malfunction handling, sloping ground landings. Light Wpn attack operations using Aircraft Training Mode. 1 x 1.5 hour sortie per pilot. Observer student to witness via aircraft rear seat. |

Table 11 - FDT Course 1A (Aircrew)

| FDT Course 1B |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 OME Maintainers |  | 2 x RN Sqn Instructors (Proposal for a joint MOD/LH course consisting of 2 x RN Instructors plus 4 x WIST Instructors) |  |  |
| Serial | Lesson Phase | Delivered By | Location | Remarks |
| 1.6 | Classroom | TA(UK) supported by OEM FASGW(H&L) personnel | TA(UK) LH Yeovil | Course content to include: FASGW H&L, Depth Charge and Stingray component location & function, OME role equipment and test equipment. |
| 1.7a OR | Practical – using live aircraft (Depending on delivery of WLST) | TA(UK) supported by RN ATEC/LH Test &Trials Weapons Team | LH Yeovil | Lesson to include: Weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |
| 1.7b | Practical – using WLST (Depending on delivery of WLST) | TA(UK) supported by RN ATEC/LH Test &Trials Weapons Team | WTC Yeovilton | Lesson to include: Weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |

Table 12 - FDT Course 1B (Maintainers)

| FDT Course 2A |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4 Aircrew |  | 1 x WTC RN Pilot/Observer crew 1 x RN Sqn QHI/QOI crew |  |  |
| Serial | Lesson Phase | Delivered By | Location | Remarks |
| 2.1 | Classroom | TA(UK) supported by OEM FASGW(H&L) personnel | TA(UK) LH Yeovil | Lesson to include: Component location & function, armed aircraft safety, role change and restrictions on role, FASGW HMI (SMS, WCP, EODS Wpn page). |
| 2.2a OR | Practical - using live aircraft (Depending on delivery of WLST) | TA(UK) supported by RN ATEC/LH Test &Trials Weapons Team | LH Yeovil | Safety pin routine and component location. Witness FASGW H&L, Depth Charge and Stingray loading/unloading procedures. |
| 2.2b | Practical – using WLST (Depending on delivery of WLST) | TA(UK) supported by RN ATEC/LH Test &Trials Weapons Team | WTC Yeovilton | Safety pin routine and component location. Witness FASGW H&L, Depth Charge and Stingray loading/unloading procedures. |
| 2.3 | Engineering Simulator | RN ATEC/LH flight/weapons trials test crew | LH Yeovil | Crew-based FASGW H&L attack operations, including malfunctions and hang-up drills. 2 x 2 hour sorties per crew. |
| 2.4 | Live flying serial – Pilot student | RN ATEC/LH flight/weapons trials test crew | Merryfield | Dummy weapon flying serial to include: normal carriage load handling, malfunction handling, sloping ground landings. Light Wpn attack operations using Aircraft Training Mode. 1 x 1.5 hour sortie per pilot. Observer student to witness via aircraft rear seat. |

Table 13 - FDT Course 2A (Aircrew)

| FDT Course 3B |  |  |  |  |
| --- | --- | --- | --- | --- |
| 8 OME Maintainers |  | Initial IOC Ships Flight Maintainers |  |  |
| Serial | Lesson Phase | Delivered By | Location | Remarks |
| 2.5 | Classroom | TA(UK) / WIST Instructor | tbc | Lesson to include: Weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |
| 2.6a OR | Practical - using live aircraft (Depending on delivery of WLST) | TA(UK) Instructors | tbc | Conduct weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |
| 2.6b | Practical – using WLST (Depending on delivery of WLST) | WIST Instructors | WTC Yeovilton | Conduct weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |

Table 14 - FDT Course 2B (Maintainers)

| FDT Course 3A |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4 Aircrew |  | 1 x Initial IOC Ships Flight crew 1 x WTC RN Pilot/Observer crew |  |  |
| Serial | Lesson Phase | Delivered By | Location | Remarks |
| 3.1 | Classroom | TA(UK) / WIST Instructor | tbc | Lesson to include: Component location & function, armed aircraft safety, role change and restrictions on role, FASGW HMI (SMS, WCP, EODS Wpn page). |
| 3.2a OR | Practical - using live aircraft (Depending on delivery of WLST) | TA(UK) Instructors | tbc | Safety pin routine and component location. Witness FASGW H&L, Depth Charge and Stingray loading/unloading procedures. |
| 3.2b | Practical – using WLST (Depending on delivery of WLST) | WIST Instructors | WTC Yeovilton | Safety pin routine and component location. Witness FASGW H&L, Depth Charge and Stingray loading/unloading procedures. |
| 3.3 | Engineering Simulator | RN ATEC/LH flight/weapons trials test crew | LH Yeovil | Crew-based FASGW H&L attack operations, including malfunctions and hang-up drills. 2 x 2 hour sorties per crew. |
| 3.4 | Live flying serial – Pilot student | RN Sqn QHI (Dependant on delivery of FASGW modified a/c to FLC) | Merryfield | Dummy weapon flying serial to include: normal carriage load handling, malfunction handling, sloping ground landings. Light Wpn attack operations using Aircraft Training Mode. 1 x 1.5 hour sortie per pilot. Observer student to witness via aircraft rear seat. |
| 3.5 | Live flying serial – Observer student | RN Sqn QOI via rear seat. Aircraft flown by Pilot student. (Dependant on delivery of FASGW modified a/c to FLC) | tbc | FASGW Heavy, Light and Existing Wpn attack operations using Aircraft Training Mode. 1 x 1.5 hour sortie per observer. |

Table 15 - FDT Course 3A (Aircrew)

| FDT Course 3B |  |  |  |  |
| --- | --- | --- | --- | --- |
| 8 OME Maintainers |  | 2nd IOC Ships Flight Maintainers |  |  |
| Serial | Lesson Phase | Delivered By | Location | Remarks |
| 3.6 | Classroom | TA(UK) / WIST Instructor | tbc | Lesson to include: Weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |
| 3.7a OR | Practical - using live aircraft (Depending on delivery of WLST) | TA(UK) Instructors | tbc | Conduct weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |
| 3.7b | Practical – using WLST (Depending on delivery of WLST) | WIST Instructors | WTC Yeovilton | Conduct weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |

Table 16 - FDT Course 3B (Maintainers)

- Factory Delivered Training Non-baseline Options

For contingency planning, the ITT proposal response shall also include additional FDT courses as detailed in Tables 17 through 19, as non-baseline costed options. If the FASGW courseware and ATE/MTE have been delivered and accepted by the Authority, the relevant FDT Course shall be delivered in the WTC by the WIST/WTC cadre instructors.

| Option No: FDT Course 4A |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4 Aircrew |  | Additional IOC Ships Flight crews |  |  |
| Serial | Lesson Phase | Delivered By | Location | Remarks |
| 4.1 | Classroom | TA(UK) / WIST Instructor | tbc | Lesson to include: Component location & function, armed aircraft safety, FASGW HMI (SMS, WCP, EODS Wpn page) |
| 4.2a OR | Practical - using live aircraft (Depending on delivery of WLST) | TA(UK) Instructors | tbc | Safety pin routine and component location. Witness FASGW H&L, Depth Charge and Stingray loading/unloading procedures. |
| 4.2b | Practical – using WLST (Depending on delivery of WLST) | WIST Instructors | WTC Yeovilton | Safety pin routine and component location. Witness FASGW H&L, Depth Charge and Stingray loading/unloading procedures. |

Table 17 - Non-baseline FDT Course 4A (Aircrew)

| Option No: FDT Course 4B |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4 Aircrew |  | Additional IOC Ships Flight crews |  |  |
| Serial | Lesson Phase | Delivered By | Location | Remarks |
| 4.3 | Engineering Simulator | RN ATEC/LH flight/weapons trials test crew | LH Yeovil | Crew-based FASGW H&L attack operations, including malfunctions and hang-up drills. 2 x 2 hour sorties per crew. |

Table 18 - Non-baseline FDT Course 4B (Aircrew)

| Option No: FDT Course 4C |  |  |  |  |
| --- | --- | --- | --- | --- |
| 8 OME Maintainers |  | Additional IOC Ships Flight Maintainers |  |  |
| Serial | Lesson Phase | Delivered By | Location | Remarks |
| 4.4 | Classroom | TA(UK) / WIST Instructor | tbc | Lesson to include: Weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |
| 4.5a OR | Practical - using live aircraft (Depending on delivery of WLST) | TA(UK) Instructors | tbc | Conduct weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |
| 4.5b | Practical – using WLST (Depending on delivery of WLST) | WIST Instructors | WTC Yeovilton | Conduct weapon wing installation/removal and loading/unloading drills for FASGW H&L, Depth Charge and Stingray. Conduct OME test set procedures. |

Table 19 - Non-baseline FDT Course 4C (Maintainers)