Title: Supply and Installation of New Multi-Use Games Area (MUGA) and Playing Surface.

Description of Requirement:
Laverstock & Ford Parish Council is inviting quotes from qualified contractors for the supply and installation of a new Multi-Use Games Area (MUGA) and playing surface. The project is to include:

- Excavation and removal of the existing playing surface material

- Removal and disposal of the existing sports wall

- Installation of an 18.5m x 15.5m tarmac playing surface with concrete edgings at ground level

- Installation of MUGA goal ends and fencing as specified

Location of Delivery:
Old Sarum Community Centre, Pheasant Drive, W3W: ///envoy.instead.revived

Contract Value / Budget:
£40k to £60k

Procurement Type:
Works / Construction Services

Heras fencing, secure storage, welfare facilities as required, construction site, and disposal of packing and exiting.

Excavate 18.5m x 15.5m area to approximately to a minimum of 150mm and dispose of all arisings off site, including existing surface where required. Install geotextile membrane, concrete pin curb edgings fully haunched in concrete. Supply and lay 100mm of 40mm Type 1 stone base. Reinstate

Supply and install IAE Duex Rebound 868 MUGA approximately 18 x 15m. 2 x recessed goal ends with basketball hoops, one goal end to have side open for access. 2 x 3m high ends and 1 x 3m high side. 1 x 1m high side with spectator rail. 1 x side @ 1m high with 1200mm high spectator rail. I x Chicane entrance within the 1m high side. Supply and lay new surface using Porous Macadam with the appropriate volume to curb level. Install painted line markings on finished surface for basketball and five-a side football.

Contract Duration:

- Expected start date: Spring 2026

- Completion required by: Summer 2026

Eligibility / Requirements:

- Proven experience in construction of MUGAs or sports surfaces

- Appropriate insurance and public liability cover

- Compliance with relevant health and safety regulations

How to Submit a Quote:
Suppliers should submit a detailed quote including:

- Breakdown of costs (materials, labour, disposal)

- Estimated timeline for completion

- Any additional recommendations or considerations

Deadline for Quotes:
05.02.2026

Further Information / Contact:
Jon Sloan
Parish Clerk
parish-clerk@laverstockford-pc.gov.uk
01722 433881