Incident Data Standard for ASC

Network Delivery and Development Directorate

- Document Control

| Document Title | Incident Data Standard |
| --- | --- |
| Author | Nigel Dowden |
| Owner | Janet Sivorn, Asset Management Office/PG |
| Distribution | TBA |
| Document Status | Revision v1.0 |

- Revision History

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 0.1 | 20-Mar-2012 | For review/approval by HA Stakeholder | Nigel Dowden |
| 1.0 | 29-Mar-2012 | Incorporating stakeholder feedback | Nigel Dowden |

- Approvals

| Name | Document Version | Approval Date |
| --- | --- | --- |
| Jan Sivorn | 1.0 – Initial Release | 29-Mar-2012 |

- Contents

Document Control	2

Revision History	2

Approvals	2

Contents	3

1.	Purpose	3

2.	Data Dictionary Overview	3

3.	Monthly Data File – Structure and Naming Convention	3

3.1	Performance Data – ASC Contract Types	3

4.	Incident Data Standard Data Dictionary	3

4.1	Performance Data – ASC/Retrofit Contracts	3

5.	HA Reference Data	3

5.1	INCIDENT_REPORTED_BY	3

5.2	INCIDENT LOCATIONAL DETAILS	3

5.3	INCIDENT_TYPE	3

6.	Service Provider Reference Data	3

6.1	RESPONSE_UNIT_NAME and INCIDENT_SUB_AREA	3

# Purpose

The purpose of this document is to define the Incident Data that the Highways Agency (HA) requires each Supply Chain contractor to provide in accordance with AMOR Appendix 3 (Incident Response Operational Requirement) in support of Performance Management Framework (PMF) on a monthly basis.

It should be noted that there are additional reporting requirements contained within AMOR Appendix 3 which are not set out in this document (e.g. reporting Critical/Major Incidents to the nominated HA representative immediately following each occurrence). These should be met by the ASC / Retrofit MAC.

# Data Dictionary Overview

The Incident Data Standard for ASC below is in ‘Data Dictionary’ format.

Each data column that needs to be supplied by the SP on a monthly basis is described as an individual row containing the following information:

| Information | Description |
| --- | --- |
| Name | This is the column name required in the monthly data file |
| Data type | Describes the data type each column should be presented as within the monthly data file |
| Mandatory | When this is Y then the column must be populated in accordance with the described rules. If it is blank then the data load will not fail, however the MAC should follow the guidance in relation to how to populate the field – in some cases the field should be left blank, whilst in others a default value is defined. |
| Comment | Describes any rules pertaining to population of the column. |

# Monthly Data File – Structure and Naming Convention

The monthly data file should be produced as a number of rows (one per Incident Response) within an Excel Spreadsheets (Excel 2003).

## Performance Data – ASC Contract Types

The Performance Data file should contain 64 columns of data – starting with HA_AREA, then REPORTING_PERIOD and so on, with the final column being FLEXIBLE_ATTRIBUTE_10. The file naming convention should be as follows:

- ASC_Incident_AA_YYYYMM.xls where AA is the Area including a leading-edge zero if necessary, YYYY is the year, and MM is the Reporting Month

- E.g. File name for Area 2 reporting in July 2012 would be ASC_Incident_02_201207.xls

# Incident Data Standard Data Dictionary

## Performance Data – ASC/Retrofit Contracts

The data elements required are detailed below.

| Name | Data Type | Mandatory | Comment |
| --- | --- | --- | --- |
| HA_AREA | Characters (2) | Y | E.g. 03 or 12. Must contain leading zero |
| REPORTING_PERIOD | Number (6) | Y | Must be of format YYYYMM |
| IMS_INCIDENT_REFERENCE NUMBER | Characters (8) | Y | Unique ID from MAC Incident Management System (IMS) |
| RCC_UNIQUE_REFERENCE_NUMBER | Characters (8) | Y | Unique ID from RCC Command and Control System – mandatory where available |
| ASC_DEFINED_INCIDENT | Text(1) | Y | Y/N |
| IDENTIFIED_OR_NOTIFIED | Text (10) | Y | One of two values: either IDENTIFIED or NOTIFIED. No other values are valid. |
| CRITICAL_OR_MAJOR | Text (8) | Y | One of three values: CRITICAL, MAJOR, NEITHER |
| INCIDENT_IN_LIVE_LANES | Text(1) | Y | Y/N |
| INCIDENT_REPORTED_BY | Text (50) | N | Defined by the HA INCIDENT_REPORTED_BY spreadsheet. A list of values is shown in the Reference Data section. The default value OTHER (DETAILED DESCRIPTION REQUIRED) should be used where MAC cannot populate this field for any given Incident Response. |
| INCIDENT_DATE_TIME | Date & Time | Y | Must be of format: DD/MM/YYYY HH:MI:SS. Where the INCIDENT_DATE_TIME is not available, the MAC should not populate this field (i.e. a null field). It should not contain blanks/spaces etc. |
| LOG_DATE_TIME | Date & Time | Y | Must be of format: DD/MM/YYYY HH:MI:SS. This is the time when the notifying phone call was complete for in the case of NOTIFIED incidents; and the time when the incident was logged in the Service Provider’s IMS for IDENTIFIED incidents. |
| DEPLOYMENT_DATE_TIME | Date & Time | Y | Must be of format: DD/MM/YYYY HH:MI:SS |
| ON_SITE_DATE_TIME | Date & Time | Y | Must be of format: DD/MM/YYYY HH:MI:SS. There is an exception to this rule where the Incident Response has been cancelled. In these cases the MAC should not populate this field (i.e. a null field). It should not contain blanks/spaces etc. |
| OFF_SITE_DATE_TIME | Date & Time | Y | Must be of format: DD/MM/YYYY HH:MI:SS. There is an exception to this rule where the Incident Response has been cancelled. In these cases the MAC should not populate this field (i.e. a null field). It should not contain blanks/spaces etc. |
| RESPONSE_CANCELLED_FLAG | Text (1) | Y | Upper case Y or N only |
| INCIDENT_SUB_AREA | Text (20) | N | Must match the MAC Area ISU spreadsheet provided by the MAC to HA. The default value ALL should be used where MAC chooses not to populate this field for any given Incident Response. |
| SECTION_REF | Text (25) | Y | Reference in accordance with NMM Part 2, Annex 2.3.1 and Annex 2.4.1 e.g. 1900A1M/25, 4720A6210/103, 1900M62/105 |
| ROAD_CODE | Text (5) | Y | Details pertaining to the HAPMS Section e.g. S2/D2/D4M |
| SECTION_FUNCTION | Text (25) | Y | E.g. MAINLINE, ENTRY SLIP, EXIT SLIP, JUNCTION, ROUNDABOUT |
| DIRECTION_CODE | Text (5) | Y | Constrained values from REFERENCE DATA – e.g. N, S, E, W, CW, AC, RB |
| XSP_CODE | Text (5) | Y | Constrained values from REFERENCE DATA – e.g. NL, CL, OL |
| INCIDENT_OSGR_NORTHING | Number(6) | Y | OSGR Northing of Incident |
| INCIDENT_OSGR_EASTING | Number(6) | Y | OSGR Easting of Incident |
| INCIDENT_LOCATION_DETAIL | Text (255) | N | If the MAC does not have INCIDENT_LOCATION_DETAIL for an Incident then it should not populate this field (i.e. a null field). It should not contain blanks/spaces etc. |
| RESPONSE_UNIT_NAME | Text (20) | Y | Has to link to MAC Area ISU spreadsheet. |
| RESPONSE_VEHICLE_TYPE | Text(20) | Y | One of REFERENCE DATA defined list including: MOTORCYCLE, CAR, VAN, TRUCK |
| RESPONSE_UNIT_ORDER | Number (2) | Y | 1 to 99 – must not contain zero. The MAC must ensure that (at a minimum) all ISU Responses for each Incident are provided. |
| RESPONSE_DETAIL | Text (255) | N | If the MAC does not have RESPONSE_DETAIL for an Incident then it should not populate this field (i.e. a null field). It should not contain blanks/spaces etc. |
| INCIDENT_TYPE | Text (100) | Y | Must conform to data type in HA Incident Type reference data spreadsheet. |
| INCIDENT_DETAIL | Text (255) | N | If the MAC does not have INCIDENT_DETAIL for an Incident then it should not populate this field (i.e. a null field). It should not contain blanks/spaces etc. |
| INCIDENT_IN_ROADWORKS_FLAG | Text (1) | Y | Upper case Y or N only. |
| EMERGENCY_SERVICES_PRESENT | Text (1) | Y | Upper case Y or N only. |
| ROAD_TRAFFIC_LEVEL | Text (5) | Y | One of ALL, LIGHT or HEAVY |
| NOTIFYING_AGENCY | Text (4) | Y | One of TOS, EMS, NONE |
| TIRP_PRODUCTION_DATE_TIME | Date & Time | Y | Must be of format: DD/MM/YYYY HH:MI:SS |
| TIRP_ESTIMATED_CLEARANCE | Date & Time | Y | Must be of format: DD/MM/YYYY HH:MI:SS |
| TIRP_INCIDENT_TERM | Text (6) | Y | Constrained to values of SHORT, MEDIUM or LONG. No other values are valid. |
| LEAD_AGENCY | Text (3) | Y | One of TOS, EMS, SP |
| ROAD_WEATHER_CONDITION | Text(4) | Y | Constrained to values of DRY, WET, SNOW or ICE. No other values are valid. |
| LANE_CLOSURE_DATE_TIME | Date & Time | Y | Must be of format: DD/MM/YYYY HH:MI:SS |
| LANE_OPENING_DATE_TIME | Date & Time | Y | Must be of format: DD/MM/YYYY HH:MI:SS |
| LANES_CLOSED | Text(255) | C | Conditional – Mandatory if LANE_RESTRICTIONS_APPLIED = Y. Selected from HA reference data (e.g. 1 LANE, WHOLE CARRIAGEWAY) |
| CMD_HANDOVER_DATE_TIME | Date & Time | Y | Must be of format: DD/MM/YYYY HH:MI:SS |
| LANE_RESTRICTIONS_APPLIED | Text (1) | Y | Y/N |
| LANE_RESTRICTION_TYPE | Text (3) | C | Conditional – Mandatory if LANE_RESTRICTIONS_APPLIED = Y. One of either ETM or TTM. |
| FIRST_RESTRICTION_ON_DATE_TIME | Date & Time | C | Conditional – Mandatory if LANE_RESTRICTIONS_APPLIED = Y. Must be of format: DD/MM/YYYY HH:MI:SS |
| LAST_RESTRICTION_OFF_DATE_TIME | Date & Time | C | Conditional – Mandatory if LANE_RESTRICTIONS_APPLIED = Y. Must be of format: DD/MM/YYYY HH:MI:SS |
| NOTIFIED_FIRST_ON_DATE_TIME | Date & Time | C | Populated in the event that a change of TM is made during an Incident. Must be of format: DD/MM/YYYY HH:MI:SS |
| NOTIFIED_LAST_OFF_DATE_TIME | Date & Time | C | Populated in the event that a change of TM is made during an Incident. Must be of format: DD/MM/YYYY HH:MI:SS |
| DCP_RECOVERAB:E_INCIDENT | Text (1) | Y | Upper case Y or N only. |
| DCP_ESTIMATED_COST | Number (6) | Y | Estimated value in £. |
| DCP_ACTUAL_COST | Number (6) | Y | Actual value in £. |
| DCP_COST_UPDATE_PROVIDED | Text (1) | Y | Upper case Y or N only. The whole record must be supplied again when the costs are known i.e. it will occur in >1 Reporting Month. |
| FLEXIBLE_ATTRIBUTE_1 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_2 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_3 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_4 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_5 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_6 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_7 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_8 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_9 | Variable |  | This fields is not yet defined |
| FLEXIBLE_ATTRIBUTE_10 | Variable |  | This fields is not yet defined |

# HA Reference Data

There are a number of HA Reference Data types which are required in order to standardize the responses from the ASC / Retrofit MACs. These are described below, including the list of valid values which has been agreed at the point of document issue. These may change over time; in which case the Service Manager will issue an updated list.

## INCIDENT_REPORTED_BY

The following list shows the valid data element values:

## INCIDENT LOCATIONAL DETAILS

There are a number of HA Reference Data sets that define the Incident Location Details. The method chosen to locate incidents is aligned to the current Pavement Management System (HAPMS) and is centered on Section Codes. The PMF-Plus system will not initially have a dynamic link to HAPMS, however each SECTION_REF will be checked for valid Road, Road Type and Local Authority Area Codes in accordance with the latest HA Guidance.

It is the Service Provider responsibility to ensure that Road Type, Section Function, Directional Code and XSP Code associated with the Incident are accurate given the INCIDENT_OSGR_NORTHING, INCIDENT_OSGR_EASTING and SECTION_REF.

The following constrained attributes will be checked against the current possible values:

- Road Types

- Section Functions

- Direction Codes

- XSP Codes

## INCIDENT_TYPE

The following list shows the valid data element values:

# Service Provider Reference Data

The ASC / Retrofit MACs are being given the opportunity to define one aspect of their operational organisation by means of supplying reference data pertaining to their ISUs and any associated in-Area geographic/other groupings that there may be (e.g. allocating a given ISU to EAST or CENTRAL or NIGHT). It is the responsibility of the ASC / Retrofit MAC to notify the NPU via the NPU inbox if there are changes to the initial set-up.

## RESPONSE_UNIT_NAME and INCIDENT_SUB_AREA

The following list shows the valid data element values: