# Risk Register

This Risk Register is for the activities which are considered specific to this project and does not detail the most common/ general risks associated with all building contracts.  This will be added to as necessary during the construction and refurbishment works.

| Produced by: Paul Brough |  |  |  | Project: Refurbishment of retail units, ground floor and first floor storage areas. |  |
| --- | --- | --- | --- | --- | --- |
| Location: 104 – 116 Church Street, Stoke |  |  |  | Date: 4th May 2018 |  |
| Activity | Hazards | At risk | Controls to mitigate the hazards |  | Owner of hazard and controls |
| Work on steels | Work at height Power tools Manual handling | Operatives Others | Only competent operatives to carry out work. Tools and equipment subject to regular maintenance regime. All operatives shall have instruction in safe manual handling techniques. Access scaffold shall be utilised, when not feasible then Class 1 access equipment shall be used. Gloves and safety glasses shall be worn when cutting materials. Area below works shall have suitable physical exclusion zone. Warning signs shall be displayed. |  | Principal Contractor |
| Repairs/renew brickwork | Use of cement Manual handling Work at heights Fall of materials | Operatives Others | Only competent operatives to work at height. Tools and equipment subject to regular maintenance regime. All operatives shall have instruction in safe manual handling techniques. Access scaffold shall be utilised, when not feasible then Class 1 access equipment shall be used. Suitable safety glasses and gloves to be worn. When working from scaffold materials and equipment shall be raised and lowered by pulley system. Area below works shall have suitable physical exclusion zone. Warning signs shall be displayed. |  | Principal Contractor |
| Asbestos containing materials | Fibre release | Operatives Others | No works shall be carried out on or near any known asbestos containing materials. Contractor to develop and “Emergency Action Plan” for any event where unidentified asbestos is discovered. All contractor employees shall review the Asbestos Survey report provided as part of the Pre-Construction Information pack. Contractors are to review the buildings “Certificate of Re-occupation” where asbestos containing materials have been removed. |  | Principal Contractor |
| Utility services | Fire/burns Explosion/leak Flooding | Operatives Others | Only trained and competent plumbers, gas fitters and electricians shall carry out any works to utility services (where this does not come under the remit of the utility supplier. CAT scanners and cable tracers shall be used before any “intrusive” works are undertaken. All services shall be isolated and protected prior to work commencing. The contractor shall notify the relevant utility provider if any uncontrollable damage is inflicted on any utility service. |  | Principal Contractor |
| Dropping and/or repairing of ceilings/walls | Manual handling Dust Work at height | Operatives | Operatives received suitable instruction in safe manual handling techniques. Ensure adequate ventilation. Suitable face mask to a minimum of FFP3 standard. Safety glasses, gloves and helmet shall be worn during dropping of ceilings/walls and clean-up operations. Only competent operatives shall work from any step ladder, hop up or trestle. |  | Principal Contractor |
| Plastering of surfaces | Manual handling Dust Work at height Power tools Hazardous materials Power tools | Operatives | Suitable coshh assessment to be produced for gypsum products. Operatives trained in safe manual handling techniques. Only competent persons shall work from any item of access equipment, including ladders and step ladders. Power tools shall be subject to regular inspection, testing and maintenance regime. Ensure adequate ventilation. Suitable face mask to FFP3 shall be worn during all mixing operations. |  | Principal Contractor |
| Rewire Electrical works | Manual handling Work with electricity Work at height | Operatives | Only suitably trained, competent and experienced electricians shall carry out the works. Electricians trained in safe manual handling techniques. Power supply shall be isolated prior to works commencing. Only competent electrician shall carry out work at height. Permit to work system shall be implemented as necessary. Access and step ladders shall be of fibre construction. |  | Principal Contractor |
| Installation of heating and water supply | Manual handling Utility service (gas) Hot Works | Operatives | Only suitably trained plumbers and heating engineers shall certificate shall carry out the works. Permits to work shall be utilised as necessary. Operatives trained in safe manual handling techniques Utility services to be isolated prior to work commencing. Water supply isolated prior to work commencing. Where necessary pipework shall be earth bonded. |  | Principal Contractor |
| Non-construction persons (client/residents/visitors | Personal injury Damage to property | Operative sand others | Demarcation zones set up around work areas. Physical barriers and warning signs shall be utilised. Visitors to the works area shall be accompanied at all times. Visitors to the project site shall complete the site attendance register Visitors to the project site shall wear all necessary PPE. The site shall be secure at all times. |  | Principal Contractor |
| Noise/dust Vibration | Hand arm vibration White finger Hearing loss both temporary and permanent Inhalation irritation | Operatives | Levels controlled as much as is reasonably practicable. Noisy tasks carried out at suitable times of the working day. Hearing protection available for personnel. Radios, personal music players etc. shall not be permitted. The Code of Practice ‘Control of pollution and noise from demolition and construction sites’ provided by the Principal Designer shall be adhered to. Suitable hearing protection shall be worn during noise producing works. Suitable dust/face masks shall be worn during all dust producing works. Powered equipment shall be inspected, tested and maintained. Ensure adequate ventilation. Only 110v or batter operated power tools are permitted. Time spent using vibration equipment shall be limited. |  | Principal Contractor |
| Fire | Flammable materials Sources of ignition |  | The Principal Contractor shall take all reasonable precautions to avoid the outbreak of fire, particularly in work involving the use of naked flames. Where necessary a “Hot Work” permit system shall be implemented. The Principal Contractor will provide suitable fire-fighting equipment for the duration of the project. Extinguishers shall be located appropriately, i.e. near any hazardous substances and at junctions within the building. An emergency evacuation procedure will be in place throughout the project and all personnel will be provided with information and instruction on the procedure at the site Induction. Any flammable materials will be suitably stored when not in use and shall be removed completely from the work sites at the end of each day. Smoking shall be strictly prohibited within the project site. |  | Principal Contractor |