Pendennis Headland, Falmouth, TR11 4LP.

Works: Resurfacing of lower car park area incorporating installation of ecogrid heavy duty permeable ground reinforcement grid with compacted granite chippings.

Description: Falmouth Town Council invite quotations relating to the above works to provide an environmentally friendly, maintenance free solution to stabilise the existing car parking area. The works have been subject to Scheduled Monument Consent as the area is a listed site and will require a watching brief during surface preparation works (this arrangement and cost will be managed by FTC).

Specification: Prepare ground to enable the installation of recycled polyethylene, UV stabilised, weather resident, non toxic, free draining paving crates. Load capacity in excess of 300 tons per m2 (Enviropave or similar approved) based upon the following:

Based upon an area of 740m2.

- Site set up, welfare and security arrangements.

- Excavate the existing top layer to a suitable depth, and remove all spoil from site via a licenced waste carrier

- Supply and lay fresh, clean Type 1 (clause 803) hard core base.

- Supply, lay and roll suitable levelling medium (0-4mm sharp sand)

- Supply and lay porous geotextile weed barrier.

- Supply and install SUDS-compliant 100% recycled polymer crates / pavers.

- Supply and infill with 10-14mm granite chippings. (To prevent the occasional loose chippings from sticking in the grooves of both walking boots and car tyres)

- Clean site and return to operational condition.

- Quotation to identify guarantee period.

Site contact;

Andy Medlin

Facilities Manager

Falmouth Town Council, The Old Post Office,

The Moor, Falmouth. TR11 3QA.

07485379879

01326 315559

AndyMedlin@falmouthtowncouncil.com