Langport Town Council

Langport River Project

Lot 4

Construction of Walkway

North bank, River Parrett, Langport

Somerset

TENDER PACKAGE

Closing Date: Thursday 09 April 2020

FAO: Deborah Speed, Town Clerk

Langport Town Hall

Town Hall

Bow Street

Langport TA10 9PR

01458 259700

townclerk@langport.life

The European Agricultural Fund

For Rural Development:

Europe investing in rural areas

Timescale of works:

Start Date: 07 September 2020

Completion Date:  before 02 October 2020

Description of the Works:

The works are for the construction of an all year-round accessible walkway between Whatley ST 42061 26561 towards Huish Bridge as far as ST 42443 26358 adjacent to the north bank of the River Parrett.   Approx. 500m – The construction of the walkway is to be 2m wide where possible. See Appendix One.

The contractor shall apply for a Flood Risk Activity Permit (FRAP) through the Environment Agency (EA).  Langport Town Council has already held several preliminary discussions with the EA and the following specifications have been agreed as to the completed height of the walkway as follows:

- The walkway can be constructed to 8.2m where the height of the bank is currently lower. However, the bank can be levelled to provide a level surface for the walkway to match the highest point of the bank, normally at the river’s edge except for the 30m spillway section. (See bullet point 3)

- Where the bank is currently higher than 8.2m the walkway can be constructed at the highest point, i.e. in no circumstances should the height be lowered.

- The construction of the walkway at the 30m spillway section shall not exceed 8.0m and the bank can be raised to 8.0m where it is currently lower.

- The width of the 2m walkway may need to be adjusted to meet the topography of the site.

- Any gradient from the outer edge of the constructed walkway should be taken into consideration access to EA plant machinery and safety of users.

- The length of the walkway is approx. 500 – 525m.

The Contractor shall consult with Langport Town Council about the final design and measurements of the works before and during construction and advises a site visit prior to quotation.  Access to site is via Huish Bridge, Huish Episcopi TA10 9HG

Appendix One:  Site map and location

Appendix Two:  Walkway survey data (Electronic data available)

Please supply a copy of:

Public Liability Insurance with a minimum of £5 million

VAT Registration number (if applicable)

Company registration number (if applicable)

Please provide two itemised quotations (satating Lot 4) on Company headed notepaper for the two options as follows:

COSTS ONE:  Gridlok Elite Plastic Paver

- Mobilise plant and equipment - remove on completion

- Application to Environment Agency for a Flood Risk Activity Permit (FRAP)

- Earthworks and site preparation as required for Gridlok Elite plastic paver surface in accordance with manufacturers specification and Appendix Two.

- Supply and lay Gridlok Elite plastic paver 2m wide in accordance with manufacturers specifications and Appendix Two.

- Supply and lay Gridlock edging in accordance with manufacturers specification.

- Supply, place and compact 6mm to 10mm clean angular stone in Gridlok Elite plastic paver.

COSTS TWO: Grassblock

- Mobilise plant and equipment - remove on completion

- Application to Environment Agency for a Flood Risk Activity Permit (FRAP)

- Earthworks and site preparation as required for Grassblock in accordance with manufacturers specification and Appendix Two.

- Supply and lay Grassblock 2m wide in accordance with manufacturers specifications and Appendix Two.

- Supply and lay Grassblock edging in accordance with manufacturers specification.

- Supply, place and compact 6mm to 10mm clean angular stone in Grassblock.

Please quote for these optional extras:

- Supply and fix 0.50m long galvanised steel pins to secure plastic paver. (Costs 1)

- Dress path with 20mm dust topping (Costs 1 & 2)