RFQ049 Appendix B1, Lot 1 Biology

Please see below the specifications for the following

- Precision &Analytical Balance

- Digital microscopes

- Advanced Colony counter

- Microscope Camera

- Phase contrast microscope

- Data log kit biology

- NDD Easy On-PC (TrueFlow) PC Based Spirometer

- Water baths 18l Digital

- Binocular Microscopes 100x oil immersion lenses

- Portable Balance (500g) 0.01g

Precision &Analytical Balance

- RS232 Interface

- Weighing modes: Grams and Newtons

- Multiple application modes and units

- Weigh-below hook

- Integral security bracket

- Integral menu lock

- AC Adaptor

Digital microscopes

- Total magnification: 40x, 100x, 400x

- Eyepiece: Wide Field 10x/18mm

- Head: Monocular digital observation head, 360° rotating, with 45° inclined eyepiece tube

- Nosepiece: Quadruple revolving nosepiece, rotation on ball bearings

- Objectives: Anti-fungal treated achromatic DIN

- 4x A.N. 0.10, W.D. 18mm

- 10x A.N. 0.25, W.D. 7mm

- 40x A.N. 0.65, W.D. 0.53mm

- Stage: : Double layer with mechanical sliding stage with specimen holder, 125 x 116mm. X-Y movement range 76 x 30mm. Vernier scale on the two axes, accuracy 0.1mm

- Focussing: Coaxial coarse and fine focusing mechanism (graduated 0.002mm) with upper stop. Adjustable tension of coarse focussing knob

- Condenser: Abbe condenser, N.A. 1.2 pre-centred

- Illumination: 1W LED with adjustable intensity

- LED Details: 1W, 6300K colour temperature, average lifetime approx. 50000h

- Dimensions: H350 x W170 x D210mm

- Weight: 4kg

- Voltage: External power supply 100/240V, 50/60Hz, output 6V D.C. 2.5A. Internal NiMH rechargeable battery (up to 15 hours of use from a single charge, LED at medium intensity)

- Accessories included: Power supply, OPTIKA Vision Lite software disc, 1.8m USB cable, dust cover and instructions

Digital Camera Specification:

- Resolution: 1280 x 1024 pixels, 5:4 aspect ratio

- Pixel size: 2.8μm x 2.8μm

- Sensor: 1/3.2”CMOS

- Output: USB 2.0 port

- Framerate: 15fps (1280 x 1024) or 30 fps (640 x 480)

- S/N Ratio: 42.3dB

- Dynamic Pro Range: 71dB

- Exposure Time: 20s - 1/1000s

- Effective Digits: 10bit - on chip

- Sensitivity: 1.0 V/lux-sec (550nm)

- Software: OPTIKA Vision Lite for Windows XP, Vista, 7, 8 (32/64bit)

Advanced Colony counter

Stuart SC6 Plus Colony Counter

The SC6+ colony counter is easy to use. Simply place the petri dish on the electronic pressure pad and touch the dish with a felt tip pen to mark each colony.

- Pressure sensitive counting

- Average count facility

- Digital readout from 0 to 999

- Bright white energy saving LED lighting

- With BioCote antimicrobial protection

- Audible confirmation

- Choice of light or dark background

- Connectivity to printer or computer

Phase contrast microscope

Labomed 9126005 Advanced Phase Contrast Microscope, Binocular

- Magnification  100x/200x/400x/1000x

- Maximum Magnification  1000x

- Head Configuration:  Binocular

- Objectives  Plan phase

- Eyepiece Diameter (mm)20

- Eyepiece Magnification 10x

- Eyepieces Widefield

- Stage Dimensions160 x 200 mm

- Illuminator(s)Halogen with rheostat, 30 W

- Power (VAC) 100 to 240

- Power (Hz) 50/60

Data log kit biology

- 1x Logbook GLE Data Logger (connection via USB cable)

- 1x Oxygen Atmospheric Sensor

- 1x Pulse Sensor

- 1x Humidity Sensor

- 1x Advanced Light Sensor

- 2x Temperature Sensors

- 1x CO2 Sensor

- 1x Skin Temperature Sensor

NDD Easy On-PC (TrueFlow) PC Based Spirometer

- The NDD Easy On-PC is a small, lightweight and easy to use PC Based Spirometer

- Plugs directly into a PC or laptop for real time spirometry diagnostics

- Optional clinical integration software offers full integration with the InPS VIsion Clinical System

- Automatic transfer of Read coded information (FEV1, FVC, FEV1%, PEF etc) into Vision.

- Automatic filing of PDF spirometry reports to Vision.

- Saves time by automatically entering patient details into spirometry software from clinical system.

- Quick and simple to use allowing the nurse to focus on nursing rather than data entry.

- Eliminates data entry errors.

- 2 Year Warranty

- Easy on-PC flowhead sensor

- 2 x Spirettes

- Easy on-PC Software (on USB stick)

- Supplied in hard case

Water baths 18l Digital

Grant Unstirred Digital Thermo Bath - 18L

- JBN Unstirred Digital Thermo Water Bath

- B8R05659

- Set and Forget™ technology - fast heat-up, reliable temperature control

- Temperature stability ±0.5ºC

- Simple to use controls, clear bright display

- 3 year warranty

- UK design and manufacture

- Display lock - disables front panel controls

- Drain tap, tray and lid included

- Advanced dry start and run dry protection - prevents costly service repairs

- Size: 18 Litres

Binocular Microscopes 100x oil immersion lenses

- Total magnification: 40x, 100x, 400x, 1000x.

- Eyepieces: Wide field WF10x/18

- Head: Binocular, 360° rotating, with 30° inclined eyepiece tube. Diopter adjustment on left eyepiece. Interpapillary adjustment 48-75mm.

- Nosepiece: Quadruple revolving nosepiece, rotation on ball bearings.

- Objectives: Anti-fungal treated Achromatic (DIN) objectives with mechanical tube length 160mm

- 4x, A.N. 0.10, W.D. 18mm

- 10x, A.N. 0.25, W.D. 7mm

- 40x, A.N. 0.65, W.D. 0.53mm

- 100x, A.N. 1.25, W.D. 0.13mm (oil immersion)

- Stage: Double layer with mechanical sliding stage with specimen holder, 125 x 116mm. X-Y movement range 76 x 30mm. Vernier scale on the two axes, accuracy 0.1mm.

- Focussing: Coaxial coarse and fine focusing mechanism (graduated 0.002mm) with upper stop. Adjustable tension of coarse focussing knob.

- Condenser: Abbe condenser, N.A. 1.2 pre-centred

- Illumination: 1W LED with adjustable intensity.

- LED Details: 1W, 6300K colour temperature, average life time approx. 50000h.

- Observation Mode: Brightfield

- Voltage: External power supply 100/240V, 50/60Hz, output 5V D.C. 500mA.

- Accessories included: Power supply, dust cover and instructions.

Portable Balance (500g) 0.01g

- RS232 Interface

- Weighing modes: Grams and Newtons

- Multiple application modes and units

- Weigh-below hook

- Integral security bracket

- integral menu lock

- AC Adaptor