Construction of New Footpath – Chelmer Valley, Great Dunmow

- To set out line and levels of proposed footpath with Client.

- To excavate area, up to 200mm depth, for proposed footpath as required and stockpile on site.

- To supply and fix permanent 44mm x 100mm (approx) tantalised timber edgings securely pegged either side (and end) of footpath.  Timbers ‘half cut through’ to bend on curves as necessary.

- To supply and lay geobabric membrane on exposed surface.

- To supply and lay granular sub-base material, nominal depth of 200mm fully compacted.

- To spray compacted stone base with bitumen emulsion tack coat and thereon supply and lay granite fine chippings or gravel chippings gently rolled.

- To spread topsoil either side to new footpath as required to grade adjacent ground.

- To clear and tidy site and remove all surplus materials.  All excavated material to remain on site.