STATEMENT OF SERVICE REQUIREMENTS FOR THE PROVISION OF TECHNICAL SUPPORT ON GEOTECHNICAL INVESTIGATION AND INTERPRETATION ASSESSMENT - WYLFA NEWYDD

- BACKGROUND TO THE PROJECT

  - Introduction

The Generic Design Assessment (GDA) of the Hitachi-GE UK Advanced Boiling Water Reactors (UK ABWR) is currently underway.  Hitachi-GE is the Requesting Party with respect to the GDA process.  The assessment of the final GDA Step 4 civil engineering designs is expected to be completed by late 2017.

An explanation of the Office for Nuclear Regulation’s (ONR) GDA and licensing processes is available in [Reference 1].

Horizon Nuclear Power (HNP) has applied for a nuclear site license to construct and operate two UK ABWRs at Wylfa Newydd on Anglesey, and intend to become the Licensee.  In conjunction with the site licensing process, HNP have undertaken a range of on-site and off-site geotechnical investigations at the proposed location of the construction site.  The reports and interpretations of the geotechnical investigations and their supporting documents will support and inform the design of the UK ABWR civil engineering structures.  A number of these structures have nuclear safety functions.

  - Scope of Civil Engineering Structures

The structures include reinforced concrete and steel structures and nuclear containments that support and contain safety related nuclear plant and other equipment.  The structures will be designed to withstand a range of normal plant and environmental loads, fault loads and extreme environmental loads including earthquakes.  A number of below ground structures, foundations and tunnels are included.  There are also significant off-shore structures.

  - Assessment Methods

ONR will assess HNP submissions to support the licensing process and evaluate if HNP has demonstrated that the site, Wylfa Newydd on Anglesey, is suitable to build and operate two UK ABWRs.

HNP has issued the Detailed On-shore Ground Investigation (DOnGI) report in support of their License application.

The DOnGI report forms the basis of the civil engineering submission. The submission should be assessed in line with the ONR SAPs [Reference 2] and other relevant good practice (including [Reference 3]).

ONR will engage with a technical support contractor (TSC) who will provide technical support in assessing the onshore geotechnical investigation report, interpretations, and supporting documents.

- SCOPE OF THE SERVICES REQUIRED

The successful TSC is required to provide technical support to ONR in order to assess independently the following document:

- Horizon Nuclear Power, Wylfa Newydd Nuclear Power Station, Detailed On-shore Ground Investigation (DOnGI), Final Interpretative Ground Investigation Report, Issue 7.0

  - The scope of services required includes:

- Familiarisation with the Wylfa Newydd site and the proposed outlined site of the UK ABWR structures.

- ONR will hold an initial meeting with the successful TSC in order to provide familiarisation with the nuclear safety functions of the safety related nuclear plant structures.

- Assessment of the following:

  - Specifications used to undertake the ground investigation and produce the final Interpretative ground investigation report. The applicability of the ground investigation methodology used should also be considered.

  - Codes and standards used for the specification and execution of the site investigations and interpretations; and the provision of advice on whether they are appropriate.

  - Specifications for additional boreholes being undertaken in 2017/18 and their location suitability.

- Advice on whether the codes and standards used have been correctly and appropriately applied.

- An assessment of the recommended characteristic values provided to the structural’ designers to confirm that the recommendations are appropriate to the nuclear safety functions of the safety related nuclear plant structures. Note that the DOnGI report follows the structure provided in Eurocode 7: Geotechnical engineers from HNP will be expected to develop geotechnical design reports from the DOnGI report.

- The TSC shall estimate the allowable ground bearing pressures and settlements in two locations (location to be confirmed) using data derived from the DOnGI

- TSC to provide an indication of the variability of the geotechnical profile within the licenced site boundary using the DOnGI.

  - The scope of the work should also include, where appropriate:

- The identification of any necessary project specific information required from HNP.  ONR will formally submit these to HNP as Regulatory Queries, but the TSC is expected to identify the information required.

- Review the responses provided by HNP in answer to any queries raised in conjunction with completing the work package.  Advise the ONR on the adequacy of the responses.

    - Meetings

A number of meetings are required. ONR has identified the following meetings:

- One day start-up meeting with ONR at TSCs office

- Progress meetings with ONR: weekly telecom meetings/contact (2 hours) plus face to face meeting half-way through the project at TSCs office (1 day)

- One day close out/final meeting with ONR at TSCs office

- In addition assume 3 half day meetings with the future Licensee, at the Licensee’s premises in Gloucester, to discuss the information provided and seek clarification if needed.

  - Supplementary Information

The DOnGI consists of 336 pages of main text with 17 appendices.  It is the intention of the ONR to provide the winning bidder with Issue 7.0 of the DOnGI Report upon award of the contract. The DOnGI investigation included:

- 96 cable percussive boreholes to depths of between 2m and 26mbgl;

- 278 vertical rotary cored boreholes to depths of between 11m and 188mbgl;

- 32 inclined rotary cored boreholes to a length of 132m;

- 4 vertical rotary cored boreholes in superficial deposits to depths of between 6.6m and 18.7m;

- 85 machine excavated trial pits;

- Groundwater monitoring installations in 59 boreholes;

- In situ tests:

  - Surface geophysics (seismic refraction, surface wave ground stiffness and electrical resistivity);

  - Downhole geologging in 278 rotary cored boreholes (optical and acoustic televiewer logging, calliper, natural gamma, gamma-gamma density, dual neutron porosity, fluid temperature, conductivity, impellor flow meter, heat pulse flow meter, full wave sonic in all boreholes, and suspension logging in selected boreholes. Heat pulse and flow meter velocity testing were not carried out in inclined holes);

  - Crosshole testing – two arrays;

  - 53 high pressure dilatometer tests;

  - Downhole seismic testing in 21 boreholes;

  - Variable head and soakaway permeability tests in superficial deposits;

  - Standard Penetration Tests (SPT) in boreholes;

  - Shear vane tests in boreholes and trial pits;

  - 195 double packer tests; and,

  - 76 hydrotests.

- Laboratory testing:

  - Classification;

  - Strength;

  - Static stiffness;

  - Seismic velocities and soil dynamic properties (carried out by Fugro GeoConsulting);

  - Earthworks testing;

  - Rock aggregate;

  - Chemical aggressivity testing; and,

  - Contamination.

To assist with assessment of the DOnGI Interpretative report issue 7.0 the following will be provided:

- Combined NavisWorks 3D Ground Model

- 3D Ground Model READ ME File

- (3 pages) Checking Plan for Final Interpretative Investigation Report

For tendering purposes assume that technical queries relating to this SSR that can be addressed by the HNP Civil Engineering Team will require up to 10 working days for a response.  For comments that require wider business engagement or engagement with the supply chain, a holding response will be used to allow due process.

Any required access to the Wylfa Newydd site in support of the assessment will be undertaken in line with agreement with HNP and follow their company processes.

The successful TSC should develop a forward work programme, with planned tasks, deliverables, milestones, timeframes and project costs.  The TSC should:

- Obtain ONR acceptance of the technical review scope, supporting programme and project costs.

- Deliver the defined scope as set out in the programme and to the project cost.

- Routinely review the programme scope, deliverables, milestones, timeframes, project costs and indicative invoice forecast.

- Attend regular brief progress meetings with ONR that will be required during the course of the contract, mainly via teleconference or at the TSCs office.

- Supply a high level overview of the DOnGI report and discuss this with the ONR and other TSCs.

- Report the overall results of this work in a report to ONR.  The report should contain full details of the work undertaken, discussion of the review, summary, conclusions and a full reference list

  - Exclusions

The following sections of the report should be excluded from assessment:

- Assessment of Shear Wave Velocity; which includes sections 5.5, Appendix I, Appendix P and Appendix Q.

- OBJECTIVES

In summary, the overall objective of this work is to provide technical support to ONR, during its assessment of the reports of the geotechnical investigations, interpretations and their supporting documents that will support and inform the design of the civil engineering structures of the UK ABWR.

This will be done via:

- Review HNP’s report, detailed in Section 2, against ONR’s SAPs and international guidance and relevant good practice. These include:

  - IAEA Safety Standards, Geotechnical Aspects of Site Evaluation and Foundations for Nuclear Power Plants, http://www-pub.iaea.org/MTCD/publications/PDF/Pub1195_web.pdf

  - CIRIA W30, Good practice guidance for site characterisation, http://www-pub.iaea.org/MTCD/publications/PDF/Pub1195_web.pdf

  - U.S. Nuclear Regulatory Commission, Site Investigations For Foundations Of Nuclear Power Plants, https://www.nrc.gov/docs/ML0327/ML032790474.pdf

  - U.S. Nuclear Regulatory Commission, General Site Suitability Criteria For Nuclear Power Stations, https://www.nrc.gov/docs/ML1218/ML12188A053.pdf

  - BS 5930: 1999+A2 (2010) British Standard – Code of practice for site investigations, Amendment No. 2;

  - BS EN 1997-1:2004+A1 (2013) - Eurocode 7 Geotechnical design - Part 1: General Rules

  - BS EN 1997-2:2007 (2010) – Eurocode 7 Geotechnical design - Part 2: Ground Investigation and Testing

  - BS EN ISO 14688-1:2002+A1:2013 Geotechnical investigation and testing. Identification and classification of soil - Part 1: Identification and description

  - BS EN ISO 14688-2:2004+A1: 2013 Geotechnical investigation and testing. Identification and classification of soil - Part 2: Principles for a classification

  - BS EN ISO 14689-1: 2003 Geotechnical investigation and testing. Identification and classification of rock – Part 1. Identification and description

  - BS EN ISO 22475-1:2006 Geotechnical investigation and testing – Sampling methods and groundwater measurements

  - ICE UK Specification for ground investigations (Site Investigation Steering Group, 2012)

- Support to ONR (including any required preparation) during teleconferences, videoconferences and / or face-to-face meetings (in the UK) with HNP to discuss, findings from the assessments and / or to request additional information and / or analysis from HNP.

- Drafting queries and / or observations to request additional information and / or analysis from HNP (to be formally submitted by ONR) and advise ONR on the adequacy of HNP’s responses.

- Reporting to ONR any shortfalls identified during the assessment of HNP’s reports and supporting documents and commenting on their significance.

- Documenting the work completed and the conclusions. The TSC will be required to produce approved technical assessment reports.

- Project management, by reporting monthly progress (short summary of progress and expenditure per task and team member).

- Delivering the work in accordance with the agreed scope, programme timescales and project costs.

- CONSTRAINTS

Constraints related to regulatory judgement:

- Whilst the successful TSC will undertake the detailed technical reviews, it will be done under direction and management of ONR.  Regulatory judgement on the adequacy of reports of the geotechnical investigations, interpretations and their supporting documents for the UK ABWR shall be made exclusively by ONR.

Constraints related to the TSC’s technical capabilities:

- The TSC should put forward a team with expert knowledge in geotechnical engineering, foundation designs and safety assessment, and with good knowledge of the UK regulatory expectations regarding safety related civil engineering structures.

Meetings:

The successful TSC should assume attendance at three half day meetings in the UK between ONR and the Licensee during the course of the work, these are likely to take place in the Licensee’s premises in Gloucester. This might include, for example, discussions on information required to undertake the assessment. Additionally, it is expected that the TSC engage with ONR in a one day start up meeting, a one day intermediate meeting and a one day closeout/final meeting. Progress meetings/phone calls are expected and will likely account to 2 hours per week.

Timing constraints:

- A start date of December 2017 is initially proposed for the assessment of the geotechnical investigations, interpretations and their supporting documents. It is expected that the TSC supplies a high level overview of the DOnGI report by the end of January 2018 for discussion. The TSC’s draft report is required by end of March 2018 (this is for the purpose of ONR’s reporting).

Security constraints:

- HNP’s material and, as a consequence, the TSC’s review reports may include information of a sensitive nature that may attract the UK protective marking “OFFICIAL – SENSITIVE”.  This material will therefore need to be handled appropriately.  In addition, work cannot commence until the required security clearances of individuals working on the project have been established and the TSC’s IT systems have been demonstrated to be suitable for the protective marking applied to the information.

Conflict of interest constraints:

- If the TSC has previously supported HNP (directly or indirectly), in developing or reviewing the UK or Japanese UK ABWR Civil Engineering safety case, full details must be declared.  Tenderers must complete Schedule C.

Other constraints – distribution of the TSC’s reports and usage of the TSC’s information included in the UK ABWR review reports:

- It is ONR’s intention to provide the TSC’s reports to HNP for confirmation of factual accuracy.  Interim outcomes of the TSC’s review will be used by ONR, if appropriate, to inform the preparation of Regulatory Observations requesting the HNP to make improvements to the UK ABWR Civil Engineering safety case during site specific design.

- TENDER SUBMISSIONS

To submit a compliant bid, tenderers must provide the following within their tender:

- A description of how you will deliver the work and the resources you will use to meet the Scope and Objectives stated;

- The CVs and details of any staff who will be undertaking the work,

- Address the security constraints detailed above (bullet 5 of Item 4);

- Arrangements for managing delivery of the work and dealing with disputes (including details of who will act as the liaison point between the you and ONR);

- A demonstration of undertaking similar work for other customers;

- A project plan and timetable covering proposed tasks and deliverables;

- A detailed breakdown of all proposed costs associated with this work (presented by fiscal year: ONR financial year starts in April);

- REFERENCES

- A guide to the Regulatory Process. Revision 0 September 2013.

http://www.onr.org.uk/new-reactors/ngn01.pdf

- Safety Assessment Principles for Nuclear Facilities  2014 Edition Revision 0 http://www.onr.org.uk/saps/saps2014.pdf

- IAEA Safety Guide Series No. NS-G-3.6 Geotechnical Aspects of Site Evaluation and Foundations for Nuclear Power Plants, 2005.  ISBN:92-0-107204-X

- Appendix 1

- codes used for uk abwr ANALYSIS AND DESIGN

Seismic Analysis and Design

Codes and standards used for the seismic analysis and design are as follows:

- ASCE 4-98: ASCE Standard for Seismic Analysis of Safety Related Nuclear Structures

- ASCE 43-05: Seismic Design Criteria for Structures, Systems, and Components in Nuclear Facilities

- NUREG-0800: USNRC Standard Review Plan for Review of Safety Analysis Reports for Nuclear Power Plants-LWR Edition, Revision 4

  - Section 3.7.1, Seismic Design Parameters, Revision 4

  - Section 3.7.2, Seismic System Analysis, Revision 4

  - Section 3.7.3, Seismic Subsystem Analysis

- RG 1.61: Damping Values for Seismic Design of Nuclear Power Plants, Revision 1

- RG 1.92: Combining Modal Responses and Spatial Components in Seismic Response Analysis, Revision 2

- EUR Volume 2:General Nuclear Island Requirements, Chapter4 Design Basis, Revision D

- BS EN 1998-1:2004 Design of structures for earthquake resistance, with UK National Annex to Eurocode 8

- IAEA NS-G-3.6:2004 Geotechnical Aspects of Site Evaluation and Foundations for Nuclear Power Plants

Civil Structural Design

Codes and standards used for the civil structural design are as follows:

- ACI 349-13: Code Requirements for Nuclear Safety-Related Concrete Structures and Commentary

- ANSI/AISC N690-12: Specification for the Design, Fabrication and Erection of Steel Safety-Related Structures for Nuclear Facilities

- ASCE 4-98: Seismic Analysis of Safety-Related Nuclear Structures and Commentary

- ASCE 7-05: ASCE Standard for Minimum Design Loads for Buildings and Other Structures

- ACI 350.3-06: Seismic Design of Liquid-Containing Concrete Structures

- NUREG-0800: USNRC Standard Review Plan for Review of Safety Analysis Reports for Nuclear Power Plants-LWR Edition;

  - Section 3.3.2 Tornado Loads, Revision 3

  - Section 3.7.2 Seismic System Analysis, Revision 4

  - Section 3.8.4: Other Seismic Category I Structures, Revision 4

  - Section 3.8.5: Foundations, Revision 4

- RG 1.92: Combining Modal Responses and Spatial Components in Seismic Response Analysis, Revision 2

- RG 1.142: Safety-Related Concrete Structures for Nuclear Power Plants (Other Than Reactor Vessels and Containments) , Revision 2

- ANSI/AISC N690-12: Specification for the Design, Fabrication and Erection of Steel Safety-Related Structures for Nuclear Facilities

- ASME B&PV Code (Boiler & Pressure Vessel), Sections II, III, V, VII, IX and XI

- Appendix 2

- Table of contents extract from the Detailed Onshore Ground Investigation (DOnGI), Final Interpretative Ground Investigation, Report issue 7.0

- Please see supporting document.