Industrial Dwellings Society.

Contract for:

Internal Works Programme – Kitchens & Bathrooms Replacement

Ref: IDS 001				Date: July 2023

Contract Administrator:	Alvin Sum

Specification for Kitchen Replacement

Document 9b

1st Floor, Anna House,

214-218 High Street

London, N15 4NP

GENERALLY - Kitchens

3/B/01	Scope of the works  
Set aside tenants ‘white goods’ temporally fix in another location; Remove and dispose, existing kitchen units; all wall & floor finishes; remove existing electrical supply & fittings: Design new kitchen layout & supply and fit all new kitchen units complete; new electrical supply & fittings; new gas supply to cooker; new floor wall & ceilings finishes; + associated minor carpentry repairs & finishes.

The works are to be carried out to occupied dwellings. Always comply with the requirements set out in the preliminaries; include for all necessary protection to floor coverings, furniture and the like.

- The kitchen units are to be delivered to site (the individual house or flat) by the kitchen supplier, to suit the programme.

- To minimise movement of dust and dirt between rooms allow to temporarily seal or cover doors as appropriate during “dirty” work and use “dustless” drilling and cutting tools.

- Please note a kitchen install should be completed within 15 working days from start to Hand Over and the resident should be updated regularly regarding progress. .

3/B/02	The new kitchens are to be priced on ‘all in’ rate on a floor area basis and to include the full scope of works as above and as specified. Prices to be inserted in section 4 Provisional Quantities.

- The three floor areas are:

- n.e 6.0 sqmtrs

- exc 6.0 sqmtrs ne 9.0 sqmtrs

- exc 9.0 sqmtrs

- For the purposes of pricing, the total quantity of kitchens to be replaced has been split between the three sizes noted above.

3/B/03	The following items are to be included in the overall cost of an ‘all in’ rate.

      - New Kitchen Wall & Base units

      - New Worktops

      - New inset Sink

      - All pipework, wastes and connections

      - Floor tiling

      - Wall tiling

      - Re-plastering up to (16 sqmtrs for walls & 8 sqmtrs for ceiling)

      - Full redecorations

      - Light switch & ceiling fitting replacement

      - Adjust doors

      - Repair/replacement to duct covers etc

      - Fit new door closures

      - Minor repairs/replacement of skirting’s ne 5 l/m & architraves ne 5 l/m.

      - New electrical supply wiring S.D.S.O & S.S.S.O with isolating spurs

      - Gas cooker supply move Ne 1.0 LM from original position

3/B/04 	The following items are to be excluded from the overall cost of an ‘all in’ kitchen rate and to be priced individually. See Document 4 for pricing quantities

- EO Plastering above 16 sqmtrs for walls & 8 sqmts for ceiling

- New ceilings

- Skirting’s & Architraves exceeding 5 l/m each.

- Demolition of Built in Larders

- E.O. Service Ducts

- Extract fans

- Take off & reposition radiators

- EO for fitting hobs & built-in ovens

- E O for new gas cooker supply

- Fire Protection Works

- Additional S.D.S.O above worktop

- Additional S.S.S.O with isolating spur engraved for ‘tumble dryer’ or ‘dishwasher’ or ‘freezer’.

3/B/05	Disconnect all residents’ appliances (e.g. cookers, fridge/freezers, washing machines, dishwashers, tumble dryers and microwaves etc.) as required to carry out the work.  Reposition as necessary in the dwelling as agreed with resident and make temporary connections as required.  At completion refix in new positions; connect to new services Include for temporary connection(s) to provide cooking facilities during the works and for permanent use of fridge/freezers.

- Deliver a minimum of 4 No plastic crates approx. 500x500x600mm at least 2 days before works are due to commence for temporary storage of kitchen utensils, food etc.  Collect crates from residents at least two days after completion of works.  Assist residents with movement of filled crates prior to and on completion of works.

- If any residents’ appliances are found to be unsafe and not capable of simple repair or adaptation do not reconnect.  Mark appliance(s) with warning label(s), explain what is wrong to the resident(s) and forward written details or certificates to the CA.

- Carefully take down and set aside net curtains, roller and Venetian blinds, tracks and the like; refix on completion of work.

- To minimise movement of dust and dirt between rooms allow to temporarily seal or cover doors as appropriate during “dirty” work and use “dustless” drilling and cutting tools.

KITCHEN FITTINGS/SINKS

3/B/06	Supply of Kitchen Units:

- All units are to be factory assembled and delivered to site suitably protected with door and drawer fronts wrapped in a protective bag, including the worktops Plinths etc.

- Preferred kitchen supplier:

Howdens

Meridan Trading Estate, Unit 2A

Charlton

Se7 7sw

Tel: 0208 293 4284

The use of an alternative kitchen supplier will need to be approved by the Client and any such supplier would need to meet the kitchen specification fully and samples need to be produced when seeking approval.

- Kitchen Unit Range: choice of 4 colours (to Include Natural Oak & 3 Gloss Colours)

- All hinges and adjustable drawer runners are to be from a single supplier for the complete contract (ie Blum or Salice).

3/B/07	Sink & Base Units:

- Style: HIGHLINE

- Size: Various widths of single and double units, approx. 870mm high and 600mm deep with minimum 65mm service void.

- Base Units: The design, SHALL include for, 1 No unit (500mm min), ‘all drawer pack’.

- Carcass: 18mm core board to BS EN 312 Type P3 with hardwearing white melamine finish and lipped on all edges.

- Top and rear horizontal rails with vertical rails to double units, front vertical and top rails to be pre-finished white.

- Carcass end panels to match colour of unit doors

- Dry fixed with locating dowels and cam fasteners (and or glued).

- Adjustable legs with 4 legs per single unit and 5 per double unit.

Fully removable 8mm one piece pre-finished “white” back panels, for double units fit central horizontal back rail on service void side to prevent distortion of panel.

- Associated corner posts and white pre-finished blanking panels to corner base units.

Intermediate shelf: 18mm core board to BS EN 312 Type P3 with hardwearing white melamine finish and lipped on all edges.  Central shelf supports required for double units.

- Hinges: Blum or Salice adjustable concealed hinges with 100o hinges to corner base units and 170o to all other units.

- Drawers: 2No Blum or Salice white pre-finished metal sided panels with fully adjustable roller action runners and minimum 15mm (18mm preferred) core board to BS EN 312 Type P3 to base and backs with hardwearing white melamine finish and fully lipped.

- Tall Broom/Larder Units etc: To be of similar design as above.

- Plinths: Removable 150mm high continuous clip-on plinths, 18mm core board to BS EN 312 Type P3 with hardwearing colour to match unit doors and lipped on all edges.

- Provide matching laminate edging strips for site “cut” plinths.

3/B/08	Wall Units:

- Size: Various widths of single and double units, approx.720mm high and 300mm deep.

Carcass:

-	18mm core board to BS EN 312 Type P3 with hardwearing white melamine finish and lipped on all edges.

-	Dry fixed with locating dowels and cam fasteners.

-	White pre-finished vertical rails to double units.

-	One piece fixed pre-finished “white” back panels.

-	Adjustable corrosion resistance metal wall hanging brackets.

- Intermediate shelf: 15mm core board to BS EN 312 Type P3 with hardwearing white melamine finish and lipped on all edges.  Central shelf supports required for double units.

- Hinges: Blum or Salice adjustable 100o concealed hinges.

3/B/09	Door & Drawer Fronts to Wall, Base & Tall Units:

- 18mm “flush” core board to BS EN 312 Type P3 and faced both sides with high pressure laminate and minimum 2.5mm PVC coloured impact resistant lipping to all edges (colour co-ordinated to match door fronts).

- Stainless steel “D” handles either rectangular or curved (ie deep sweep) as agreed with residents.

- End Panels: 18mm core board to BS EN 312 Type P3 with hardwearing white melamine finish with lipping to all edges.

3/B/10	Worktops:

- Size/profile: Nominal 40mm thick, single post-formed.

- Core/finish: Core board to BS EN 312 Type P3 with high pressure hardwearing and easy to clean laminate to BS EN 438 Part 1 as certified by FIRA (matt textured finish) certified by FIRA to top face and wrapped around front edge. Back edge and underside face sealed with balancing layer.

- Accessories: SAA capping strips to ends of worktops at cooker positions with SAA “L” shape jointing strips to corners.

- Provide matching laminate edging strips for site “cut” worktops.

3/B/11	Sinktops & Taps:

- Sinktop/type: 0.9 gauge 18/10 stainless steel nominal 950x508 single-bowl and single drainer inset sinktop with two tap holes to BS1244.  Handing to suit design.

- Mixer tap: Ideal Standard “Tantadisc” cp quarter turn disc 15mm sink deck mixer tap with swivel spout ref “E7811AA”.

- Waste: 38mm cp waste and overflow assembly with plug and chain. Flexible connections to taps are not permitted.

3/B/12	Kitchen Design Generally

- During the pre-condition survey the kitchen supplier (employed by the contractor) shall provide plans and details of the new layout for approval by the resident and submit one copy to the CA for approval.  This new layout shall be designed by a kitchen supplier and shall take full account and allowance of all the resident's own kitchen fixtures, fittings and appliances. Tall fridges or fridge freezers should be sited at the ends of runs of units or be free-standing at appropriate positions.

- The agreed kitchen layout plan and colour choices are to be approved and signed by each resident. One copy to be handed to the resident and another copy to the CA. for approval. Upon agreement of both parties the kitchen can be ordered & programmed scheduled into work process.

- List of properties which are to receive new central heating and electrical re-wiring are to be handed to contractor and Kitchen designer.  Liaise and agree positions of boilers and extract fans where applicable.

- Inform CA of any obvious defects in the existing structure and/or retained heating installations, pipe runs or size of kitchens where 600mm deep base units are impractical.

3/B/13	General Layout Requirements

For kitchens are summarised as follows:

- Layout design must allow a work sequence of worktop - cooker - worktop - sink – worktop where practicable. Wall units can be swopped for base units where practicable.

- 1 sink unit with stainless steel single bowl and drainer over worktops, infill’s and tubular supports as appropriate

- As alternative option omits 1 wall unit and 1 base unit and add 1 larder/broom cupboard.

- Wall units shall be of 720mm nominal height; position approx. 450mm above worktop level (ie 3 course tiled splashback).

- Do not position wall units above sinktops, cooker or fridge spaces. Set back wall units by at least 100mm from the sides of cooker spaces.

- Worktops shall be 600mm deep.  500mm deep worktops and base units may be considered if a kitchen is too small for the 600mm depth but this will only be in exceptional circumstances.  Extend worktops over all base units and washing machine spaces.

- Allow a minimum of 500mm of work surface each side of the sinktop.  Allow space for a washing machine, below a worktop or drainer, with a minimum clear width of 620mm (a greater width may be needed to suit some residents' appliances).

- A cooker space shall be provided between 620-640mm wide with a minimum of 500mm of worktop each side.  The space shall not be in front of a window or in a location that requires access over the cooker top.  Protect worktops on each side of the cooker with alloy edge trims.

- Sink bowls and cooker spaces shall be positioned a minimum of 600mm from any return.

- Double hinged folding doors for corner wall and base units shall are not permitted.  Single hinged type sturdy doors with easy access to spaces behind are required.

- Space for a refrigerator or fridge-freezer, nominal clear width of 600mm shall be provided either at the end of a worktop run or in a corner of the room.  Where possible continue worktops over low fridges and freezers.

- The contractor is to consider & make allowances for the inclusion and design of a new electrical supply within the new kitchen layout. See for numbers etc.

3/B/14	Residents' Built-In Equipment

- Kitchen designer to ascertain if resident wishes to keep any built-in equipment such as hobs and ovens.  Where residents request that these be refitted, allow for providing carcass base units and additional worktop(s) to suit. See Document 4 for pricing quantities

- The ready-assembled kitchen units, worktops, sinktops, accessories etc for each kitchen will be delivered to site (the individual house or flat) on the day work will commence in accordance with the agreed design drawings prepared by the kitchen supplier.  The kitchen supplier will offload the units and agree storage with the kitchen installer and resident.

- The design drawings will also include the residents agreed colour choices for the vinyl floor coverings and wall decorations.

- On delivery of units, check delivery tickets and immediately inform the MC and kitchen supplier of any discrepancies and/or missing or damaged components and accessories etc.

- Kitchen supplier will also forward all installation instructions in advance of the works and liaise with the supplier and MC when required to discuss any problematic areas.

3/B/15	Installation of Kitchen Units

- Methods of fixing and fastenings and all installation is to be in accordance with the manufacturer’s instructions as supplied by the kitchen supplier. Units and worktops are to be fixed together with proprietary fasteners.

- Base & Wall Units: Level and square units; include for all packers, drilling/cutting holes for services and the like to properly complete the installations.

- Worktops:  Cut worktops to minimise wastage and scribe to walls and the like; wherever possible worktops are to be in unjointed lengths between angles and ends of runs.  Treat all cut and exposed edges, including edges to receive SAA jointing or capping trims with two coats of PVA (or similar).

- Trim exposed cut edges with laminate edge strips as supplied by the kitchen supplier fix with moisture resistant adhesive.

- Provide softwood battens, plugged and countersunk screwed to walls to support ends of “open” worktops above appliance spaces as shown on the design drawings.  Decorate battens in gloss paint.

- Sinktops & Taps: Cut and fit worktops for inset sinks; seal exposed edges with two coats of PVA (or similar).  Bed sinktops on clear silicone sealant or use manufacturer’s gasket complete with fixing clips.

- Install mixer tap and cp waste and overflow assemblies as recommended by the manufacturers.

- Connect mixer tap to hot and cold feeds with copper pipes and cp servicing valves in accessible position. 38mm white plastics bottle trap (combined sink/washing machine type trap where appropriate); run plastic waste and connect to existing waste or gully or stack (white wastes inside dwelling and black externally).

- Plinths:  Cut & scribe in plinths; treat all cut and exposed edges with two coats of PVA (or similar) and clip to feet of units.

- Trim exposed cut edges with laminate edge strips as supplied by the kitchen supplier fix with moisture resistant adhesive.

- Completion: Ensure that doors and drawers are accurately aligned and do not bind.  Adjust as necessary to ensure smooth operation.

- Check & adjust ironmongery as necessary to ensure correct functioning.

- Fit colour co-ordinated caps to all redundant holes in kitchen units.

3/B/16	Stripping Out

- Strip out kitchen of any size to remove all wall tiles, base/wall cupboards, full height larder units (where required).   Worktops, sinktops and the like to leave room clean and clear for re-fitting.  Isolate plumbing as necessary and remove or cut back to extent needed for new work.

  - It is the responsibility of the contractor to ensure that basic cooking & washing facilities are maintained/provided at the end of every working day. RLO/Site manager to be informed if this is not attainable.

  - Disconnect and reposition white goods appliances

  - Deliver and collect packing crates as preamble clause.

  - Temporarily reconnect sink top and base unit to maintain facility during the works; OR provide temporary sink top and base unit with connections to hot/cold water and waste.

  - Temporarily reconnect cooker safely or provide temporary cooking facilties

  - Make good wall surfaces where disturbed by removal of units and tiling.  Patch damage with one-coat gypsum plaster flush with adjacent.  Where full height larder units are removed make good all floor and ceiling finishes to match existing.

3/B/17	Assist residents with packing and unpacking the crates.

3/B/18	Skirtings, Architraves & Trims

Renew, missing or damaged sections s. w. architraves. Splice in with new to match existing (prime backs and edges before fixing): In small sections ne 5 l/m

Renew, missing or damaged sections s. w. skirtings. Splice in with new to match existing (prime backs and edges before fixing):  In small sections ne 5 l/m

See Document 4 for pricing quantities

3/A/19 Duct Cover, Pipe Casings & Trims

- Supply & fix new 25x30mm tanalised s.w. timber framing to all previously encased pipework/ductwork or new pipework. Clad with 6mm Marine plywood. All pipework/ductwork fall within the tiled area must be clad with new ceramic wall tiles. Include for 1.4 meter x 350mm x 200mm of new or replacement ductwork.
See Document 4 for other pricing quantities for ductwork

- Where a duct contains services that run from ground to roof level passing through compartment floors. They are to be clad in ½ hour fire protection board (not plasterboard) ‘superlux’ or similar approved.
See Document 4 for pricing quantities for fire protection works

3/B/20	Floor Finishes

- Remove all layers of carpet and/or tiles or sheet, except for firmly bonded asbestos tiles (see also below), from timber floor in kitchen and any adjoining cupboards complete with any board underlay; prepare and level boarded floor, ply underlay and re-cover floor with sheet vinyl as below; seal at perimeters with sealant etc.

- Ditto but solid floors as below.

- Make good floor any type and apply latex levelling compound to concrete floor or 6mm WBP plywood to floors as appropriate. Surface should be free of damp, dust and primed with Uni Bond super PVA Adhesive sealer and primer. EAN.5010383000158.

- Provide and lay POLYFLOR XL PU 3900 300 x 300 mm thermoplastic tiles (equal and approved) to Kitchen and built-in larders. complete including under plinths new or existing skirtings and mastic seal on joint, all fitted as per to manufacturer specification.

#### Polyflor Ltd

Tel: +44 (0)161 767 1111
Fax: +44 (0)161 767 1100
Email: info@polyflor.com

www.polyflor.com

- Use only manufactures approved adhesives.

- Residents to be given a choice of 4 colour’s. and included within kitchen design approval.

- Finish at door thresholds with new metal edge trim

- During the demolition works, if Asbestos material is suspected - work is to be immediately stopped, the area vacated and reported to RBG. For removal by specialist sub-contractor.

- Provide alloy edge trim to vinyl tiles at door frame threshold for all floor finish types

- Provide alloy edge trim to vinyl between kitchens and carpeted dining areas of any width.  Relay/trim carpet as necessary.

- All as section 3/B/21 below:

- 3/B/21	VINYL TILING

- Vinyl tiling On Existing Screed Or Quarry Tiles:

  - Location: Kitchens and any adjoining cupboards.

  - Base: Existing screed or quarry tiles on concrete base.

  - Preparation: Remove various loose and fixed resident’s coverings complete (i.e. carpets, vinyl/lino etc); make good any cracks or holes in screed.  Prepare as clause 30, clean and degrease as recommended by chosen floor-covering manufacturer.  Where existing asbestos thermoplastic/pvc floor tiles exist and are firmly bonded these are to be left in-situ, remove loose tiles bag up and remove; comply with RBG asbestos procedures. Prepare as clause 48, clean and degrease as flooring manufacturer's recommendations.

  - Underlayment: Smoothing levelling compound.

  - Floor Coverings: Heavy-duty marbleised vinyl tiled flooring with a polyurethane surface treatment.

  - Thickness: 2.5mm

  - Colour/pattern: Residents' choice from a range up to four colours selected from the chosen floor-covering manufacturer’s standard range as displayed on the sample boards as provided by the kitchen supplier, who shall obtain residents’ agreement to their selected colour choices as part of their pre-access survey.

  - Adhesive and primer: As recommended by chosen floor-covering manufacturer.

  - Finishing: Clean as recommended by chosen floor-covering manufacturer.

  - Special requirements:

    - Extend vinyl below kitchen/bath unit/ plinths up to junction of walls

    - Clear silicone sealant around perimeters and at junctions with plinths etc.

    - Proprietary SAA edging trim across dividing floor areas/carpet finishes in open plan kitchen/diners.

- VINYL TILES ON EXISTING TIMBER FLOOR:

- Location: Kitchens and any adjoining cupboards.

- Base: Existing floorboards, plywood or chipboard.

- Preparation: Remove various loose and fixed residents’ coverings complete (i.e. carpets, vinyl/lino etc) including plywood/hardboard underlay where fitted.  Inspect floor surface and joists below for suitability to receive new floor finish; where existing asbestos thermoplastic/pvc floor tiles exist and are firmly bonded these are to be left in-situ, remove loose tiles bag up and remove; comply with RBG asbestos procedure. Prepare as clause, clean and degrease as recommended by chosen floor-covering manufacturer.

  - Underlayment: Plywood as previous.

  - Sheet: All as previous.

  - Adhesive (and primer if recommended by manufacturer): As recommended by chosen floor-covering manufacturer.

- Finishing: As recommended by chosen floor-covering manufacturer.

- Special requirements: as previous.

- Preparation Of Concrete/Screed Bases:

  - Bases must be dry and clean.

  - Remove all high spots, nibs and irregularities.  Apply smoothing compound as necessary to provide a smooth even surface free from abrupt changes in level.

  - Apply primer/bonding coat or agent if recommended by the adhesive manufacturer and allow to dry thoroughly before laying floor.

- Smoothing Compound: Fast drying self-levelling latex and smoothing underlayment on primer as recommended for the purpose by the manufacturer.  Mix and lay in accordance with manufacturer’s recommendations.

- Existing Floor Covering To Be Removed: Remove covering and as much adhesive as possible.  Skim with smoothing compound to give a smooth, even surface.

- Existing Floor Covering To Be Overlaid: Make good by patching or filling with smoothing underlayment compound to give a smooth, even surface.  Clean and degrease floor coverings to remove traces of polish and any other contaminants etc.

- Plywood Underlay For Sheet Finishes:

- To an approved national standard.

- Bonding quality to BS EN 314:Part 2:Class 3.

- Appearance class to BS EN 635: Class II.

- Finish: Sanded.

- Thickness: 4mm.

- Sheet size: 1200 x 1200.

- Ensure that existing floorboards are securely fixed and acceptably level. Remove or fill any gross irregularities. Punch in any protruding fastenings.

- Lay sheets with cross joints staggered such that no joint within the base and underlay is coincident and with a 0.5-1mm gap between sheets.

- Fix with 25mm annular ring shanked or twisted shank nails or divergent staples, commencing at the centre of one side of each sheet, at 150mm grid centres over the area and 100 mm centres along perimeter, set in 12mm from edge.

- Ensure that fastenings are driven well in, with heads set flush with surface and do not project through underside of base.

- Laying Coverings:

- Bases must be rigid, dry, smooth and free from grease, dirt and other contaminants before coverings are applied.

- Use a primer where recommended by adhesive manufacturer. Allow to dry thoroughly.

- Adhesive, when not specified otherwise, to be as recommended by chosen floor-covering manufacturer. or, in the absence of such recommendation, to be approved.

- Before laying commences thoroughly condition materials as recommended by relevant manufacturer.

- Finished coverings must be accurately fitted, tightly jointed, securely bonded, smooth and free from air bubbles, rippling, adhesive marks, stains, trowel ridges and high spots.

- Finishing Vinyl Floors:

  - Protect flooring to prevent damage before completion.

- Remove all scrap, dust and dirt.  Carefully remove adhesive and other marks from coverings and adjacent surfaces, using approved cleaning agents and methods.

- Wash floor using mops dampened with water containing neutral detergent.  If necessary, lightly scrub heavily soiled areas using a brush or synthetic fibre web pad.

- Thoroughly rinse with clean water, removing surplus and allow to dry.

3/B/22	Hot & Cold Water Plumbing

Adapt/extend copper hot and cold feeds in any sized kitchen; tee off and fit hot and cold copper washing machine supply with male connectors AND blanking caps in base unit next to appliance space; drill base unit for waste and water pipes.

Insulate concealed 15mm and 22mm dia hot and cold pipework within service void of base units with pre-formed split tube insulation.

3/B/23	Gas Fittings & Pipework

Disconnect existing gas feed serving cooker; adapt/extend and re-run to new cooker position in any sized kitchen, branch shall be brought to 600mm above floor with plug-in bayonet to receive flexible hose; secure to wall with 2No brackets. Ne 1.0 LM from original position. Supply and securely fix new restraining chain for cooker

Test and purge pipework, in accordance with BS6891 and issue gas safety test certificate (see also central heating specification).

Disconnect existing gas feed serving residents hob or coven units; adapt/extend copper feed and re-run to new appliance position; connect with approved gas isolating valve in any sized kitchen. To be priced as provisional quantity only

3/B/24	Waste Pipes & Fittings

Connect/re-connect to new kitchen sink and washing machine (with new combined sink/washing machine plastic trap) to existing gully(s) and/or waste/soil stack(s) with new solvent-weld plastic pipes. All connections, bends, junctions and fittings, with access points to allow future rodding.  Re-use of, and connection to, sound existing waste pipes is allowed.

3/B/25	Wall Tiling

- Tiling to kitchen of any size as 3/B/27 with manufacturer’s glazed edge or rounded tiles to all exposed edges/corners.

3/B/26	Wall Tiling

- Prepare wall Supply & fix white ceramic wall tiles 150 x 150 mm, 3 courses above all worktops, and full infill to ‘cooker’ area and splash back above sink and drainer areas.

- Clad all pipe casing built in cupboards etc. with new tiles where they occur within the prescribed areas.

- All cuts and fixings to be strictly in accordance with manufacturer’s instructions.

- Tiling to kitchen of any size as below with manufacturer’s glazed edge or rounded tiles to all exposed edges/corners

- Where sink is above the height of the window cill provide & fix new back board clad with tiles.

- All as detailed in 3/B/27 below

3/B/27	CERAMIC TILING

Tiling to Kitchens:

- Description: 3 courses above worktop, including returns and ductwork.

- Tiles: Gazed ceramic wall tiles to BS 6431, white “satin” finish.

- Size and thickness: Nominal 150mm x 150mm x 6.5.mm

- Background/Base: Varies - a mixture of existing plaster/plasterboard on brick/block or stud walls or timber faced ducts etc.

- Preparation: remove existing tiles.

- Adhesive: Ribbed adhesive bed as clause 50 using “Bal-Wall Blue Star” adhesive by Norcros Adhesives Ltd (or equal and approved).

- Joint width: 3mm spacer lugs.

- Grout: “Bal-Superflex” by Norcros Adhesives Ltd (or equal and approved).

- Accessories: White mould-resistant silicone sealant at joints with pipe ducts and at internal corners.

- Other requirements: Manufacturer’s round-edge or glazed edge tiles to all exposed edges/corners etc.

Existing Backgrounds/Bases Generally:

- Remove efflorescence, laitance, dirt and other loose material by thoroughly dry brushing.

- Remove substances incompatible with bedding using suitable emulsion cleaner then wash with clean water.

- Remove all loose or defective areas and repair with materials compatible with background/base and bedding.

- Clean paint and other nonporous surfaces by washing with water containing detergent then with clean water.

- Where existing paint does not have satisfactory adhesion, remove by mechanical means.  The use of paint strippers within enclosed rooms is not allowed. Should there be circumstances where this is the only way to achieve the required finish etc. The contractor must apply to the CA in writing and present details, COSHE assessments. They must receive the agreement in writing from the CA before proceeding.

Fixing Generally:

- Avoid unintended colour/shade variations within the tiles for use in each area/room.

- Check that adhesive is compatible with background/base.

- Cut tiles neatly and accurately.

- Fix tiles so that there is adhesion over the whole of the background, base and tile backs.

- Before bedding material sets make adjustments necessary to give true, regular appearance to tiles and joints.

- Variations in gap under a 2m straightedge (with feet) placed anywhere on the surface to be not more than 3mm.

- Clean surplus bedding material from joints and face of tiles.

Setting Out:

- Joints to be true to line, continuous and without steps.

- Joints on walls to be truly horizontal, vertical and in alignment round corners.

- Cut tiles to be kept to the minimum, as large as possible and in unobtrusive locations.

- Joints in walls to be in alignment.

Ribbed Adhesive Bedding To Walls:

- Apply 3mm floated coat of adhesive and trowel to a ribbed profile using the recommended notched trowel.  Press tiles firmly into adhesive with a twisting sliding action.

Grouting:

- Allow bedding to harden sufficiently before grouting.

- Ensure that joints are 6mm deep (or the depth of the tile if less), and are free from dust and debris.

- Fill joints completely, tool to an approved profile, clean off surface and leave free from blemishes.

- Polish wall tiling with a dry cloth when joints are hard.

Material, Satin White Ceramic

- Minimum 6.5mm Thick

- EN ISO 10545 tolerances, complies

- EN ISO 10545 stain resistance Class 3 or better

- EN ISO 10545 chemical resistance Class B or better

3/B/28	Internal Decoration

- Wash down walls, ceilings, woodwork and copper pipework; fill small holes, cracks etc, prepare and redecorate rooms with emulsion paint to walls, ducts and ceilings and gloss paint or stain to woodwork and pipework all as below:

- Strip paper (including multi-layers) to walls and partitions etc; line with new lining paper as below to receive paint finish.

- Strip paper (including multi-layers) to ceilings; line with new lining paper as M52/10 to receive paint finish

- Strip paper (including multi-layers) to walls and partitions etc; cross line with new lining paper as below to leave ready for wallpapering by residents on completion of works.

- Strip polystyrene tiles from ceilings; remove adhesive and scrape/sand down level; make good, apply sealer to any remaining adhesive and leave ready for decoration. Where adhesive is impracticable to remove CA to be informed with a request for further instruction

- All existing ‘Textured coatings’ to ceilings are to be treated as containing asbestos. They are not to be disturbed, drilled through or removed. This is to be carried out by RBG’s specialist contractor. Should the ceiling appear to be sound, and free from any visual defects. It is to be gently washed with non abrasive cleaner & cloths. Allowed to dry & paint with 2 coats of white matt emulsion.

- All operatives attending site, must have must have had UKATA certificated Asbestos Awareness training in the last 12 months.

- All as detailed in 3/B/29 below:

3/B/29 	Painting/Clear Finishing

Decorations & Wall Finishes

- Generally - Prepare all surfaces and decorate all new and previously decorated surfaces with base coat plus 2 coats of Crown bathroom paint and gloss to woodwork

Gloss Paint To New Internal Joinery & Metalwork:

- Manufacturer: Dulux – Trade Water Based System or similar approved.

- Surface(s): Sanded softwood and paint grade plywood. Copper pipework.

- Preparation: As clauses ‘preparation generally’

- Initial coat(s): Knot and prime.

- Finishing coats: 1 undercoat and 2 quick drying gloss finish coat.

Gloss Paint To Existing Internal Joinery & Metalwork:

- Manufacturer: As above.

- Surface(s): Previously painted softwood and plywood. Copper pipework etc.

- Preparation: As clauses ‘preparation generally’ Remove mould growth etc as above.

- Initial coat(s): Knot and prime or patch prime and bring forward.

Walls:

- Manufacturer: As above.

- Surface(s): Previously painted surfaces/repaired plaster or new lining paper Preparation: As clauses ‘preparation generally’. Remove mould growth etc as clause.

- Initial coat(s): 1 mist coat.

- Finishing coats: 2 coats of fungicidal vinyl silk emulsion.

Ceilings:

- Manufacturer: As above..

- Surface(s): As clause 15 (or sound plaster/existing textured finishes).

- Preparation: As clauses ‘preparation generally’, Remove mould growth etc

- Initial coat(s): 1 mist coat.

- Finishing coats: 2 coats of silk emulsion.

Open Joist Ceilings:

- Timber Surface(s): Previously painted or stained joists.

- Preparation: As clauses ‘preparation generally’

- Paint finish: Manufacturer as clause

- Initial coat(s): Knot and prime or patch prime and bring forward.

- Finishing coats: 1 undercoat and 1 quick drying gloss finish coat.

- Stain finish: Manufacturer Sikkens - Water Based Interior Woodstain (or similar).

- Finishing coats: 2 or 3 coats of “Cetol – BL Décor” depending on the existing colours.

- Ceiling Surface(s): Previously painted boarded ceiling linings.

Colours:

- Colour/pattern: Paint colours for the wall decorations will be chosen by residents from the Dulux “Colour Dimensions” range up to four colours as displayed on the sample boards as provided by the RLO, who shall obtain residents’ agreement to their selected colour choices as part of their pre-access survey.

- Ceiling: colours will be brilliant white.

- Internal joinery: will generally be brilliant white although ducts and the like are to match adjacent colours.

Preparation - Generally

- To BS 6150, Section 4.

- Preparation materials: Types recommended by their manufacturers and the coating manufacturer for the situation and surfaces being prepared.

- Substrates: Sufficiently dry in depth to suit coating.

- Remove: Efflorescence salts, dirt, grease and oil:

- Joints, cracks, holes and other depressions: Fill with stoppers/fillers.  Abrade to a smooth finish.

- Surface irregularities: Abrade to a smooth finish.

- Remove: Dust, particles and residues from abrasion:.

- Doors and other moving parts: Ease before coating.  Prime resulting bare areas.

M60/Mould Removal

- Surface mould growth: Wash down with one or two coats of household bleach and rinse with clean water or remove with assistance of biocidal solution.  Apply residual effect biocidal solution to inhibit re-growth.

Previously Coated Surfaces Generally

- Preparation: To BS 6150, Section 6.

- Removing coatings: Do not damage substrate and adjacent surfaces or adversely affect subsequent coatings.

- Loose, flaking or otherwise defective areas: Carefully remove to a firm edge.

- Alkali affected coatings: Completely remove.

- Discovery: Give notice to MC and CA and agree decorating procedures of:

  - Coatings suspected of containing lead.

  - Substrates suspected of containing asbestos.

  - Significant rot, corrosion or other degradation of substrates.

- Retained coatings: Thoroughly clean.  Abrade gloss coated surfaces to provide a key.

- Partly removed coatings: Apply additional preparatory coats.

- Completely stripped surfaces: Prepare as for uncoated surfaces.

Fixtures:

- Before commencing work, remove all existing and retained fixtures and fittings, set aside and replace on completion.

Timber Preparation:

- General: Abrade to a smooth, even finish with lightly rounded arrises.

- Degraded or weathered surface timber: Abrade to remove.

- Degraded substrate timber: Repair with sound timber of same species.

- Heads of fasteners: Countersink sufficient to hold stoppers/fillers.

- Resinous areas and knots: Apply two coats of knotting.

- Defective primer: Abrade back to bare timber.

Steel Preparation

- Corrosion and loose scale: Abrade back to bare metal.

- Residual rust: Treat with a proprietary removal solution.

- Primer: Apply as soon as possible.

Copper:

- Clean surfaces, remove grease and lightly abrade with fine abrasive paper and white spirit; apply etching primer where recommended by the coating system manufacture

Plaster Preparation

- Nibs, trowel marks and plaster splashes: Scrape off.

- Over-trowelled ‘polished’ areas: Abrade lightly.

Coating Generally

- Application: To BS 6150, Section 5.

- Conditions: Maintain suitable temperature, humidity and air quality.

- Surfaces: Clean and dry at time of application.

- Thinning and intermixing: Not permitted unless recommended by manufacturer.

- Priming coats: Apply as soon as possible on same day as preparation is completed.

- Finish: Even, smooth and of uniform colour.  Free from brush marks, sags, runs and other defects.  Cut in neatly.

3/B/30	Sealant Pointing

- To Wall Tiling/Bath/Flooring Etc:

- Apply silicone sealant to all exposed edges around bath basin W.C. ducts pipe casing etc where water can damage the surface below and around. Apply to other areas to give decorative finish.

- Sealant: Silicone based to BS 5889, Type B with fungicide. Dow Corning 785+ or similar approved.

- Colour: White.

- Application: - Substrate: Dry.

- Sealant application: Fill joints completely and neatly, ensuring firm adhesion to substrates.

- Sealant profiles: - Butt and lap joints: Slightly concave.

- Fillet joints: Flat or slightly convex.

- Protection: Protect finished joints from contamination or damage until sealant has cured.

Concealed Joinery Surfaces:

- General: After priming, apply additional coatings to surfaces that will be concealed when fixed in place.

Doors:

- Prime and paint bottom edges and re-hang.

Completion:

- Ensure that doors and other moving parts move freely.  Remove all masking tape and temporary coverings.

- 3/B/31	Plastering Existing walls

Hack of defective plastering to walls Make good walls and ceilings including re-plastering where required. Include for coat of uni-bond or similar

- Allow for up to 24 sqmtrs skim coat per Kitchen

- Allow for up to 1.0 sqmtrs 1 coat patch repair per Kitchen

- See Document 4 Plastering for all ‘Extra Over’ pricing quantities

All as detailed in 3/B/32 below:

- 3/B/32	Plastered Coatings to Walls & Ceilings

  - One-Coat Gypsum Plaster: To be used in patch repairs.

  - Background: Existing brick or blockwork.

  - Plaster proprietary reference: Constructor’s discretion.

  - Thickness: to be as required to match existing finish.

  - Finish: Smooth.

- Plasterboard & Skim:

  - Background: Joists - Plasterboard backing:

  - 12.5mm gypsum wallboard with grey paper face, or other as scheduled,

  - nail fixed. -

  - Skim coat: Board finish plaster: To BS 1191:Part 1, Class B.

  - Thickness: 3mm

  - Finish: Smooth.

- Mixing:

  - Measure materials accurately by volume using gauge boxes

  - Do not allow contamination of one type of material with another, or by any set material.

  - Do not use admixtures unless specified or approved.

  - Mix materials thoroughly.

- Adverse Weather

  - Take precautions to prevent damage to internal work when air temperature is below 3°C.

- Backgrounds Generally:

  - Ensure that backgrounds are secure, free from contamination and loose areas, reasonably dry and in a suitable condition to receive specified coatings.

Repairing Existing Plaster:

  - Remove plaster that is loose, soft, friable, badly cracked or affected by efflorescence.  Gently tap all remaining intact surfaces and remove hollow sounding areas of plaster. Remove stained plaster to 300mm beyond last point of visible staining. Cut back to straight horizontal and vertical edges

  - Advise CA if any built-in timbers, structural deficiencies or sources of damp are revealed

  - Thoroughly dry brush the background and edges to remove dust, loose material and efflorescence.

Plasterboard Backings:

- Plasterboard: To BS 1230:Part 1.

- Fixings: Galvanised plasterboard nails of length not less than 3 times the thickness of board being fixed.

- Ensure that noggins, bearers, etc. to support fixtures, fittings and services are accurately and securely fixed.

- Ensure that the perimeter and unbound or cut edges of boards are supported by additional noggins in accordance with board manufacturer's recommendations.

- Stagger end joints between rows.  Gap between boards to be not more than 3mm.  Do not use damaged boards

- Fill and tape (scrim) all joints and angles between boards (except where coincident with a metal bead) using tape recommended by board manufacturer.

- Beads/Stops:

- Use appropriate corrosion resistant beads/stops at external angles and stop ends.

- Application Generally:

- All coatings to be firmly bonded, of even and consistent appearance, free from rippling, hollows, ridges, cracks and crazing.

- Finish surfaces to a true plane with all angles and corners to a right angle and with walls and reveals plumb and square.

- Prevent excessively rapid or localised drying out.

- Dubbing Out:

- If necessary to correct background inaccuracies, dub out in layers of not more than 13mm in same mix as first coat.  Allow each coat to set before the next is applied.  Scratch the surface of each coat.

Fire Stopping To ducts

3/B/33	Fire Sealant around pipes and services

- Where duct and pipes are exposed with a gap through the compartment wall and or floor, with a risk of smoke/fire passing through, these must be sealed with materials from Nullifire:

  - Large gaps – B747-50 coated batts

  - Smaller gaps – M701 intumescent acrylic sealant to gaps not exceeding 10mm from one side for items passing through void

  - Application may require use of Firetherm Intumsatic, Firetherm intubatt, Firetherm Intucompound,Firetherm Intucollar, Firetherm Intusleeve depending on location and property type.

- Must be installed strictly as per manufacturer’s instructions

- Where service pipes/duct pass through one compartment floor (or wall) or dwelling and into another or into a common area the gaps around the pipes must be sealed from the passage of flame and smoke using intumescent pipe wrap or pressure exerting graphite mastic.

- Plastic pipes insulated pipes with combustible lagging, cables sealed with Intumescent Pipe Jacket (single collar rate (based on collar diameter) or Pressure Exerting Graphite Mastic.

- Must comply with current Building Regulations Part B3

- Must also take into account the prevention of vermin travel & attack by vermin.

- To ensure that these areas have been fire sealed they must be inspected by CA/COW before duct covers etc are boxed in and have a signature of sign off and evidential picture for library and record purposes

- See Document 4 for pricing quantities

Fire Certification

- All the above fire sealant works must be issued with the relevant Fire Certificates for all works under a third-party Accredited Scheme with Warrington Fire and Research (FIRAS).

Recommended Manufacturers; Details – A suitable and compliant alternative can be put forward but NOT used in Operation until signed off by Head of Risk & Compliance for RBG.

- Nullifire,

A Division of Tremco illbruck Coatings Ltd
Torrington Avenue
Coventry
CV4 9TJ
England

Tel: +44 (0) 2476 855 000

Fax: +44 (0) 2476 469 547

Email: protect@nullifire.com


Electrical Works to Kitchens.

3/B/34	Electrical Works to Kitchens (to be read in conjunction with Rewire 3/D/ Section)

- See Rewire section for wiring details etc. The below numbers of SO overrides any stated Electrical Rewire section.

- Provide 3no switched double socket outlets (S.D.S.O) at high level (above worktop)

- Provide 3no single socket outlets (S.S.O) at low level below worktop. All with isolating spurs engraved with, ‘fridge’  ‘washing machine’, ‘cooker’ ‘dishwasher’, ‘tumble dryer’ & ‘freezer’ at high level (above worktop.) First 3 engraved (S.O.O), are compulsory. Should there be a need for a 4th 5th or 6th (S.O.O). A Request for CVI must be put forward to the C.A. at the time of the design approval. See Document 4 for pricing quantities

- 1no 45-amp cooker box with a combined single socket outlet. on their own ring circuit rewired back to Consumer Control Unit (C.C.U.)

- The new 6no S.O’s to be on their own ring circuit rewired back to Consumer Control Unit (C.C.U.)

Extractor Fans

Generally – Kitchen (to be read in conjunction with Rewire Section

3/B/35 Existing Fan

- The contractor is to check & test the running operation of the existing fan. This is to include:

  - Electrical test; Test & ensure the fan is connected & working correctly & safely.

  - Maintenance; Removal of access covers, dismantling & cleaning filters & re-fixing

  - Report any defects to site team

  - See Document 4 for pricing quantities

3/B/36	Existing Fan Replacement (to be read in conjunction with Rewire Section)

- The contractor must be aware of the requirements and variations of fitting an extract fan:

  - Replacement of existing fan – As existing through the wall.

  - Complete new installation - through the wall.

  - New installation, insufficient walls space - through the wall in the Kitchen - ducted. (Insufficient wall space)

  - Not required – where through the double glazed unit installed.

  - Not required – where over window head fan installed.

3/B/37 Existing Fan - through the wall

- Carefully remove existing extract fan and cables from existing wall back to spur point and cart away

- See Section 3/D/ for all rewire specification details. Any contradiction/differences or queries between the rewiring details in this section and the details in 3/D/ should be brought to the attention of the CA/PM as soon as possible.

- Supply & fit into existing hole new SELV rated extract fan from Vent-axia model nos VA140 including flue terminal kit. See Document 4 for pricing quantities

- Before connection test existing circuit & connect new fan & leave running as per manufacturers recommendation’s

- Test new installation & issue certification upon completion.

- All fan units and their associated wiring and controls shall be installed in accordance with the Current edition of BS 7671 IEE Wiring regulations, the current edition of the Building Regulations part P Electrical safety, and part F ventilation

- It shall be the contractor’s responsibility to site units to final positions and ensure that units do not compromise, any Gas appliances, flu outlets/termination, door or window openings, structures, lintels, etc. and carry out as required spillage tests, to confirm compliance.

3/B/38 New installation through the wall

- Carefully drill 102mm dia hole through wall of various types brick, brick/block, reinforced concrete. Carry out pre-inspection to ensure that the wall is able to be drilled successfully and safely. It is the responsibility of the contractor to ensure the above drilling is safe if there is any doubt this must be referred to the CA/PM. Core drill must be fitted with dust collection system. Clean up all debris etc. and cart away.

- Supply and fit new SELV rated extract fan from Vent-axia Lo-carbon Tempra single room heat recovery fan connected to spur point. See Document 4 for pricing quantities

  - or

- Supply and fit new SELV rated extract fan from Vent-axia model nos VA140 to spur point. See Document 4 for pricing quantities

- Before connection test existing circuit & connect new fan & leave running as per manufacturers recommendation’s

- Test new installation & issue certification upon completion.

- New installation to comply with the 17th edition of IEE regulations.

- Before connection test existing circuit & connect new fan & leave running as per manufacturers recommendation’s

- Existing airbricks shall not be used to house extractor fan units without the specific permission of the CA

- Final positions of the extract units will be made in consultation with tenants and the CA.

- The Contractor should be aware and allow for within their costs, where tenants are elderly, infirm or disabled they shall have the option of a more accessible position of controls for extractor fan units.  The wiring to these units shall be installed in white high impact mini trunking installed within room corners, skirting boards and door architraves so as to be as neat and inconspicuous as possible and avoid the need for excessive trunking runs.

- See Document 4 for pricing quantities

Manufacturers Details

Vent-Axia Limited,
Fleming Way,
Crawley,
West Sussex,
RH10 9YX,
United Kingdom

- Domestic and Commercial Enquiries

- Tel: +44 (0)844 856 0590
Fax: +44 (0)1293 565169

3/B/39 Wiring and fittings

- The bathroom shower unit circuit shall be protected by means of a 30mA residual current device.  The RCD shall form a Modular Section of the Consumer Unit

- The lighting installation within each dwelling shall comprise the lighting points and switches and shall be wired using flat twin and CPC PVC/PVC cable throughout.

- Particular attention is drawn to the Constructor that 1 flat twin and CPC PVC/PVC cable with RED conductor insulation shall be used for wiring to all one way switch positions.

- General lighting switches shall be of the flush mounted, rocker pattern with all insulated white plates.  Single pole, two way and multi-gang switched shall be provided as necessary.  Lighting switches shall be in accordance with BS EN 60669-1.

- Lighting switches shall generally be positioned in existing positions or at 1200mm (AFL) to the centre of the switch or at a height as existing to minimise disturbance of decoration.

- Each lighting point shall comprise a white ceiling rose, 300mm length of white PVC sheathed flexible cord and a white heat resisting PVC lamp holder, complete with integral automatic safety switch cable grip and skirt.  In bathrooms, walk-in cupboards and separate toilets, a white batten type lamp holder shall be provided complete with H.O. skirt and patress.

- Bathroom light fitting with low energy bulkhead fitting 2D shall comply with BS EN 61184, and BS EN 60238. Light fitting to be enclosed fitting complete with an energy saving bulb.

- Ceiling roses shall be white all insulated pattern with shrouded ‘loop in’ terminals and supplied with a separate flange ring.  Ceiling roses shall comply with BS 67.

- Break joint rings shall be used in conjunction with all ceiling roses/batten holders.

- Where existing flush switch points cannot be reused, the Contractor shall install a surface box suitable for terminating mini-trunking to the right, left or centre.

- Where tenants have installed their own lighting fittings to an existing lighting point, the Constructor shall remove the fitting, rewire the point and re-connect the fitting.  Before removing the fittings, the Constructor shall inform the tenant that the fitting is being removed at the tenant’s risk and that neither the Client nor the Constructor accepts any responsibility for damage to the fittings.

- Where a fitting is faulty or does not comply with the current standards (e.g. earthing terminal etc.,) the Constructor shall inform the tenant, disconnect the fitting, rewire the point and fit a ceiling rose and pendant or batten holder.

- The method of wiring throughout the installation shall comprise PVC insulated and sheathed flat twin cable with bare CPC.  No junction boxes shall be used and all connections shall be made at switches, sockets, ceiling roses etc.  The Constructor shall use existing conduits, roof spaces and floors voids where possible to install the new wiring which shall be concealed as much as possible.  At locations where this is not possible, surface mounted mini trunking shall be non-intrusive.  The installation shall be in accordance with this specification and to the approval of the CA

3/B/40 EARTHING & BONDING

- All earthing and bonding shall be carried out strictly in accordance with this specification and the particular requirements of the local Electricity Supply Company in respect of PME installations.

- The Constructor shall ensure that main equipotential bonding is available at the main earthing terminal linking with gas, water and central heating services in accordance with the IEE Wiring Regulations.  Otherwise, the Constructor shall supply and install main equipotential bonding to these services.

- Supplementary Bonding shall be supplied and installed by the Constructor at all sink, baths, showers, tanks and immersion heater pipes as detailed in this specification.  At positions where a radiator is accessible with exposed conductive parts or other extraneous conductive parts, the Constructor shall provide and install supplementary bonding unless the connecting pipes are electrically connected to main equipotential bonding through negligible impedance.

- The existing installation at the dwellings is TNC-s.  The earthing conductors, supplementary bonding conductors and circuit protective conductors shall be segregated from the neutral conductor within the consumer installation.  All exposed conductive parts shall be connected to their respective circuit protective conductors.  All such conductors shall be identified with green/yellow sleeving

- See Section 3/D/ Rewires for details