DBOM Facilities Requirements, Technical Specification and Zone Data Sheets

Facilities Requirements

CONTENTS

Clause	Subject matter	Page

1.1	Introduction	3

1.2	Core Requirements	3

[1.3	Reception Area	5

[1.4	Secure Storage Area (Reception)	7

[1.10	Sports Hall	20

[1.11	Dry Side Change Area (Sports Hall)	23

[1.12	Fitness Suite	26

[1.13	Dry Side Change Area (Fitness Suite)	28

[1.14	Aerobics/Dance Studio	30

[1.15	Community Information Point	32

[1.16	Café		33

[1.17	Kitchen and Store	34

[1.18	Toilets		35

[1.19	Cleaner's Store (non-public support accommodation)	36

[1.20	Flexible Space	37

[1.21	Creche	38

[1.22	Duty Manager’s Office	39

[1.23	General Office	40

[1.24	IT/Communications Room	41

[1.25	Staff Store	42

[1.26	Staff Rest Room	43

[1.27	Staff Change Area	44

[1.28	Internal Circulation (public support accommodation)	45

[1.29	Plant Room	46

1.1	Introduction

The Facilities Requirements identify the requirements for the design, architecture and building services installations of the Facilities. This document should be read in conjunction with the Zone Data Sheets.

The descriptions of facilities and outputs set out in this document provide an overview of the Authority’s requirements with regard to the Facilities and are intended to assist the Contractor in designing facilities which respond to local requirements and fulfil local need.

Nothing in these Facilities Requirements shall remove the obligation of the Contractor to meet the requirements of all relevant Guidance and Legislation.

The Contractor shall comply with any methodologies identified in these Facilities Requirements and with all relevant professional and trade guidance as set out in these Facilities Requirements.

1.2	Core Requirements

The Contractor is required to provide high quality sports and leisure facilities with the aim of promoting accessible sport and physical activity opportunities in the area.

The Facilities shall provide Users with a safe and comfortable environment in which to participate in a range of activities.

The Facilities shall be designed to a standard consistent with all relevant and current technical design guidance published by Sport England and the national governing bodies of sport (NGBs)

The Core Requirements for the Facilities include:

Sports Hall –linked to Spectator Stand/Club house

https://www.sportengland.org/facilities-planning/tools-guidance/design-and-cost-guidance/sports-halls/

- Reception Area

- Secure Storage Area (Reception)

- Entrance Foyer

- Fitness suite???

- 8 badminton court Sports Hall – 40.0 x 34.5 x 8.3

- Dry Side Change Area (Sports Hall)

- Toilets

- Cleaners Store (non-public support accommodation)

- Plant Room

Club House/ Spectator Stand

https://www.sportengland.org/facilities-planning/tools-guidance/design-and-cost-guidance/clubhouses/

- Stand for 500 [seated?] spectators

- Healthy eating café/bar

- Bar cellar

- Kitchen and Store

- Outdoor sports changing accommodation.

- Toilets

- Duty Manager’s Office

- Staff Rest Room

- Staff Change Area

- Internal Circulation (public support accommodation)]

Main Pitches

- 3G Artificial Grass Pitch compliant with SE guidance (Sept 2015)

https://www.sportengland.org/facilities-planning/tools-guidance/design-and-cost-guidance/artificial-sports-surfaces/

- Floodlighting compliant to SE guidance (Sept 2015)

https://www.sportengland.org/facilities-planning/tools-guidance/design-and-cost-guidance/artificial-sports-facilities/

Grass Pitches

- all compliant with SE guidance [standard of play i.e. division]

https://www.sportengland.org/facilities-planning/tools-guidance/design-and-cost-guidance/natural-turf-for-sport/

  - 3 Rugby Pitches (including junior pitch)

  - 2 football pitches

  - 1 mini football pitch

The above required Zones are described in more detail below. For each Zone the following further facility requirements detail is provided:

- General Requirements

- Dimensions

- Description of Activities/Functions

- Key Design Requirements

- Adjacencies

- Occupancy/Capacity.

1.3	Reception Area

General Requirements

- The external entrance foyer shall be easily identifiable and have a prominent position in the general arrangements for the building. An entrance lobby shall be provided to reduce air changes and loss of heat and the migration of dirt and water into the main foyer. The lobby shall be provided with a full width threshold mat and well and automatic doors.

- The position of the reception counters and access control systems shall allow Users to immediately recognise the correct admission points for the Facility. Users shall be channelled such that they pass the reception desk and/or the reception desk should be designed to allow optimum supervision and minimise staffing.

- The reception desk shall be visible upon arrival, at an appropriate height for wheelchair users, fitted with an induction loop and signed appropriately.

Dimensions

- To meet the Authority's Requirements [ZDS] and allow suitable circulation and waiting space for peak occupancy.

Description of Activities/Functions

- The Reception Area shall serve as a meeting point for Users before entering the Facility.

- The Reception Area should be capable of holding up to [xx] people based on anticipated peak occupancy.

- The reception desk shall be the main control and security point to the Facility.

- The layout of the Reception Area and position of the reception desk shall provide Users with space to queue at busy periods.

Key Design Requirements

The Reception Area shall:

- Be of a sufficient size to allow people to meet before using the facilities.

- Meet the requirements set out in the Zone Data Sheet.

- Allow for plenty of natural light.

- Have provision of a convenient and secure store for baby buggies/prams which can be seen.

- Have a clear view of the external cycle stands.

- Have a convenient store for disability buggies, clearly visible from reception.

- Have automatic operation of the main doors.

- Have access which is level.

- Have high quality external windows and doors, floors and ceiling finishes that create a good impression and contribute to lower maintenance, energy and replacement costs.

The reception desk shall:

- Be of an ‘open’ design, with a lower level for wheelchair users and children.

- Incorporate storage for lost property and items for sale or hire.

- Make provision for the monitoring of security and fire alarm systems.

- Allow for the use of CCTV monitors.

- Allow for IT equipment and data-gathering systems.

- Include tills and other equipment as identified in the Zone Data Sheets.

- Include an induction loop that complies with BS EN 60118-4 and BS7594.

Adjacencies

- The Reception Area shall provide access to all parts of the building.

- The reception desk shall afford good views of Users entering the Facility, and a high level of surveillance should be possible from the reception desk.

Occupancy/Capacity

- The Reception Area shall accommodate groups waiting to access the Facility, or those waiting for other activities to be held in the vicinity.

- The reception desk will need to accommodate a minimum of [x staff].]

1.4	Secure Storage Area (Reception)

General Requirements

- The Secure Storage Area (Reception) shall be used to store general items.

Dimensions

- To meet the Authority's Requirements based on the outline schedule of equipment ( i.e 4 demountable stands (1800mm high x 600mm wide) and 20 stacking chairs])

Description of Activities/Functions

- Used to store exhibitions, equipment, stands, chairs etc.

Key Design Requirements

- The Secure Storage Area (Reception) must be of robust design to deter thieves.

- Doors should not open directly into circulation spaces.

Adjacencies

- The Secure Storage Area (Reception) shall allow easy access to the Reception Area.

Occupancy/Capacity

- For use by staff and exhibitors.

1.5 Entrance Foyer

General Requirements

- To provide air-lock between internal/external environments.

Dimensions

- To relevant building standard based on [xxxx] number of users.

Key Design Requirements

- DAT compliant entrance system which allows for access and egress to and from the building whilst minimising loss of heat/cooling from the building to the exterior.

- Dirt trap matting system to be employed between the doors.

Adjacencies

- Reception area and reception desk

Occupancy/Capacity

- For use by all centre users

.

1.6	Sports Hall

https://www.sportengland.org/media/4330/sports-halls-design-and-layouts-2012.pdf

General Requirements

- The overall dimensions of the Sports Hall shall be derived from the optimum arrangement of badminton courts compatible with the minimum spatial requirements for a variety of sports as set out in the table below.

- The space and safe area required for most games depends on the standard of play, and an indication of standard has been included in the table below.

- The Sports Hall shall be used for a number of different sporting activities as set out in the table below:

| Activity | Minimum Designated Standard of Play |
| --- | --- |
| Badminton Courts 8 and short tennis | Premier |
| Basketball Courts 2 –cross and 1 arena style | Premier |
| Volleyball Courts 2 | Club |
| Gymnastics | Club |
| 5-a-side Football/ Futsal Courts 3 | Community |
| Netball Courts 2 cross and 1 arena style | Premier |
| Sports Hall Athletics | Community |
| Various Martial Arts | Club |
| [Table Tennis Courts [minimum of x]] |  |
| Indoor Hockey 2 | Club |
| Handball 2 | Club |
| 2Korfball 2 | Club |

Dimensions

- A clear space of [1380m2] (in 8 x badminton court Sports Hall) with a minimum clear height of [8.3m] all in accordance with Sport England Guidance.

- Equipment storage accommodation at least 5m in height, equivalent to at least 5% of the floor area of the Sports Hall shall be provided.

Description of Activities/Functions

- Sports shall be accommodated in the Sports Hall as set out in the table above.

- Spectators shall view the sports from a viewing area.

Key Design Requirements

The design shall satisfy the following requirements:

- All surfaces to be flush and of a consistent colour in accordance with Sport England Guidance.

- A semi sprung floor finish that complies with BSEN 14904 and BS8201 all in accordance with the Sport England document ‘Sports Hall Design’. The floor shall comply with impact and flatness requirements, whilst still being robust enough to deal with external shoes.

- A robust and maintenance free surface to the walls, free from projections.

- Any doors shall open out of the Sports Hall whilst having a flush fit and flush ironmongery to the inside doors of the Sports Hall.

- Glare shall be kept to a minimum in accordance with Sport England Guidance.

- Court and guide markings for all of the sports listed in the table above to both regional and recreational standards.

- Allow for segmentation of the Sports Hall.

- Provide a means of escape for Users of the Sports Hall which satisfies all Legislation and Guidance.

- Provide a dedicated mat store of sufficient size to accommodate full size mats on a trolley and which is fire rated and mechanically ventilated to satisfy the local fire officers' requirements.

- Include an induction loop that complies with BS EN 60118-4 and BS7594.

- Meet the requirements set out in the relevant Zone Data Sheets.

- Comply with the stipulated court and safety margin dimensions and unobstructed clear height requirements stipulated by Sport England for the activities and standard of play set out in the table above.

- Provide spectator accommodation that provides a comfortable environment in which to watch the Sports Hall activity. The spectator seating shall be capable of allowing simultaneous viewing of x badminton courts.

- Fixtures, fittings, floor markings and equipment are required to enable the sports listed in the table above to be played and will be detailed in the Zone Data Sheets.

- The floor of the Sports Hall shall be constructed in such a manner and with the appropriate materials to provide a safe environment for all sporting activities specified in the table above and be flush, impact resistant and of a consistent colour.

- The wall colour shall provide a clear contrast with playing equipment such as shuttlecocks and basketballs and shall contrast with the floors.

- Glare from natural light shall be avoided.

- Intelligibility of speech within the hall is required to enable coaching and training.

- Storage doors shall open outwards from the Sports Hall.

- The mat store shall be separate from the equipment store, and shall comply with fire regulations.

Adjacencies

- Direct access shall be provided from spectator seating to the courtside.

- Secure storage for all sporting equipment shall be provided and shall be directly accessible from within the Sports Hall.

- The equipment store shall provide direct access to the Sports Hall and shall be located along the side of the Sports Hall to enable easy access from both halves of the Sports Hall when there is a central partition for activities.

Occupancy/Capacity

- The Sports Hall shall be able to accommodate a combination of Users from different sports as per the table above.

1.7	Dry Side Change Area (Sports Hall)

General Requirements

- The Dry Side Change Area (Sports Hall) shall be designed to reflect the needs of the majority of Users of the Sports Hall, and the activities undertaken.

- The Dry Side Change Area (Sports Hall) shall comply with Sport England standards as specified in the relevant guidance notes.

- The Dry Side Change Area (Sports Hall) shall include separate male and female changing areas. The Dry Side Change Area (Sports Hall) shall be designed to accommodate:

- Single people

- Classes

- Ethnic minorities

- Disabled individuals and groups

- The Dry Side Change Area is required to service all of the Sports Hall area.

- The finishes used shall provide an environment that can be easily maintained to a high standard of cleanliness and hygiene.

- The Dry Side Change Area (Sports Hall), showers and toilet area shall be suitable for use by disabled users as set out in the Sport England Guidance note ‘Accessible Sports Facilities’.

Dimensions

- To meet the Authority's Requirements within the space available.

Description of Activities/Functions

- The Dry Side Change Area (Sports Hall) shall support the following and shall support coaching and briefing sessions as part of a competitive home and away changing provision:

- Gymnastics club training

- Exercise classes

- Exercise on prescription

- Five a side football

- Badminton

- Indoor tennis

- Basketball

- Other sports as listed in the Sports Hall table of activities at paragraph 1.6

Key Design Requirements

- The Dry Side Change (Sports Hall) shall meet the requirements of the Handbook of Sports and Recreational Building Design and shall have the capacity to accommodate different ratios of males/females through the use of buffer or individual changing units.

- Toilet and shower facilities shall be provided in accordance with Legislation or, if higher, Sport England Guidance.

- Specific provision for young persons shall be provided (e.g. height of urinals, toilets and wash basins) and for people with disabilities.

- Cubicles for male and female Users who require privacy shall be provided.

- Sanitary facilities shall be provided and positioned suitably.

- Separate vanity, hair drying and baby change facilities shall be provided. Use of customers’ own hairdryers in ‘wet’ areas shall not be permitted and therefore shall not be catered for.

- All sinks and showers shall have hot and cold water and [(sinks only)] plugs that work. Soap dispensers shall be installed [at each sink] [and] [in each shower].

- Appropriate floor finishes shall be used in the Dry Side Change Area (Sports Hall), in particular, floors shall be slip resistant when wet.

- Full length ceramic tiles shall be installed in the toilets and shower area.

- The security of those using the Dry Side Change Area (Sports Hall) shall be ensured and in particular observation of access to the Dry Side Change Area (Sports Hall) shall be incorporated into the design.

- Lockers shall be secure and provided in zones allowing appropriate observation.

- The Dry Side Change Area (Sports Hall) shall be designed to reflect the needs of the Users, and activities they undertake.

- Means of securing the Dry Side Change Area (Sports Hall) shall be provided when teams of Users are participating in sporting activity.

Adjacencies

- Access shall be provided to the Dry Side Change (Sports Hall) from the general circulation route.

- The Dry Side Change Area (Sports Hall) shall provide direct access to any external sports facilities.

- The Dry Side Change Area (Sports Hall) shall incorporate a direct access to the Sports Hall that can be isolated from other Users of the dry side change areas.

Occupancy/Capacity

- The Dry Side Change Area (Sports Hall) shall have a minimum capacity of 2.5 times the access occupancy levels and shall meet Sport England Guidance.

- A minimum of 2  shower cubicles for male and female customers who require privacy shall be provided.

- Secure coin-return clothes storage lockers shall be provided.number based on assessed occupancy levels

- Toilet accommodation shall be provided at the following rates:

- Male: one WC, one washbasin, one urinal per 15 -20  Users

- Female: one WC per 7 -10 Users, one washbasin per 15 Users.

- Other specific requirements:

- Gymnastics club classes require dry side change rooms to accommodate groups of up to [xx] children and their parents. Most children do not use the dry side change areas, though specific provision for children and parents may increase the number using dry side change areas

- Older age groups, require provision for [xx] adults, approximately xx% of which are likely to be female

- Exercise classes [insert number of people]

- Exercise on referral [insert number of people]

- Five a side football [insert number of teams]

- Badminton [insert number of people]

- Indoor tennis [insert number of people]

- Basketball [insert number of teams].]

1.08	Fitness Suite

General Requirements

- The Fitness Suite shall consist of a gym with areas dedicated to aerobics, weights and stretching activities.

- The Fitness Suite shall be equipped with a minimum of [xxx] pieces of equipment including cardiovascular, resistance and free weight machines.

- The Fitness Suite shall have IFI (Inclusive Fitness Initiative) accreditation.

- The sports equipment in the Fitness Suite shall cater for people with limited movement in accordance with the guidelines from the English Federation of Disability Sport and IFI, and shall be suitable for fitness testing and GP referral.

- The Fitness Suite shall accommodate use by schools and those under 16 years of age.

Dimensions

- There shall be a minimum clear height of 4.0m

Description of Activities/Functions

- The Fitness Suite shall include (as a minimum) the following:

- Cardio-vascular equipment

- Resistance equipment

- IFI accredited stations

- Free weights zone

- Spinning zone

- Warm up/warm down stretching area with mats

- A space for gym assessments to take place in privacy

- Areas/equipment for school use, those under 16 years of age

- Areas/equipment suitable for GP referrals

- Programming complimentary to the community hospital.

Key Design Requirements

- Floor loadings shall be calculated in accordance with the Sport England document ‘Fitness Suites and Aerobic Studios’.

- A square or rectagonal design is preferred.

- Clear aisles shall be provided to ensure good access to all equipment for Users of all abilities, and shall comply with Legislation and Guidance relevant to the means of escape.

- The Fitness Suite shall meet the requirements of the Handbook of Sports and Recreational Building Design.

- A high quality sound and vision system shall be provided.

- The Fitness Suite shall include an assessment/consultation area/room.

- 2 water fountains shall be provided in optimum locations.

- Any change in floor level shall comply with the Equality Act 2010 and the Sport England Guidance note “Accessible Sports Facilities”.

- Induction loop shall be provided that complies with BS EN 60118-4 and BS7594.

- The Fitness Suite shall incorporate external views or views over other activity areas.

- Temperature cooling shall be supplied to comply with the requirements of the Zone Data Sheets.

- Storage areas in the Fitness Suite shall be a flexible space that is able to accommodate all the storage requirements of the Fitness Suite.

- There shall be a separate mats store that complies with fire Guidance and Legislation.

- Doors shall not open into the gym/aerobics studio.

Adjacencies

- The Fitness Suite shall be located close to, and on the same floor as, the Dry Side Change Area (Fitness Suite).

- All storage areas shall be located adjacent to the Fitness Suite and shall be accessible by staff members/trainers only.

Occupancy/Capacity

- The Fitness Suite shall accommodate [approximately/at least] [x] Users per hour.

- A minimum of xm2 and a maximum of xm2 per station shall be allowed for each station, as per Sport England Guidance.]

1.09	Café/bar

General Requirements

- The Café shall be capable of:

- Have a distinct area providing refreshments and hot and cold meals to Users of the Facility

- Have a distinct area providing refreshments, hot and cold meals and alcoholic beverages

- Providing catering for major sports and social events that take place at the Facility e.g. swimming galas.

Dimensions

- To meet the Authority's Requirements within the space available.

Description of Activities/Functions

- The consumption of snacks and meals.

- The consumption of alcoholic beverages

- Provision of vending machines.

Key Design Requirements

- The Café/bar shall:

- Have healthy option vending

- Range of alcoholic and non alcoholic beverages

- Give unhindered access to disabled people, who should be able to use the Café independently or with companions

- Have seating which is a combination of with/without armrests

- Have a public telephone

- Have an induction loop that complies with BS EN 60118-4 and BS7594 at the cash till

- Locate tables and chairs so there is a view into the Main Pool

- Be designed to ensure that standards of décor match successful high street equivalents.

- The layout and all furniture and seating shall be Equality Act 2010 compliant.

Adjacencies

- A physical link with the Kitchen, Store  and cellar is required to ensure easy operation of the Café.

- There shall be easy access to the vehicle delivery area and refuse collection area.

Occupancy/Capacity

- Approximately [xx] covers shall be provided.]

1.10	Kitchen, Store and Cellar

General Requirements

- The Kitchen, Store and cellar shall service the Café/bar.

Dimensions

- To meet the Authority's Requirements within the space available.

Description of Activities/Functions

- The Kitchen shall be used for the provision of food and beverage making facilities in a clean, safe environment.

- The Store shall provide a secure storage area for food, utensils and equipment used in the Kitchen and Café.

- The cellar will provide for the storing and tapping of alcoholic beverages including casks

- The Kitchen may be used for catering training, educational courses and by community groups.

Key Design Requirements

- [To be specified]

Adjacencies

- The Kitchen, Store  and cellar must be adjacent to the Café/bar

- Storage and servery areas shall be serviced from a nearby vehicle delivery area.

Occupancy/Capacity

- The Kitchen and Store shall be of a sufficient size to cater for approximately [xx] covers.

- The bar shall be of sufficient size to cater for [xx] customers

- The Kitchen must be capable of accommodating at least [xx] staff.

- The Store shall be accessed by staff/authorised personnel only.

- The Kitchen shall have the capacity to provide for orders from the [community hospital, community and voluntary groups]

1.11	Toilets

General Requirements

- Toilet provision in the Dry Side Change Area (Sports Hall) and Dry Side Change Area (Fitness Suite) are detailed in this Schedule 1 Part 1A (Facilities Requirements). There is an additional requirement for WCs to be provided in the following further areas:

- [to be specified].

Dimensions

- To meet the Authority's Requirements within the space available.

Description of Activities/Functions

- Toilet provision for general use by those not using the Facility but may be entering the Reception Area, to include a toilet for disabled Users.

- Dedicated toilets within the Facility for disabled Users.

- Additional toilet provision that complies with the Equality Act 2010 for visitors to the Café.

Key Design Requirements

- Full height ceramic tiles shall be used in all toilets.

- There shall be at least one clearly signposted unisex accessible toilet cubicle on each floor of the Facility.

- All disabled toilets shall comply with the Sport England Technical Guidance Note ‘Accessible Sports Facilities.’

Adjacencies

- In or near to the Reception Area and Café/Bar

- The disabled Users’ toilets shall be accessible from all areas of the Facility and shall open out onto a corridor with each access to the leisure facilities.

- Every part of the Facility shall be within [xx m] of a unisex accessible toilet.

Occupancy/Capacity

- The toilets shall be sufficient for use by members of the public.

- The disabled toilets shall comply with the requirements of the Equality Act 2010 and all relevant Guidance.]

1.13	Cleaner's Store (non-public support accommodation)

General Requirements

- There shall be separate storage for cleaning equipment and materials.

Dimensions

- To meet the Authority's Requirements within the space available.

Description of Activities/Functions

- Storage of cleaner’s equipment and materials including chemicals.

Key Design Requirements

- The door shall have a lock to prevent access to hazardous chemicals.

- A bucket sink shall be fitted.

- Wet/dry cleaner’s store may be combined.

Adjacencies

- The Cleaner’s Store shall be located close to the Dry Side/Wet Side Change Areas.

- The Cleaner’s Store shall be easily accessible from all parts of the building.

Occupancy/Capacity

- Staff access only.

1.14	Duty Manager’s Office

General Requirements

- Office for the Duty Manager.

Dimensions

- To meet the Authority's Requirements within the space available.

Description of Activities/Functions

- The Duty Manager shall be based here and will use it as an office and for personal discussion with staff.

Key Design Requirements

- The Duty Manager’s Office shall have a meeting space capable of holding meetings of [x] people in a private environment.

Adjacencies

- The Duty Manager’s Office shall be located near to the Reception Desk, the General Office and the IT/Communications Room.

Occupancy/Capacity

- Duty Manager and meeting of up to [x] people.]

1.15	General Office

General Requirements

- The General Office shall be where the bulk of the Facility administration is carried out.

Dimensions

- To meet the Authority's Requirements within the space available.

Description of Activities/Functions

- Administration, IT, cashing up by staff.

Key Design Requirements

- The General Office shall:

- Have a safe

- Be capable of monitoring CCTV

- Be the host for main alarm and public address system

- Have broadband connection

- Be secure with controlled access.

Adjacencies

- The General Office shall allow easy access to the Reception Desk and shall be near to the Duty Manager’s Office and the IT/Communications Room.

Occupancy/Capacity

- A minimum of x staff, including the Duty Manager.]

1.16	Staff Rest Room

General Requirements

- The Staff Rest Room shall enable staff to eat meals and rest in an appropriate and comfortable environment.

Dimensions

- To meet the Authority's Requirements within the space available.

Description of Activities/Functions

- Staff shall use the Staff Rest Room and shall require some catering/dining facilities.

Key Design Requirements

- The Staff Rest Room shall:

- Have dimensions and facilities that comply with [BCC] Health and Safety Regulations

- Be suitable for disabled staff

- Have a relaxing and comfortable environment.

Adjacencies

- The Staff Rest Room shall be adjacent to the Staff Change Area.

Occupancy/Capacity

- Staff use only.]

1.17	Internal Circulation (public support accommodation)

General Requirements

- Internal circulation refers to areas such as corridors. The materials used shall be robust and easy to clean, maintain and replace.

- Internal circulation must comply with relevant Sport England Guidance, in particular regarding width of corridors, levels, hand rails and door openings.

Dimensions

- To meet the Authority's Requirements within the space available.

Description of Activities/Functions

- Circulation areas serving all the main rooms and areas of the Facility.

Key Design Requirements

- Painted plaster-finished walls.

- Long corridors shall be avoided where possible.

- The use of natural light shall be maximised.

Adjacencies

- None.

Occupancy/Capacity

- For use by staff and the public.]

1.18	Plant Room

General Requirements

- A Plant Room shall be required to contain the machinery needed to run the Facility.

Dimensions

- To meet the Authority's Requirements within the space available.

Description of Activities/Functions

- Generator or equivalent.

- Combined Heat and Power Unit (CHP).

Key Design Requirements

- The Plant Room shall be secure.

- Design shall be flexible to enable periodic replacement of major plant items.

Adjacencies

- The Plant Room shall be located near to vehicular access to facilitate the delivery materials

Occupancy/Capacity

- Restricted access-authorised personnel only.]

A full list of minimum requirements for equipment shall be completed and set out in an appendix on a project specific basis.