ALEXANDER RECREATION GROUND

for the Resurface-paving repairs of

The Alexander Recreation Ground

King Charles Road

Surbiton

Surrey

KT5 9BE

For and on behalf of

Trinity Homes

ALEXANDER RECREATION GROUND RESURFACE-PAVING REPAIRS CONTRACT

SPECIFICATION PRELIMINARIES

PART 1

- PRELIMINARIES

1.1	Site  The address of the site is Alexander Recreation Ground, King Charles Road, Surbiton, Surrey, KT5 9BE.

1.2	The Employer  The employer is Trinity Homes.

1.3	Visit Site No appointment required.

- Description   The project comprises the following works to the existing football changing rooms:

- to renovate and upgrade of the roof mounted water storage tank enclosure,

- upgrade existing roof coverings,

- repair of perished parapet and other brickwork

- construction of a new toilet extension to one side of the football change room

- Refurbishment of internal fittings and fixtures

- Supply and installation of security fencing.

- Drawings  The Contractor may or may not provide the drawings if he believes it is required. Bidders are strongly advised to visit the site to ascertain the existing condition of the recreation park.

- Contract   The Articles of Agreement & Conditions for the Contract will be as described in Trinity Homes Conditions of Contract and Form of Tender.  The copy of this contract may be inspected by appointment with Trinity Home Authorities. The Contract period will be agreed prior to commencement, including the Liquidated Damages Clause of £50 per day for non-completion.  The defects liability period will be 12 months on a 2.5% retention preceded by a 5% retention during the contract period.

- Contingencies   Allow the specified Contingency Sum in the tender price; this sum or part of sum to be expended only with the written agreement of the Surveyors by way of a Contract Instruction.

- Costs   The  Contractor  shall  allow  for  all  costs incurred  in respect of workpeople and site staff  for the following:

- National Insurance Contributions

- Pensions

- Annual & Public Holidays

- Travelling time, expenses, fares, transport

- Guaranteed time

- Non-productive time and other expenses in connection with overtime

- Incentive and Bonus schemes

- Severance pay and obligations under the Redundancy Payments Act 1965

- Any other cost arising from the employment of labour.

- Workmanship and Materials:  Materials, goods and workmanship shall be of the best quality of their respective kinds and those for which there is a British Standard shall comply therewith unless otherwise stated. Preambles and descriptions of materials, goods and workmanship given in any one section or trade shall apply throughout this specification unless otherwise described.  Workmanship shall comply with the relevant Code of Practice where such standards exist.

The Contractor shall carry out everything necessary for the proper execution of the works, whether or not shown on the drawings or described in the specification, provided the same may reasonably be inferred there from.  Should the Contractor find any discrepancies he shall, before tendering, refer the same to the Surveyor who shall decide which shall be followed.  Figured dimensions shall be followed in preference to scaled dimensions and all dimensions and particulars shall be taken from the actual work where possible.

- Scaffold   Where applicable the Contractor shall provide and install all necessary hoists, ladders,  scaffolding, staging, tackle,  tarpaulins, tools, vehicles and plant and allow for altering, adapting and maintaining them as necessary and eventually removing and making good.  The scaffold to be erected and approved by a competent scaffolder who holds a current scaffolding certificate.

- Setting out.  Where applicable the Contractor shall take dimensions from the existing premises and verify all dimensions given on the drawing.  He shall set out and level the works where necessary and amend any errors arriving there from at his own expense.

- Work shown or indicated on the drawings and not mentioned in this specification shall be deemed to be carried out under this specification and vice versa.

- The terms 'Approved', 'Directed' and 'Selected' shall mean approval, direction and selection of or by the Surveyors.

- Access to Site & Workshops.  The  Surveyors and any person authorised by them to have at all times free access to the works, and to be at liberty to send tradesmen to execute work not included in the Contract.  The owners and their representatives are also to have liberty to send Tradesmen to execute work not included in the Contract.  The owners and their representatives are also to have access to the workshops of the Contractors and other places where work is being prepared for the building.

- Access to Site: The Access to the rear and to the front of the park are available: 8:00 am to 5:00 pm.

- Foreman Where applicable a competent general or working Foreman is to be employed continually on the site, who shall be authorised to receive and carry out instructions given by the Surveyor. This condition is to be strictly adhered to and the Contractor shall replace the Foreman by a more competent man should the Surveyor request this action.

During the contract period the Foreman shall be contactable at all times by the Client Officer or Surveyor to answer any queries that might arise.

The Foreman shall have delegated administrative powers from the Contractor in relation to the effective running and management of this contract and as such will be supplied by the Contractor with a mobile telephone which must be switched on during the working day, the telephone number to be supplied to the Client Officer and Surveyor.

Should the Contractor employ more than one gang on the contracts he shall be expected to provide a working mobile telephone for each gang when working on site within the town, and provide the Client Officer and Surveyor with their numbers.

- Responsibility The Contractor shall keep all persons, including those employed by Sub-contractors, under control and within the boundaries of the area allocated to him and shall keep his men from trespassing on adjoining property and lands around the site.  He will be held responsible for the care of the existing premises and of the works generally until their completion including all works executed and materials, goods and plant on the site together with all risks arising from the weather, carelessness of workpeople, damage or loss by theft or any other cause and he shall make good at his own expense all such damage or loss.  He shall also at his own expense uncover for inspection any work already covered up if required to do so by the Surveyor and reinstate the work.

1.18	Limits to work area  Prior to commencement of the Contract areas of working will be discussed and agreed but, initially, the Contractor is limited to a working area of 5m zone around the perimeter of the park in order to minimise damage to the green areas, or unless otherwise directed for  storage of materials, etc. N.B. - The Contractor shall, at all times, keep all building operations away, and clear of, any garden or cultivated areas.

  - Insurance  The clause of the Contract to be used is Clause 6.3B requiring the Employer to insure the existing premises and all unfixed materials and goods intended for, delivered to, placed upon, or adjacent to the works and intended therefore.  The Contractor shall insure temporary buildings, plant, tools and equipment owned or hired by him or any Sub-contractor.

1.20	Approaches and Paths Where applicable the Contractor shall keep approaches to the site clear of mud, sand, ballast and other materials through-out the execution of the works and  shall make good or pay for the reinstatement of any damage caused directly or indirectly by the execution of the works by either himself or any Sub-Contractor.

1.21	Water. The Contractor shall be allowed to draw off at the site as much water as may be necessary for the execution of the works.

1.22	Electricity. The Contractor shall be allowed to connect to the existing electrical supply any tools, etc. used in the execution of the works and all charges incurred will be met by the Employer provided that the use thereof is neither excessive or wasteful.

1.23	Lighting Should it be necessary for the Contractor to provide artificial light to expedite the work, he is to do so at his own expense.

1.24	Telephone The Contractor is advised that there is no telephone for his use at the site.

1.25	Temporary Buildings.  Where applicable if he requires them, the Contractor shall provide all temporary buildings, etc. which shall be situated in approved positions.  If required the Contractor shall provide proper sheds, etc. for the storage and protection of materials, goods, tools, etc.  Suitable office accommodation is to be provided for the Foreman and the safe keeping of any drawings, etc. supplied by the Surveyor.  The Contractor shall provide all necessary sanitary accommodation for the work people and site staff due to the fact the existing toilets will be out of order for a considerable part of the works.

1.26	Quality of materials No materials of foreign origin are to be used without consent.  Materials described under one trade and only referred to under another to be the same in quality and composition.

1.27	Carriage and Hauling Unless specified as delivered to site, pay carriage on all goods forming part of this Contract, including those covered by the PC  sums allowed for hauling.  The Contractor will be held responsible for the safe delivery of all goods and will be required to reinstate or replace any damaged in transit or delivery.

1.28	PC Sums & Subletting   The words Prime Cost or the initials PC shall mean the net amount paid by the Contractor after deducting all trade discounts, except discounts for cash.

The Contractor is to add for profit and fixing at the national current rates applicable at the time of acceptance and agreed by the employer and shall not be varied throughout the works.

Prime Cost sums shall be credited in whole or in part if not expended on completion and the Contractor shall furnish the Surveyor's office with receipted accounts.

The Contractor shall not, without the written consent of the Surveyor, sub-let any portion of the works. All specialists, merchants, tradesmen or others executing any work or supplying any goods for which prime cost prices are included in this Specification, who may at any time be nominated, selected or approved by the Surveyor, are hereby declared to be sub-contractors employed by the Contractor and (the Contractor) shall be responsible to the Surveyor  for their work and materials.

The Contractor is to satisfy himself that sub-Contractors nominated by the Surveyor will contract to do the work or supply the materials under the same condition of contract by which he is bound.  Should these conditions be unobtainable he is to notify the Surveyor immediately and in any case within seven days of the receipt of instructions from the Surveyor as to who the sub-Contractor shall be.

The Contractor is to supply the sub-Contractors with all the necessary dimensions and information to enable them to properly complete the work of their sub-contract or he is to arrange with the sub-Contractors to obtain the same.

1.29	Provisional Sums and Extras   Where provisional sums are included in this specification the amounts stated are intended to cover the cost of any work ordered by the Surveyor which is impracticable fully to specify until the works are in hand.  The net cost of all materials supplied or of labour in carrying out the work under this heading shall be carefully segregated by the Contractor who shall submit to the Surveyor all time sheets and receipts.  In the final settlement of accounts the Contractor shall be allowed these costs plus current national percentage rates on materials and labour which will be agreed with the employer before the commencement of the work and will not be varied throughout.

The Surveyor will require the Contractor to submit an estimate for all extra work likely to exceed £10.00 and if his estimate is accepted it will be final and will vitiate the conditions mentioned in the previous paragraph.

1.30	Weather Conditions.  Where applicable, the Contractor shall cover up and protect the works from the weather and suspend all operations during weather conditions which would be detrimental to the works. No concrete work setting of masonry or brickwork to be executed when the temperature is below 2°C/35°F.

The Contractor shall cover up and protect the works every night or when work is not in progress.  The Contractor shall, by the use of securely tied down tarpaulins, render all exposed parts of the building and works totally watertight.  The Contractor shall make good any damage caused by the weather due to insecure protective materials entirely at his own expense.

1.31	Clearing away   The Contractor shall take down and clear away all plant and temporary work, including sheds, etc. and make good.  The Contractor shall remove all rubbish, debris and surplus materials from the site as they accumulate and at the completion of the work and clean all surfaces, including those of the existing premises, internally and externally, remove stains and touch up paint work and polished work and leave the premises clean and to the satisfaction of the Surveyor.

No wood shavings or other rubbish is to be buried in the garden.  On completion wash all floors and pavings, clean down faces of brickwork, clean windows inside and out, oil hinges, locks, etc., bale out gullies and flush drains.

1.32	Antiquities.  Any coins, antiquities or other valuable items found on site remain  the property of the Employer.

1.33	Notices   The works are to be executed in accordance with the Building Regulations of the Local Authority or other authority and the Contractor is to give all notices required and pay all fees legally due.  The Contractor shall be responsible for advising the Surveyor of any requirements by such authorities that may vary or modify instructions already given.

1.34	VAT.  The Contractor shall exclude from his tender any allowance for Value Added Tax.  VAT payable by the Employer is to be calculated and assessed by the Contractor in conjunction with the Surveyor and such amounts as are agreed payable shall be added to interim payments as work proceeds.

1.35	Day-Works.  No work whatsoever is to be carried out on a Day-works basis without the written  consent  of  the Surveyor.

1.36	Tenders.  The Employer does not bind himself to accept the lowest nor any tender, nor will he be responsible for any costs incurred in the preparation of the tender.  The Contractor, whose tender is being considered for acceptance will be required to provide a copy of the Specification fully priced item by item and with the price on the summary agreeing with the amount of this tender.

The Contractor's attention is drawn to the fact that no variation to the Contract Sum will be made in respect of any fluctuation in the cost of labour and/or materials.

1.37	Construction (Design and Management) Regulations 2015.  The Contractor shall comply in all respects with the provisions of the current Construction (Design and Management) Regulations 2015 and will, in accordance with the provisions of Clause 6, be appointed as Principal Contractor for the duration of the works.  See the Pre-Tender Health and Safety Plan attached to this document.

## General

All works when finished must comply with the current British and European Standards and recommendations.

Risk Assessments and Method Statements showing how the Contractor proposes to undertake the works in question, for Client Officer approval shall accompany the tender documents.

## Working Hours

The working day shall be between the hours of 08.30 a.m. to 4.00 p.m. Monday to Friday unless otherwise agreed in writing with the Client Officer.

The Contractor shall identify in the Method Statements his normal weekly and daily work times on which his tender is based.

No weekend or overtime working shall be permitted without the previous written authorisation from the Client Officer.

## Risk Assessments

The Contractor shall provide copies of his risk assessment forms relating to his working practices of his employees operating on and covering the nature of this contract.

The Contractor shall fully acquaint themselves of the nature of this contract and provide a full risk assessment of how they will maintain the integrity of property boundaries around the site.

## Customer Care

During the contract period the Contractor shall be expected to adhere to the Council's Customer Care Policy, copies of which shall be provided upon award of the contract.

## Maintenance Period

From the date of Practical Completion and Handover the Contractor shall be liable for any snagging work that results from the refurbishment for a period of 12 months.  To enable this the Retention identified in the Conditions of Contract Clause 5 Contract Period, will be held by the Council and only released at the end of this period subject to any claims made upon it during this time.

ALEXANDER RECREATION GROUND RESURFACE-PAVING REPAIRS CONTRACT

SPECIFICATION SCHEDULE OF WORKS

BILL OF COSTS

Please list all costs in the following table and therefore if necessary add additional lines to this table.

| Number | Description | Quantity | Total |
| --- | --- | --- | --- |
|  |  |  | £ |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  |  | Sum |  |
|  | Project Total | £ |  |

# Pre-Tender Health and Safety Plan

Drawn up under the

Construction (Design & Management) Regulations 2015

for

ALEXANDER RECREATION GROUND

Pre-tender Health and Safety Plan drawn up under the Construction (Design and Management) Regulations 2015 for Premises situated at Alexander Recreation Ground, King Charles Road, Surbiton, Surrey, KT5 9BE.

CONTENTS

| Part 1 | INTRODUCTION & PLAN DEVELOPMENT & REVIEW |
| --- | --- |
| Part 2 | NATURE OF PROJECT |
| Part 3 | EXISTING ENVIRONMENT |
| Part 4 | EXISTING DRAWINGS |
| Part 5 | THE DESIGN |
| Part 6 | SITE WIDE ELEMENTS |
| Part 7 | SITE OCCUPIERS & CLIENTS UNDERTAKINGS |
| Part 8 | SITE RULES |
| Part 9 | CONTRACTORS HEALTH & SAFETY PROPOSALS |
| Part 10 | CONTINUING LIAISON |
| Part 11 | UNFORESEEN EVENTUALITIES |

- INTRODUCTION & PLAN DEVELOPMENT & REVIEW

  - Introduction

Clients of construction work, including maintenance and alterations, have new legal duties assigned to them under the Construction (Design and Management) Regulations 2015 (CDM). The Planning Supervisor is appointed to assist with compliance, to help ensure that others appointed under contract are competent from a health and safety perspective, and to co-ordinate and manage the health and safety aspects of the design and planning phase. Designers are also assigned legal duties under CDM, to co-operate with the Planning Supervisor, to select design elements with consideration for the health and safety aspects and to provide information on significant risks inherent in executing the design. Contractors retain ultimate responsibility under CDM for ensuring that site work is managed effectively to minimise the risk of harm arising to site works, other occupiers, neighbours or other who may be affected by the works.

  - Creating the Plan

This Health and Safety Plan has been prepared in compliance with CDM. It seeks to convey information provided by the Client, Designers and other and efforts have been made to produce a document which is as comprehensive as possible. It should be noted, however, that this outline Plan does not seek to be exhaustive - there may be unidentified hazards which are present on the site, in the existing structures or arise during the planned works. The focus in this document is to highlight known major hazards, particularly those which may be regarded as non-standard or abnormal in some way. The absence of a reference to a specific hazard should not be taken as a warranty that such a hazard is not present or cannot and will not arise during the works.

  - Plan Development & Review

CDM impose legal duties on Planning Supervisors. It is necessary to be able to demonstrate that due diligence has been employed in the creation of the pre-tender Health & Safety Plan. The Plan will also be used as a record of its own development and the management of the communication strategy between the Planning Supervisor and the Client, Designers and Principal Contractor. Accordingly, once the Plan begins to take shape it is necessary to log the process of its amendment, update and completion. This section creates that log and should be used to indicate what aspects of the Plan have been added or amended on what date. This will also assist in establishing the status of different versions as that with the latest amendment date will be the latest version.

Date				Amendments & Additions			Signature

- NATURE OF PROJECT

  - Type of Project

This project includes work to the footpaths, benches, toilets and storage.

  - Site Location

Alexander Recreation Ground, King Charles Road, Surbiton, Surrey, KT5 9BE.

  - Nature of Work

Resurface-paving repairs.

  - Time Scale

Four to five weeks (approx.) for the total project.

- EXISTING ENVIRONMENT

  - Known Previous Use of The Site

The site has been used as a sport facilities and public recreation.

  - Current Use of Site

Sport facilities and public recreation.

  - Existing Services (Electricity, gas, water, sewage, telcoms, etc.)

All mains services are connected – Electric, water and sewerage.

Internal paths of main incoming services unknown.

  - Existing Structures

Metal, wood, block, concrete, soil etc.

  - Ground Conditions

Variety.

  - Surrounding Land Use and Related Restrictions

Public and private lands.

  - Previous Use of Adjacent Sites

Not relevant.

  - Current Use of Adjacent Sites

Public and private.

  - Road and Traffic Systems Adjacent to the Site

There is no restrictions for vehicle access to the site.

  - Planning and other Restrictions Applicable to the Site

The premises is located within the boundaries of a Public and private lands.

- EXISTING DRAWINGS

No original drawings.

- THE DESIGN

  - Significant Hazards

Working At Heights.  Ref: HSE Information:

- Construction work at heights.

- Transportation of tools and materials from the delivery points to the working area.

- Removal of dismantled plumbing, electrical, wall, floor materials etc.

  - Access.  Ref HSE Construction Sheets 2 and 3 revised.

- Safe Use of Ladders, General Access Scaffolds.

- Transportation of tools and materials from the site area to the working areas.

  - Design & Safety Erection Principles

The Contractor’s workmen should take care to keep tools and equipment safe.  Works should at all times be screened off from the general public.

  - Special Precautions for Material Use

Contractors COSHH Assessment and Safe Method of Work required for use of following substances:

- Solvent based paints and cleaners.

- Transportation of heavy materials.

- Removal of masonry and other redundant materials.

  - Special Precautions for Operations

The Public Recreation Park will be occupied at all normal working times and members of the public have access at all times of the day. Therefore, full separation from the works must be maintained at all times.

  - Contractors Rules

The Principal Contractor shall undertake to notify all of his work force, whether directly employed or sub contracted of the arrangements and issues addressed in this document prior to their arrival on site, and that under the Construction (Design and Management) Regulations 2015 a copy of the notification of these works should be displayed on the site.

The Principal Contractor and all sub-Contractors shall undertake works with due care and attention, and adhere to current legislation and Health and Safety Regulations.

The Principal Contractor shall make sure all workman and sub-Contractors are competent and have relevant qualifications and experience to undertake the works. All electrical works should be carried out by a NICEIC registered electrical Contractor. Any works to a gas appliance should be carried out by a CORGI registered Contractor.

- CONTRACTORS HEALTH & SAFETY PROPOSALS

  - The Principal Contractor shall address all issues raised by other sections of this Health and Safety Plan in addition to the specific issues raised below which should form part of the tender submission.

  - The Contractor(s) detailed proposals for the safe management and execution of the works, are required for the following aspects:

PROPOSED METHOD FOR:

WELFARE

To include First Aid, W.C., Washing, Messing.

MANAGEMENT OF HEALTH & SAFETY

To include liaison.

  - Contractors Health and Safety Plan

The appointed Principal Contractor shall submit a health and safety plan that satisfied the requirements of Regulation 15(4) before construction works start.

In particular, the plan should be sufficiently developed to incorporate the following (Ref gas):

- incorporates the approach to be adopted for managing health and safety by everyone involved in the construction phase;

- includes the assessments prepared by Contractors under the Management of Health & Safety at Work Regulations (see reg.19(1)(b) and other legislation;

- incorporates the common arrangements (including emergency procedures and welfare); these may be imposed by the client or developed by the Principal Contractor;

- includes arrangements for fulfilling the Principal Contractor’s duties under Regulations 16-18;

- includes reasonable arrangements for monitoring compliance with health and safety law;

- includes, where appropriate, rule for the management of the work for health  and safety, and

- can be modified as work proceeds according to experience and information received from the Contractors (reg.19(1)(b) and (e)).

- SITE WIDE ELEMENTS

  - Site Access

Site access is from the public roads.

  - Location of Site Area

The site shall be limited to the building only. The Contractor shall not obstruct the use of the entrance by others.

  - Location of Temporary Site Accommodation

To be agreed with the premises occupants/surveyors.

  - Location of Unloading, Layout and Storage Areas

To be agreed with the premises occupants/surveyors.

  - Traffic/Pedestrian Routes

Exit routes must not be obstructed.

  - Access To Working Areas

During the course of the works all access routes are to be kept clear of materials and tools at all times.

- SITE OCCUPIERS AND CLIENTS UNDERTAKINGS

- To ensure clear vehicle and pedestrian access at all times.

- To train your Contractors in the site emergency procedures.

- To submit a register each morning (to the Surveyors) of persons on site.

- SITE RULES

- All accidents to be reported to the Surveyors.

- All spillage’s or accidental contamination of drains to be reported immediately to the Surveyors.

- Smoking is not permitted in areas of the site.

- Daily work programme to be discussed with the Surveyors.

- Noise and dust disturbance caused by removal works should be minimised, where unavoidable these operations should be screened off.

- CONTINUING LIAISON

  - Design by Contractors (procedures for considering health and safety implications for design elements of the Principal Contractor or other Contractors).

Under CDM, each element of design including temporary works, designed access equipment, individual specialist Contractor and specialist designer packages is required to be developed with due consideration for the health and safety of all concerned both during construction and subsequent occupancy and maintenance. Each element of design is to be co-ordinated for health and safety aspects for the Planning Supervisor. To facilitate this the following shall apply

- each appointed designer shall be advised to the Planning Supervisor as soon as possible

- the Principal Contractor shall ensure that any designer appointed by him shall be provided with all necessary information to enable the design to take into account other works and other design elements in selecting the most appropriate options

- each element of design shall be developed by the designer on the basis of risk assessments regarding execution, maintenance, repair and subsequent demolition and dismantling. If required, the Planning Supervisor shall be provided with evidence of the competency of the personnel carrying out the work and of the risk assessments themselves

- each element of design shall be submitted in sufficient time to permit proper consideration of health and safety, and approval of the design itself

- the Principal Contractor shall be required to update, amend and modify as necessary the detailed construction phase Health and Safety plan to address any health and safety risk associated with individual design element developed during the works, so that at all times the Contractors Plan may be considered suitable and sufficient. Such amendments and modifications shall require the approval of the Planning Supervisor prior to commencement of the relevant phase of work

- the Principal Contractor shall provide all necessary information to the Planning Supervisor to enable him to adequately prepare the health and safety file. Such information shall be provided progressively throughout the contract period but shall be submitted completely by the date for completion. The information provided by the Principal Contractor shall include, but not be limited to:

  - one copy of record or ‘as built’ drawings and plans used and produced throughout the construction process along with the design criteria where these are produced by the Principal Contractor, his sub-contractors (whether nominated or otherwise) or consultants;

  - general detail of the construction methods and materials used;

  - details of the structures equipment and maintenance facilities;

  - manuals produced by specialist Contractors and suppliers which outline operating and maintenance procedures and schedules for plant and equipment installed as part of the structure;

  - details on the location and nature of utilities and services, including emergency and firefighting systems.

- UNFORESEEN EVENTUALITIES (procedures for dealing with unforeseen eventualities during the execution phase resulting in design change and which might affect resources)

If due to unforeseen circumstances there are aspects of design which require modification post tender, the following procedure shall apply:

- as soon as it becomes known that a design element is to be modified, the Planning Supervisor shall be notified

- the Planning Supervisor may check that whoever is instructing the Design Team has ensured the provision of all necessary information to enable the design to take into account other works and other design elements in selecting the most appropriate options when modifying the design

- each modification shall be developed on the basis of risk assessments regarding execution, maintenance, repair and subsequent demolition and dismantling. If required, the Planning Supervisor shall be provided with evidence of the risk assessments

- each modification of design shall be submitted in sufficient time to permit proper consideration of health and safety, and approval of the modification itself

- the Principal Contractor shall be required to update, amend and modify as necessary the detailed construction phase Health and Safety Plan to address any health and safety risk associated with individual design elements modified during the works, so that at all times the Contractors Plan may be considered suitable and sufficient. Such amendments and modifications shall require the approval of the Planning Supervisor prior to commence of the relevant phase of work.