Compound Fencing and Site Segregation Fencing

Compound Fencing

Perimeter Fencing to Site Compound –

Supply and erect: V-Mesh type fence, finished height 50.11 of 1.80m above ground, panel width of 3.0m, posts

supported with concrete footings. Finish: Black

(RAL9005).

Provide single and double mesh gates as per the V mesh data sheet and / or equivalent as approved

Site Segregation

Bridge to Ch950': 300 Meters  To supply and install 1200mm high stock

fencing comprising of HT8/80/15 netting with two lines

of plain wire above. Strainers & struts to be driven at

ends and corners of runs. Intermediate posts to be

"Cundey" 75-100mm treated timber posts driven at 3m

ctrs.

Stockpile 'C': 278 meters To supply and install 1200mm high

stock fencing comprising of HT8/80/15 netting with two

lines of plain wire above. Strainers & struts to be driven

at ends and corners of runs. Intermediate posts to be

"Cundey" 75-100mm treated timber posts driven at 3m

ctrs.

Stockpile 'D': To supply and install 1200mm high Post & Three rail fencing. Intermediate posts to be "Cundey" 75-100mm treated timber posts driven at 1.8m ctrs. Rails to be treated timber 88x38mm 290 meters ..