# Statement of Work (Machine Specifications)

- The requirement is for an X-Ray Scanner machine which is fit-for-purpose in its intended role of screening checked-in baggage

- The tow cart and machine must meet EU specifications in all applicable areas detailed below:

- Tow Cart Specification

- Body Length – between 4000mm - 5000mm

- Body Width – between 2000mm - 2300mm

- Body Height – between 2000mm - 2500mm

- Maximum weight – 2300Kgs

- Coupling = Towball at approximately 520mm

- X-ray Machine Specifications

General:

- Tunnel opening shall be between 1000mm - 1250mm wide x between 1000mm - 1250mm high

- Conveyor Belt Height shall be between 900mm – 1000mm

- Power Requirements – 240 Volts AC @ 50Hx – using a 3-pin UK style plug

- Conveyor speed – Between 0.20m/sec – 0.24m/sec

- Conveyor capacity: Between 130kg – 150Kg

- X-Ray

- Voltage – Approximately 160KVp within a reasonable tolerance

- Beam – Horizontal

- Dose per inspection shall be no greater than 0.1mR / 1 μSv

- Imaging

Must include the following features:

- Variable zoom functionality (zoom in & zoom out)

- High/low penetration

- Organic and inorganic identification/differentiation

- Minimum 19” display size monitor