Cemetery Requirements for brick lined graves.

Specification for double depth, one-off, brick lined burial chamber

Base of Chamber

On a bed of 20mm washed gravel 75mm thick lay 9 x 255mm wide x 1050mm long and 65mm deep pre-stressed concrete lintels butted together and sealed with mastic.

Main Chamber

Using solid concrete 7.3kn blocks 215mm high x 450mm long x 150mm wide laid in 1:3 sand cement mortar with fully bedded perpends to provide and internal size of 550 x 850 mm and three courses high. on top of this lay three courses of 7.3kn blocks 215mm high x 450mm long x 100mm wide (mortar spec as above)   to give a total internal height of 1350 mm.

Middle of chamber

On the mid ledge created between the 150mm wide blocks below to the 100mm blocks above, lay 9 x 255mm wide x 1050mm long and 65mm deep pre-stressed concrete lintels and one 100 mm wide x 1050mm long and 65mm deep butted together and sealed with mastic.

On top of this to seal lower chamber after first internment lay 50mm cement and sand screed

Top of Chamber

Sealed with 9 x 255mm wide x 1050mm long and 65mm deep pre-stressed concrete lintels butted together and sealed with mastic.

Waterproofing/sealing the Chamber

Encapsulate the complete construction in 1200mm gauge DMP quality polythene with lapped and taped joints, allow sufficient surplus folded down one side to completely cover the top once the lintels have been sealed, and allow for the remaining joints to be lapped and taped.   The price tendered must include the contractor attending at the time of the interment to fit mid lintels after the funeral service, seal and screed, before fitting top lintels and laying of floral tributes.  Which includes weekends and bank holidays (Excluding Christmas Day/Boxing Day and Easter Sunday/Monday)

Excavation

The excavation of the trench is not required as this will be carried out by the cemetery contractor.

Safety

Shoring

The safe shoring of the grave should be included in the quote with any materials at cost price.  The contractor shall liaise with the digging contractor to shore the grave as require.

Grave to be shored with 2 sheets of 25mm marine ply 2400mm x 1200mm  longitudinally and2 sheets of 18mm marine ply 1200mm x 1200mm laterally. These to be joined at all four corners by 8no 75mm screws at each corner into 50 x 75 c16 timber to form a corner post. All Ply and timber cut to fit trench size exactly.

The 25mm ply to be installed with top edge at ground level. 18mm ply to sit firmly on the bottom of the excavation. Two mid bearers to be temporarily installed laterally and to be removed from ground level once chamber is built.

All shoring to be complete at the end of the working day.

All shoring to be left in-situ when the grave is back filled.

Errus fencing to be installed at the end of every working day to prevent unauthorised access.

Cement

Any Chamber built to be used the same day to be built using rapid hardening cement according to manufacturers recommendations and left for a minimum of one hour to set sufficiently for safe use.

Water

Generator and 110v electric submersible pump with a minimum 30m delivery hose to be supplied to keep works dry during excavation and construction of the chambers.

Fencing

Errus fencing shall also be supplied and erected prior to the commencement of the excavation work and removed upon completion of backfilling and reinstatement works.

Waste Material

All waste to be removed from site and will require the issuing of a Waste carriers certificate at the time of waste removal. And a copy of the contractors waste carriers license updated annually or as required to be included with the tender

Regulations

All health and safety regulations and current building regulations must be adhered to during the construction work.

A valid comprehensive £5 million pound Public Liability Insurance Cover must also be produced by the contractor

Advance Notice

The advance notice to construct a brick lined burial chamber can vary from 7 days to the morning of a burial scheduled later that afternoon.  Therefore, provision must be made in the tender to allow for this and sufficient materials held in stock.

Price Tendered

The price tendered should be per chamber, including all materials and construction work (including shoring and dpc) based on constructing one chamber and taking into account all the details specified above.