Cemetery Requirements for brick lined graves.

Specification for single depth brick lined burial chambers constructed in a trench – minimum of 9 brick chambers.

Muslim burial specifies burial as soon as possible after death, often on the same day.  Therefore, where a brick lined grave is required these have to be constructed in advance to ensure there is always a grave ready for burial.  To maximize the space available the brick chambers are constructed in a trench that will accommodate at least 9 individual chambers.  The specification is as follows:

Base of Chamber

On a bed of 20mm washed gravel 75mm thick 9 x 255mm wide x 1050mm long and 65mm deep pre-stressed concrete lintels butted together and sealed with mastic.

Main Chamber

Using solid concrete 7.3kn blocks 215mm high x 450mm long x 100mm wide laid in 1:3 sand cement mortar with fully bedded perpends to provide and internal size of 950 x 2050 mm and three courses high to give an internal height of 675mm.

Top of Chamber

9 x 255mm wide x 1050mm long and 65mm deep pre-stressed concrete lintels butted together and sealed with mastic.

Cement

Any Chamber built to be used the same day to be built using rapid hardening cement according to manufacturer’s recommendations and left for a minimum of one hour to set sufficiently for safe use.

Waterproofing/sealing the Chamber

Encapsulate the complete construction in 1200mm gauge DMP quality polythene with lapped and taped joints, allow sufficient surplus folded down one side to completely cover the top once the lintels have been sealed, and allow for the remaining joints to be lapped and taped.   The price tendered must include, if required, the sealing of the chamber at the time of the burial which includes weekends and bank holidays (Excluding Christmas Day/Boxing Day and Easter Sunday/Monday)

Excavation

The excavation of the trench is not required (this will be carried out by the cemetery contractor) but as the cemetery is a working site funerals can take place on a daily basis.  This can entail the contractor having to stop work on the trench to excavate graves so please allow up to 10 working days to complete 9 brick chambers.

Safety

Shoring

The safe shoring of the grave should be included in the quote with any materials at cost price.  Where a trench is being excavated for multiple brick chambers the contractor shall liaise with the digging contractor to shore the trench as and when required on a daily basis.   When sufficient space has been excavated and shored for at least three chambers, and it is safe to do so, construction of the chambers can commence as the remaining section of the trench is excavated.  To enable soil to be back filled between chambers each chamber shall be shored as follows:

Grave to be shored as digging progresses with 2 sheets of 25mm marine ply 2400mm x 1200mm  longitudinally and and 2 sheets of 18mm marine ply 1200mm x 1200mm laterally. These to be joined at all four corners by 8no 75mm screws at each corner into 50 x 75 c16 timber to form a corner post. All Ply and timber cut to fit trench size exactly.

The 25mm ply to be installed with top edge at ground level. 18mm ply to sit firmly on the bottom of the excavation. Two mid bearers to be temporarily installed laterally and to be removed from ground level once chamber is built.

When building multiple chambers in rows the shoring to be repeated as above as each grave is excavated.

All shoring to be complete at the end of the working day.

All shoring to be left in-situ when the grave is back-filled.

Shoring to be fitted on SHORT NOTICE  at the end of each working day at the request of the Cemetery Contractor No trench is to be left overnight un-shored.

Errus fencing to be installed at the end of every working day to prevent  unauthorised access.

Water

Generator and 110v electric submersible pump with a minimum 30m delivery hose to be supplied to keep works dry during excavation and construction of the chambers.

Fencing

Errus fencing shall also be supplied and erected prior to the commencement of the excavation work and removed upon completion of backfilling and reinstatement works.  Where a trench has been excavated for several Muslim brick chambers this could be up to a month after building has been completed.

Notice

The construction of the brick chambers may be required with only 2 weeks’ notice so sufficient materials must be kept in stock/accessed at short notice for 9 chambers.

Waste Material

All waste to be removed from site and will require the issuing of a Waste carriers certificate at the time of waste removal. And a copy of the contractors waste carriers license updated annually or as required to be included with the tender

Regulations

All health and safety regulations and current building regulations must be adhered to during the construction work.

A valid comprehensive £5 million pound Public Liability Insurance Cover must also be produced by the contractor

Price Tendered

The price tendered should be per chamber, including all materials and construction work (including shoring and dpc) based on constructing at least 9 chambers and taking into account all the details specified above.