FOL16/615: RESURFACING AND RECONFIGURATION OF TENNIS COURTS

LOT 2 – RESURFACINGAND RECONFIGURATION OF TENNIS FACILITIES INCLUDING PADEL COURT CONSTRUCTION AT THE REGENT’S PARK TENNIS CENTRE

TABLE OF CONTENTS

1.	GENERAL INFORMATION, CONDITIONS & PRELIMINARIES........................................4

1.1.	Parties and Consultants	4

1.2.	Timing and Duration of Works	4

1.3.	Site Access and Temporary Roads, Hard Standings etc.	4

1.4.	Limitations of Working Space	4

1.5.	Trespass and Nuisance	5

1.6.	Inspection of the Site	5

1.7.	Advertisements	5

1.8.	Publicity	5

1.9.	Programme	5

1.10.	Plant, Tools and Vehicles	5

1.11.	Site Administration	5

1.12.	Site Security and Temporary Fencing	6

1.13.	Temporary Accommodation for Use by the Contractor	6

1.14.	Safety, Health and Welfare of Work people	6

1.15.	Maintenance of Roads etc.	7

1.16.	Keeping Roads etc.  Clean	7

1.17.	Removing Rubbish etc. and Cleaning Works on Completion	7

1.18.	Police Regulations	7

1.19.	Control of Noise and Pollution	7

1.20.	Planning Consent & other Statutory Approvals	8

1.21.	The Construction (Design and Management) Regulations 2015	8

1.22.	Ground & Site Investigation	8

1.23.	Inspections	9

1.24.	Failure of Tests	9

1.25.	Contractor Responsibilities	10

1.26.	Definitions	10

1.27.	Quality of Materials and Workmanship	10

1.28.	Sanitary Accommodation	11

1.29.	Site Location	11

1.30.	Reinstatement	11

1.31.	Site Access	11

1.32.	Design Criteria	11

2.	SPECIFICATION APPLYING TO ALL OPTIONS	12

2.1.	Constructional and Performance Requirements	12

2.2.	Detection of Roots during Excavation	13

2.3.	Site Dimensions and Layout	13

2.4.	Health and Safety	13

3.	OPTION 1 – SPECIFIC REQUIREMENTS	13

4.	OPTION 2 – SPECIFIC REQUIREMENTS	16

5.	CONSTRUCTION PARTICULARS (BOTH OPTIONS)	16

5.1.	Preparation and Construction for Courts 11 and 12 and the Mini Tennis Courts	16

APPENDIX 1 - SITE LOCATION PLAN	23

APPENDIX 2 - TENDER DRAWINGS (ATTACHED SEPARATELY AS PDFs)	24

APPENDIX 3 - CONSTRUCTION (DESIGN AND MANAGEMENT) REGULATIONS 2015	25

APPENDIX 4 - PHOTOS OF THE EXISTING SITE	27

APPENDIX 5 - PADEL COURT CONSTRUCTION REQUIREMENTS	28

APPENDIX 6 - BURIED SERVICES DATA	32

GENERAL INFORMATION, CONDITIONS & PRELIMINARIES

Parties and Consultants

| Employer: | The Royal Parks |
| --- | --- |
| Contact: | Ms Jane Pelly |
|  | Head of Landscape |
|  | The Royal Parks |
|  | The Old Police House |
|  | Hyde Park |
|  | London |
|  | W2 2UH |
|  | T: 0300 061 2000 |

| Project Consultant: | Mr Bob Froud |
| --- | --- |
|  | MSc - Consultants Ltd |
|  | Unit 3, Greenwich Centre Business Park |
|  | 53 Norman Road |
|  | London |
|  | SE10 9QF |
|  | T; 020 8293 6655 |
|  | Fax 020 8269 0440 |
|  | e: b.froud@msc-global.co.uk |

| Contract Administrator: | Mr Steve Riley |
| --- | --- |
|  | Will To Win Tennis |
|  | The Regent’s Park Tennis Centre |
|  | The Regents Park |
|  | York Bridge |
|  | Inner Circle |
|  | London |
|  | NW1 4NUT |

Timing and Duration of Works

Works will commence on the 6th February and finish on the 31st March, 2017.

Site Access and Temporary Roads, Hard Standings etc.

The Contractor shall allow for forming a suitable site access to allow the works to be undertaken during the agreed contract period.  This shall include providing all temporary roads, hard standings, crossings and the like necessary for carrying out the whole of the Works.  On completion of the works the Contractor shall remove any temporary haul roads and fully reinstate the access way.

Limitations of Working Space

The Contractor shall confine everything pertaining to the Contract within the area of the proposed works and surrounding areas.  The Contractors operations are to be confined to the minimum area required to carry out the works, which shall be executed carefully so as to cause minimum nuisance and inconvenience to the users of adjoining facilities.

No storage of materials will be allowed on the adjoining roads or pavements.

Trespass and Nuisance

All reasonable means shall be used to avoid inconveniencing adjoining owners and occupiers.  No persons employed on the works shall be allowed to trespass on adjoining properties.

The Contractor shall indemnify the Employer against any claims or action for damage on account of any trespass or other misconduct of the Contractors' employees.

The Contractor or any of its sub-contractors, their staff and workmen shall not park any vehicles or plant on roads surrounding the Employer’s site.

Inspection of the Site

The Contractor is recommended to visit the site and surrounds, as no claim due to lack of knowledge that could have been obtained by such a visit will be entertained.  Details may be obtained from the Contract Administrator, Steve Riley.

Site visits will be hosted on the following date:

| The Regent’s Park Tennis Centre | 10:00-14:00, Wednesday, 21st December |
| --- | --- |

Advertisements

All rights of advertising on the site shall be reserved by the Employer and the Contractor shall take all necessary measures to ensure that no unauthorised advertising takes place.

Publicity

No information, either written or verbal, nor photography or drawings concerning the Contract shall be supplied by the Contractor to any third party without the written authority of the Employer.

Programme

When required by the conditions of contract, or as requested by the Employer the Contractor shall amend and revise its programme or works to show all works completed at the time of revision and the anticipated phasing and completion of all uncompleted works.

Plant, Tools and Vehicles

The Contractor shall allow for providing all plant, tools and vehicles necessary for the completion of the Works.

Site Administration

The Contractor shall allow for all necessary site administration for the proper execution of the Works.  Prior to commencing the works on site the Contractor shall confirm to the Employer the name of the person in charge of the site together with brief details of their experience.  This person is not to be changed without the prior agreement of the Employer, which will not be unreasonably withheld.

Site Security and Temporary Fencing

The Contractor shall ensure that the works and the site are properly protected and secured at all times, including any works outside the site boundary, and that the Employer is indemnified against any claim for loss, damage, theft or the like.  Where required the Contractor shall provide temporary and permanent fencing as necessary.

The site is to be maintained in a secure state until completion, at which time all temporary fencing, building materials and equipment shall be removed and the site made good.  The site is also to be left in a safe and tidy state at the end of each working day.

Guard dogs shall not be used.

Temporary Accommodation for Use by the Contractor

The Contractor shall allow for providing and maintaining all necessary temporary offices or storage for materials for its own requirements.

No offices, stores or temporary buildings shall be erected on site without first obtaining the consent of the Employer as to the position in which they are to be erected.

Contractor to photograph condition of site prior to setting temporary accommodation.

All huts and other temporary facilities shall be removed, and contaminated soil disinfected and all damage made good on completion of the works.

Safety, Health and Welfare of Work people

The Contractor shall allow for providing and maintaining all welfare and safety measures to a standard not inferior to that laid down in statutory instruments, rules and orders and subsequent amendments thereto for all workmen employed on the site including the employees of sub-Contractors.

Sanitary accommodation for workmen and staff shall be provided by the Contractor, connected to existing drainage where practicable, and maintained in a thoroughly clean, deodorised and orderly condition.

The Contractor shall provide industrial safety helmets complying with the latest BS for the use of all personnel including sub-Contractors and authorised visitors attending the site.  The Contractor shall be expected to use its best endeavours to ensure that safety footwear, helmets and other necessary protective clothing are worn be all personnel on site in accordance with current legislation.  The Contractors' attention is particularly drawn to its obligations under the Health and Safety at Work Act etc., 1974.

Maintenance of Roads etc.

The Contractor shall maintain all public and private roads, footpaths, paved areas, boundary walls and fences on or adjacent to the site in their present condition and on completion, make good any damage arising from the works and reinstate to the satisfaction of the Employer.

The Contractor shall carry out a photographic survey of the site boundaries, footpaths and roads and agree this with the Employer prior to works commencing.

Keeping Roads etc.  Clean

The Contractor shall keep any public, private and existing roads, footpaths and paving on or adjacent to the site or used by traffic entering or leaving the site in a clean and unobstructed and safe state to the satisfaction of the Employer, the Police and the Local Authority.  He shall use all means to prevent mud or rubbish of any kind being carried on to such roads, footpaths and paving, by vehicles belonging to himself or any other sub-Contractor to the reasonable satisfaction of the Employer.

Where, however, in spite of such precautions, mud or rubbish is carried on to the roads, footpaths or paving, the Contractor must immediately clean up such mud or rubbish at the Contractor’s own expense by scraping, brushing, shovelling and removing to tip.  Special attention must be given to prevent mud becoming embedded in the road and footpath surfaces.

Removing Rubbish etc. and Cleaning Works on Completion

The Contractor shall allow for removing all rubbish, protective casings, coverings and debris from the site.  All materials shall be disposed of in accordance with all appropriate environmental and health and safety legislation.

Rubbish, etc shall not be burnt on site.

Police Regulations

Police regulations shall be ascertained and the Contractor shall allow for complying with any such regulations or requirements concerning pedestrian or vehicular traffic control, the loading and unloading of or waiting by vehicles on the public highway, site ingress and egress, safety precautions and other matters affecting the works.

Control of Noise and Pollution

The attention of the Contractor is drawn to the provision of Section 60 of the Control of Pollution Act 1974 with references to the control of noise in relation to any construction works, and must comply therewith.  The Contractor is recommended to confer with the local Chief Environmental Officer in relation to proposed method of work and resultant noise level.  The attention of the Contractor is drawn to the provision of Section 60 of the Control of Pollution Act 1974 with references to the control of noise in relation to any construction works, and must comply therewith.  The Contractor is recommended to confer with the local Chief Environmental Officer in relation to proposed method of work and resultant noise level.

Work can only take place between 07:00 and 18:00 hrs from Monday to Friday.  No work is permitted on weekends or on Bank Holidays.  The park by-laws must be adhered to at all times.

Planning Consent & other Statutory Approvals

The Contractor is to allow for all necessary liaisons with the relevant planning authority and for complying with any requirements of the planning authority and all other statutory authorities notified to him by the Employer or that he may reasonably be expected to identify, before any works commence on site.

The Construction (Design and Management) Regulations 2015

This Contract will be executed strictly in accordance with the Construction (Design and Management) Regulations.  The Contractor will be deemed the Principal Contractor.

The Principal Designer is:

Mr Bob Froud

MSc - Consultants Ltd

Unit 3, Greenwich Centre Business Park

53 Norman Road

London

SE10 9QF

T: 020 8293 6655

F: 020 8269 0440

e: b.froud@msc-global.co.uk

The Contractor shall:

- Submit its Health and Safety Plan for the project prior to works commencing and shall execute the whole of the Contract strictly in accordance with Health and Safety Plan.  The Plan must be approved in writing by the Employer before works can commence on site.

- Coordinate with the Principal Designer and execute the Health and Safety Plan.

- Contribute to the Health and Safety File as required by the regulations on completion of the works or within one month of practical completion.  The Contractor shall ensure that all sub-Contractors do the same.

- Allow the Principal Designer to access to the Works and afford him every reasonable facility for the performance of his duties.

Ground & Site Investigation

Limited hand dug trial hole excavation has been conducted on courts 2 and 3 and the findings are as follows;

- 90mm depth of 6mm nominal aggregate sized porous macadam wearing course in three layers

- 40mm depth of 6mm nominal aggregate sized red pigmented porous macadam wearing course.

- 40mm depth of 14m nominal aggregate sized base course

- An undefined depth of fine ash and clinker with brick fragments.

Limited hand dug trial hole excavation has been conducted between courts 6 and 7 and the findings are as follows:

- 70mm depth of 6mm nominal aggregate sized porous macadam wearing course in two layers

- 20mm depth of 6mm nominal aggregate sized red pigmented porous macadam wearing course.

- 30mm depth of 14m nominal aggregate sized base course

- An undefined depth of fine ash and clinker with brick fragments

Limited hand dug trial hole excavation has been conducted on court 11 and the findings are as follows;

- 30mm depth of 6mm nominal aggregate sized porous macadam wearing course.

- 30mm depth of 6mm nominal aggregate sized porous macadam wearing course.

- 30mm depth of grey green surfacing (bitumen bound fine graded aggregate)

- A least a 200m depth of fine ash and clinker with brick fragments

Limited hand dug trial hole excavation has been conducted on court 4 (mini tennis court) and the findings are as follows;

- 65mm depth of porous macadam

- At least 100m depth of 28m aggregate sized limestone chippings.

There is slight puddling on courts 1-4 during heavy rain.

The surface regularity of the courts has not been measured.

It is however the responsibility of the Contractor to satisfy himself as to the nature of the existing site and grounds and presence of overhead and buried services to allow him to construct the works as required under the contract.

On the award of the contract the successful Contractor will be deemed to have undertaken their own site investigations and utility company investigations.  No claims will be accepted by the Employer for lack of information or inaccuracies in this respect.

Inspections

Where the Contractor has given notice that a particular operation or stage of the works will be ready for inspection on a specified date and the Employer or its agent then find that the works are not complete on that date so that the inspection cannot be carried out or completed necessitating a further visit, the additional cost incurred by the Employer, including all expenses, will be borne by the Contractor.

Failure of Tests

Should any intermediate stages of construction or samples tested be found, in the opinion of the Employer or its agent, to be unsatisfactory or likely to produce unsound work, the defective material, workmanship or consignment which the sample represents shall be removed from the site or suitable and approved corrective action taken.

Any sample or intermediate work stage, which has been accepted by the Employer’s or its agent, may subsequently be rejected if he shall decide that the quality has in any way deteriorated.

The Contractor shall, at its own expense, remove and replace all rejected materials, or correct any intermediate work stage shown to be outside specification.  Any delays consequential upon the rejection of any sample or work stage shall not in any way relieve the Contractor from its responsibility with regard to completion within the Contract Period.

Work corrected or materials replaced for these reasons will be re-checked by the Employer or its agent as determined by the Employer.  The additional costs of testing any material 	replaced for this reason or re-inspecting any work stage subjected to remedial works shall be recovered from the Contractor.

Contractor Responsibilities

The Contractor shall assume full responsibility for the construction of the whole of the project as described in the Employer’s Requirements.

Any materials, fixings, foundations or drainage detail indicated in these documents are typical only and shall not relieve the Contractor in any way of any of its design or other responsibilities.

The Contractor shall conform to the mandatory data and any dimensions indicated in the Works Information.

Definitions

| PPA | Area within the boundaries of the court markings |
| --- | --- |
| TPA | Area within the boundaries of the court fence |
| CPCMTC | ‘Code of Practice for the Construction and Maintenance of Tennis Courts’ published by the Sports and Play Contractors’ Association, Second Edition, March 2007 |
| ITF CS | Test method specified in the Initial ‘ITF Study on Performance Standards for Tennis Courts Surfaces’ published by the International Tennis Federation, June 1997 |

Quality of Materials and Workmanship

Where and to the extent that materials, products and workmanship are not fully specified they are to be:

- Suitable for the purposes of the Works stated in or reasonably to be inferred from the contract documents.

- In accordance with good building and/or engineering practice, including the relevant provisions of current British and European Standards.

- In accordance with the CPCMTC.

Sanitary Accommodation

Washing and toilet facilities will not be available to the Contractor within the site.  The Contractor shall provide portable washing and toilet facilities within its working compound and shall maintain these facilities in a clean and sanitary condition all to the satisfaction of the Employer's Agent.

Site Location

The site of the proposed works (see Appendix 1) is on land that is owned by the Monarch in right of the Crown and managed by the Secretary of State for the Department of Media, Culture and Sport (DCMS).

Reinstatement

Following completion of the works, Contractor will reinstate all surfacing, hard and soft, to the Employer’s satisfaction.

Site Access

Site access will normally be via Chester Gate subject to local roadworks and redirections.

For work on courts 11/12/13/14 it is suggested to use the site access route that runs past the eastern end of court 12 (see Appendix 1).  The compound will be adjacent to court 12.  For work on courts 1-10 there will also be access over the York Bridge but no lorries are permitted to go over the bridge which has weight restrictions.

The Contractor shall allow for forming a suitable compound within the site to be agreed with the Employer and shall convey by power-barrow and/or dumper, all materials to the working site.  All landscaping around the courts must be protected to minimise damage to soft landscaping.  All damage to soft landscaping must be reinstated on completion of the works.

Design Criteria

Drawings showing the dimensions and layout of the proposed facility are shown in Appendix 2.  The existing facility comprises a block of ten colour coated porous macadam tennis courts divided into a block of four of overall dimensions 36.9m to 37.2m x 57.5m and a block of six of overall dimensions 36.85m to 37.2m x 84.49m.

There are also two single tennis courts to the north of the block of six courts orientated end to end.  The overall dimensions of each of the single courts are 33.55 x 18.2m and 33.4m x 18.2m.

There are also two mini tennis courts red to the north of the block of four courts orientated end to end.  The overall dimensions of the two mini tennis courts are 31.2 x 9.2m.

The gradients of the court have not been measured.

There is currently a lighting installation on the courts which is to remain.  The current layout comprises 10 No. 10m high columns supporting a total of 16 No 2kW luminaires lighting 4 courts.

The primary sport on the new courts will be Tennis i.e. intended for school, club, recreation and community use.  The courts are free to access to junior for a period in the afternoon but at all other times court use is bookable for a fee.  There will be provision for five netball courts on site (as existing).

The Contractor is asked to price for three different options.  The Employer’s selected option will depend on whether planning permission is received prior to contract award.

- Option 1 - resurface the block of ten courts with a two layer macadam system and replace the chain link fencing using the existing fence posts.  The existing divider fence is to be taken down and a new divider fence erected to create a block of six and block of four courts.

The 2 No. single courts are also to be resurfaced with a two layer macadam system and the fencing replaced using the existing fence posts.

The 2 No. mini tennis courts are also to be resurfaced with a two layer porous macadam system and the fencing replaced using the existing fence posts.

All courts are to be colour coated.

All nets post sockets to be replaced and new nets and posts required.

A root barrier is to be installed along the southern side of court 3 (5m) using manual digging where required and following agreement of suitable method statement as are five runs of plastic ducts across the courts for future floodlight provision.

- Option 2 - As option 1 but with the addition at a later date (subject to planning permission) of planing up the existing 2 x singles courts and add 150mm of stone plus a two layer macadam system.  The gradient is to be corrected.  A root barrier is to be installed - price based on linear meter.

Conversion / planing up of the 2 x mini tennis courts into a mini tennis court and a Padel tennis courts.

The dimensions of all new facilities are shown on the plan in Appendix 2.  There are many mature trees located around the courts which have caused root damage to the courts.  Root barriers and root trimming are proposed as part of the works.

- Option 3 - Options 1 and 2 delivered together concurrently but with an eight week construction period (i.e. 8+3+1).

The specification for the layout and fencing of a Padel court are shown in Appendix 5.  The installation of posts and walls to the Padel court form part of this contract.

SPECIFICATION APPLYING TO ALL OPTIONS

Constructional and Performance Requirements

All resurfaced courts including the Padel court are to meet the constructional and performance requirements of a ‘Type 1’ Open Textured Macadam MUGA as described in Sport England/SAPCA publication A guide to the Design, Specification and Construction of Multi-Use Games Areas (MUGA’s) including Multi Sport Synthetic Turf Pitches (STP’s) ;  Part 1 (of 3) General Guidance and Design Considerations; Dimensions and Layouts.  In addition to the requirements of a type 1 MUGA, the Employer wishes the macadam surface to be colour-coated with a slip resistant, texturised, spray-applied polyurethane paint.

The constructional and performance requirements are as follows:

| Property | Test Method | Test Condition | Requirement |
| --- | --- | --- | --- |
| Slip Resistance | AENA/ITF CS/02/01 | Dry and Wet | ≥60* |
| Permeability | BS 7044 Section 2.5 Method 4 | N/A | ≥100mm/hour at practical completion and for the following 24 months |
| Surface Regularity | ITF CS/08/01 & CPCMTC | N/A | ≤ 6mm** |
| Gradients | N/A | N/A | Single plane with no fall greater than 1:120 (0.83%)*** |
| Deviation from Design Level | N/A | N/A | ±25mm |

| * | Line markings and colour coating to meet this requirement |
| --- | --- |
| ** | The Employer has tightened the SE/SAPCA specified tolerance from 8mm to 6mm with no allowance for permitted deviations above 6mm |
| *** | Disregard the 1% referred to in the SE/SAPCA document |

Detection of Roots during Excavation

Where roots are detected in any excavation, the Contractor shall be required to employ hand digging and possibly air spade excavation in that location.  Once exposed, the Employer’s Tree Officer must be allowed to inspect the roots before placement of any stone.  Roots cannot be cut or trimmed without first obtaining approval from the Employer’s Aboricultural Officer.  The Contractor’s rate for a membrane type root barrier will be used in case it is required.  The Employer appreciates that over time, roots may push the macadam upwards.

Site Dimensions and Layout

The proposed site is within The Regent’s Park.  There are trees and shrubs in close proximity to the court boundaries although there will be no need to remove trees for this project.  The Employer will cut back tree canopies prior to works commencing.

Health and Safety

CDM regulations 2015 apply and a CDM Principal Designer has been appointed.  All current relevant health and safety regulations must be strictly adhered to.  Our designer’s risk assessment is shown in Appendix 3.

OPTION 1 – SPECIFIC REQUIREMENTS

The requirement under this option is to resurface the block of ten courts with a two layer macadam system and replace the black chanlink fencing using the existing fence posts.  The existing divider fence is to be taken down and a new divider fence erected to create a block of six and block of four courts.  The new kerbing on courts 7-10 is to finish 10mm above the macadam to allow for the possibility of a 10mm thick needle punch carpet being installed in the future.

The 2 No. single courts are also to be resurfaced with a two layer macadam system and the fencing replaced using the same fence posts.

The 2 No. mini tennis courts are also to be resurfaced with a two layer porous macadam system and the fencing replaced using the existing fence posts.

All courts are to be colour coated.

All nets post sockets to be replaced and new nets and posts required (see drawings in Appendix 2 for proposed layout).

The resurfacing method for the block of ten courts will be to pierce them to assist drainage and overlay with a 40mm base course porous macadam and 25m thick wearing course porous macadam.  All edging kerbs will be broken out and replaced, and moved to a licensed skip.  There is tree root damage within the court block on the southern perimeter.  Any roots uncovered over 25mm in size need to be identified in a Method Statement and treatment to be agreed with the TRP Arboricultural Officer.  Where this has occurred, the existing macadam is to be carefully broken out in these locations and the roots trimmed back where possible.  The Contractor shall propose a method to allow the root that remain beneath the courts to have free volume to grow without immediately pushing the macadam upwards through a suitable geotextile membrane added.  The Employer does accept the roots will eventually push the macadam upwards with localized repairs likely.  Tennis net post sockets are to be installed and new nets and posts supplied.

A similar resurfacing method is to be carried out for the two single courts and mini courts.  Tennis net post sockets are to be installed and new nets and posts supplied.

The new fencing to the block of ten courts and two single courts is to be 3m high black plastic-coated chainlink11 gauge mounted on the existing angle iron fence posts.  The existing fence posts are to be painted black prior to cladding with new black chainlink. 7 single and 1 double gate are required in the block of ten and a single gate into each of the single courts.  There are some bent fence posts in the SW corner of the block of 10 which need replacing.

New 3m high black chainlink is also required around the mini tennis courts on two sides and 1m high on the southern side keeping the existing fence posts.

The macadam surfacing on courts 1-12 and the Padel court is to be colour-coated with a slip resistant, texturised, spray applied Polyurethane paint.  The colour scheme is to be light green surrounds and dark green playing areas as shown on the photo in Appendix 5.  The playing surface shall then be marked with the tennis courts and a total of 5 No netball courts.

The mini tennis court shall have a red PPA and a green surround.

The Contractor shall also include within his price the cost of supplying 12 sets of tennis nets and posts.  The construction period shall be no longer than 6 weeks.  The Employer accepts that there is to be a 3 week curing period for the macadam before it is colour coated and a further 1 week to colour coat and line mark in addition to the 6 week construction period (i.e. 6 + 3 + 1).  If the colour coating cannot be carried out due to unsatisfactory weather conditions, the Contractor shall provide temporary lines on the courts and return in the spring to colour coat the courts.  This eventuality shall be included in the contract price.

OPTION 2 – SPECIFIC REQUIREMENTS

The Contractor is to price for the following works assuming they will be undertaken under a separate contract following completion of Option 1 i.e. the Contractor has to return to site at a later date;

Plane up the existing 2 No singles courts macadam and spread and add 150mm of stone plus a two layer macadam system.  The gradient is to be corrected to 1:120.

Convert the 2 mini tennis courts into one mini tennis court and one Padel tennis court.  The Padel court is to have a concrete ring beam of 500mm wide cast around it for the installation of the Padel court posts and glass walls, which forms part of this contract.

The dimensions of all new facilities are shown on the plan in Appendix 2.  There are many mature trees located around the courts which have caused root damage to the courts.  Root barriers and root trimming to be included in the contract price

The two single courts shall be planed to break up the macadam into smaller pieces, regraded and overlaid with Terram geotextile membrane.  Onto this is then installed a 150mm layer of MOT Type 1X frost resistant limestone finished with 65mm of porous macadam (40mm depth of 14mm aggregate and 25mm depth of 6mm aggregate).  Construction to be retained within 150mm x 50mm pcc edgings (Tennis net post sockets are to be installed and new nets and posts supplied.  A membrane type root barrier is required on the northern perimeter.  The height of the retaining wall surrounding the single courts is to be increased by two courses to retain the new construction.  The court entrance is to be adapted with soldier bricks to match existing.

The two mini courts shall be planed to break up the macadam into smaller pieces, re-graded and overlaid with Terram geotextile membrane.  Onto this is then installed a 150mm layer of MOT Type 1X frost resistant limestone finished with 65mm of porous macadam (40mm depth of 14mm aggregate and 25mm depth of 6mm aggregate).  Construction to be retained within 150mm x 50mm pcc edgings or larger edgings as required suitable for raising the courts by 215mm (as specified by the Contractor in its tender).  Tennis net post sockets are to be installed and new nets and posts supplied.  A root barrier is required on the northern and western perimeter.

The macadam surfacing on the courts to be colour-coated with a slip resistant, texturised, spray applied Polyurethane paint.  The colour scheme is to be light green surrounds and dark green playing areas.  The playing surface shall then be marked with the tennis courts.

The mini tennis court shall have a red PPA and a green surround.

The construction period shall be no longer than 6 weeks.  The Employer accepts that there is to be a 3 week curing period for the macadam before it is colour coated and a further 1 week to colour coat and line mark in addition to the 6 week construction period (i.e. 6 + 3 + 1).  If the colour coating cannot be carried out due to unsatisfactory weather conditions, the Contractor shall provide temporary lines on the courts and return in the spring to colour coat the courts.  This eventuality shall be inlcuded in the contract price.

CONSTRUCTION PARTICULARS (BOTH OPTIONS)

Preparation and Construction for Courts 11 and 12 and the Mini Tennis Courts

Extended area to the north and west of the mini tennis courts

The Employer will remove the self seeded small trees in the vicinity of the excavation.

The existing topsoil and subsoil from the mini tennis court extension is to be excavated and removed from site to a licensed tip.

The formation shall be well rolled and consolidated using a roller equal or greater in weight to that used to consolidate the new playing surface.  Herbicide shall be applied to the formation, in accordance with the manufacturer’s instructions, to kill any moss and weed growth which is present.  The surface regularity tolerance on the formation shall be 25mm from design level.

There are shrubs surrounding the courts.  The Contractor shall not remove any shrubs or cut any tree roots without the Employer’s permission.  Any damage to vegetation or shrubbery will be reinstated to the Employer’s satisfaction.

Any soft spots found in the formation, shall be excavated, back-filled with stone/Terram and consolidated to stabilise the affected area.  Weedkiller shall then be applied to the formation.

Preparation

The existing macadam surface shall be planed up into MOT Type 1 grading and spread back over the courts to form a cross fall of 1:120.

Herbicide shall be applied to the existing stone/macadam base in accordance with the manufacturer’s instructions, to kill any moss and weed growth which is present.

The existing stone/macadam base shall be well rolled, to consolidate, using a roller equal or greater in weight to that used to consolidate the new macadam surface.  Any soft spots found in the formation, shall be excavated, back-filled with stone/Terram and consolidated to stabilise the affected area.

Terram 1000, or equivalent, geotextile membrane shall be laid over the exposed formation before installation of stone sub-base.  Joints shall overlap by at least 300mm.

The retaining wall height needs to be increased around the single courts by two brick courses to retain the new construction.

Excavated Area on Mini Tennis Courts

The Contractor shall construct a 450mm thick layer of free-draining, frost-resistant MOT Type 1X limestone above the formation.  Alternative fill material will be considered for any deeper excavations, below formation level.  The Contractor shall take full account of the site ground conditions.  The stone shall be at least 450mm deep.  The stone shall not be applied and consolidated in layer depths exceeding 150mm.

The Contractor shall state in his submission precisely what depth of sub-base is proposed, together with the grade or grades of stone to be employed.

Any soft spots shall be excavated and back filled using granular fill material, as specified in Clause 608 of the Department of Transport Specification for Highway Works (Class 6F1 or 6F2).  Following preparation the regularity of the stone base shall not exceed +10mm/-30mm from design level.  The sub-base shall be treated with a total weedkiller in accordance with the manufacturer’s application rates.

Stone layer above regarded planings on courts 11 and 12 and Mini Tennis Courts

The Contractor shall construct a 150mm thick layer of free-draining, frost-resistant MOT Type 1X graded limestone above the existing macadam.  Alternative fill material will be considered for any deeper excavations, below formation level.  The Contractor shall take full account of the site ground conditions.  The stone shall be at least 150mm deep.  The stone shall not be applied and consolidated in layer depths exceeding 150mm.

The Contractor shall state in his submission precisely what depth of sub-base he proposes, together with the grade or grades of stone to be employed.

The construction will then following the steps below for the block of 10 courts (piercing not applicable).

Block of 10 courts

Preparation

Arisings from the breaking out works are to be excavated and removed from site to a licensed tip at an agreed location.

The existing macadam and formation shall be well-rolled and consolidated using a roller equal or greater in weight to that used to consolidate the new playing surface.  Herbicide shall be applied to the macadam in accordance with the manufacturer’s instructions, to kill any moss and weed growth which is present.

Any soft spots found in the formation, shall be excavated, back-filled with stone/Terrram and consolidated to stabilise the affected area.  Weed killer shall then be applied to the formation.

Geotextile membrane

Terram 1000, or equivalent, geotextile membrane shall be laid over any exposed formation before installation of stone sub-base.  Joints shall overlap by at least 300mm.

Quality of baseworks

Any soft spots shall be excavated and back filled using granular fill material, as specified in Clause 608 of the Department of Transport Specification for Highway Works (Class 6F1 or 6F2).  Following preparation the regularity of the stone base shall not exceed +10mm/-30mm from design level.  The sub-base shall be treated with a total weed killer in accordance with the manufacturer’s application rates.

Drainage

The existing macadam surfacing shall be drilled at 450mm centres using a 37mm bit.  The holes shall be back filled with clean rounded gravel.

Following piercing the surface shall be power washed and the permeability of the MUGA checked to ensure that all areas are free draining.  If drainage problems are found to occur they shall be investigated and proposals made to the Employer as to how they may be overcome.

Perimeter Edgings

The new edgings are to be 900mm x 150mm x 50mm precast concrete kerbs, they shall be laid on edge and finish flush (+ 3mm) with the finished level of the playing surfacing.  The maximum gap between the outer kerb face and adjacent perimeter fencing shall be 10mm.  The kerbs shall be laid so that court gradients do not exceed 1:120 (0.83%) in a single plane.

Edgings shall be well-haunched in concrete in accordance with clause C7.5P of the Department of Transport Specification for Highways Works, Section 1101.  Top of kerb to finish flush with the top of the macadam wearing layer except for courts 7-10 where the top of the kerb is to finish 10mm above the macadam to allow for the future installation of a 10mm thick needle-punch carpet.

Macadam base course

The macadam base course shall comprise 20mm or 14mm nominal size 200 pen. open grade base course coated macadam produced and laid in accordance with Clause 2.14 of the CPCMTC.  The material shall be hot laid to a minimum consolidated depth of 40mm.

Macadam wearing course

The wearing course shall comprise 6mm 200 pen. open grade latex modified coated macadam produced and laid in accordance with clause 2.14 of the CPCMTC.  The material shall be hot laid to a minimum consolidated depth of 25 mm.  Cut back oils shall not be used.  There shall be no ridges, changes in the surface texture or bay joints that result in erratic ball behaviour.  The macadam surface shall meet the regularity and porosity performance requirements in 1.06 above.  The Contractor is to provide an extra/over cost for Bardon Leisuretex Multi Use 6mm porous macadam as the wearing layer.

Colour Coating

The macadam layer is to be colour coated with a texturised polyurethane coating meeting the performance requirements in 1.06 above.  The surrounds shall be light green and the playing area dark green.

The colour coated macadam surface shall meet the regularity, porosity and slip resistance performance requirements in 1.06 above.

The Contractor will supply a guarantee for the colour coating.

A minimum of two coats of paint shall be applied in transverse directions to provide uniform appearance.  The quality of the paints shall ensure that there are no significant signs of fading , flaking or cracking for a period of three years.  Contractor to provide appropriate certification or guarantee.

Line Markings

Courts 1-12 shall be marked out with 12 full-sized tennis courts (white markings).  All line markings to meet the slip resistance requirements set out in the Sport England/SAPCA document (see 1.06 above).  All line markings to meet the specific dimensional requirements of the sports governing body.

When measured with a calibrated steel tape all line markings shall be within 10mm of their true position, as specified in the appropriate rule.  Lines shall not deviate by more than 10mm from a line joining their ends, nor deviate by more than 5mm over a distance of 3.0m, nor include any sudden steps.

Line marking edges shall be parallel and uniform.

3 x netball courts are also required to be marked out in yellow on courts 7-10 as shown on the drawing in Appendix 2.  Netball court markings in yellow are also required on each of the single courts 11 and 12.  The slip resistance requirement will be the same as tennis – as this is to be used 90% tennis 10% netball

The Padel tennis court shall be marked as shown in Appendix 5 including a mini tennis red court marked on it.

The mini tennis red court is to have white line markings.

Court Fixtures

The new tennis net post sockets shall be bedded in concrete footings dimensioned as detailed in clause 2.22 of the CPCMTC.  New sockets shall be aligned so that the posts are vertical when under tension.  The sockets shall have captive hinged metal covers that lie flush with the surrounding court surfacing.  The metal covers shall be of a slip resistant variety.

The new posts shall comply with BS EN 1510.  	Unless aluminium or brass, all metal work shall be galvanised after cutting, welding, fabrication and drilling in accordance with BS 729.  The centre band anchor shall be installed at the new playing surface level, flush with the surface.

The Contractor shall supply a combined bootscraper/brush unit outside each of the gates, at the side of the paths.

Fencing

The new fencing system to the block of ten courts and two single courts is to be 3m high black chainlink on the existing angle iron fence posts.  The existing fence posts are to be painted black prior to cladding with new black chainlink. 7 single gates are required in the block of ten and a single gate into each of the single courts.  There are some bent fence posts in the SW corner of the block of 1 which need replacing.

The fencing specification for the Padel court is shown in Appendix 5 (to be installed by Verde Padel under a separate contract).

The mini tennis court is to have 1m high black chainlink fencing around it on the the southern perimeter and 3m on the other two perimeters with new tubular fence posts with a top and bottom rails.

Root Barrier

The Contractor is to install a 600mm deep membrane type root barrier in affected areas.

Any roots exposed over 25mm need to be included in a method statement to be approved by the TRP Arboricultural Officer.

Access Paths

The Contractor shall ensure DDA access to the courts at the eastern end of the courts.  The ramp is to be installed with a gradient to meet DDA requirements (including handrails and landings as applicable).  The path shall be sloped so as not to hold water and the path shall be free of ridges, steps and trip hazards.  Surround landscaping shall meet the path edges so that there are no vertical drops from the path edges.  The existing access steps shall be reconstructed to form a ramp and a black powder coated handrail fitted to one side.

Ducting

To future proof for any future lighting installation, the Contractor is asked to price for the installation of five No plastic ducts across the main court block, and two across the mini court block.

Site Rules

All of the Contractors staff are to abide by the site rules which will be supplied by the Employer and are to work fully clothed at all times.  Drugs and alcohol are strictly forbidden on site.  Operatives must wear PPE at all times during the works.

Reinstatement & Landscaping

The Contractor shall leave the site everyday in a clean and tidy condition.  All damage caused to surrounding areas and surfaces shall be reinstated in full to the satisfaction of the Employer.  Any fencing or gates which are removed to gain access to the site or enable the works to be carried out must be reinstated as found.  All hard areas shall be reinstated using similar materials to existing and to the satisfaction of the Employer.  The Contractor shall remove all rubble and sub-soil material completely from site, unless otherwise agreed by the Employer.

Warranty

The Contractor shall provide a written warranty that the properties of the combined surface and foundation construction shall be retained for the 12 months defects liability period plus a further four years after practical completion subject to the correct maintenance and the stated patters of usage.

Testing on completion

The facility will be tested on completion and practical completion will not be awarded unless the facility meets the requirements set out in 1.06 above.  The facility shall also be tested prior to the end of the defects liability period.  Should any of the test results fail to meet the requirements of 1.06, the Contractor shall immediately carry out rectification works to the satisfaction of the Employer.  The test regime on the facility is as follows;

| Property | Testing Requirements |
| --- | --- |
| Slip Resistance (colour coating and line markings) | Shall be determined in at least six locations including all court colours and line marking colours. Tests shall be made under both wet and dry conditions |
| Permeability | The permeability rate of the surface shall be determined in not less than three locations |
| Regularity | The entire surface shall be assessed |
| Gradients | The principal gradients of the courts shall be measured and the design levels calculated |
| Levels | Levels to be taken on a 5m grid to determine deviation from design level |

Planning Approval

The court resurfacing under option 1 does not require planning approval.

The Employer is applying for planning approval for the padel court.

- APPENDIX 1 - SITE LOCATION PLAN

(SEE SCHEDULE 3 – SITE INFORMATION)

- APPENDIX 2 - TENDER DRAWINGS

- (ATTACHED SEPARATELY AS PDFs)

- APPENDIX 3 - CONSTRUCTION (DESIGN AND MANAGEMENT) REGULATIONS 2015

Designer’s Risk Assessment

| MSc HEALTH & SAFETY CHECKLIST FORM CDM 1 |  |  |  |
| --- | --- | --- | --- |
| Employer: | Will To Win Tennis | Section of work: | Tennis Court Reconstruction at Regents Park |
| Project No: | 116-0705 | Type of check: | Pre-tender design |
| 1 | ## CONCEPT DESIGN |  |  |
| 1.1 | Does the conceptual design satisfy health & safety considerations as required by regulation 13? | YES |  |
| 1.2 | Does the relevant Planning Authority impose any special requirements? | NO |  |
| 2 | SITE CONSIDERATIONS | Are risks present? | Comments |
| 2.1 | Proximity of boundaries | YES | Site within a public park |
| 2.2 | Adjacent water courses | N/A |  |
| 2.3 | Existing structures on or adjacent to site | YES | N/A |
| 2.4 | Adjacent railways | N/A | N/A |
| 2.5 | Existing roads and access to site | YES | See plan in Appendix 1 Banksman required |
| 2.6 | Services including |  |  |
|  | Gas/Electricity/BT Cables | YES | Not known |
|  | Water mains / Sewage | YES | Not known |
|  | Electricity cables (overhead/buried) | YES | Not known |
|  | Cable TV | YES | Not known |
| 2.7 | Public footpaths and roads | YES | Public Footpaths within park |
| 2.8 | Occupation of site by Employer or others | YES | Park Staff and the public |
| 2.9 | Other Contractors | N/A |  |
| 2.10 | Site clearance/demolition works | NO |  |
| 3 | GROUND CONDITIONS |  |  |
| 3.1 | Contaminated land | N/A |  |
| 3.2 | Dangerous ground conditions | N/A |  |
| 3.3 | Volatile gases | N/A |  |
| 3.4 | Polluted ground/surface water | N/A |  |
| 3.5 | Underground structures/Mining | N/A |  |
| 4 | CONSTRUCTION DESIGN |  |  |
| 4.1 | Method of construction | YES |  |
| 4.2 | Trenches >1.2m | YES |  |
| 4.3 | Scaffold & Access | YES |  |
| 4.4 | Temporary works | N/A |  |
| 4.5 | Confined spaces | N/A |  |
| 4.6 | Work on or near water | N/A |  |
| 4.7 | Work on or near gas or electricity | N/A |  |
| 4.8 | Height of works | N/A |  |
| 4.9 | Alterations to existing structures | NO |  |
| 4.10 | Special Construction techniques | N/A |  |
| 4.11 | Special Construction materials | N/A |  |
| 4.12 | Lifting operations | YES |  |
| 4.13 | Traffic management | YES |  |
| 4.14 | Building and fire regulations | N/A |  |
| 5 | DESIGN POST + CONSTRUCTION | ## Are risks present? | ## Comments |
| 5.1 | Access of maintenance | NO |  |
| 5.2 | Working space of maintenance | NO |  |
| 5.3 | Safe operating conditions for equipment | NO |  |
| 5.4 | Safe cleaning particularly for glazing | YES |  |
| 5.5 | Demolition | NO |  |
| 6 | DUTIES UNDER COSH REGULATIONS | Comment |  |
| 6.1 | Have alternative substances hazardous to health been considered | YES – Concrete, macadam. polyurethane paint |  |
| 6.2 | Are substances hazardous to health noted in health and safety plan? | YES |  |
| 7 | GENERAL |  |  |
| 7.1 | Have specific risks and special construction requirements being noted on drawings and or in Health & Safety Plan? | Yes |  |
| 7.2 | Have all other possible aspects that could require checking been considered? | Yes |  |

Name: Bob Froud MIM, I.Eng, Dated: 12/08/16

The Contractor shall submit an acceptable site-specific health and safety statement and risk assessment to be assessed and approved by the Employer before any works can commence.

- APPENDIX 4 - PHOTOS OF THE EXISTING SITE

(SEE SCHEDULE 3 – SITE INFORMATION)

- APPENDIX 5 - PADEL COURT CONSTRUCTION REQUIREMENTS

ISOMETRIC VIEW

SPECIFICATION AND QUOTATION

QUOTATION

SPECIFICATIONS

CONCRETE RING BEAM DETAIL

PADEL COURT LAYOUT

- APPENDIX 6 - BURIED SERVICES DATA

(TO BE ASSESSED BY THE CONTRACTOR ON SITE)