RFQ057 - Appendix E - SAN Technical Requirements

M/D - Mandatory/Desirable

M - meets requirement

P - partially meets requirement

N - Not meet requirement

| # | M/D | Requirement | M/P/N | Response |
| --- | --- | --- | --- | --- |
| 1 | M | Latest technology featuring a fast system disc storage system with a high level of redundancy, resilience and availability (99.995% minimum). Hence should have low latency, disc RAID, multiple power supplies, fan, management modules etc. There should be no single point of failure so management modules, PSUs must be doubled up and hot swappable. Rack space should be described. |  |  |
| 2 | M | Solution should be sized for approx. 33Tb of usable data without compression or de-duplication. |  |  |
| 3 | M | Solution must provide 30,000 IOPS at <1ms latency (based on 50/50 read/write, 4kb random) |  |  |
| 4 | M | Must have Active/Standby controller architecture. A single controller must be capable of handling the entire workload. Failover must be automatic, seamless and without intervention |  |  |
| 5 | M | Minimum of 4 x 1Gb ports initially plus a further 1Gb port out of band management report. To include 2 x 10Gb port for future use. An option would be to allow the 10Gb to operate at 1Gb |  |  |
| 6 | M | Must be able to handle multiple disk failures without data loss. State limitations. |  |  |
| 7 | M | Solution must be self-configuring without intervention, to maximise performance and minimise management. This must also extend to applications such as SQL and potentially Oracle) |  |  |
| 8 | M | Any compression should be in-line with no impact on performance |  |  |
| 9 | M | Must be capable of interfacing with and able to use the existing VEEAM backup system. |  |  |
| 10 | M | Licensing - all products should be fully included so no hidden surprises. Any options should be clearly defined along with projected annual costs for 5 years |  |  |
| 11 | M | SAN must offer encryption of selected data sets or all |  |  |
| 12 | M | Solution must be easily scalable by adding new discs etc. Suppliers should state how this achieved |  |  |
| 13 | M | System upgrades must be achievable with no downtime. |  |  |
| 14 | M | 24 x 7 Pro-active monitoring and feedback of potential system failure should be included as standard. State how achieved. |  |  |
| 15 | M | Reporting to include: detailed disc usage, current and predicted growth, errors. Must also include ability to monitor VMWare from host to storage. |  |  |
| 16 | M | Training for up to 4 IT staff should be included. |  |  |
| 17 | M | 24 x 7 4hr response Maintenance Support for 5 years. Indicate additional year costs beyond 5 years. Include option for 12 x 7 NBD or similar. |  |  |