241 Thameside, Chertsey KT16 8LD

We require the replacement of the existing conservatory roof with a new lightweight insulated pitched roof to take into account these matters:

- The new design needs to resolve the existing problems, specifically:

  - The existing window frames are being distorted by the existing roof structure

  - There are various water leaks caused by the distorted frames and at other locations

- The new roof needs to be light enough to prevent the distortion of the window frames and for the existing foundations to support.

- The design needs to ensure that the load is spread between the existing masonry to the corners and the main house with a new wall plate, avoiding point loads to the non-load bearing window heads.

- The design should ensure a suitable junction between the existing roof to the house and the new roof, to ensure good drainage and avoid pooling water.

We will require submission of a roof design for evaluation.

Please also note the requirement to submit information about health and safety (see evaluation of tenders document included)