Schedule of Site Works & Construction of an All-weather Path, 2 metres wide, around Western Road Recreation Ground, Hailsham.

| Section Ref: | Length . m |  | Site Works | Construction | Notes |
| --- | --- | --- | --- | --- | --- |
| A to B | 136 | Alignment to be agreed with HTC….behind existing seats? Excavate grassed surface avge. 250mm deep , load & dispose of arisings off site estimated 85 cu.m |  | 150x50mm Pc concrete edgings Non-woven Geotextile underlay 150mm Type 1 sub-base material 60mm – AC14 bin 40mm - AC10 Surf. | Cross section of standard construction, Ref: I |
| Gate to B | 12 Excl. from overall total of perimeter path | Break up pc edgings, both sides & dispose of arisings. Excavate to progressively widen the entrance path (between new edgings) from 1.1 to 2.0 metres over the 12 metre length. |  | Install new pc edgings (150 x 50mm,) , allowing for 40mm new oversurfacing . Infill the widening with 100/125mm of 12 :1 leanmix concrete compacted and finished flush with existing path construction Apply tack coat and oversurface with 40mm AC 10 AC Surf, finished flush with tops of new edgings. | Widening the access path from Western Road to join with new 2 metre path towards the Cricket Pavilion |
| B to C | 27 | Break up and cart away existing concrete path 1.1 m wide x say 100mm thick with 40mm bitmac surfacing & pc edgings both sides. |  | Standard Construction, as A-B, on new formation after removal of existing concrete. Depth of excavation say 150 mm &set aside for re-use on site. | Cross section of standard construction, Ref: I |
| C + 4 metres | 4 | Splay leading to Pavilion…leave as existing, tying in new surfacing levels |  | No new construction needed for this short section..in good condition & alignment | Leave as existing |
| C to D | 19 | Break out & dispose of existing concrete edgings & foundation/backing. Excavate 210mm deep in verge area to widen path adjoining Pavilion 1.5 to 2.0 metres, & set aside soil for re-use on site. |  | Construction all as shown on cross section III-b, including new 40mm new surfacing & tack coat. Break out & dispose of existing surfacing if needed to control level against building …(damp course?) | Cross section III-b refers |
| D to E | 4 | Existing concrete Hardstanding to remain as existing, agreed with Cricket Club 24/1 |  |  |  |
| E to F | 98 | Break up and cart away existing concrete path (including 32 metres of pc slabs 900x 600mm) & in-situ 900 m wide x say 100mm thick. Excavate average 150m, deep, load and dispose of off site..estimated 40 cu.m |  | 150x50mm pc concrete edgings Non-woven Geotextile underlay 150mm Type 1 sub-base material 60mm – AC14 bin 40mm - AC10 Surf. | Cross section of standard construction, Ref: I : with 100mm less depth excavation |
| F to G | 34 | New construction as Standard, 2 metres wide. |  | 150x50mm pc concrete edgings Non-woven Geotextile underlay 150mm Type 1 sub-base material 60mm – AC14 bin 40mm - AC10 Surf. | Cross section of standard construction, Ref: I |
| G to H | 11 | New construction as No-Dig , 2 metres wide…excavate 200mm deep @ G & reduce to 100mm @H…transition of levels past cricket nets, raising path surface to protect shallow tree roots. |  | 150x50mm pc concrete edgings Non-woven Geotextile underlay 150mm Geocells filled with 4-20mm open graded aggregate. 50mm AC10 porous Asphalt Surfacing | Cross section of No-Dig construction, Ref: II |
| H to I | 90 | New construction as No-Dig , 2 metres wide…excavate 100mm deep (H to I) Store excav. materials for re-use on site |  | Construction 200 mm thick, all as above (G-H). Pathway surface generally @ 100mm above existing cricket ground | Cross section of No-Dig construction, Ref: II |
| I to J | 90 (approx.) | Ditto , all as above (( I to J) Store excavated materials for re-use on site in regrading shallow batters to raised path levels. |  | Ditto , all as above . Path alignment between the trees and hedge on eastern side…to be agreed with HTC | Cross section of No-Dig construction, Ref : II |
| J to K | 22 | New construction as Standard, 2 metres wide. Transition between No-Dig @ J (OGL + 100mm) to meet surface level of entrance path to fenced Playground. |  | 150x50mm pc concrete edgings Non-woven Geotextile underlay 150mm Type 1 sub-base material 60mm – AC14 bin & 40mm - AC10 Surf. | Cross section of standard construction, Ref: I |
| K to A | 30 | Break out & dispose of existing pc edgings concrete backing & foundations on both sides of existing path..existing width 1.1 metres |  | Provide and install new 150x50mm pc edgings raised, by 40mm on western side as per existing alignment, and allow for 2.5% crossfall to top of pc edgings on the hedge side. Excavate on eastern (hedge) side average 1.1 metres wide and 210mm deep & set aside clay/soil for re-use on site. Oversurface resulting 2 metre path with AC10 Surf. finished flush with tops of pc edgings | Construct widening to path to standard detail Ref : III -a |
| Total Perimeter | 565 | Subject to actual alignments and re-measured on site, on completion. |  |  |  |

Refer also to accompanying Check List for pricing, with Site Plan and various sketches, detailing cross sections:

Updated version 17 Feb 2020