The Maritime and Coastguard Agency (MCA) is an Executive Agency of the Department for Transport.  The MCA is responsible throughout the UK for implementing and developing the UK Government’s maritime safety and environmental protection policy.  That includes co-ordinating Search and /Rescue at sea through Her Majesty’s Coastguard 24 hours a day, and checking that ships meet UK and international safety rules.  The MCA work to prevent the loss of lives at the coast and at sea, to ensure that ships are safe, and to prevent coastal pollution: Safer Lives, Safer Ships, Cleaner Seas.

The MCA provides a full range of search and rescue, counter pollution, survey, inspection and enforcement activities and has 12 major business activities:

| Survey | Seafarers’ Services |
| --- | --- |
| Inspection | Search and Rescue |
| Enforcement | Pollution Response and Salvage |
| Ship Registration | Stakeholder Communication |
| Navigation Services | Ministerial Services |
| Strategic Prevention Design/Development | Regulatory Process |

These activities are maintained by support services responsible for providing a range of administrative functions including: infrastructure, MCA people, financial management and administration and corporate management.

In accordance with the 2010 Equality Act, our capacity as a public body means we have a statutory duty to eliminate unlawful discrimination, promote equality of opportunity and promote good relations between people of different backgrounds.  Contractors will be expected to ensure that the service they provide promotes good relations between the MCA and its customers and does not directly or indirectly discriminate on the grounds of any of the protected characteristics specified in the Act.

You are invited to submit a tender for the following project:

MCA REFERENCE:  TCA 3/7/1022

SMALL COMBINATION SKIMMER SYSTEMS

General Requirement

1. 	The MCA wishes to purchase five small, shallow draft, hydraulically driven combination skimmer systems with transfer pumps and power packs. The transfer pumps should either be integrated with the skimmer or power pack.

2. 	The skimmers should utilise changeable modules (e.g. brush, disk, drum) so as to be able to recover most levels of oil viscosity. Included in the package for each skimmer system should be brush and disk modules.

3.	The skimmers will be used in shallow waters close to the shoreline and in the collection area of the MCA’s Current Buster 2. Its operating draft should therefore not be greater than 200mm and its longest surface dimension no greater than 1.5m.

4.	This skimmer will be light enough to be handled manually by two people for short lifts.

5.	The skimmer will incorporate a pump capable of dealing with most oil viscosities and be able to tolerate small pieces of debris taken in by the skimmer.

6.	In optimum conditions the skimmer should have a proven recovery rate of at least 10m3 per hour using disk banks and 25m3 using brush banks.

7.	Long-term durability of the equipment is a key requirement for the MCA, so the skimmer must also:

- Have a minimum expected service life of 15 years (barring its use in a pollution incident);

- Be easy to maintain

8.	The MCA wishes to purchase a proven solution, so evidence of the skimmer having been tested, either in use or in a realistic simulated environment, must be provided.

Storage and Distribution

9. 	The MCA intend to store the skimmers in designated shoreline deployment packages, in either 10ft containers or cages located in one of our three warehouses. 	However, the MCA would still appreciate recommendations from the Tenderer for alterative storage solutions, which will be considered. The recommended storage solution should be priced separately.

10. 	The tenderer should detail the ideal storage conditions for the skimmers, including the temperature range.

Deployment

9.	The skimmers will be deployed from the shoreline or from the deck of a vessel into the collection area of the MCA’s Current Buster 2 it should therefore be possible to handle safely by hand for short lifts, by two operators and have appropriate handholds. The weights of components and the user instructions for deployment should be fully compliant with UK health and safety legislation. Lifting slings or attachments for lifts by crane should also be provided.

10.	The skimmer head itself will be free floating.

Guarantee

11. 	We regard warranties as an important component of achieving overall best value for money.  Full details of your warranty on the tendered equipment is required, including the period of cover.  Please also explain if in-house maintenance affects your warranty terms.

Training / Commissioning

12.	Training/commissioning on this equipment is not required. Instead, a comprehensive user manual should be provided giving clear instructions for the deployment, operation and maintenance of the skimmer.

Sustainability

13.	The MCA is committed to sustainable procurement.  This means making the necessary decisions to operate our procurement activity in an economically, socially and environmentally responsible way, in accordance with the policy published at https://www.gov.uk/government/collections/greening-government-commitments.

14.	The MCA considers that there are three main areas of sustainability risk in this contract, and suppliers should state in their tender what steps they would take to minimise these risks in the delivery of the contract.  The identified risk areas are:

- Emission of greenhouse gases in the transport of goods to the MCA site;

- Emission of greenhouse gases, use of energy, water and materials in the manufacture of the product; and

- Poor working conditions and/or breach of equality and diversity principles, in the supplier’s workforce and supply chain.

Delivery

15.	The MCA has counter pollution equipment stockpiles at Barnsley, Bristol and Dundee.  However, the price for this tender should be for delivery costs to Barnsley.

16.	Suppliers should state in their tender their lead time for delivery to Barnsley, which must be by 31st March 2018.

Payment

17.	Payment shall be made upon delivery of the goods to Barnsley in accordance with the ‘General Conditions of Contract for the Supply of Goods and Associated Services’, and subject to the receipt of a valid and correctly submitted invoice.  The MCA pays undisputed invoices 30 days in arrears.

Contractors should note that the MCA has migrated its invoicing activity to the DfT Shared Service Centre (Swansea). Invoices should be sent to the address below and include a case reference number and transaction account code (to be assigned by the Agency). Invoices also need to detail the purchase order number issued by the MCA and a full description of items provided:

DfT Shared Service Centre

Arvato Bertelsmann

Sandringham Park,

Swansea Vale,

Swansea, Wales,

SA7 0EA.

The MCA will comply fully with statutory legislation on late payment on the basis of claims submitted by the successful tenderer.