# Gringley on the Hill Parish Council

## Multi-Use Games Area (MUGA) Refurbishment

### Tender Specification

## 1. Project Overview

Gringley on the Hill Parish Council invites tenders for the refurbishment of the existing Multi-Use Games Area (MUGA) located at:

Gringley Play Area, Sunny Bank Gardens / Finkle Street, Gringley on the Hill, DN10 4RZ.

The existing facility measures approximately 36m (L) x 18.3m (W), total area approx. 659m².

The refurbished surface must support informal and recreational football, basketball, tennis and similar sport types.

The facility is publicly accessible and free to use.

## 2. Scope of Works

The Contractor shall provide a full design, supply and installation service including site inspection, preparation, surfacing, line marking, and handover documentation.

## 3. Preparation Works

Chemically treat existing porous macadam to remove moss and algae.

Thoroughly clean existing surface.

Repair minor surface defects including cracks and fretting edges.

Confirm structural suitability of base prior to installation.

Allow for localised levelling where required.

Report any significant base failures before proceeding.

## 4. Surfacing Specification

Shockpad Layer: TBC tenderers to price in correspondence with Parish Council.

Wearing Course: TBC tenderers to price in correspondence with Parish Council.

Colour to be confirmed; tenderers to price black and one standard colour option.

UV stable, permeable system suitable for year-round recreational multi-sport use.

## 5. Line Marking

Provide durable line markings for 5-a-side football, basketball court, and optional tennis court.

Line markings must be contrasting colour and suitable for long-term wear.

## 6. Equipment (If Required)

Optional pricing for replacement basketball hoops and backboards.

All equipment must be vandal resistant and compliant with relevant BS/EN standards.

## 7. Drainage

Surface must remain fully permeable.

Contractor to confirm no ponding risk and that existing falls remain appropriate.

Minor drainage adjustments to be included if required.

## 8. Access & Working Conditions

Vehicle access available within 3–5m of court.

Contractor responsible for site safety, welfare, and safeguarding.

Site must remain secure during works.

## 9. Quality & Standards

Minimum 5 years’ experience in sports surfacing required.

Provide three comparable project references.

Minimum 5-year product warranty and installation guarantee.

Compliance with relevant BS EN standards.

## 10. Programme

Tenderers must provide mobilisation period, installation duration, and total programme length.

Target completion within 6 months of contract award.

## 11. Health & Safety

Provide Risk Assessment and Method Statement (RAMS).

Public Liability Insurance.

Evidence of staff training and competency.

## 12. Maintenance & Handover

Provide maintenance manual and cleaning guidance.

Provide warranty documentation.

Provide as-installed system specification.

## 13. Pricing Format (Mandatory)

Lump sum total (net of VAT) with VAT clearly identified.

Breakdown of preparation works, shockpad, surfacing, line marking, colour upgrade, and optional equipment.

## 14. Evaluation Criteria

Price.

Compliance with specification.

Warranty length.

Experience and references.

Programme and whole-life value.

The Parish Council reserves the right not to accept the lowest tender.