DAMEN 4207

SECTION L

SEA TRIAL PROGRAMME

1. 	List of onboard personnel posted ashore.

2. 	Record fuel and fresh water levels both before and after trials.

3. 	Record draught marks both before and after trials.

4.	Record weather conditions, sea state and trials area.

5. 	Ensure all safety equipment is on board, in date and certificated.

6.	Operate emergency steering.

7. 	Record ETD and ETA to competent person of the contractor’s staff.

8.	A full power trial of at least 2 hours duration is to be carried out in weather conditions that will allow free movement of personnel around the cutter including weather decks, the trial will consist of the following:

(a) 	Work up to full engine power (1880 rpm) in stages agreed by the

onboard Caterpillar Engine Service Engineer who will record main 		engine instrumentation readings.

(b) 	Two 360 degree circles turning to Port at full speed.

Two 360 degree circles turning to Starboard at full speed.

Two figure of eight turns at full speed.

An Emergency Stop from ahead at 1500 RPM.

(NOTE: full ahead to full astern is not to be attempted, unless in the

case of a real emergency).

(c) 	A run of approx 5 minutes duration at full PRM on the Port Main Engine only.

(d) 	A run of approx 5 minutes duration at full RPM on the Starboard Main Engine only.

(e) 	An astern run of 1 minute duration at 1200 RPM is to be carried out on both main engines.

(f) 	Bowthruster (Main engines clutched in)

One complete 360 turn clockwise.

One complete 360 turn anti-clockwise.

(g) 	Anchor trial

Carry out anchoring trial and recovery

9.	Equipment manufacturer’s personnel to be present during the Sea Trial 	Programme:

- Caterpillar Service Engineer.

- Rolls Royce

- Vosper Marine

- Electrical contractor

NB

1. Damen EPSE Dept are to ensure that all safety equipment is onboard and extant prior to proceeding for sea trial.

2. Damen EPSE Dept are  to carry out a safety drill and appoint nominated personnel familiar with the operation of the LSA and fire fighting appliances

a:\asma