# SPECIFICATION FOR HM ARMED FORCES VETERANS BADGE AND BOX

The HM Forces Veterans Badge shall consist of a 3 colour enamelled round shaped badge surmounted with the St Edward’s crown and with an arced scroll in dark blue with legible word VETERAN in gold lettering below. The badge bears the insignia of the armed forces with HM ARMED FORCES centred around the top of the circle in gold. The parameters are as follows:

Badge design

- Round with obverse showing, as Diagram 1 shows
(No variation to this design will be permitted)

- Centre circle 16 mm +/- 0.5 mm diameter

- Height from top of crown to bottom of scroll 26 mm +/- 0.5 mm

- Max width of scroll 19.2 mm +/- 0.5 mm

- Metal finished gilt and three colour (red (within crown), white (within the circle of the badge) and blue (within the scroll) hard enamel with no marks or blemishes.

- Reverse- Polished gilt coloured finish

- Squeeze clutch pin on reverse

- Abbreviated Design Registration number impressed on reverse of badge 
– REGD. DES 274493.1 as shown in Diagram 2

Box

Outer

- Metal frame covered with blue vinyl

- Hinged along one side incorporating a spring to hold the lid open or closed

- Lid top, sides and base edged in gold, as shown in Diagrams 3 and 4

- Protective white cardboard sleeve

- Diagram 3 provides a plan view of the top of the box

- Size 52 mm by 47 mm by 30 mm +/- 10mm

Inner

- The inside of the lid to be padded covered in white satin to hide the hinge.  The manufacturer may place their logo on the lid lining.

- Base lined with white card

- Navy coloured flock base pad folded in half to facilitate fitting of Lapel Badge.  An appropriate hole to hold the badge in a secure upright position for presentation, as shown in Diagram 4.

HM Armed Forces Veterans Lapel Badge

| Diagram 1 | Diagram 2 |
| --- | --- |
| Diagram 3 | Diagram 4 |

Packaging

- Commercial Packaging is to be used.

- The sleeved, boxed badges are to be packed snugly in boxes of 20 to ensure movement and possible damage is restricted.

- The boxes containing 20 sleeved, boxed, badges are to be packaged in larger boxes.

- All boxes should be robust to ensure that the contents are not damaged in transit.