STATEMENT OF REQUIREMENT

Introduction:

- The Sovereign Base Area (SBA) Police and Customs are seeking to develop their Technical Surveillance Capability to one that is more in line with UK Police Forces.

Single Statement of Need

- The SBA Police and Customs require a ‘Start-up Pack’ of a fully compatible equipment, with Subject Matter Expert (SME) support to facilitate the development of Technical Surveillance capability to allow identification, monitoring, and prosecution of illegal activities within the SBA.

Background

- In line with the UK Regulation of Investigatory Powers Act (RIPA) SBA Police regularly carry out surveillance either by means of operatives and / or technical means. During the last two years the use of technical means has been proven to be a valuable tool in the hands of the police. The surveillance equipment is used in a variety of criminality / operations such as offences related to Game & Wild Birds Ord, illegal immigration through SBA boundaries, Stealing etc. As a result, several successful operations were carried out resulting in arrest and convictions.

- The Operational Intelligence Unit (OIU) officers without any proper training have learned to deploy and camouflage surveillance equipment which was gradually purchased locally. In addition, a small basic workshop has been established where equipment is prepared before deployment.

Technical Evaluation

- Bidders are requested to provide a proposal, based on the recommendation report at Appendix 1 to SC1B - Annex A.  This is for a ‘Start-up Pack’ of fully compatible equipment, with Subject Matter Expert (SME) support to facilitate the development of Technical Surveillance capability to allow identification, monitoring, and prosecution of illegal activities within the SBA in line with UK Police Forces best practice.

Recommendation Report Summary of Surveillance Equipment/Methods/Tools/Training

- The Crime & Intelligence steering group and the SBA Police Senior Management Team  identified the need to expand the capabilities of the SBA Police in the areas of surveillance by providing the operatives the necessary tools.

- It was therefore arranged for two OIU officers (one from each Division) to visit the Technical Surveillance Unit at East Midlands, Derbyshire Police Technical support unit during 12-15/2/20. The aim of the visit was to see first-hand the equipment used there and what can be used here. Following the visit, the officers were tasked to submit a detailed report with recommendations of surveillance equipment/methods/tools/training that fits the needs of the SBA Police and the terrain we are operating within.

- During the two-day attachment the following areas of improvement where identified:

- Increase use of smaller cameras;

- Requirement for improved Video recorder capability;

- Better equipped workshop and equipped it with various tools;

- Battery packs that will last longer when deployed;

- Need for training in Technical surveillance equipment.

- The following key findings on requirement outputs were identified:

| Ref | Findings |
| --- | --- |
| 1 | A variety of surveillance cameras that collectively enable imagery to be obtained in low light conditions. |
| 2 | The ability to remotely operate those cameras with a pan tilt zoom (PTZ) capability (up to a x36 zoom) along with various lens options to enable filters to be utilised. |
| 3 | Ability to house cameras within ruggedized weatherproof casings. |
| 4 | All cameras must be digital HD specification with the ability to utilise memory cards and connect to a HD DVR recording device and the ability to connect to an external power supply. |
| 5 | Power sources (variety of alkaline and lithium rechargeable batteries) that enable cameras/ DVR’s to operate for extended periods (96 hours). Batteries must be able remain stable and retain charge at temperatures between 0 and 45 degrees centigrade. Batteries must be supplied with protective casings to allow them to be concealed beneath soil and debris for extended periods. |
| 6 | HD & SD compatible DVR’s that can record imagery from a number of cameras simultaneously, compress video onto SD cards. DVR’s to have the ability to be operated remotely and can facilitate live video streaming. |
| 7 | Workshop tools to maintain and adapt equipment as required. |
| 8 | Through Life Supply of Consumables such as Memory cards and spares. |
| 9 | Training Support on equipment uses and maintenance. |
| 10 | All equipment provided complies with all relevant current UK & EU legislation. |

- The following user requirement needs were identified:

| Serial | User Requirement | Recommendation |
| --- | --- | --- |
| General Requirements |  |  |
| 1 | All equipment shall have an ability to operate in 0°C to + 45°C ambient air temperature. | 1 – Essential. |
| 2 | All cameras and associated transmitting equipment shall be weatherproofed and ruggedized to withstand persistent rain for periods of up to 5 days. | 1 – Essential. |
| 3 | All equipment (less viewing monitors) shall use weatherproofed and ruggedized rechargeable batteries, to withstand persistent rain, that can provide sufficient power for a minimum of 96 hours and maximum of 120 hours. Batteries shall also be required to be buried or camouflaged by operatives for the duration of a task. | 96 hours – 1 – Essential 120 hours – 2 – Highly Desirable |
| 4 | All mobile equipment shall have suitable transit cases to be protect it whilst being transported between operations. | 2 – Highly Desirable |
| 5 | Batteries should be of a single-use type in order to improve longevity. Sufficient batteries for a 3-year period should be provided, with adequate shelf-life. | 1 – Essential. |
| 6 | All associated cabling shall be provided to connect interoperable equipment. | 1 – Essential. |
| Fixed No Zoom Camera Requirement |  |  |
| 7 | Qty 20 ‘fixed no zoom’ capability cameras. In addition to ‘General Requirements’ section; | 1 – Essential. |
| 8 | All ‘fixed no zoom’ cameras shall be able to record in high definition. | 1 – Essential. |
| 9 | Qty 18 of the cameras shall be capable of operating and capturing colour imagery at a minimum low-level light level of 0.006 lux or below. | Minimum of 0.006 lux - 1 – Essential |
| 10 | Qty 2 of the cameras shall be capable of operating and capturing black and white imagery at a minimum low-level a minimum of 0.00009 lux or below. | Minimum 0.00009 lux - 1 – Essential |
| 11 | Camera shall have the ability to operate for a minimum of 4 days unsupported (96 hours) and desirably 5 days (120 hours) | 1 – Essential. |
| 12 | The camera shall be adjustable and should the ability to provide imagery that can deliver facial recognition at a range of between 5 metres and 75 metres from the target area. | Distances have room for some negotiation. |
| 13 | Camera shall have the ability to connect to a separate HD DVR and modem. | 1 – Essential |
| 14 | Camera should have the capability to operate utilising a Passive Infra-Red (PIR) detector. | Highly Desirable. |
| Pan Tilt Zoom (PTZ) Camera Requirement |  |  |
| 15 | Qt 10 - Total PTZ capability requirement Pan Tilt Zoom (PTZ) camera. In addition to ‘General Requirements’ section; | 1 – Essential. |
| 16 | All PTZ cameras shall be able to record in high definition. | 1 – Essential. |
| 17 | PTZ cameras shall be remotely adjustable and controlled. | 1 – Essential. |
| 18 | Qty 5 of the PTZ camera shall be capable of operating and capturing black and white imagery at a minimum low-level light level of 0.006 lux | Minimum of 0.006 lux - 1 – Essential |
| 19 | Qty 5 of the PTZ camera shall be capable of operating and capturing black and white imagery at a minimum low-level light level of 0.00009 lux | Minimum of 0.00009 lux - 1 – Essential |
| 20 | PTZ cameras shall have the ability to connect to a separate HD DVR and modem. | 1 – Essential. Evaluation: Pass/Fail |
| 21 | PTZ camera shall have the ability to operate for a minimum of 4 days unsupported (96 hours) and desirably 5 days (120 hours) | 96 hours – essential. 5 days - desirable |
| 22 | The cameras shall be adjustable and should the ability to provide imagery that can deliver facial recognition at a range of between 5 metres and 100 metres from the target area. | Distances have room for some negotiation. |
| Miniature lens ultra-light camera |  |  |
| 23 | Qty 20 miniature lens cameras are required for use in confined spaces and hidden in objects if necessary. In addition to ‘General Requirements’ section; | 1 – Essential. |
| 24 | Camera should have the ability to operate for a minimum of 4 days unsupported (96 hours). | 1 – Essential. |
| Multi-format test monitors. |  |  |
| 25 | Qty 4 test monitors that shall be used to ensure cameras are setup and functioning correctly prior to extraction of the installing team. Different monitor size options should be provided. | 1 – Essential. |
| Mobile hard drive recorders (HDR) |  |  |
| 26 | Qty 20 ruggedized DC-powered DVR for recording footage during covert surveillance operations. In addition to ‘General Requirements’ section; | 1 – Essential. |
| 27 | HDR shall have at least one HD input for high definition imagery. | 1 – Essential. |
| 28 | Capability shall have an integral minimum 4G modem capable of operating with Cypriot-sourced Sim cards that can transit ‘live-feed’ imagery from fixed no zoom and PTZ camera systems. | 5G acceptable |
| 29 | Mobile HDR shall have the capability of an RF input to allow antenna to be remotely located. | 1 – Essential. |
| Drop down audio devices |  |  |
| 30 | Qty 4 ‘drop down’ audio devices that can be secreted within for example a cup, or rubbish bag. | 1 – Essential. |
| 31 | Drop down devices shall be capable of recording audio integrally and for a minimum of 60 mins. | 1 – Essential. |
| 32 | Drop down devices shall be weatherproof. | 1 – Essential. |
| Mounts |  |  |
| 33 | Qty 40 - A variety of different professional camera mounts shall be provided to accommodate the supplied cameras for covert and non-covert installations. | 1 – Essential. |
| Surveillance Analysis |  |  |
| 34 | Qty 4 High Definition Monitors shall be required to review ‘live-streamed’ footage and recovered media. Screens shall be a minimum of 27” and compatible with HDMI and SVGA | 1 – Essential. |
| 35 | Qty 2 computers with latest Windows operating system pre-installed. | 1 – Essential. |
| 36 | Viewing software / web-based service shall be installed / provided. | 1 – Essential. |
| 37 | Computers shall have the ability to connect to the internet. | 1 – Essential. |
| 38 | Qty 2 optical mouse and key board shall be required. | 1 – Essential. |
| 39 | Qty 10 portable hard drives with USB connection, that shall have a storage capacity of 1TB, for use in evidence preservation. | 1 – Essential. |
| 40 | Qty 10 portable hard drives with USB connection, that shall have a storage capacity of 2TB, for use in evidence preservation. | 1 – Essential. |
| 41 | Qty 10 portable hard drives with USB connection, that shall have a storage capacity of 4TB, for use in evidence preservation. | 1 – Essential. |
| Training |  |  |
| 42 | A ‘Train the trainer’ package shall be provided for 2 pax with classroom and practical scenarios to familiarise and train in the use and daily maintenance of all supplied equipment. | 1 – Essential. |
| Warranty |  |  |
| 43 | Equipment shall have 12-month manufacturer’s warranty. Options for a 24/36-month warranty for all electrical equipment shall be provided. | Desirable option for 24/36 month |