# Annex A

| Title of project: | Identifying the possible impacts of rock dump from oil and gas decommissioning on Annex I mobile sandbanks. |
| --- | --- |
| Date and time for return of tenders: | 26 October 2016 at 16:00 hours |
| Contract Reference No: | C16-0287-1046 |
| Address for tender submission: | 1 electronic copy to be sent to TenderResponse@jncc.gov.uk PLEASE DO NOT SEND TENDERS DIRECTLY TO LOUISE PELL-WALPOLE, BECKY HITCHIN, DORA IANTOSCA OR GORDON GREEN VIA THEIR PERSONAL EMAIL ADDRESSES, AS THIS WILL INVALIDATE YOUR TENDER Tender responses must be less than 10 MB in size. On receipt of your tender, you will receive an automated e-mail to confirm receipt by JNCC Support Co. If you do not receive this automated email, please contact, in the following order: Sue Wenlock (00 44 1733 866880) Chris Downes (00 44 1733 866877) |
| Contacts for technical information relating to this project specification: | Louise Pell-Walpole (Primary Lead) Joint Nature Conservation Committee Email: Louise.Pell-Walpole@jncc.gov.uk Tel: 01224 266576 OR Becky Hitchin (Co-Lead) Joint Nature Conservation Committee Email: becky.hitchin@jncc.gov.uk Tel: 01224 266560 |
| Contact for any queries regarding the tendering procedure: | Dora Iantosca or Gordon Green Finance Team Joint Nature Conservation Committee Email: Dora.Iantosca@jncc.gov.uk or Gordon.green@jncc.gov.uk Tel: 01733 866894 or 01733 866806 |
| Proposed start-date: | Week beginning 7 November 2016 |
| Proposed end-date: | 28 February 2017 |

Title of Project

Contents

1.	Joint Nature Conservation Committee	4

2.	Project Aims	4

3.	Project Background	4

4.	Project Objectives	8

5.	Project Objectives: Detailed tasks	8

6.	Outputs	9

7.	Dissemination	10

8.	Timescale	10

9.	Health and safety	10

10.	Product specification	11

11.	Project management	11

12.	Instructions for tender submission	11

13.	Evaluation Criteria (evaluation table below is a guide and can be adapted to suit individual project requirements)	13

14.	Payment	14

15.	Additional Contractor requirements	14

## Joint Nature Conservation Committee

The Joint Nature Conservation Committee (JNCC) is the statutory adviser to the UK Government and devolved administrations on UK and international nature conservation. Its work contributes to maintaining and enriching biological diversity, conserving geological features and sustaining natural systems.

Our role is to provide evidence, information and advice so that decisions are made that protect natural resources and systems. Our specific role is to work on nature conservation issues that affect the UK as a whole and internationally:

advising Government  on the development and implementation of policies for, or affecting, nature conservation in the UK and internationally;

providing advice and disseminate knowledge on nature conservation issues affecting the UK and internationally;

establishing common standards throughout the UK for nature conservation, including monitoring, research, and the analysis of results; and

commissioning or supporting research which it deems relevant to these functions.

## Project Aims

This project aims to evidence possible impacts associated with rock dumping during the decommissioning of oil and gas infrastructure on mobile Annex I sandbanks which are slightly covered by seawater all the time, using North Norfolk Sandbanks and Saturn Reef cSAC/SCI as a case study.

## Project Background

The next three decades will see a major decommissioning of UK oil and gas infrastructure with >8,000 platforms, pipelines and wells needing decommissioning around the UK. Within offshore Marine Protected Areas (MPAs), there are challenges and complexities beyond that of decommissioning activities out with MPAs as the potential impacts of human activities must be assessed against the conservation objectives established and advised by JNCC for the site. There are considerable scientific uncertainties around the impacts associated with the process of decommissioning, none more so than where decommissioning occurs on or near Annex I tidal current (mobile) sandbanks.

Industry, regulators and Statutory Nature Conservation Bodies (SNCBs) alike must comply with obligations set out in environmental legislation, and face individual sets of challenges in understanding potential impacts within MPAs. JNCC‘s role within national and European legislation is to establish and advise on conservation objectives for MPAs, and with the SNCBs, to provide advice on whether potential impacts associated with human activities may have an adverse effect on protected features or site integrity. Industry faces challenges regarding fulfilling their environmental obligations and regulators need to be able to introduce risk-based procedures for assessing effect in consenting operations. Within MPAs, competent authorities, having considered advice given from the appropriate statutory nature conservation body, must assess the proposed operations against the site’s conservation objectives.

Improving our understanding of how decommissioning operations can effect Annex I mobile sandbanks would support the provision of more robust advice to regulators and other interested parties, support site specific and national assessments of impacts and allow competent authorities to undertake a more robust decision-making process.

  - North Norfolk Sandbanks and Saturn Reef cSAC/ SCI

The Habitats Directive (EC 92/43/EEC) requires the designation of special areas of conservation to contribute to a coherent European ecological network. Sites that are part of this network host the natural habitat types listed in Annex I of the Habitats Directive.

### North Norfolk Sandbanks and Saturn Reef (NNSSR) Candidate Special Area of Conservation/ Site of Community Importance (cSAC/ SCI) is designated in its entirety as a representative functioning example of Annex I feature ‘Sandbanks which are slightly covered by sea water all the time’ and also contains Annex I ‘Reefs’ (Sabellaria spinulosa reef). The site contains the most extensive example of the offshore linear ridge sandbank type in UK waters including a series of ten main sandbanks and associated fragmented smaller banks formed as a result of tidal processes. Whilst there are ten discrete sandbank structures, the whole cSAC/SCI is designated and viewed as one integrated sandbank system. This site-wide integration has been further validated by recent biological community analysis, which suggests that biological communities associated with sandbanks occur across the site including areas deeper than 20m (JNCC report pending publication).

Sandbank areas within NNSSR were surveyed by JNCC/ Cefas in 2013 (http://jncc.defra.gov.uk/page-7140) and three EUNIS level 3 habitat types were identified; Sublittoral Sands, Sublittoral Mixed Sediments and Sublittoral Coarse Sediments. The majority of the biological communities present on the sandbanks were representative of the infralittoral mobile sand biotope. The series of sandbanks within the cSAC/ SCI were similar in terms of the biological communities present, fewer species were recorded on the inner banks and eastern most end of the outer banks. Increasing species numbers were recorded on the outer most banks, particularly on the Indefatigables Bank and the western-most end of the Swarte Bank, likely to be related to the change in hydrodynamic regime with increasing distance from the coast.

Recent unpublished biological community analysis within JNCC reviewed the 2013 survey data collected across the entire site and draft findings suggest that sediment type has the greatest influence on the composition of biological communities present rather than predicted topography (i.e. crest, flank, trough).

Areas of Sabellaria spinulosa reef are known within the site, though location and extent seems temporally variable. Whilst rock dump operations associated with oil and gas decommissioning are capable of affecting Sabellaria spinulosa reefs, this feature only concerns the proposed contract in as much as it forms part of the biological communities associated with the sandbanks.

Further information regarding the designation, protected features and the conservation objectives of NNSSR and activity-pressure relationships that may be relevant in understanding impact on the features and site can be found on the JNCC website: http://jncc.defra.gov.uk/page-6537 and http://jncc.defra.gov.uk/page-7136.

  - Conservation objectives for NNSSR

The conservation objectives are presented in the Regulation 18 advice package for this site which can be downloaded from the Site Information Centre (links above).

The current Conservation Objectives for North Norfolk Sandbanks and Saturn Reef SAC (sandbanks which are slightly covered by seawater all the time, and reef), are:

Subject to natural change, restorea the sandbanks which are slightly covered by seawater all the time and reefs to favourable condition, such that the:

- The natural environmental qualityb, natural environmental processesc and extentd are maintained

- The physical structuree, diversityf, community structureg and typical speciesh, representative of sandbanks which are slightly covered and reefs in the Southern North Sea are restored

a) Restore implies that the feature is likely to have been degraded to some degree. Even in the absence of direct evidence of damage or deterioration, where activities associated with pressures to which the feature is sensitive overlap the feature, they may need to be managed to reduce or eliminate potential negative impact(s). The first step for a restore objective may be to seek new information on the current condition of the site feature and/or more-detailed information on the potential overlap of activities with the feature(s) at the site. Restoration in the marine environment generally refers to natural recovery to favourable condition through the reduction or removal of adverse impacts.

Maintain implies that, based on our existing understanding, the feature is regarded as being in favourable condition and will, subject to natural change and to existing levels of usage, remain in this condition at designation.

JNCC considers that maintenance or restoration of the following parameters (b - h) will take account of the maintenance or restoration of natural structures and functions and ecological processes.

b) Natural environmental quality e.g. chemical quality parameters of water, suspended sediment levels, etc. should not deviate from baseline levels at designation (if data is available) or from reference conditions.

c) Natural environmental processes e.g. circulation, sediment deposition and erosion etc. should not deviate from baseline levels at designation (if data is available) or from reference conditions, subject to natural change.

d) Extent - the area covered by the habitat and communities.

e) Physical structure - the shape, form and composition of the habitat and its substrata.

f) Diversity - the number of different biological communities or number of species within a given community.

g) Community structure - e.g. age classes, sex ratios, distribution of species, abundance, biomass, reproductive capacity, recruitment, range and mobility.

h) Typical species – see Appendix III for draft criteria for identifying typical species.

  - Human Activities in NNSSR cSAC/SCI:

There is a considerable amount of human activity occurring within NNSSR cSAC/ SCI. Key activities include:

- Oil and gas – the Southern North Sea contains the UK’s largest gas reserves, and this area has been exploited for over 50 years. The fields are considered mature, and some are no longer producing. The site contains numerous pipelines, wells, surface and subsurface infrastructure, some of which are currently subject to decommissioning programs, and the majority of which will be decommissioned in the near future.

- Two areas are licensed for aggregate extraction, though activity is not thought to be underway at the current time.

- Dredge disposal sites – two dredge disposal sites are located within the cSAC/ SCI boundary, on the Ower and Leman Banks.

- There is evidence of mobile demersal, static and pelagic fishing effort within the cSAC/ SCI. UK and non-UK registered vessels have been active in the area. Work is ongoing to create areas of fisheries management within the site.

- Cables - three telecommunication cables currently cross through the cSAC/ SCI.

  - Oil and Gas Decommissioning within North Norfolk and Saturn Reef cSAC/SCI

Current oil and gas decommissioning projects within NNSSR cSAC/SCI involve the removal of small-scale gas platforms and their associated infrastructure. Comparative assessments conducted by industry have suggested that the platforms should be removed and associated pipelines should be decommissioned in situ. In a number of the decommissioning projects, considerable quantities of rock dump (sediments ranging from coarse sand to cobbles) are being deposited to provide protection to pipelines and to act as stabilising material for jack up rigs.

Rock dump can be a significant input into protected sediment habitats (including sandbanks) and at present proposed decommissioning works are resulting in larger post-decommissioning footprints than the infrastructure footprints during operation. In MPAs with a restore objective (such as NNSSR), these decommissioning operations may not contribute to the restoration of those features and sites to Favourable Condition, and could lead to declines in resource and the favourable conservation status of Annex I habitats at national level.

## Project Objectives

To meet the overall aim of this project (Section 2), the objectives are to:

To improve our understanding of the similarities and differences between typical sediment profiles, and sediment characteristics associated with oil and gas rock dumping operations and the natural sediment profiles, and sediment characteristics within North Norfolk and Saturn Reef cSAC/ SCI.

Improve our understanding of the potential physical impacts that rock dump associated with oil and gas decommissioning may have on mobile sandbanks and their biological communities, having taken into account their size and natural variability. North Norfolk and Saturn Reef cSAC/ SCI shall be used as a case study.

## Project Objectives: Detailed tasks

Primary Objectives:

Propose (where appropriate), and implement, a methodology investigating sediment characteristics (e.g. Particle size analysis and rugosity) used in rock dump operations on mobile sandbanks. Compare these results to the typical NNSSR sediment particle size, including the whole range of sediments that make up part of the feature (i.e. sands, mixed and coarse sediments). Identify any evidence gaps and propose further work to address these deficiencies.

Propose (where appropriate), and implement, a desk based methodology to investigate rock dump sediment profiles (I.e. their height and shape) used by the oil and gas industry for decommissioning operations on mobile sandbanks, both within the NNSSR cSAC/ SCI and within other marine protected areas that have Annex I sandbanks as a designating feature. Identify any evidence gaps and propose further work to address these data deficiencies.

Propose (where appropriate), and implement, a desktop based methodology comparing sediment profiles from rock dump operations to the typical NNSSR site sediment profile. Identify any evidence gaps and propose further work to address these data deficiencies.

An investigation as to how the introduction of rock dumping could affect the natural supporting processes, sandbank movement and the biological communities of the NNSSR cSAC/ SCI. Identify any evidence gaps and propose further work to address these limitations.

A desk based literature review investigating if previous rock dumps in mobile sandbanks have remained on the surface or have been buried (permanently or cyclically) including any available decommissioning monitoring reports. Identify any evidence gaps and propose further work to address these limitations.

Secondary Objectives:

- A discussion as to how the outputs of primary objectives 1-5 can be applicable to other features of conservation interest, and identification of areas of future work that could be undertaken to inform our advice on the impacts that rock dump associated with oil and gas and other applicable industries could have on NNSSR cSAC/ SCI, and other MPAs.

## Outputs

There is one key output required from this study:

- A technical report that will include all of the information and findings from the study, which will be published in the JNCC Report Series (http://jncc.defra.gov.uk/page-2132). We expect the length of this report to be appropriate considering the length of the project.

We expect at least one draft report to be submitted for consultation with the project steering group prior to the production of the final technical report.

The report should clearly set out the review methods used and the outcomes; clearly cite all evidence and information sources so that readers can easily refer back to original sources; and include technical annexes if appropriate. The report should also include a statement describing quality assurance procedures used in their development (e.g. peer review processes).  Project methods, management protocols and final outputs must comply with the JNCC Evidence Quality Assurance Policy, which can be found on the JNCC website: http://jncc.defra.gov.uk/default.aspx?page=6675.

The JNCC Project Officers reserve the right to be listed as co-authors on all materials produced in the project and JNCC and its Project Officers must be acknowledged in all materials (including published papers) and presentations resulting from the work.

## Dissemination

The report produced under this contract will be a JNCC product and shall not be published or disseminated without the written permission of JNCC. It may at some point be published on the JNCC website and all material supplied as part of this contract shall remain copyright of JNCC. The findings from this contract will also be made available to JNCC, the UK Conservation Agencies (Natural England, Natural Resources Wales, Scottish Natural Heritage, Department of Agriculture, Environment and Rural Affairs and Defra.

## Timescale

Provisional dates for delivery of the contact outputs are set out below. Exact dates are to be agreed at start-up meeting based on Contractor and JNCC staff availability.

| Milestones | Provisional dates |
| --- | --- |
| Start-up meeting Aberdeen (In person at JNCC Aberdeen if possible) | Week beginning 7 November 2016 |
| Progress meeting (by teleconference) | Week beginning 5 December 2016 |
| Progress meeting (by teleconference) | Week beginning 9 January 2017 |
| Progress meeting (by teleconference) | Week beginning 23 January 2017 |
| Progress meeting (by teleconference) | Week beginning 20 February 2017 |
| Draft report submitted to Project Officer | Week beginning 6 February 2017 |
| Internal peer-review of outputs to be managed by JNCC Project Officer | Week beginning 13 February 2017 |
| Final report supplied to JNCC for publication on JNCC website | 28 February 2017 |

In agreeing exact dates please note that the potential for work under this contract to be funded into 2016/17 is subject to availability of funds. Please note however that the potential for additional work to be undertaken is subject to a continuing need, availability of funds, and satisfactory contractor performance. For the avoidance of doubt, no guarantee can be given, that you will be asked to undertake the potential additional work outlined within this Annex A document. Tenderers are not asked to provide detailed project plans for these follow-on aspects at this stage.

## Health and safety

The Contractor is expected to follow appropriate Health & Safety procedures and undertake appropriate risk assessments, evidence of which should be supplied to JNCC. (NB under no circumstances should any work or service commence prior to the receipt of written approval of the risk assessment by JNCC H&S advisor)

Any incidents occurring within the contract should be immediately reported to JNCC.

## Product specification

The final report must adhere to the JNCC report template and house style guidance unless stated otherwise. The draft and final reports should be provided electronically via email both as a Microsoft Word document and an Adobe PDF.

Copies of documentation associated with the case studies should be provided in electronic format, with an associated reference catalogue.

## Project management

The Contractor shall nominate a project manager, who shall be responsible for ensuring the project is completed satisfactorily and who shall be the main contact point for JNCC.

JNCC’s main contact points will be:

Louise Pell-Walpole

Email: Louise.Pell-Walpole@jncc.gov.uk

Tel: +44 (0)1224 266576

Becky Hitchin

Email:  Becky.Hitchin@jncc.gov.uk

Tel: +44 (0)1224 366560

## Instructions for tender submission

The tender submission should include the following:

A brief summary of the potential Contractor’s experience in relation to the requirements of this contract;

A proposed approach for achieving the objectives of the contract (and delivering the detailed tasks identified within each objective). This should be sufficiently detailed allow assessment against the evaluation criteria (Section 14);

A detailed project plan (including Gantt chart), including the proposed work programme and an estimate of time required to achieve each objective;

A draft Table of Contents for the final report;

Details of Quality Control procedures to be followed;

Details of the Contractor’s own internal Quality Management System;

Details of the Project Team, including their roles and experience, an estimate of their time input into each task, and CVs of all personnel involved in the contract;

Availability of the Project Team for a start-up meeting in Aberdeen week beginning 7 November 2016.

Overall quote for the contract, to include:

  - Daily rates for all members of the Project Team;

  - Rates for attending start-up, interim and final meetings in Peterborough or Aberdeen (costs for travel and accommodation are attached and should be used. These rates are analogous to the civil service rates);

  - Costs and time allocation should be clearly allocated to specific tasks within this contract; and

  - VAT if applicable. The contractor is to specify whether VAT at the prevailing rate would be applicable to this project and to provide their company’s VAT registration number.

The following documentation:

  - Copies of health and safety policy statements where available or a note regarding such items as lone working, emergency procedures and accident reporting;

  - Copies of current public and employer liability insurance certificates; and

  - Copies of any appropriate risk assessments.

  - Copies of any environmental policies should you have them

In addition, note that the tender submission should provide sufficient information to allow assessment against the criteria outlined in Section 14.

## Evaluation Criteria

JNCC are not bound to accept the lowest priced or any tender.  Having the technical expertise and experience to complete the work to a high standard, and being able to complete it within the timescale, are of the essence for this contract.

Tenders will be evaluated using the following criteria:

|  | EVALUATION CRITERIA | Max Score | Score |
| --- | --- | --- | --- |
| 1. Quality of proposal, (50% of the total for the three assessment categories) |  |  |  |
|  | Clarity of proposal (particularly work plan and deliverables) | 10 |  |
|  | Understanding of, and relevance to, the requirements (in particular, the adequacy of outputs and understanding of particle size analysis, sources of industry information, physiological and biological processes on sandbanks | 10 |  |
|  | Soundness and logicality of methods | 7 |  |
|  | Realism and measurability of milestones | 6 |  |
|  | Identification and proposed solutions to potential problems/risks | 6 |  |
|  | Serious weaknesses which threaten success | 6 |  |
|  | Probability of success | 5 |  |
|  | Sub Totals | 50 |  |
| 2. Details of Contractor (25 % of the three assessment categories) |  |  |  |
|  | Expertise, experience and balance of team | 9 |  |
|  | Risks if important team members drop out | 7 |  |
|  | Adequacy of subcontractors (if any) | 9 |  |
|  | Sub Totals | 25 |  |
| 3. Cost (25% of the total for the three assessment categories) |  |  |  |
|  | Transparency and correctness of presentation | 6 |  |
|  | Fairness/reasonableness for the level of work and expertise required | 6 |  |
|  | Appropriateness of ratio of senior to junior staff time | 6 |  |
|  | Clarity of each team member’s contribution and value added | 7 |  |
|  | Sub Totals | 25 |  |
| Total score |  | 100 |  |

## Payment

Payment will be made on completion of the objectives, following the submission of an invoice(s); and based on satisfactory undertaking of the contractual elements to the agreed standard of the JNCC Project Officer. Stage payments against deliverables shall be discussed and agreed at the start up meeting.

## Additional Contractor requirements

All tenderers are requested to carefully read the Terms and Conditions applying to this contract. Payment will only be made upon delivery of key milestones.

It is assumed that all costs associated with the production of figures, reproduction of photographs and the final report are accounted for within the rates and fees given.

The Contractor is expected to supply all necessary equipment, software, licences etc. to carry out the obligations required under the contract.