PROPERTY

SERVICES

# Specification Document

## Internal Alterations &

Refurbishment to Offices, Toilets and Kitchen

## Ref: insert

Location

Jubilee House

Arnold, Nottingham

Property Services Section

Civic Centre, Arnold

Nottingham

NG5 6LU

May 2017

# CONTENTS

1	Preliminaries

2	Pre Construction Health & Safety Information

3	Specification of Works

4	Schedule of Works

5	Quotation Form

Appendix A – Form of Contract

# 1– PRELIMINARIES

1.1	GENERAL

1.1.1	The Contractor is to read and note the requirements within this section, to consider the financial implications of each item, and to include within the quotation for compliance.

1.2	CONTRACT CONDITIONS.

1.2.1	The form of contract is provided as Appendix A to this document

1.3	CLIENT/COUNCIL’S REPRESENTATIVE

1.3.1	The Client for the project is-

Gedling Borough Council

Civic Centre, Arnold,

Nottingham, NG5 6LU

1.3.2	The Refurbishments have been agreed and designed through the Council’s Property Services Section. From that Section the Property Services Manager shall be the Council’s Representative.

1.4	OVERVIEW/DESCRIPTION OF THE WORKS

1.4.1	The work comprises internal refurbishment and alterations to offices, corridors, toilets and kitchen within one of the Council’s main office accommodation buildings, Jubilee House, located on Nottingham Road, Arnold, Nottingham.

1.4.2	The work is restricted to the areas as detailed within this document. It is proposed that all other areas of the building and external areas will remain in use by Office staff and visitors to the building.

1.4.3	It is a requirement that a single Main Contractor (acting as Principal Contractor under the CDM Regulations) is contracted  to carry out the Works. The Main Contractor shall appoint his own Sub-Contractors accordingly.

1.4	ASSIGNMENTS

1.4.1	The Principal Contractor shall be prohibited from transferring or assigning directly or indirectly, to any person or persons whatever, any portion of this Contract without the written permission of the Council. Sub-letting of any part(s) of the Work, except to the extent permitted in writing by a Corporate Director for the Council, shall be prohibited.

1.5	ASSOCIATED WORKS

1.5.1	The following Works are in association with the main refurbishment Works and are to be carried out by the Council’s Specialist Contractors-

- Security alarm system

- Fire alarm system

- Door access systems

- Heating system

- Asbestos removal

The Council will obtain costs for these works separately and issue orders directly to the Specialist Contractors

1.5.2	The Contractor shall allow for attendance on all Specialist Contractors and shall also allow for incorporating the work of these Specialist Contractors in to the Works Programme.

1.5.3	The Council has nominated Goodbrooks Electrical Ltd to carry out the electrical work associated with these refurbishments.  The Contractor shall allow for attendance on the nominated contractors and shall also allow for incorporating the work of these nominated contractors in to the Works Programme

1.6	SPECIFICATION, SCHEDULE OF WORKS & PLAN DRAWINGS

1.6.1	The Specification, Schedule of Works and Plan drawings are intended to convey the extent and nature of the works required for quotation purposes. The descriptions do not itemise every piece of equipment, fixing, or method of fixing associated with the undertaking of the works. It is expected that the Contractor will use experience and knowledge to incorporate within the quotation for such things as timber sub frames and grounds, sealants, trunking, conduit, cable trays, elbows, bends, adaptors, supports, brackets, clips, valves, stop cocks etc, as it would be reasonable to predict from the description of the works.

1.6.2	The Contractor shall be expected to carry out the works in accordance with Statutory Regulations and ‘standard’ specifications as may be readily obtained from manufacturers.

1.7	HOURS OF WORK, TIMESCALES & PROGRAMME OF WORKS

1.7.1	It is proposed to start the Works in June 2017, and will take place over a maximum 7 week period.

1.7.2	During the Contract period, the building will be open as standard Monday to Friday 7.00am to 7.00pm and the Contractor may work any hours and shifts within these times.  The building will also be opened on a Saturday subject to prior arrangement. Saturday working is proposed for certain works (for example isolation of electrical circuits and removal of debris out of the building) to minimise disruption to the weekday activities within the building.

1.7.3	The Contractor will be required (in consultation with the Council’s Representative) to produce a realistic Programme of Works based upon the above dates and times and to enable sufficient time for pre-construction planning and for the works to be carried out in a safe manner within an occupied building.

1.8	CONTRACT MEETINGS

1.8.1	At least 1 week before the Start of Works on Site, the Contractor shall attend a Site Meeting with the Council’s Representative to confirm:

Site Arrangements

Programme of Works

The Construction Phase Plan & associated measures in place/in hand

First day induction arrangements

Any Specification queries

1.8.2	During the Construction Phase the Contractor shall attend Site meetings as requested by the Council’s Representative to discuss:

Progress against Programme

Variations

Any Specification queries

Health & Safety Reports

1.9	LABOUR AND PLANT

1.9.1	The Contractor shall not carry out or permit to be carried out on the site, any 	trade or work not forming part of this Contract. The Contractor shall provide, 	install and maintain all necessary plant and equipment for the proper and 	expeditious undertaking of the Works.

1.9.2	The Contractor shall ensure that operatives employed by him and his sub-contractors are adequately trained and competent for the work being undertaken.

1.9.3	Before Sub Contractors work on the site the Principal Contractor shall provide full details of the company including membership of trade bodies and health and safety policies as necessary. The Electrical sub-contractor shall be N.I.C.E.I.C, E.C.A, N.A.P.I.T registered (or similar approved body) to carry out the electrical work contained in the Contract works.

1.10	QUALITY STANDARDS

1.10.1	A representative of the Contractor shall be in attendance at all reasonable 	times to ensure quality standards and to liaise with the Contract Administrator 	for compliance to standards of workmanship and good construction practice.

1.10.2	The building is a major asset for the Council and as such the Council demands the very highest standards of management and quality of installation. It is essential that the Contractor has a proven track record in building refurbishment and alterations similar to this Project.

The Contractor shall comply with:

Current B.S Specifications and Codes of Practice

BSRIA Commissioning guides

EEC Legislation

Current Building Regulations

Water Supply Regulations 2010

Current Gas Safety Regulations

HSE ACOP L8 Legionella Disease

IEE 17th Edition Wiring Regulations

Manufacturer’s recommendations

1.11	SITE ORGANISATION AND TEMPORARY PROVISION

1.11.1	There is no manned reception at jubilee House.  The Contractor shall arrange a suitable system of signing in operatives and visitors on arrival to site and signing out on departure.  Access to security badges will be arranged at the start of the Contract and will be the responsibility of the Contractor for the duration of the Contract.  On completion of the works the Contractor shall hand back all security badges.

1.11.2	Access arrangements for the Contractor’s operatives and for removing waste and bringing in new materials shall be discussed with the Contractor during the pre-quotation visit, so that the Contractor may include for compliance within the quotation. It is proposed that the Contractor will provide temporary screens and/or barriers and signs to prevent unauthorised access into the working areas and to minimise the spread of dust etc.

1.11.3	The Contractor shall liaise closely with the Council’s Representative as regards delivery vehicles, in order to agree times and suitable locations and arrangements for the purposes of loading and unloading. Any delivery vehicles shall be positioned to avoid blocking access to Jubilee House for visitors, both in vehicles and on foot.

1.11.4	The Contractor shall be provided with 2 parking spaces within Jubilee House car park for the Contract Works. Any additional works vehicles shall not be parked in parking spaces directly outside the building as these are required for visitors during the day. Vehicles may be temporarily parked for off-loading small materials and tools etc., but then parked away from the building.

1.11.5	Jubilee House is used by East Midlands Ambulance Service (EMAS) as a rest stop for its operatives.  It is important that the Contractor ensures that the exit route from the EMAS parking bay at the end of Jubilee House to Nottingham Road is not blocked at any time.

1.11.6	The Contractor is responsible for assessing his own needs as to storage of materials on the site, which shall in all cases be in accordance with manufacturer’s recommendations.

1.11.7	The Contractor shall determine requirements for temporary provisions on the site during the construction period, although the Contractor shall be able to use the toilet and welfare facilities within Jubilee House.

1.11.8	On completion of the Works the Contractor shall remove all temporary provision and make good all surfaces disturbed.

1.12	SITE SECURITY

1.12.1	The Contractor shall be jointly responsible for the security of the Building in respect of maintaining security of the Working areas at all times. External doors to the building shall not be left open when the Site is unattended and any external compound/storage area and waste skip shall be fenced off and protected against unauthorised access and misuse.

1.13	PREVENTION OF NUISANCE

1.13.1	The works are to be undertaken and planned in such a manner as to cause as little inconvenience as possible to the staff and users of the building, resulting from smoke, dust, rubbish, noise, etc.

1.13.2	The Contractor shall remove debris from the site to prevent unsightliness and to maintain good housekeeping practices.

1.13.3	The Contractor is required to keep roads, car parks and footpaths in the vicinity of the building in a clear, unobstructed, safe and clean condition. Should any materials be deposited on the roads or footpaths in the car park area, it shall be removed immediately by the Contractor.

1.14	SITE RULES

1.14.1	On the first day of the Construction Phase Works the Contractor’s Site Representative shall be advised of the Site Rules in respect of-

- Site familiarisation, location of welfare facilities etc

- Emergency evacuation procedure

- Caretaking and security arrangements

- Vehicle access and parking arrangements

- No Smoking policy (strictly no smoking on Council property & land)

1.14.2	The Contractor’s Representative shall ensure that the Site Rules are conveyed to all Operatives on Site throughout the course of the Works Programme.

1.15	INSURANCE

1.15.1	The successful Contractor will be required to submit written confirmation that the relevant Employer and Public Liability Insurances are in place, to cover the period of the Contract Works and to provide adequate cover in relation to the works being undertaken

1.16	COMPLETION

1.16.1	On completion of the works the Contractor shall clear from the site all remaining rubbish, debris, unused materials, temporary equipment, plant etc.

1.16.2	The Contractor shall clean down all existing surfaces from dust and dirt resulting from the works, remove all protective wrappings and stickers from fittings and clean all new fixtures, fittings and surfaces.

1.16.3	The Electrical Contractor shall provide a certificate of completion for the electrical works.

1.16.4	The Contractors shall provide any relevant maintenance manuals, equipment specifications etc. for the completion of the Health and Safety file.

The Contractor will be expected to make good defects resulting from the works, up to 3 months after the agreed date of completion of the Works.

1.17	VARIATIONS

1.17.1	The Contractor shall bring to the attention of the Council’s Representative as soon as discovered during the carrying out of the works, any variations to the original Contract as implied by this document.  No additional expense should be incurred without the written consent of the Council’s Representative which will normally only be given for works incurring an extra cost which could not have reasonably been foreseen at the time of providing the quotation.

1.17.2	The Contractor is strongly advised to visit the site prior to submitting the quotation, as no additional costs can be agreed for the Contractor having failed to correctly interpret the works. The Contractor shall be responsible for checking all on site measurements and dimensions to determine quantities.

1.17.3	A contingency sum of £5,000 is to be included in the quotation to allow for unforeseen works or additional works agreed as work progresses.

1.18	FURTHER INFORMATION AND QUERIES

1.18.1	At any time prior to the date for receipt of quotations, the Contractor may contact the Building Services Section to arrange site visits and to answer any queries via-

Phil Goodwin

0115 9013782

phil.goodwin@gedling.gov.uk

# 2 – CDM REGULATIONS

# PRE-CONSTRUCTION HEALTH & SAFETY

INFORMATION

2.1	INTRODUCTION

2.1.1	The Contractor should be aware and understand the CDM Regulations 2015 and the associated HSE Guidance on the Regulations (L153) Managing Health and Safety in Construction and be prepared to accept the role and responsibilities of Principal Contractor under the Regulations.

2.1.2	The purpose of this section of the Specification Document is to provide information to Contractors on the Health and Safety and welfare matters to be taken in to account when pricing for and in due course planning for, the Site Works as detailed in the Specification, Schedule of Works and any associated plan drawings. The Principal Contract shall use and develop this information as part of the duties and responsibilities under the CDM Regulations.

2.1.3	This section should provide specific information obtained from both the Client and Designers during the course of the design and early planning phases of the Project, so that Health and Safety issues are identified and communicated effectively.

2.1.4	The absence of reference in this section to a hazard does not mean that such a hazard does not exist, or may arise. Any ‘Method of Working’ described in this section, as prepared by the Principal Designer, may be varied by the Principal Contractor as long as it represents an acceptable, good practice, alternative, agreed with the Principal Designer prior to Works, or that particular part of the Works, commencing.

2.2	DESCRIPTION OF THE PROJECT

2.2.1	Location

Jubilee House, Nottingham Road,

Arnold, Nottingham, NG5 6LU

2.2.2	The Works

Internal refurbishment and alterations to offices, corridors, toilets and kitchen at Jubilee House as described in more detail in this Specification Document.

2.2.3	Timescale

A start on Site is proposed for June 2017. It is anticipated that the works the work should take place over a maximum 8 week period of time.  A Programme of Works is to be agreed.

2.3	PROJECT DIRECTORY

2.3.1		Client

Gedling Borough Council

Civic Centre,

Arnold, Nottingham, NG5 6LU

2.3.2		Principal Designer

Mrs K Walters

Property Services Manager

Gedling Borough Council

Arnold, Nottingham

2.3.3		Designers

Building Services Section, Gedling Borough Council,

Arnold, Nottingham, NG5 6LU

2.3.4	Principal Contractor

To be confirmed

2.3.5	Area H.S.E Office

900 Pavilion Drive

Northampton Business Park

Northampton, NN4 7RG

2.3.6	Local Authority

Gedling Borough Council

Civic Centre,

Arnold, Nottingham, NG5 6LU

2.4	COMMUNICATION AND LIAISON WITH CLIENT, OTHER CONTRACTORS & NOMINATED CONTRACTORS

2.4.1	The Principal Contractor shall ensure co-operation between Contractors (including any of the Client’s Nominated Contractors) on Site for the purposes of Health and Safety.

2.4.2	Upon appointment the Principal Contractor shall be prepared to attend regular meetings (to be scheduled and agreed with the Principal Designer) and these meetings shall include Health and Safety matters to ensure information on Health and Safety is passed and enforced between all Contractors on Site and visiting personnel.

2.4.3	The Principal Contractor shall inform the Principal Designer if it becomes apparent that any proposals of any party in connection with the Works could present risk to the health or Safety of Workers undertaking the Construction, of Workers potentially undertaking maintenance work in the future and of visitors to and users of the building’s facilities, either during the Works or in the future.

2.4.4	For this specific Project the Principal Contractor shall note the intention of the Client to appoint a Nominated Contractor to undertake amendments to the electrical systems.

2.4.5	Should it be necessary, the Principal Contractor will need to incorporate the work by the Nominated Contractor in to the Programme of Works and the Construction Phase Health and Safe Plan.

2.5	PROJECT PROGRAMME

2.5.1	The Principal Contractor will be invited to prepare a realistic Programme of Works for the Project based upon the timeframes included in paragraph 2 of this Section of the Document.

2.5.2	In preparing the detailed Programme of Works the Principal Contractor must allow for the co-ordination and rationalisation of all Contractors (including the nominated Contractors) and trade activities to avoid the creation of uncontrollable hazards due to clashes of activities. Adequate provision must be made in the Contractor’s programme to allow for the adoption of safe working procedures.

2.5.3	The minimum period required from the Principal Contractors appointment (issue of a Works Order) to a start of Works on Site is proposed as 2 weeks. This is to allow for adequate pre-planning and preparation and ordering of materials.

2.6	PROJECT NOTIFICATION

2.6.1	The Project is not notifiable.

2.7	EXISTING ENVIRONMENT

2.7.1	General Site Description- Jubilee House is an office building used by Gedling Borough Council staff. It is open from 7.00 am to 7.00 pm. It is not intended to shut the building during the Construction period, but of course, the specific areas of work in each stage of the works shall no longer be in use during that part of the project.

The building is also used by East Midlands Ambulance Service (EMAS).  Jubilee House has a car park attached which also serves adjacent buildings which are used by the ‘Men in Shed’ charity and by Gedling Borough Council as a small vehicle workshop carrying out taxi inspections and equipment maintenance. Given the use of the buildings the Contractor can expect that there will be members of public from all age ranges either in the building, around the outside of the building and within the surrounding car parks.

Parking arrangements will be agreed with the Principal Designer as part of the Site Rules Induction.

In order to work in certain parts of the buildings whilst others are open to the public and staff and taking in to account that all external spaces are permanently accessible to the Public, the Principal Contractor shall agree in advance the sequence and method of work which includes

- to barrier or close off unauthorised access as necessary to the Working area and to any external storage compounds and waste skips,

- to use low use staff entrances and/or emergency exits as the means for operatives to access the working areas and to bring in, or remove materials, waste and equipment

- to ensure that H.G.V delivery vehicles are avoided on the minor access roads around the Park and that Contractors vehicles and Workers cars are safely parked and are driven carefully around the Park, respecting all speed restrictions.

Access to the Working Area- The proposed working areas within the building are within the staff only, office areas of the building. They can be accessed from the main reception via internal doors, but access through these doors is restricted by a card reader system to staff members only.

The working area can also be accessed via external staff entrance doors and emergency exit doors. These doors are proposed, where possible, as the intended route in and out of the Working area (except in emergency) for Site Workers, for removal of debris and for delivery of materials in to the Working Area.

Space at the front of the building will be made available for a site compound to include space for a waste skip.  Delivery vehicles (assuming not large HGV) can be directed to the most convenient access door for the purpose of unloading/loading materials

Fire Precautions- As a public building jubilee House has a fire alarm system and fire evacuation procedure. There is a Fire Risk Assessment for the building. As part of that assessment jubilee House operates the corporate permit to work system for works that represent an increased risk of fire such as hot works.

The individual work areas will not be in use during the refurbishment, however corridors and access routes outside the individual areas are designated emergency exit routes and therefore the Principal Contractor shall ensure that corridors are left free of debris and stored materials at all times.

Emergency evacuation procedures for the building will be provided by the Principal Designer as part of the Site Rules Induction.

Site Security- The working area is to be confined to the individual offices being refurbished at the time, other than when removed debris is being transported from the office space via the corridors and exit doors to the skip outside, or materials are being delivered and taken to the working area, again by the agreed route.  The corridors and other offices will be in use during the construction phase by members of staff and visitors, therefore the transport of debris and materials shall be strictly limited to times agreed in advance.

As the access doors all lead directly into a public park, Contractors shall ensure that when not in use they are kept locked at all times.

2.7.5	When preparing the Construction phase risk assessments the Principal Contractor shall note that the existing environment implies the following risks:

- Injury to members of the public and Council staff from collision with or access to works vehicles, delivery vehicles and construction materials around the perimeter of the Site.

- Unauthorised access to the Working Area resulting in injury to that person(s)

- Injury to Staff and invited guests from collision with objects/materials being taken to and from the working areas via the ‘shared’ sections of corridors and access routes.

- Fire starting within the Working area

- Workers and building users affected by fire if emergency procedures and Site Rules not adhered to

2.7.6	At the pre-tender/design stage the following measures have been taken or are proposed to eliminate or reduce and control the risks:

- It is proposed that the Principal Contractor shall arrange for as much delivery of materials as practical at times agreed with the Principal designer, when adjacent areas are not in use.

- Site workers, visitors and delivery companies shall be made aware of and comply with all on-site speed restrictions in the Park.

- Other than in an emergency the Principal Contractor shall direct all operatives and site visitors for access to the working area via the most convenient access door.

- The Principal Contractor’s representative shall instigate a system of signing in and out for all workers and visitors to the works area

- The Principal Contractor shall advise Site visitors and delivery vehicles in advance of the access arrangements to the Site and/or arrange for Site workers to direct delivery vehicles to the required access point.

- The Working Area shall be identified as out of bounds from access within the building, by ‘keep out-Construction works in progress’ signs on the outer doors of each area of work.

- Unauthorised access to the corridor area by the general public shall be prevented by keeping the internal security doors and the external doors closed at all times unless in use and/or attended.

- The Principal Contractor shall be wholly responsible for the security of the Working Area from unauthorised access and shall maintain a signing in book for Site Workers and Visitors

- The Principal Contractor shall note the positions of emergency exits from the building(s) around the vicinity of the works and ensure that egress from those exits are not obstructed by works vehicles, delivery vehicles, stored materials, plant and equipment and any temporary fencing/barriers etc.

- The Principal Contractor shall ensure that all routes required for emergency vehicle access to the building are maintained at all times.

- All workers on Site shall be made aware of the Site specific fire evacuation and fire alarm procedure through the Principal Contractor’s Site representative. On the first day on Site the Principal Designer shall provide the Principal Contractor’s Site representative with a Site induction to include the Emergency evacuation procedures.

- For any Hot Works the Client’s corporate Permit to Work arrangement shall be applied and strictly conformed with.

- The Principal Contractor shall provide and maintain suitable and sufficient fire- fighting equipment within the Working Area.

2.8	ASBESTOS

2.8.1	The Building is known to contain asbestos containing materials (ACMs) used within its original construction.

2.8.2	A survey has been carried out by a competent Contractor. The full survey is available for inspection and will be provided to the successful Contractor prior to works starting on Site. There is Asbestos confirmed but and some ACMs presumed-

- Cast iron waste pipes.

- Safe in Office 3

- Floor tiles.

- Redundant flue in Ladies Toilet ceiling space

2.8.3	When preparing the Construction phase risk assessments the Principal Contractor shall note that the construction of the building implies the following risks:

- Lung disease from inhaling air borne Asbestos fibres.

2 8.4	At the pre-tender/design stage the following measures are proposed or have been taken to eliminate or reduce and control the risks:

- An R&D Asbestos survey has been carried out by a competent person.

- Asbestos is known to be in other parts of the building and the Facilities Manager holds the Local Asbestos Management Plan & Register for the building.

- Should Workers require access for working outside the designated work areas as covered by the R&D survey, then this will only be permitted by prior arrangement with the Principal Designer. If the worker is to access an area identified as containing ACMs, then the Client’s corporate Permit to Work arrangement shall be applied and strictly conformed with.

- All workers on Site must have received Asbestos awareness training prior to working on Site.

- The Contractors on Site are requested to isolate electric, water and waste services to the Water Bunny, but the unit itself will be removed from the wall and disposed of from Site by others.

- The specified works do not imply that any other identified areas as presumed to contain ACMs will be disturbed as a result of those Works.

2.9	EXISTING SERVICES

2.9.1	Prior to Demolition and Removal works commencing on Site, the Contractor shall arrange for the Mechanical & Electrical Contractor(s) to carry out an investigation of the existing M&E systems design, particularly with regard to electrical circuits.

The purpose of the electrical investigation is to identify the means for safely isolating circuits to enable work to proceed and to identify any circuits shared with areas outside of the working area. In such cases the Electrical Contractor will liaise with the Principal Designer for a suitable time to separate the shared circuit(s), allowing occupied areas to continue to function whilst circuits are isolated to the working area.

2.9.2	There is natural light and natural ventilation in the working area. Isolation of circuits will remove the existing artificial lighting.  The Contractor shall determine and allow for adequate lighting and ventilation as necessary.

2.9.3	Electrical circuits within the Working area are run from a local distribution boards positioned around the building. The distribution boards are not within secure enclosures, or locked cupboards. As such the boards are outside of the working area and outside of the direct control of the Electrical Contractor.

2.9.4	When preparing the Construction phase risk assessments the Contractor shall note that the arrangement and information available of the existing building services implies the following risks:

- Electrocution from live electrical circuits

- Lose of lighting and essential services to other parts of the building, occupied and in use by the public

- Loss of emergency lights to aid an emergency evacuation

- Lack of adequate light levels in the working area

2.9.5	At the pre-tender/design stage the following measures are proposed or have been taken to eliminate or reduce and control the risks:

- An electrical system investigation to be carried out by a competent person(s) prior to any alteration and refurbishment works being carried out.

- All relevant electrical equipment and circuit cabling to be safely isolated prior to removal and demolition of adjacent structures.

- The Principal Contractor shall ensure an appropriate communications system is in place between the site Workers and electrical Contractor to ensure that any live circuits and equipment is identified and prevented from unauthorised access and/or disturbance.

- The Electrical Contractor shall allow to provide temporary low voltage lighting as necessary

- The Distribution boards are not in locked cupboards. The Electrical Contractor shall devise a means for preventing isolated circuits being inadvertently switched back on.  These circuits shall be suitably locked off or safely disconnected. The Client operates a corporate Permit to Work arrangement for electrical work which shall be applied and strictly conformed with.

- The Electrical Contractor shall mark-up existing and new circuits as necessary during the undertaking of the works, to enable ease of identification of circuits for isolation and reference purposes.

2.10	COSHH

2.10.1	When preparing the Construction phase risk assessments the Principal Contractor shall note that the use of products and materials, whether specified or not, implies the following significant risks from the potential use of hazardous substances:

- Respiratory problems from inhalation or swallowing

- Eye irritation or damage from contact

- Skin irritation or damage from contact

- Fire from flammable materials

2.10.2	At the pre-tender/design stage the following measures are proposed or have been taken to eliminate or reduce and control the risks:

- For all substances and materials to be used on Site, the Principal Contractor shall obtain the manufacturer’s product data sheets and work within the guidance given.

- Low VOC level gloss paint is specified for internal use

- Mechanical and natural ventilation to be utilised as much as practically possible.

- For unspecified materials and products the Principal Contractor should consider those that eliminate the use of hazardous substances.

- For specified materials and products the Principal Contractor may propose alternatives that eliminate or reduce the use of and exposure to hazardous substances.

2.11	SITE SPECIFIC HAZARDS- DEMOLITION

2.11.1	When preparing the Construction phase risk assessments the Principal Contractor shall note that the Demolition Work as specified implies the following significant risks:

- Noise from compressors, percussion tools and pneumatic breakers used during demolition and removal.

- Dust from demolition of concrete block and plaster partitions, taking down of suspended ceiling tiles, grid and light fittings.

- Injury from ‘flying’ debris during demolition and removal.

- Electrocution.

- Injury from premature collapsing structures.

2.11.2	At the pre-tender/design stage the following measures are proposed or have been taken to eliminate or reduce and control the risks:

- Noise generating tools and equipment to be fitted with effective silencers as recommended by the equipment manufacturer.

- Noise producing activities to be programmed in to convenient time slots and to be limited in duration to minimise disturbance to the day to day operations within the Centre

- Mechanical and natural ventilation to be utilised as much as practically possible to help clear dust.

- Surfaces to be wetted to supress excessive dust levels

- Workers engaged in demolition and removal work to wear full P.P.E

- Access to the working areas during demolition and removal works to be carefully controlled and limited to workers wearing full P.P.E

- Demolition and removal to be done ‘carefully’ to minimise ‘flying’ debris. Eye and skin protection must be worn at all times during this activity.

- A suitable level platform to be provided for access to taking down ceilings

- All relevant electrical equipment and circuit cabling to be safely isolated prior to removal and demolition of adjacent structures

- The Principal Contractor shall ensure an appropriate communications system is in place between the site Workers and electrical Contractor to ensure that any live circuits and equipment is identified and prevented from unauthorised access and/or disturbance.

- Existing partition walls and suspended ceiling systems shall be taken down in manageable sections, systematically and logically to prevent uncontrolled collapse.

2.12	SITE SPECIFIC HAZARDS- HOT WORKS

2.12.1	When preparing the Construction phase risk assessments the Contractor shall note that the Plumbing alteration Work as specified implies the following significant risks:

- Fire resulting from hot works

2.12.2 At the pre-tender/design stage the following measures are proposed or have been taken to eliminate or reduce and control the risks:

- For any Hot Works the Client’s corporate Permit to Work arrangement shall be applied and strictly conformed with.

- All workers on Site shall be made aware of the Site specific fire evacuation and fire alarm procedure through the Principal Contractor’s Site representative. On the first day on Site the Principle Designer’s representative shall provide the Contractor’s Site representative with a Site induction to include the Emergency evacuation procedures.

- The Contractor shall provide and maintain suitable and sufficient fire- fighting equipment within the Working Area.

2.12 	SITE SPECIFIC HAZARDS- WORKING AT HEIGHTS

2.12.1	When preparing the Construction phase risk assessments the Principal Contractor shall note that the Alteration Work as specified implies the following significant risks from working at height:

- Fall whilst installing the new ceiling level and new services in the ceiling void

- Preventing unauthorised access to heights

2.12.2 At the pre-tender/design stage the following measures are proposed or have been taken to eliminate or reduce and control the risks:

- Access equipment should not be left unattended in public areas and should not be left in position out of working hours.

- All access equipment used on Site shall be suitably checked for damage before use.

- All work at ceiling height and above, inside the building, shall be carried out using adequately sized working platforms on low level scaffold frameworks or mobile towers.

2.13	WELFARE

2.13.1	The Principal Contractor shall provide welfare facilities in the Working Area, as laid down in the CDM Regulations 2015, Schedule 2 from the beginning of the Project. Note that Site operatives are invited to use the existing toilet  and kitchen facilities within the Civic Centre. The Principal Contractor shall confirm the details and locations of welfare arrangements within the Construction Phase Plan.

2.14	CONSTRUCTION PHASE

2.14.1	The Principal Contractor shall review and develop the Pre-Construction Information in in this section of the Document in formulating Risk Assessments and Method Statements for work to produce the Construction Phase Health and Safety Plan as required by the Regulations.

2.14.2	Risk Assessments and Method Statements may be reviewed, developed and modified during the Construction Phase to include and significant changes to the design or scope of the Works as specified and scheduled in this Document.

2.14.3	It is expected that future liaison over the Construction Phase Health & Safety Plan will take place at a pre-contract meeting and at site progress meetings as and when arranged.

2.15	HEALTH AND SAFETY FILE

2.15.1	The Contractor is to provide all relevant information on the new installations as may be required for the Health and Safety file, including any safe systems of working for future use and maintenance of equipment.

2.15.2	It is expected that the Health and Safety File will contain:

- A brief description of the Works carried out

- Any hazards not eliminated through the design and construction process

- All hazardous substances used during the Construction process

- Information regarding the removal or dismantling of plant and equipment installed

- Health and Safety information regarding the on-going maintenance and cleaning of plant and equipment installed

- As built drawings including location of services and circuit labelling details

- Electrical certificates and as installed wiring diagrams

2.16	CONTINUING LIAISON

2.16.1	The Contractor will be required to develop the Plan and submit method statements for site operations in accordance with the requirements of Health and Safety legislation.

It is expected that future liaison over the Safety Plan will take place at a pre-contract meeting and at site progress meetings as and when arranged.

The Contractor is to provide all relevant information on the new installations as may be required for the Health and Safety file, including any safe systems of working for future use and maintenance of equipment.

# 3 – SPECIFICATION OF WORKS

3.1	GENERAL

3.1.1	For the purposes of the Specification, where the term Contractor is used this is intended to be the Principal Contractor and/or the Principal Contractor’s sub-contractors, carrying out the particular work item.

3.1.2	The term Electrical Contractor refers to the Council’s nominated Electrical Contractor, Goodbrooks Electrical Ltd. All electrical work and communications system work is to be carried out by the nominated Electrical Contractor working in conjunction with and as a Sub Contractor to the Principal Contractor.

3.2	DEMOLITION & REMOVALS- GENERAL

3.2.1	The Contractor shall take down or remove all internal structures and fittings as detailed in the Schedule of Works and indicated by the relevant plan drawings.

3.2.2	Prior to taking down sections of the existing structure, the Contractor shall confirm the removal and /or isolation of all associated electrical switches, sockets, cable conduit and equipment to ensure they are made safe. This work is to be carried out by the Electrical Contractor. The Electrical Contractor shall also take down redundant light fittings and if necessary remove obsolete cabling and/or temporarily make safe cable connections, pending the installation of new fittings within the suspended ceiling voids.

3.2.3	Prior to taking out sections of pipework and radiators to the existing heating system, the Contractor shall confirm the isolation and draining down of the relevant parts of the system. This work is to be carried out by the Specialist Heating Contractor. The Specialist Heating Contractor shall remove all redundant sections of pipework to prevent dead-legs and shall make good pipework connections to enable the remaining sections of the heating system to operate as normally. Temporary capping off of pipework should not solely rely upon existing valves to be closed off overnight.

3.2.4	Prior to taking out the existing sanitary-ware and kitchen fittings, the Contractor shall confirm the removal and/or isolation of all associated water and waste connections. The Plumbing Contractor shall ensure that any redundant sections of pipework are completely removed to avoid dead-legs of pipework being left within the system.

3.2.5	Prior to the demolition and/or taking down of structures, wall and floor coverings, fixtures and fittings, the Contractor shall assess the existing condition and method of construction This assessment will be of sufficient detail to identify structural problems, and identify risks associated with flammable substances, or substances hazardous to health. Note also the refurbishment and demolition Asbestos survey results, which indicates the presence of ACMs in some areas of the works. The method of demolition and taking down should then be chosen which should gradually reduce the height or size of the structure or component, or arrange for parts to be completed in a controlled collapse so that work can be completed at ground level.

3.2.6	All redundant materials taken out are to be immediately removed from the site to a waste skip. The Contractor to estimate for and include the cost of all skips required to remove rubbish from the site.

3.3	DEMOLITION & REMOVALS- EXISTING EQUIPMENT & MATERIALS

3.3.1	This information on existing equipment and materials is intended to assist the 	Contractor in determining the implications and costs associated with 	demolition and removals work as detailed in the Schedule of Works.

3.3.2	As detailed in the Schedule of Works and shown on the relevant plan 	drawings, the Contractor shall take down sections of internal partition walls. 	The Contractor is to assume that all walls, whether masonry or metal and 	timber and stud partition walls to be demolished are non-load bearing. The 	Contractor is to assume that all door openings formed within masonry walls 	will require suitable sized lintel support and shall allow for such in the cost of 	the works.

3.3.3	As detailed in the Schedule of Works, the Contractor shall take up and 	remove existing carpet and any associated fixing residue from the sub floor. 	The office and corridor floor coverings are a mix of carpet tiles (500mm x 	500mm, 8mm thick carpet tiles) which have been laid less than 5 years ago 	and the Contractor may assume that there is minimal sub floor repairs and/or 	preparation prior to laying new coverings, and broadloom carpet. This carpet 	is at least 10 years old and some sections may be original to the building 	construction (30 years old). In such instances the Contractor may assume that 	the sub floor will require repairs and/or preparation prior to laying new floor 	coverings.

3.3.4	The Contractor shall take down and dispose of all shelving, notice board and 	blinds 	as listed in the Schedule of Works and shown on the relevant plans. 	The Contractor shall allow within the quotation for general making good of 	plastered wall and ceiling surfaces following removal of partitions, masonry, 	fixtures and fittings. Therefore making good holes in walls from removal of 	screw fixings and plaster patching of isolated areas up to 0.05sqm is deemed 	to be included within the cost of the Works.

3.3.4	In rooms as indicated in the Schedule of Works the existing textured wall 	paper is to be removed.  The Contractor shall allow within the quotation for 	general making good of walls following removal of the paper. Making good 	holes in walls after removal of textured wall paper in isolated areas up to 	0.05sqm is deemed to be included within the cost of the Works.

3.3.5	The Contractor shall allow within the quotation for minor adaptations and 	making good of timber skirtings as a result of removal works.

3.3.6	For areas of re-plastering that may arise, over and above the quantities 	indicated above, the Contractor shall allow a Provisional Sum of £500 within 	the Quotation, which will also include for making good surface decoration.

3.3.7	As indicated in the Schedule of Works the Contractor shall allow to take down 	existing suspended ceilings.  The Contractor shall allow to take down all tiles, 	framework and timber edgings.

3.3.8	The Contractor shall allow to take down and dispose of internal doors as 	detailed in the Schedule of Works.  Where new doors are to be hung in the 	existing frames, the Contractor shall make an allowance for any minor 	alterations to the frame as is required.

3.3.8	The Contractor shall allow for the removal of sanitary fittings and kitchen 	fittings as indicated in the Schedule of Works.  The Contractor shall allow 	within the quotation for all works to isolate fittings as is necessary.

3.4	NEW BUILDING & JOINERY WORKS

3.4.1	All new non loadbearing partitions shall be constructed in Gypwall Classic System, i.e 48mm gypframe ‘C’ studs at 600mm centres with two layers of 12.5mm plasterboard each side, plaster skim finished ready to receive decoration. Cavity between studs to be infilled with 50mm insulating blanket material to improve sound reduction quality.

3.4.2	New partitions are to be built up to the underside of the concrete floor slab 	soffit to enable the suspended ceiling to be finished flush to the new partition 	both sides.  In Office 5 the suspended ceiling grid is to be cut and reformed to 	suit with edge trim running both sides of the new false beam. The existing 	ceiling tiles are to be carefully removed and cut to size for the new grid layout 	and refitted on completion.

3.4.3	As detailed in the Schedule of Works and shown on the relevant plan 	drawings, the Contractor shall form a suitably sized opening for a door in an 	existing internal masonry wall to the new Male Locker Room.  It can be 	assumed that this wall is load bearing and so lintel support will be required. 	The Contractor shall make good plaster finishing around the openings and 	adapt existing skirting boards to suit on completion.

3.4.4	As detailed in the Schedule of Works and shown on the relevant plan 	drawings, the Contractor shall form a suitably sized opening for a door in the 	new partition wall for the Female Locker Room.

3.4.5	New partitions shall be formed around existing service trunking/compartment trunking as necessary and all joints sealed with a suitable sealant.

3.4.6	For all new partitions and for infill sections, the Contractor shall supply and fix 	new skirting boards to both sides. New skirting to match existing profile.

3.4.7	New suspended ceilings are to be provided to all areas shown on the relevant plan drawing as replacement for the original suspended ceilings to be removed. The new ceiling system to be 600mm x 600mm lay-in grid type, with white profiles and a black shadow batten to the perimeter. New tiles to be Dune Supreme board edge type, as manufactured by Armstrong.

3.4.8	New internal doors and frames, where required, shall have a minimum 30 minutes fire resistance and clear opening minimum width of 800mm. Doors  shall have two glazed vision panels to NG9 pattern (for example as referenced on Noberne Doors datasheet), except for doors D21, D25, D26, D30 and D32 which are to be solid doors. Doors to have North American Maple Veneer finish to match existing doors. The door frame to be timber rebated with 45mm x 16mm profiled architrave both sides to match existing.

3.4.9	New doors to be provided with nylon coated furniture (colour to match existing 	doors) as supplied by Hoppe. Doors to have lever handle set Paris Series 	K138 product ref AR600/13 unless otherwise specified.  All doors to have 900mm x 200mm kick plates both sides product ref AR625 and rack and pinion door closer product ref AR1000.

3.4.10	The Contractor shall supply and fit W.C Cubicles and vanity unit from the Quadro range as manufactured by Bushboard Washroom Systems Ltd. The toilets fitted will be fitted in the same positions as the existing fittings and so the Contractor is to allow for toilet cubicles to match the positions of the existing as far as possible. Fit the manufacturer’s standard stainless steel coat hooks to the back of each cubicle door. Within the Female toilets supply and fit HPL vanity unit comprising counter top with upstand (CTU) 555mm bed with 100mm downstand and 75mm upstand, 1500mm long with left hand blanked off and under vanity panels with post formed edges. The Contractor to provide the frameworks to support the vanity units.

3.4.11	Kitchen base units, cupboards, associated lengths of worktop and white goods to be supplied and fitted within the Kitchen.  Kitchen units are from the Greenwich Ivory range and worktop from the 38mm square edged laminates range as manufactured by Howden Joinery.  The kitchen layout is still to be determined and the Contractor shall allow a Provisional Sum of £4000 for the kitchen fittings.

3.4.12	For pricing purposes the Contractor can assume that the kitchen will comprise of the following -

2 x 800 wall units

1 x sink unit

4 x base units

1 x larder cupboard unit

10m x worktop

3.4.13	The Contractor shall allow for new wall unit in LadiesToilet, in same range as kitchen units

3.4.14	The Contractor shall allow a Provisional Sum of £1000 to refit shelving, notice boards, worktops and fitted storage cupboards, adapting existing fittings to suit the space available and to fit around existing pipework and skirtings as necessary.

3.4.15	The Contractor shall allow within the quotation for minor adaptations and making good of existing timber skirtings as a result of new works.

3.5	PLUMBING WORK

3.5.1	All plumbing works are to comply with the requirements of the Water Supply (water fittings) Regulations.

3.5.2	The Plumbing Contractor shall supply and fix all specified fittings, fixings and associated waste and water supply pipes.

3.5.3	Generally water distribution pipework shall be in copper. The Plumbing Contractor shall determine the appropriate diameter pipes for supply purposes

3.5.4	The water distribution system shall include all stop cocks and isolating valves. All pipework shall be adequately clipped to prevent sagging and all pipes within the ceiling void shall be insulated with 38mm diameter tubular insulation material.

3.5.5	The existing cold water mains supply shall be amended and extended to

supply:

Sink unit to kitchen

Worktop mounted water boiler in kitchen

Worktop mounted water cooler in kitchen

Wash hand basin to Male toilet

WC to Male toilet

Urinal to Male toilet

2 x WC to Female toilet

2 x wash hand basins to Female toilet

3.5.6	The existing hot water service system shall be amended and extended as necessary to supply:

Sink unit to kitchen

Wash hand basin to Male toilet

2 x wash hand basins to Female toilet

3.5.7	WC pans to be close coupled WC cisterns, pans & seat set Option (Twyfords Bathrooms)

Urinal bowls to be Clifton with exposed flush (Twyfords Bathrooms)

Urinal cistern to be plastic auto cistern 9L

Wash basin to male toilet to be wall hung wash basin 450mm x 340mm Galerie 2 taps (Twyfords)

Wash basin to female toilet to be semi recessed wash basins 500mm x 425mm Galerie 2 taps (Twyfords)

3.5.8	Exposed waste pipes shall be in white plastic. Suitably sized waste pipes from the sanitary ware to be run to minimise lengths and to discharge into existing soil and vent pipes

3.5.9	Upon completion of the works, but before pipework is concealed with insulation, the whole system shall be sterilised, filled and tested for correct functioning and the avoidance of leaks.

3.5.10	Prior to hand over of the Works, all sanitary ware shall be thoroughly cleaned to remove building debris and manufacturers marks and tapes and left ready for use.

3.5.11	The Plumbing Contractor shall provide a copy of all operating and maintenance manuals in relation to the equipment installed and shall demonstrate the operation of the system as necessary.

3.6	ELECTRICAL WORKS

3.6.1	For the purposes of the quotation the work to be undertaken by the Electrical Contractor shall be covered by a Provisional Sum of £10000. The Contractor shall include within the quotation for all costs/overheads and profit associated with attendance on the Nominated Contractor and for incorporating the electrical work within the Construction Phase Health and Safety plan and arrangements.

3.6.2	The work of the Electrical Contractor is in association with the alteration works 	and is indicated by the relevant plan drawings. In general terms the Electrical 	Works covered by the Provisional Sum are:

- Disconnect/isolate all light fittings and electrical equipment from the existing electrical supplies to enable removal as required and to allow other demolition and removal works to take place.

- Provide any temporary lighting and electrical supplies that may be required by the Contractor in order to undertake the works.

- Relocate and/or alter light switching arrangements to suit revised office layouts

- Provide and fix new light fittings where indicated on the plan drawings

- Remove and relocate existing power and data outlets, including to amend skirting compartment trunking as required.

- Provide and fix new power and data socket outlets where indicated on the plan drawings

- Provide power supplies to new equipment where required as indicated on the plans

3.6.3	On completion of the works the Electrical Contractor shall provide:

A Test Certificate for all new work.

3.7	FINISHES

For decoration of new and existing surfaces the Schedule of works will list the 	elements of the room to be decorated. The Contractor shall prepare and 	decorate surfaces as follows:

- All ceiling areas and wall surfaces in emulsion.

- All timber door frames and architrave surrounds to both sides (gloss)

- All timber cills to window frames (gloss)

- All timber skirtings to walls (gloss)

- All timber and metal sections to skirting height service trunking (gloss)

- All exposed flow and return metal pipework to radiators (gloss)

For re-decoration of all new and existing gloss surfaces, the Contractor shall allow for one full coat of Aqua water based undercoat and one full coat of Aqua water based Acrylic Satin finish, both as manufactured by Johnstones Paints.

For re-decoration of all existing emulsion surfaces, the Contractor shall allow for two full coats of matt emulsion to cover.

For re-decoration of all new emulsion surfaces, the Contractor shall allow for  an initial  mist coat and then two full coats of matt emulsion to cover.

Where areas of broadloom carpet are being replaced and as detailed in the Schedule of works, prior to laying new tiles carpet, the sub floor shall be scraped clean of all previous carpet backing residue and glue, any loose floor screed to be removed and the whole area prepared with a suitable self-levelling screed compound or similar proprietary product.

Where areas of carpet tile are being replaced and as detailed in the Schedule of Works, any significant holes and/or indentations or excessive changes in level shall be filled and reduced using a suitable patching screed or similar proprietary product.

All new carpet covering and carpet patching, unless detailed otherwise in the Schedule of Works is to be carried out using 500mm x 500mm x 8mm thick carpet tiles as manufactured by Desso, in colour reference A147 7290. The carpet to the Entrance Lobby shall be Heckmondwike Battleship Entrance Matting, in colour to be decided.  New carpet to be secured and laid using fixing glues as recommended by the carpet manufacturer.

Altro Walkway 20 sheet vinyl safety flooring shall be laid, complete with the manufacturer’s skirting up-stand to perimeter walls in the toilets and kitchen.  All floors are to be suitably levelled and prepared before the floor covering is laid.

The Contractor shall supply and fit new ceramic wall tiles to sections of wall within the toilet areas. The choice of wall tile is to be determined, but for pricing purposes the Contractor shall assume that the tiles will be 600mm x 300mm x 11mm.  The Contractor shall price to supply and fix the new ceramic tiles on the basis of a tile cost of £30/sqm. The Contractor shall allow to tile all walls in the Gents Toilet to full height, approximate area 30 sqm.  The Contractor shall allow to tile above the vanity unit in the Ladies Toilet to ceiling height, approximate area 3.5 sqm.

A tiled splashback is to be formed along the full length of worktop in the new Staff Kitchen. The splashback to extend to a height to align with the underside of the wall unit, which is to be fixed at standard height. Tiles to be ceramic 150mm x150mm in a gloss white.  The Contractor shall allow for a maximum area of tiling to the kitchen splashback of 3.5 sqm

The Contractor shall fix the tiles with an adhesive suitable for the application and in accordance with the manufacturer’s recommendations.  Likewise the Contractor shall grout between the tile joints with a suitable product.  Any external corners shall be finished with a proprietary stop bead.

Mirrors to be in impact resistant Acrylic finish, with square edges and concealed fixings. A 1500mm x 750mm and 1000 x 500mm mirror are to be fixed above the bank of wash basins in the Female and Male toilets respectively

The Contractor shall allow a Provisional Sum of £500 to supply and install miscellaneous signage such as, fire notices, door signs and general internal signage, etc.

## 4– SCHEDULE OF WORKS

4.1	OFFICE 1

4.1.1	Remove and dispose of -

All shelving

4 x notice boards

Redundant water supply pipework

Internal door (D1)

All broadloom carpet

Window blinds

4.1.2	Supply and fit new door (D16) to existing frame.  Door D16 to have door access controls fitted (by others)

4.1.3	Replace architrave to remaining notice board.

4.1.4	Make good hole through external wall.

4.1.5	Allow to replace 6 x ceiling tiles in existing suspended ceiling.

4.1.6   Lay new carpet floor covering.

4.1.7	Decorate all surfaces within this Section of the Works.

4.1.8	Electrics -

Remove all data points and cabling, including all trunking

Remove 8 x lay in grid light fittings

Supply and fit 8 x 600mm x 600mm LED light fittings, suitable for office 	environment (2 x emergency)

Supply and fit new power supplies for new door access controls and for 	comms unit in positions as show on plan number G17/JH/PA17/002

Supply and fit 14 x data points in positions as shown on plan number  G17/JH/PA17/002 with associated cabling run in surface mounted trunking to new comms unit

Supply and fit 1 x double points in positions as shown on plan number  G17/JH/PA17/002 with associated cabling run in surface mounted trunking

4.2	OFFICE 2

4.2.1	Remove and dispose of -

Ceiling tiles and grid, including timber edging

1 x notice board

All window blinds

Internal door (D2)

All textured wall coverings

All broadloom carpet

4.2.2	Fit new suspended ceiling as detailed in Specification of Works.

4.2.3	Box in existing distribution board, allow for accessible door.

4.2.4	Make good and reline walls in textured wall covering.

4.2.5	Supply and fit new door (D17) to existing frame.  Door D17 to have door access controls fitted (by others)

4.2.6	Lay new carpet floor covering.

4.2.7	Decorate all surfaces within this Section of the Works.

4.2.8	Electrics -

Remove and dispose of 3 x light fittings

Remove all data points and cabling, including all trunking

Supply and fit 4 x 600mm x 600mm LED light fittings, suitable for office 	environment

Supply and fit 6 x data points and cable back to comms unit

Provide new power supply to door access controls

4.3	OFFICE 3

4.3.1	Remove and dispose of -

Ceiling tiles and grid, including timber edging

2 x notice board

All window blinds

Internal door (D3)

All textured wall coverings

Redundant air conditioning units

All broadloom carpet

4.3.2	Fit new suspended ceiling as detailed in Specification of Works.

4.3.3	Make good wall after removal of air conditioning units.  Allow for brickwork externally and blockwork and plaster internally.  External opening size 600mm x 400mm maximum

4.3.4	Supply and fit new door (D18) to existing frame.  Door D18 to have door access controls fitted (by others)

4.3.5	Lay new carpet floor covering.

4.3.6	Decorate all surfaces within this Section of the Works

4.3.7	Electrics -

Remove and dispose of 3 x light fittings.

Isolate redundant air conditioning units prior to removal.

Remove all data points and cabling, including all trunking.

Supply and fit 4 x 600mm x 600mm LED light fittings, suitable for office 	environment.

Supply and fit 10 x data points and cable back to comms unit.

Supply and fit 2 x double points in positions as shown on plan number  G17/JH/PA17/002 with associated cabling run in surface mounted trunking

Allow for new data point and power supply for ceiling mounted projector

Provide new power supply to door access controls

4.4      OFFICE 4

4.4.1	Remove and dispose of -

Cupboard doors and shelving

Window blinds

Ceiling tiles and grid, including timber edging

2 x notice boards

1 x radiator (by others)

All broadloom carpet

2 x internal doors (including door to office 5) (D4 and D5)

Blockwork non loadbearing wall between Office 4 and Office 5

4.4.2	Form false beam between Office 4 and Office 5

4.4.3	Form new opening in masonry wall for new door to Male Locker Room

4.4.4	Form new stud wall in position as indicated on Plan G17/JH/PA17/002

4.4.5	Allow to extend timber boxing at skirting level from Office 5 to walls of Office 4 	as indicated on Plan Number G17/JH/PA17/002

4.4.6	Install new suspended ceiling to Office 4 and new Male Locker Room

4.4.7	Supply and fit new door (D21) and frame to new opening.  Door D21 to be fitted with key pad in place of handle

4.4.8	Supply and fit 2 x internal doors (D19 and D20) to existing frames.  Door D19 to have door access controls fitted (by others)

4.4.9	Lay new carpet floor covering

4.4.10	Decorate all surfaces within this section of the works.

4.4.11	Electrics -

Remove 6 x data points as indicated on Plan Number G17/JH/PA17/001

Remove 6 x power points as indicated on Plan Number G17/JH/PA17/001

Remove 7 x light fittings

Extend skirting trunking from Office 5 through Office 4 on walls as indicated on Plan Number G17/JH/PA17/002 .  Fix trunking to boxing provided by Main Contractor

Install 12 x data and 12 x double power points to skirting trunking

Install 10 x LED light fittings, suitable for office environment (2 x emergency 	fittings) and new light switch

Reposition Air Conditioning controls

4.5	ENTRANCE LOBBY AND RECEPTION AREA

4.5.1	Remove and dispose of -

All ceiling tiles and grid

All broadloom carpet

Section of old reception counter and screen as indicated on Plan Number G17/JH/PA17/001

All shelving and cupboards in this area of the works

3 x internal doors (D6, D7 and D8)

4.5.2	Form new stud partition walls as indicated on Plan Number G17/JH/PA17/002. Allow for opening for new doorway into Female Locker Room.

4.5.3	Supply and fit new door (D25) and frame.  Door D25 to be fitted with key pad in place of specified lever handle

4.5.4	Supply and fit 3 x internal doors (D22, D23 and D24) to existing frames.  Door access controls to be removed from door D22 (by others).  Door D22 fitted with push pad and D handle in place of specified handle.  Existing key pad on Door D7 to be refitted to door D23 in place of specified lever handle.  Door access controls on door D8 to be refitted to door D24 (by others)

4.5.5	New key pad to be fitted to door D34

4.5.6	Board internally in plasterboard glazed external wall to height of high level window and prepare for decoration

4.5.7	Board both sides in plasterboard glazed screen to entrance lobby to full height and prepare for decoration.

4.5.8	Supply and fit new suspended ceiling as detailed in the Specification of Works

4.5.9	Decorate all surfaces within this area of the works

4.5.10	Supply and fit new floor covering

4.5.11	Electrics –

Remove dispose of 7 x light fittings

Remove redundant data point in new Female Locker Room area

Reposition Air Conditioning controls

Install 2 x 600mm x 600mm LED light fittings to Female Locker Room with new switch in position as shown on Plan Number G17/JH/PA17/002

Install 4 x 600mm x 600mm LED light fittings to Reception area and Entrance Lobby

Install 1 x emergency exit light fitting

4.6	GROUND FLOOR CORRIDOR

4.6.1	Remove and dispose of -

Ceiling tiles and grid, including timber edging

2 x doors (D9 and D10)

2 x fire exit signs

All broadloom carpet

4.6.2	Install new suspended ceiling

4.6.3	Supply and fit 2 x doors (D26 and D27) to existing frames.  Door D27 to be fitted with push pad and D handle in place of specified lever handle

4.6.4	Lay new carpet floor covering

4.6.5	Decorate all surfaces within this Section of the Works.

4.6.6	Electrics -

Remove and dispose of 2 x light fittings

Install 3 x 600mm x 600mm LED light fittings, suitable for office environment

Install 2 x emergency exit light fittings

4.7	STAIRS, LANDINGA AND FIRST FLOOR CORRIDOR

4.7.1	Remove and dispose of -

Ceiling tiles and grid

1 x notice board

1 x door (D11)

All broadloom carpet

4.7.2	Existing radiator to be repositioned (by others)

4.7.2	Form new stud wall with opening as shown on Plan Number G17/JH/PA17/002

4.7.3	Supply and fit 1 x door (D29) and frame into prepared opening. Door D29 to be fitted with door access controls (by others)

4.7.4	Supply and fit new door (D28) to existing frame.  Door D28 to be fitted with push pad and D handle in place of specified lever handle

4.7.5	Install new suspended ceiling

4.7.6	Lay new carpet floor covering

4.7.7	Decorate all surfaces within this Section of the Works.

4.7.8	Electrics -

Remove and dispose of 6 x light fittings

Remove 1 x single power point

Install 10 x 600mm x 600mm LED light fittings, suitable for office environment

Install new light switch and alter switching arrangements

Install 1 x emergency exit light fittings

Install power supply for door access controls

4.8	LADIES TOILET

4.8.1	Remove and dispose of -

2 x toilet cubicles

1 x cleaner’s cupboard

1 x wall unit

2 x toilets

2 x wash hand basins

1 x vanity unit

1 x hand dryer

1 x mirror

1 x internal door (D12)

All wall tiles, including floor tile skirting trunking

Redundant water supply pipe to Office 1

4.8.2	Remove and save –

2 x toilet roll holders

1 x paper towel dispenser

1 x soap dispenser

2 x sanitary bins

4.8.3	Supply and fit in positions as indicated on Plan Number G17/JH/PA17/002

2 x toilets

2 x wash hand basins

1 x vanity unit

2 x toilet cubicles

4.8.4	Supply and fit 1 x internal door (D30) to existing frame

4.8.5	New wall unit

4.8.6	Allow to supply and fit 5 x replacement ceiling tiles

4.8.7	Decorate all surfaces in this section of the works

4.8.8	Supply and fit 1 x mirror

4.8.9	Supply and fit vinyl floor covering including coved skirting

4.8.10	Allow to refit toilet roll holders, paper towel dispenser and soap dispenser.  Replace sanitary bins.  Replace sanitary bins

4.8.11	Electrics -

Isolate hand dryer for removal

Remove and dispose of 4 x light fitting

Supply and fit new hand dryer

Supply and fit new 4 x LED light fitting

4.9	GENTS TOILET

4.9.1	Remove and dispose of –

1 x privacy screen

1 x toilet cubicle

1 x toilet

1 x wash hand basin

1 x urinal

2 x internal doors (D13 and D14)

1 x hand dryer

1 x mirror

All wall tiles, including floor tile skirting tiles.

4.9.2	Remove and save

1 x toilet roll holder

1 x soap dispenser

4.9.3	Tile walls to full height

4.9.4	Supply and fit –

1 x toilet

1 x wash hand basin

1 x urinal

1 x toilet cubicle

1 x privacy screen

4.9.5	Supply and fit 2 x doors (D31 and D32) to existing frame. Doors D31 and D32 to be fitted with push pad and D handle in place of specified handle

4.9.6	Supply and fit 1 x mirror

4.9.7	Allow to supply and fit 3 x replacement ceiling tiles

4.9.8	Decorate all surfaces in this section of the Works

4.9.9	Supply and fit vinyl floor covering including coved skirting

4.9.10	 Allow to refit toilet roll holder and soap dispenser

4.9.11	Electrics -

Isolate hand dryer

Remove and dispose of 1 x light fitting

Supply and fit new hand dryer

Supply and fit 3 x LED light fittings

4.10	KITCHEN

4.10.1	Remove and dispose of –

Cleaner’s cupboard and high level cupboards

Ceiling tiles and grid, including timber edging

Wall units

2 x base units

All worktops

Sink unit and taps

Floor tiles

1 x internal door (D15)

Notice board

Tile splash back

4.10.2	Remove and retain –

Lincat water boiler

Water chiller

Soap dispenser

Paper towel dispenser

4.10.3	Fit new high level cupboards for water tanks (supplies heating system)

4.10.4	Fit new suspended ceiling as specified in Specification of Works

4.10.5	Supply and fit new vinyl flooring

4.10.6	Supply and fit 1 x internal door (D33) to existing frame.  Door D33 to be fitted with push pad and D handle in place of specified handle

4.10.7	Supply and fit new kitchen units and worktops as specified in Specification of Works

4.10.8	Refit Lincat water boiler

4.10.9	 Refit water chiller

4.10.10 Tile walls to form new splashback above all worktops, as specified in

Specification of Works

4.10.11 Decorate all surfaces in this area of the works

4.10.12 Refit soap dispenser and paper towel dispenser

4.10.13 Electrics -

Remove and dispose of 1 x light fitting

Allow for alterations to power supply to provide power points to suit new kitchen layout

Allow for new power supply to supply new cooker

Supply and fit 4 x 600mm LED light fittings

## 5– QUOTATION FORM

- 5.1	 BREAKDOWN OF QUOTATION

The Contractor is not required to submit a breakdown of the quotation for the purposes of the initial submission, however for the avoidance of doubt the following confirms the elements to be included within the Quotation Sum:

- Section							Cost (£)

Preliminaries

(Include Contingency Sum of £5,000)

Pre-Construction H & S Information

Specification of Works

(Note provisional sums)

Schedule of Works

Total:

Appendix A

Contract for Internal alterations & refurbishment

works to the Jubilee House

Between

Gedling Borough Council

-and-

This Contract is dated the     day of                        20

Parties

- Gedling Borough Council of Civic Centre, Arnot Hill Park, Arnold, Nottingham NG5 6LU (‘the Council’), and

- (‘the Supplier’)

Background

- The Supplier is in the business of providing the Services, and

- The Council wishes to obtain and the Supplier is willing and able to provide the Services on the terms and conditions set out in this agreement.

Agreed Terms

| 1. |  | Interpretation |
| --- | --- | --- |
|  | 1.1 | Definitions |
|  |  | Business Day: a day other than a Saturday, Sunday or public holiday in England when Banks are open for business. Charges: the charges payable by the Council for the supply of the Services by the Supplier as set out in Schedule 2. Conditions: these terms and conditions set out in Clauses 1 to 23 inclusive. Contract: this agreement between the Council and the Supplier for the supply of Services in accordance with the agreed terms and any Schedules. Control: shall be as defined in section 1124 of the Corporation Tax Act 2010, and the expression ‘change of control’ shall be construed accordingly. Council’s Materials: all materials, equipment and tools, drawings, specifications and data supplied by the Council to the Supplier. Deliverables: all documents, products and materials developed by the Supplier or its agents, subcontractors and personnel as part of or in relation to the Services in any form, including without limitation computer programmes, data, reports and specifications (including drafts), and the key deliverables set out in the Contract Details. Services: the services including without limitation any Deliverables, to be provided by the Supplier to the Council pursuant to this Contract as described in Schedule 1 and the appendices. Services Start Date: the day on which the Supplier is to start provision of the Services, as set out in Clause 2.1 below. |
|  | 1.2 | Interpretation |
|  |  | - A reference to a statute or statutory provision is a reference to it as amended or re-enacted. A reference to a statute or statutory provision includes any subordinate legislation made under that statute or statutory provision, as amended or re-enacted. |
|  |  | - Any phrase introduced by the terms including, include, in particular or any similar expression shall be construed as illustrative and shall not limit the sense of the words preceding those terms. |
|  |  | - A reference to writing, or written, includes fax and e mail. |
| 2. |  | Start Date and term |
|  | 2.1 | This agreement shall start on *******. |
| 3. |  | Supply of Services |
|  | 3.1 | The Supplier shall supply the Services to the Council from the Services Start Date (see clause 2.1 above) in accordance with this Contract and the Schedules. |
|  | 3.2 | In performing the Services the Supplier must meet, and time is of the essence as to, any performance dates specified in Schedule 1. |
|  | 3.3 | In supplying the Services the Supplier shall: |
|  |  | - perform the Services with the highest level of care, skill and diligence in accordance with best practice in the Supplier’s industry, profession or trade; and - co-operate with the Council in all matters relating to the Services, and comply with reasonable instructions by the Council as to the performance of the Services; and |
|  |  | - appoint or, at the written request of the Council, replace without delay a Representative who shall have authority to contractually bind the Supplier on all matters relating to the Services; and - only use personnel who are suitably skilled and experienced to perform the tasks assigned to them, and in sufficient number to ensure that the Supplier’s obligations are fulfilled; and - ensure that it obtains, and maintains all consents, licences and permissions (statutory, regulatory, contractual or otherwise) it may require and which are necessary to enable it to comply with its obligations in this contract; and - ensure that the Services and Deliverables will conform in all respects with the service description set out in Schedule 1 and that the Deliverables shall be fit for any purpose expressly or implicitly made known to the Supplier by the Council. Provide all equipment, tools, vehicles and other items required to provide the Services; and - ensure that the Deliverables, and all goods, materials, standards and techniques used in providing the Services are of the best quality and are free from defects in workmanship, installation and design; and - comply with all applicable laws, statutes, regulations and codes from time to time in force; and - observe all health and safety rules and regulations and any other reasonable security requirements that apply at any of the Council’s premises from time to time and are notified to the supplier and/or are specified in Schedule 3; and - hold all Council Materials in safe custody at its own risk, maintain Council Materials in good condition until returned to the Council, and not to dispose of or use the Council Materials other than in accordance with the Council’s written instructions or authorisation; and - not do or omit to do anything which may cause the Council to lose any licence, authority, consent or permission on which it relies for the purposes of conducting its business; and - Notify the Council in writing immediately upon the occurrence of a change of control of the Supplier. |
| 4. |  | The Council’s Obligations |
|  | 4.1 | The Council shall: - co-operate with the Supplier in all matters relating to the Services; and - provide such access to the Council’s premises and data and other facilities as may reasonably be requested by the Supplier and agreed with the Council in writing in advance, for the purposes of the Services; and - provide such information as the Supplier may reasonably request and the Council considers necessary, in order to carry out the Services in a timely manner. |
|  | 4.2 | If the Supplier considers that the Council is not, or may not, be complying with any of the Council’s obligations, the Supplier shall only be entitled to rely on this as relieving the Supplier’s performance under this contract if the Supplier, promptly after the actual or potential non-compliance, has come to its attention, has notified the Council in writing. |
| 5. |  | Title to Deliverables and Customer Materials |
|  | 5.1 | Title to any deliverables that are goods or in any physical media on which Deliverables are stored shall pass to the Council on the earlier of their delivery to the Council or payment of the charges for them. The Supplier transfers the Deliverables to the Council free from all liens, charges and encumbrances. |
|  | 5.2 | All Council Materials are the exclusive property of the Council. |
| 6. |  | Charges and Payment |
|  | 6.1 | In consideration for the provision of the Services, the Council shall pay to the Supplier the Charges set out in this clause 6 and in Schedule 2. |
|  | 6.2 | All amounts payable by the Council exclude amounts in respect of VAT which the Council shall additionally be liable to pay to the Supplier at the prevailing rate (if applicable) and subject to a VAT invoice. |
|  | 6.3 | The Supplier shall submit invoices for the Charges plus VAT (if applicable) to the Council at intervals as specified in Schedule 2. Each invoice shall include all reasonable supporting information required by the Council. |
|  | 6.4 | The Council shall pay each invoice which is properly due and submitted to it by the Supplier within 30 days of receipt, to a bank account nominated in writing by the Supplier. |
|  | 6.5 | If the Council fails to make any payments due to the Supplier under this contract by the due date for payment, then, without limiting the Supplier’s remedies under clause 9, the Council shall pay interest on the overdue amount at the rate of 4% per annum above HSBC’s base rate from time to time. Such interest shall accrue on a daily basis from the date due until actual payment of the overdue amount, whether before or after judgement. The Council shall pay the interest together with the overdue amount. |
|  | 6.6 | In relation to payments disputed in good faith, interest under clause 6.5 above is payable only after the dispute is resolved, on sums found or agreed to be due, from 7 days after the dispute is resolved until payment. |
|  | 6.7 | The Council may at any time, without notice to the Supplier, set off any liability of the Supplier to the Council against any liability of the Council to the Supplier, whether either liability is present or future, liquidated or unliquidated, and whether or not either liability arises under this contract. Any exercise of the Council of its rights under this clause shall not limit or affect any other rights or remedies available to it under this contract or otherwise. |
| 7. |  | Limitation of Liability |
|  | 7.1 | Nothing in this contract: |
|  |  | - Shall limit or exclude the Supplier’s or Council’s liability for: - Death or personal injury caused by its negligence, or the negligence of its personnel agents or subcontractors; - Fraud or fraudulent misrepresentation; - Breach of the terms implied by section 2 of the Supply of Goods and Services Act 1982 (title and quiet possession) - Any other liability which cannot be limited or excluded by applicable law. |
|  | 7.2 | Subject to clause 7.1 above, neither party to this contract shall have any liability to the other party, whether in contract, tort (including negligence), breach of statutory duty, or otherwise, for any indirect or consequential loss arising under or in connection with this contract. |
|  | 7.3 | Notwithstanding the provisions of clause 7.2 above, the losses for which the Supplier assumes responsibility and which shall be recoverable by the Council include: - Sums paid by the Council to the Supplier pursuant to this contract; - Wasted expenditure; - Additional costs of procuring and implementing replacements for, or alternatives to, the Services, including consultancy costs, additional costs of management time and other personnel costs and costs of equipment and materials; - Losses incurred by the Council arising out of or in connection with any claim, demand, fine, penalty, action, investigation or proceeding by and third party (including any subcontractor, Supplier personnel, and regulator) against the Council caused by the act or omission of the Supplier. |
|  | 7.4 | The rights of the parties under this Contract are in addition to, and not exclusive of, any rights or remedies provided by common law. |
| 8. |  | Insurance |
|  | 8.1 | During the term of this contract the Supplier shall maintain in force, with a reputable insurance company, professional indemnity insurance and public liability insurance to cover the liabilities (minimum sum insured £10,000,000) which may arise under or in connection with this Contract, and shall produce to the Council on request both the insurance certificate giving details of the cover and the receipt for the current year’s premium in respect of each insurance. |
| 9.0 |  | Termination |
|  | 9.1 | Without affecting any other right or remedy available to it, the Council may terminate this Contract with immediate effect by giving written notice to the Supplier if: - There is a Change of Control of the Supplier; or - The Supplier’s financial position deteriorates to such an extent that in the Council’s opinion the Supplier’s capability to adequately fulfil its obligations under this Contract has been placed in jeopardy; or - The Supplier commits a breach of clause 3.3(i); or - The Supplier takes any steps or action in connection with its entering administration, provisional liquidation or any composition or arrangement with its creditors (other than in relation to a solvent restructuring), being wound up (whether voluntarily or by order of the court, unless for the purposes of a solvent restructuring), having a receiver appointed to any of its assets or ceasing to carry on business. |
|  | 9.2 | The Council shall be entitled (at its option) to cancel this Contract and to recover from the Supplier the amount of any loss resulting from such cancellation if: - The Supplier shall have offered, promised or given a financial or other advantage to another person, and the Supplier intends the advantage to induce a person to perform improperly, or reward a person for improper performance of a relevant function or activity in relation to obtaining or execution of the contract or any other contract with the Council; or the Supplier knows or believes that the acceptance of the advantage would itself constitute the improper performance of the relevant function or activity in relation to the obtaining or execution of the contract or any other contract with the Council; or - If the like acts shall have been done by any person associated with the Supplier or acting on his/her behalf (whether with or without the knowledge of the Supplier); or - In relation to any contract with the Council the Supplier or any person associated with him/her or acting on his/her behalf shall have committed any offence under the Bribery Act 2010, or shall have given any fee or reward the receipt of which is an offence under section 117(2) of the Local Government Act 1972. |
|  | 9.3 | Without affecting any other right or remedy available to it, either party may terminate this Contract with immediate effect by giving written notice to the other party if the other party commits a material breach of any term of this contract and such breach is irremediable or (if such breach is remediable) fails to remedy that breach within a period of 30 days after being notified in writing to do so. |
|  | 9.4 | Termination of this Contract does not affect any of the parties’ rights and remedies that have accrued as at termination, including the right to claim damages in respect of any breach of the Contract which existed at or before the date of termination. |
|  | 9.5 | Any provision of the contract that expressly or by implication is intended to come into or continue in force on or after termination shall remain in full force and effect. |
| 10 |  | Exit arrangements |
|  | 10.1 | On termination of this Contract for whatever reason the Supplier shall: - immediately deliver to the Council any Deliverables whether or not then complete, and return to the Council all Council Materials. If the Supplier fails to do so, the Council may enter the Supplier’s premises and take possession of them. Until they have been delivered or returned, the Supplier shall be solely liable for their safe keeping and shall not use them for any purpose not connected with this Contract; and - if so requested by the Council, provide all assistance reasonably required by the Council to facilitate the smooth transition of the Services to the Council or any replacement supplier appointed by the Council. |
| 11 |  | Force Majeure |
|  | 11.1 | Neither party shall be in breach of this Contract nor liable for delay in performing, or failure to perform, any of its obligations under this Contract if such delay or failure results from events, circumstances or causes beyond its reasonable control. If the period of delay or non-performance continues for 4 weeks the party not affected may terminate this Contract by giving 7 days’ written notice to the affected party. |
| 12 |  | Assignment |
|  | 12.1 | The Supplier may not assign, transfer or sub-contract any or all of its rights or obligations under this Contract without the prior written consent of the Council. Should the Council consent to any sub-contracting by the Supplier, the Supplier shall remain responsible for all acts and omissions of the sub-contractors as if they were its own. |
| 13 |  | Data Protection |
|  | 13.1 | Where the Supplier receives and personal data (as defined in the Data Protection Act 1998 (‘the Act’)) from the Council. It shall ensure that it fully complies with the provisions of the Act and only deals with the data to fulfil its obligations under this Contract. |
|  | 13.2 | The Supplier shall indemnify the Council for any breach of the Act which renders the latter liable for any costs, claims or expenses. |
|  | 13.3 | In fulfilment of its obligations under the Act the Supplier shall have such systems in place to ensure that: - Full compliance with the Act; and - In particular, compliance with the Seventh Data Protection Principle which deals with the security of personal data; and - The reliability of all its employees who may be involved in the processing of personal data. |
|  | 13.4 | The Supplier shall take all reasonable steps to ensure that all its partners, contractors and agents comply with this clause where they are processing any personal data on behalf of the Council. |
|  | 13.5 | The Supplier shall allow the Council reasonable access to such information as is necessary to ensure that it is complying with the above provisions and the Act as a whole. |
| 14 |  | Freedom of Information |
|  | 14.1 | The Supplier acknowledges that the Council is subject to the requirements of the Freedom of Information Act 2000 (‘FOIA’) and the Environmental Information Regulations 2004 (‘EIR’) The Supplier shall: - Provide all necessary assistance and cooperation reasonable requested by the Council to enable the Council to comply with its obligations under the FOIA and EIR; - Transfer to the Council any requests for information relating to this agreement that it receives as soon as practicable and in any event within 2 Business Days of receipt; - Not respond directly to a request for information unless authorised by the Council in writing to do so. |
|  | 14.2 | The Supplier acknowledges that the Council may be required under the FOIA or the EIRs to disclose information (including commercially sensitive information) without consulting or obtaining the consent of the Supplier. The Council will take reasonable steps to notify the Supplier of any request for information to the extent that it is permissible and reasonably practical for it to do so but the Council shall be responsible for determining in its absolute discretion whether any commercially sensitive information and/or any other information is exempt from disclosure with the provisions of the FOIA and/or the EIRs. |
| 15 |  | Safeguarding |
|  | 15.1 | The Council has a statutory duty to safeguard and promote the welfare of children and vulnerable adults. This responsibility sits with all providers of services to the Council, including any services under this Contract. |
| 16 |  | Equalities |
|  | 16.1 | The Supplier shall: - Perform its obligations under this Contract (including those in relation to Services) in accordance with: - All applicable equality law (whether in relation to race, sex, gender reassignment, age, disability, sexual orientation, religion or belief, pregnancy, maternity or otherwise); and - The Council’s equality and diversity policy as provided to the Supplier from time to time; and - Any other requirements and instructions which the Council reasonably imposes in connection with any equality obligations imposed on the Council at any time under applicable equality law; and - Take all necessary steps, and in for the Council of any steps taken, to prevent unlawful discrimination designated as such by any court or tribunal, or the Equality and Human Rights Commission (or any successor organisation). |
| 17 |  | Entire Agreement |
|  | 17.1 | This Contract (including the Schedules) constitutes the entire agreement between the parties and supersedes and extinguishes all previous agreements, promises, assurances, warranties, representations and undertakings between them, whether written or oral, relating to its subject matter. |
| 18 |  | Variation |
|  | 18.1 | No variation of this Contract shall be effective unless it is in writing and signed by both parties. |
| 19 |  | Waiver |
|  | 19.1 | A waiver of any right or remedy is only effective if given in writing and shall not be deemed a waiver of any subsequent breach or default. A delay or failure to exercise, or the single or partial exercise of any right or remedy shall not: - Waive that or any other right or remedy; or - Prevent or restrict the further exercise of that or any other right or remedy. |
| 20 |  | Notices |
|  | 20.1 | Any notice or other communication (other than the service of any proceedings or documents in any legal action) given to a party under or in connection with this Contract shall: - be in writing, addressed to that party at its office as detailed in this Contract, and shall be delivered personally, or sent by pre-paid first class post or other next day delivery service, commercial courier, fax or e mail. - be deemed to have been received: if delivered personally, at that date and time of delivery; if sent by prepaid first class post or other next working day delivery service, at 10.00am on the second Business Day after posting; if delivered by commercial courier, on the date and time on the courier’s signed receipt; if sent by fax or e mail, one Business Day after transmission. |
| 21 |  | Confidentiality |
|  | 21.1 | Each party undertakes that it shall not at any time disclose to any person any confidential information relating to this Contract except as permitted by clause 21.2. |
|  | 21.2 | Each party may disclose the other party’s confidential information : - to its employees, officers, representatives, subcontractors or advisers who need to know such information for the purposes of carrying out that party’s obligations under this Contract; - as may be required by law (including in connection with the Freedom of Information Act and the Environmental Information Regulations in accordance with clause 14 above), or any governmental or regulatory authority. |
|  | 21.3 | Neither party shall use the other party’s confidential information for any purpose other than to perform its obligations under this Contract. |
| 22 |  | Third Party Rights |
|  | 22.2 | No one other than the parties to this agreement shall have any right to enforce its terms. |
| 23 |  | Governing Law |
|  | 23.1 | This Contract and any dispute or claim arising out of or in connection with it or its subject matter or formation (including non-contractual disputes or claims) shall be governed by, and in accordance with, the laws of England and Wales. |
|  | 23.2 | Each party irrevocably agrees that the courts of England and Wales shall have exclusive jurisdiction to settle any dispute or claim arising out of or in connection with this Contract or its subject matter or formation (including non-contractual disputes or claims). |

Signed by …………………………………………………….

Name…………………………………………………………..

Position:…………………………………………………………

For and on behalf of Gedling Borough Council

Signed by …………………………………………………….

Name…………………………………………………………..

Position:…………………………………………………………

For and on behalf of

Schedule 1 – Services

The Council’s Representative shall be:

Name:

Position:

Address:

Direct line telephone:

Fax:

E mail:

The Supplier’s Representative shall be:

Name:

Position:

Address:

Direct line telephone:

Fax:

E mail:

The Services:

The Services and Performance dates are set out in the following documents: - Gedling Borough Council Contract Specification Document(including plans) dated March 2017 and attached to the Contract.

Schedule 2 – Charges

The Supplier’s Bank details for payment are:

Bank name:

Address:

Sort Code:

Account number:

Charges

Price: Lump sum price for the Works as per quotation dated ***: £****

Payments

A maximum of one invoice for interim payment may be submitted by the Supplier up to completion of the Works.

An invoice for final payment may only be submitted by the Supplier upon the completion of the Works as agreed with the Council’s Representative.

Schedule 3 – Health and Safety

For the purposes of the CDM Regulations-

Client:

Gedling Borough Council

Civic Centre,

Arnold, Nottingham, NG5 6LU

Principal Designer:

Mrs K Walters

Property Services Manager

Gedling Borough Council

Arnold, Nottingham

Designers:

Building Services Section, Gedling Borough Council,

Arnold, Nottingham, NG5 6LU

Principal Contractor:

To be confirmed

In respect of Clause 3.3(i) of the Agreed Terms, refer to the Pre-construction Health and Safety information, as contained in Gedling Borough Council Contract Specification Document(including plans) dated March 2017 and attached to the Contract.